Kitty Hawk Town Code

Chapter 20

Zoning

Town of Kitty Hawk Planning Department

P.O. Box 549

Kitty Hawk, NC 27949

(252) 261-3552

Prepared and maintained by the Planning Department.

See Town Clerk for detailed amendments.

Kitty Hawk Town Code

Chapter 20 - Zoning

Prepared by the Planning Department

The following pages were compiled by inserting all the recent amendments adopted by the Town Council in 2006, up to and including Amendment No. 04-08 thru No.06-14 (dated August 7, 2006)

The planning department maintains this Chapter between the periodic re-prints from the Municipal Code Company of the entire Kitty Hawk Town Code.

# Chapter 20

## Zoning

### Article I	In General,

### Article II	Administration and Enforcement

Division 1. General

Division 2. Appeals and Variances

Division 3. Building Permits, Certificates of Occupancy, etc.

Division 4. Nonconformities

Division 5. Zoning Vested Right

### Article III	Zoning Districts Generally; Zoning Map

### Article IV	District Regulations

Division 1. Generally

Division 2. Kitty Hawk Beach

Division 3. Kitty Hawk Village

Division 4. Remedies

### Article V	Special Districts, PUD’S, Overlay Districts, etc

Division 1. Generally

Division 2. Extraterritorial Area

Division 3. Planned Unit Development

Division 4. Planned Commercial Development

Division 5. Emergency and Government Services District

Division 6.  Open Space & Recreation District (0S)

### Article VI	Supplemental Regulations

Division 1. Generally

Division 2. Off-Street Parking and Loading

Division 3. Signs

Division 4. Mobile homes, Mobile Home Parks, Trailer Parks, etc

Division 5. Buffers and Landscaping

Appendix #1   Coastal Ecosystems & Landscaping

# Chapter 20  ZONING*

## ARTICLE I.  IN GENERAL

Sec. 20-1.	Definitions.

The following words, terms and phrases, when used in this chapter, shall have the meanings ascribed to them in this section, except where the context clearly indicates a different meaning:

Accessory use means a use which is clearly incidental to and customarily found in connection with the principal use and located on the same lot with such principal use.

Aircraft means any machine or device, including airplanes, helicopters, gliders, dirigibles, hangliders (whether or not motor powered) and motor powered parachutes capable of atmospheric flight. (99-21)

Alley means a minor right-of-way, dedicated to public use, which affords a secondary means of vehicular access to the back or side of properties otherwise abutting a street, and which may be used for public utility purposes.

Animated sign means any sign that uses movement or change of lighting to depict action or create a special effect or scene.

Approval authority means the town council or other board or official designated by ordinance or this chapter as being authorized to grant the specific zoning or land use permit or approval that constitutes a site specific development plan.

Approved parking and driveway surfaces means those surfaces approved from time to time by the town planning board and the town council.

Average Foot-Candles.  The average of a number of points of foot-candle calculations or foot-candle measurements in a given area. (02-29)

Banner means any piece of cloth, lightweight fabric or other similar material bearing a design, motto, slogan or message (whether commercial or noncommercial).

Beacon means any light with one (1) or more beams directed into the atmosphere or directed at one (1) or more points not on the same lot as the light source; also, any light with one (1) or more beams that rotate or move.

Bed and breakfast facility means a single-family dwelling which consists of a single dwelling unit together with the rental of one (1) or more dwelling rooms on a daily or weekly basis. The dwelling rooms shall not be equipped to allow the preparation of meals, although a single meal may be provided in a common area by the proprietor of the establishment. Bed and breakfast shall be considered synonymous with tourist home, guest house and other such similar uses (but is not intended to include group home).

Buffer means a planted or constructed divide of material and space used to provide sight and incidental sound screening from adjoining properties. The required height and width of the buffer strip and the materials used in its construction vary according to use. "Buffer" is not a use and falls within the definition of "open space." Unless specified otherwise in the various district regulations, buffers shall be in accordance with article VI, division 5, section 20-510.

Buffer strip means a device of material and/or space used to provide sight and sound screening from adjoining properties. The required height and width of the buffer strip and the materials used in its construction vary according to use.

Building means any structure enclosed and isolated by exterior walls constructed or used for residence, business, industry or other public purposes. The word "building" includes the word "structure."

Building, accessory means a subordinate building consisting of walls and a roof, the use of which is clearly incidental to that of a principal building on the same lot.   An accessory building or structure shall not include a mobile home, trailer, or existing structure previously used as a mobile home, and mobile homes, trailers or structures previously used as mobile homes shall not be used as accessory structures within the town. (99-21)

Building footprint means the outline of the building exterior walls, including any attached additions and projections or similar attached protrusions.  It does not include roof overhangs and eaves. (99-11)

Building marker means any sign indicating the name of a commercial or historical building and date and information about its construction. Such sign shall be cut into a masonry surface or made of bronze or other permanent material. Residential cottage identification markers shall be considered building markers so long as they do not exceed six (6) square feet in area and are permanently mounted to the wall of such cottage in a location which does not interrupt architectural details.

Building, principal means a building in which is conducted the principal use of the lot on which it is located.

Building setback line means a line parallel to or concentric with the street right-of-way establishing the minimum allowable distance between such right-of-way and the nearest portion of any building, excluding the outermost four (4) feet of any uncovered porches, steps, eaves, gutters and similar fixtures; and for oceanfront lots, the oceanfront setback as established by state coastal resources commission.

Building sign means any sign attached to any part of a building, as contrasted to a freestanding sign.

Building site means an area of disturbed land and vegetation required for placement of a structure, its accessways, and utilities, including areas disturbed for parking lots, power lines, driveways, septic tank nitrification fields, cemeteries and hiking trails.

CAMA means the Coastal Area Management Act of 1974 [G.S. § 113A-100 et seq.].

Canopy sign means any sign that is a part of or attached to an awning, canopy or other fabric, plastic or structural protective cover over a door, entrance, window or outdoor service area. A marquee is not a canopy.

Changeable copy sign means a sign or portion thereof with characters, letters or illustrations that can be changed or rearranged without altering the face or the surface of the sign. A changeable copy sign shall use exposed bulbs to display the changeable message. A sign on which the message changes more than eight (8) times per day shall be considered an animated sign and not a changeable copy sign for purposes of this chapter. A sign on which the only copy that changes is an electronic or mechanical indication of time or temperature shall be considered a "time and temperature" portion of a sign and not a changeable copy sign for purposes of this chapter.

Commercial message means any sign wording, logo or other representation that, directly or indirectly, names, advertises or calls attention to a business, product, service or other commercial activity.

Commercial nudity establishment means any establishment which the public may enter, with or without admission charge or membership, wherein nudity is exhibited by employees or entertainers. For the purposes of this ordinance, nudity shall mean any exposure to public view of the human male or female genitals, pubic area or buttocks, with less than fully opaque covering, or any portion of the areola of the breast of the female with less than full opaque covering.  (96-11)

Commercial piers & boat slips means piers and boat slips that are used for commercial purposes or piers from which commercial uses are conducted in the general vicinity of the pier and boat slips.  Commercial piers and boat slips are opened to the general public.(99-24)

Community piers & boat slips means piers and boat slips that are owned by two or more residential property owners; or that are own and maintained by a property owner’s association, and the facility is used by all the property owners as part of an overall plan of development. (99-24)

Condominium means ownership in common with others of a parcel of land and certain parts of a building thereon which would normally be used by all the occupants together with individual ownership in fee of a particular condominium unit in such building.

Cottage court means the density of the cottage court shall not exceed ten (10) units per acre except existing cottage courts on August 18, 1986 may rebuild any units destroyed by accident means.

CRC means the state coastal resources commission.

Customer service area means that area of a building available for servicing customers, but not including kitchen or storage. (02-17)

Cut-Off Fixture. A flat lens, full cut-off fixture that by its design, directs a minimum of 90 percent of total lamp lumens within 80 degrees of the vertical plane of the light fixture and a maximum of 10 percent of the total lamp lumens above 80 degrees from the vertical plane, and no more than 2.5 percent of total lamp lumens above 90 degrees. Full cut-off fixtures must be installed in a horizontal position as designed. (02-29)

Density means the number of dwelling units or hotel units which may be constructed upon a parcel of land, as allowed by the various zoning districts. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for density calculations.

Directional sign means signs containing directional information about public places owned or operated by federal, state or local governments or their agents; publicly or privately owned historic, cultural, educational and religious sites; and areas of natural scenic beauty or naturally suited for outdoor recreation.

Drive-in restaurant or refreshment stand means any place or premises used for sale, dispensing, or serving of food, refreshments, or beverages in automobiles, including an establishment where customers may serve themselves and may eat or drink the food, refreshments, or beverages on the premises.

Dwelling unit means one (1) room, or rooms connected together, constituting a separate, independent housekeeping unit for owner occupancy, or rental or lease, and physically separated from any other rooms or dwelling units which may be in the same structure, and containing independent cooking and sleeping facilities for a single family.

Dwelling, multifamily means a building or portion thereof used or designed as a residence for three (3) or more families living independently of each other and doing their own cooking therein, including apartment houses.

Dwelling, single-family means a detached building designed for or occupied exclusively by one (1) family.

Dwelling, two-family (duplex) means a detached building, divided horizontally or vertically, and designed for or occupied by two (2) single-family housekeeping units contained entirely under one (1) roof and having one (1) dividing partition common to each unit, or having the ceiling structure of the lower unit the floor structure of the unit above.

Eating establishment means any establishment which provides as a principal use the sale of food, frozen desserts or beverages in a state ready for consumption within the establishment and whose design or principal method of operation includes both of the following characteristics:

(1) Customers are provided with an individual menu and are served their food, frozen desserts or beverages by a restaurant employee at the same table or counter at which said items are consumed.

(2) The food, frozen desserts or beverages are served on nondisposable plates or containers and nondisposable eating utensils are provided. Customers are not expected to clear their table or dispose of their trash.

Notwithstanding the above, a cafeteria where food, frozen desserts or beverages are: (a) generally consumed within the establishment, and (b) served on nondisposable plates or containers and nondisposable eating utensils are provided shall be deemed an eating establishment.

An eating establishment may provide a carryout service provided that such carryout service is clearly not the principal business of such establishment.

For the purpose of this chapter, a fast food restaurant shall be deemed an eating establishment.

In addition, an eating establishment shall not be deemed to include a snack bar or refreshment stand at a public or nonprofit recreational facility which is operated solely by the agency or group operating the recreational facility for the convenience of the patrons of the facility.

Entertainment which is provided for the enjoyment of the patrons shall be considered an accessory to an eating establishment, to include dancing by patrons, provided that the space made available for such dancing shall not be more than one-eighth of that part of the floor area available for dining.

For purposes of this chapter, a commercial nudity establishment shall not be deemed an eating establishment.

The sale of T-shirts and souvenirs shall be considered an accessory if the area of that secondary commercial use is less than five (5) percent of the square footage of the seating area of the restaurant. (96-11)

Façade means the front of a building facing the street from the ground level to the ceiling joist.  The roof structure shall not be included in the sign calculations. (99-11)

Family means one (1) or more persons occupying a single-family dwelling unit, provided that unless all members are related by blood or marriage, no such family shall contain more than five (5) persons.

Fast food restaurant means any establishment which provides as a principal use the sale of food, frozen desserts or beverages in a ready to consume state for consumption either within the restaurant, within a motor vehicle parked on the premises, or off-premises, and whose design or principal method of operation includes one (1) or more of the following characteristics:

(1) Food, frozen desserts or beverages are served in edible containers or in paper, plastic or other disposable containers. Eating utensils, if provided, are disposable.

(2) Food, frozen desserts or beverages are usually served over a general service counter for the customer to carry to a seating facility within the restaurant, to a motor vehicle or off-premises. If consumed on premises, customers generally are expected to clear their own tables and dispose of their trash.

(3) Forty-five (45) percent or more of the gross floor area of the establishment is devoted to food preparation, storage and related activities, which space is not accessible to the general public.

(4) Food, frozen desserts or beverages are served to the occupants of a motor vehicle while seated therein, such as through a drive-in window.

For the purpose of this chapter, a fast food restaurant shall be deemed an eating establishment. A fast food restaurant shall include: drive-in, carryout, deli, snack bar, ice cream and yogurt shop when they meet the above criteria.  (96-11)

Firewall means a fireproof or fire retardant wall that is required by the N.C. Building code to prevent fires from spreading to one section of a building from another.  The firewall may project beyond the exterior wall of a building or structure.(99-11)

Fitness Center means a building where exercise activities take place, including weight lifting (free weights and machines), aerobic machines and equipment, aerobic, yoga and dance classes or other similar physical activities.  It may include steam rooms, sauna rooms, tanning booths, and massage rooms. A fitness center can be a membership or commercial operation.  A fitness center can include an indoor or outdoor swimming pool.   It may also include sales of dietary supplements as an accessory use within the building. (03-53)

Flag means any fabric or bunting containing distinctive colors, patterns or symbols.

Flag lot or panhandle lot means a lot which has a narrow frontage on a street and a thin strip of land which provides access from the street right-of-way to a wider portion of the lot.

Floodlight. A light fixture usually capable of being pointed in any direction that is designed to project a light beam to an object or surface area to a luminance considerably greater than its surroundings. (02-29)

Floor area, gross means the total area of a building measured by taking the outside dimension of the building at each floor level intended for occupancy or storage. (02-17)

Forest canopy means the light intercepting layer formed by all of the treetops and ultimate leaf-bearing branches in a forest. It is the uppermost layer of vegetation in a forest. In the Kitty Hawk Woods, the forest canopy may be kept to near constant height by the pruning effect of salt mist nearer the ocean or it may become irregular in the height where salt impact is less.

Forest subcanopy means a light-intercepting understory layer formed by shade-tolerant saplings, shrubs, and small trees beneath the canopy of a forest. The Kitty Hawk Woods subcanopy species include dogwood, muscle wood, hop hornbeam, and holly.

Foundation includes a piling.

Foot-Candle. The unit of measure of illuminance on a surface. Foot-candles are the ratio of the quantity of light in lumens divided by the surface area in square feet on which the lumens are falling. One lumen per square foot is one foot-candle. (02-29)

Freestanding sign means any sign supported by structures or supports that are placed on, or anchored in, the ground and that are independent from any building or other structure.

Front building setback line means a line parallel to or concentric with the street right-of-way establishing the minimum allowable distance between such right-of-way and the nearest portion of any building, excluding the outermost four (4) feet of any uncovered porches, steps, eaves, gutters and similar fixtures; and for oceanfront lots, the oceanfront setback as established by state coastal resources commission. (99-11)

Fully Shielded. A light fixture that is constructed or sufficiently shielded by an opaque housing, in such a manner that all light emitted is below the horizontal plane as determined by photometric test or certified by the manufacturer. (02-29)

General use district means a zoning district with designated permitted uses and designated conditional uses.

Glare. Glare is the sensation produced by luminance within the visual field that is significantly greater than the luminance to which the eyes are adapted, causing annoyance, discomfort, or loss of visual performance. It results from high luminances or insufficiently shielded light sources in the field of view or from reflecting areas of high luminance. There are two types:

1. Disability glare affects visual performance and reduces the ability to see or identify objects. It is often accompanied by discomfort.

2. Discomfort glare produces discomfort but does not necessarily interfere with visual performance or visibility.

Golf courses does not include a miniature golf or putt-putt type of golf course.

Ground cover means the lowest layer of vegetation in a forest. Generally, these plants are within one (1) foot of the ground. Ground cover may be excluded where the canopy or subcanopy is dense. Ground cover species typically consist of weedy annual herbs, grasses, and vines. Partridge berry is a common ground cover within the dune ridge maritime forest in Kitty Hawk Woods while poison ivy is common on recently logged sites or where the canopy has been opened by ice storm or blowdowns.

Ground elevation means the mean elevation of the undisturbed land computed at the perimeter of the proposed building's foundation or piling location.

Ground stabilization plan means a plan that will assure the stabilization and subsequent re-vegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16.

Groundwater recharge area means a catchment basin or watershed underlain by layers of alternating permeable and impermeable strata such that excess rainfall not lost by evapotranspiration or runoff is retained and stored in subterranean porous layers of soil. Essentially the entire Kitty Hawk Woods acts as a groundwater recharge area since porous sandy soils permit little runoff of excess precipitation. For the purposes of this chapter, components of this system include ponds, wetland swales, bay forests, dunes and marsh.

Height means the vertical distance measured from the highest point of the top plate of the permitted structure to the ground elevation.

Height, total means the distance from ground elevation to the horizontal plane of the highest point of the building.

Home Improvement Warehouse means the wholesale and retail sales of home improvement and building construction products, residential fixtures and appliances, flooring and wall coating products, and landscaping plants, materials and supplies. (00-37)

Home occupation means a profession or occupation carried on by a member of a family or a member of a recognized profession residing on the premises (see section 20-421 of this chapter for conditions and standards).

Home occupation sign means any sign located in a district zoned for residential uses that contains no commercial message except advertising for goods or services legally offered on the premises where the sign is located, if offering such service or such goods at such location conforms with all the requirements of this chapter.

Horse as used in this code shall include pony, mule and donkey. (00-25)

Horse rental as used in this code shall include pony, mule and donkey. (00-25)

Hose lay distance means the shortest distance measured from the fire hydrant to the building over an improved public or private right-of-way or a private driveway upon which a fire hose can be dropped from a fire truck in order to provide water at the building site.  This definition does not include unimproved yards, open spaces, wetland areas, and areas not intended or appropriate for activities associated with fire suppression. (97-6)

Hotel/motel means a building or portion thereof or a group of buildings which provide sleeping accommodations for transients on a daily, weekly or similar short-term basis, whether such establishment is designated as a hotel, inn, cottage court, automobile court, motel, motor inn, motor lodge, tourist cabin, tourist court, tourist home or otherwise. A hotel or motel shall be deemed to include any establishment which provides residential living accommodations for transients on a short-term basis, such as an apartment hotel. A time share is specifically NOT a hotel or motel by definition. (96-7)

IESNA. Illumination Engineering Society of North America. (02-29)

Impervious surface means any material that prevents absorption of stormwater into the ground. (03-02)

Incidental sign means any sign, generally informational, that has a purpose secondary to the use of the lot on which it is located, such as "no parking," "entrance," "loading only," "telephone," and other similar directives. No sign with a commercial message legible from a position off the lot on which the sign is located shall be considered incidental.

Indoor ramp park means a skateboard ramp park completely enclosed within a building. (03-05)

ISO Foot-Candle Diagram. Lines plotted on a set of coordinates to show all points on a surface where equal levels of illuminance occur.(02-29)

Lamp. A bulb or tube that is a light source. (02-29)

Light Fixture. Any electrically powered illuminating device, reflective surface, lamp or any similar device, permanently installed or portable, used for illumination or advertisement, including illuminated signs. (02-29)

Lot means a parcel of land which fronts on and has ingress and egress by means of a public right-of-way or any approved private street and upon which there is located or intended to be located thereon a building or groups of buildings as provided herein with the customary accessories and open spaces. The word "lot" includes the word "plot," "parcel" or "tract."

Lot area means the total horizontal area included within lot lines.

Lot, corner means a lot with at least two (2) adjoining sides abutting for their full lengths on a street, provided that the interior angle at the intersection of two (2) such sides is less than one hundred thirty-five (135) degrees.

Lot coverage means a measure of the developed intensity of land use.  This includes, but is not limited to, all areas covered by buildings, parking areas, accessory structures, driveways, roads, sidewalks, and any area of concrete or asphalt.  (03-02)

Lot depth means the average distance from the street line of the lot to its rear line measured in the general direction of the side lines of the lot.

Lot lines means any of the following lines:

(1) Lot line, front, which is the line separating a lot from that street which is designated as the front street on the building permit, certificate of occupancy or subdivision plat.

(2) Lot line, rear, which is the lot boundary opposite and most distant from the front lot line. In the case of a pointed or irregular lot, it shall be an imaginary line parallel to and farthest from the front lot line, not less than thirty (30) feet long and wholly within the lot.

(3) Lot line, side, which is a side line is any lot boundary line which is not a front lot line or rear lot line.

Lot of record means a lot which is part of a subdivision recorded in the office of the county register of deeds, or a lot or parcel described by metes and bounds, the description of which has been so recorded.

Lot width means the width of a lot at the required building setback line measured at right angles to its depth.

Lumen. The unit of measure of the quantity of light emitted by a light source, irrespective of direction. (02-29)

Maintained Food-Candles. Foot-candles (minimum, maximum, or average) that are calculated with an adjustment for a maintenance factor that includes dirt build-up, lamp lumen depreciation, ballast factor, etc. The system is in effect over designed initially and then over time allowed to reach a maintained foot-candle level. (02-29)

Marquee means a wall sign for a theater which may have manually changeable copy to identify movies currently being screened and the times of day movies will be shown.

Maximum Food-Candle. The maximum foot-candle point calculation or measurement in a given area. (02-29)

Medical clinic means a building or structure or portion thereof where medical services are provided for outpatients only.

Migrating dune means an area of unstabilized sand subject to movement under the influence of winds. Migrating dunes occur in the Kitty Hawk Woods all along the eastern boundary of the dune ridge maritime forest.

Minimum Food-Candle. The minimum foot-candle point calculation or measurement in a given area. (02-29)

Minimum Lot size means minimum lot area as required by the various zoning districts.  Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size.(04-14)

Mobile home means a structure that has all of the following characteristics:

(1) It consists of a single unit completely assembled at the factory.

(2) It is designed so that the total structure can be transported on its own chassis.

(3) It is over thirty-two (32) feet long and over eight (8) feet wide.

(4) It is designed to be used as a dwelling unit and provides complete independent living facilities, including provisions for living, sleeping, eating, cooking, and sanitation.

(5) It is actually being used, or is held ready for use, as a dwelling.

A travel trailer is not to be considered as a mobile home.

Mobile home park means a parcel, tract or contiguous parcels of land under single ownership that has been designated and improved for the placement of mobile homes for dwelling purposes.

Modular unit means a factory-fabricated transportable building designed to be used by itself or to be incorporated with similar units at a building site into a modular structure. The term is intended to apply to major assemblies and does not include prefabricated panels, trusses, plumbing trees, and other prefabricated supplements which are to be incorporated into a structure at the site.

Multifamily dwelling development means a development composed in whole or in part of structures designed and built for occupancy by more than one (1) family unit.

Natural features site plan means a site plan and accompanying report detailing existing vegetation and topography, which must be submitted to develop any subdivision or building site.

Nonconforming sign means any sign that does not conform to the requirements of this chapter.

Nonconforming use means a use of building or land which does not conform with the regulations of the district in which such building or land is situated but was lawful before August 18, 1986.

Oceanfront setback means a line defined by state coastal resources commission regulations under the Coastal Area Management Act of 1974 [G.S. § 113A-100 et seq.].

Off-site sign means a sign, other than a directional sign or mile marker sign, that is located off the property on which the use or activity advertised is sold.

Open commercial activity means any activity done for compensation (or as a result of compensation paid to another in order to engage in such activity) where the activity undertaken is conducted outside of the perimeter walls of the structure.

Open space means an unoccupied space open to the sky.

Open storage means an unroofed storage area, whether fenced or not.

Open Vertical Fixture. An unshielded high intensity discharge fixture with an open bottom acrylic refractor. (02-29)

Parking space means a vehicular storage space of not less than ten (10) feet by twenty (20) feet, plus the necessary access space.

Pennant means any lightweight plastic, fabric or other material tapering to a point, whether or not containing a message of any kind, suspended from a rope, wire or string, usually in series, designed to move in the wind.

Permanent sign means any sign that is not a temporary sign.

Person means any association, company, corporation, firm, organization or partnership, singular or plural, of any kind.

Planned unit development means the complete development of land that is under central control or for which central control mechanisms have been established. The plan will be in accordance with such guides and objectives as may be established by the planning board and town council in article V of this chapter.

Pond means a small body of standing water with rooted plants growing across it (or at least capable of supporting plants all the way across). In Kitty Hawk Woods, ponds often exhibit moderate seasonal variations in water depth.

Portable sign means any sign not permanently attached to the ground or other permanent structure, or a sign designed to be transported including, but not limited to, signs designed to be transported by means of wheels; signs converted to A-frame or T-frames; menu and sandwich board signs; balloons used as signs; umbrellas used for advertising; and signs attached to, painted on, or placed upon vehicles parked and visible from the public right-of-way, unless such vehicle is registered, licensed and used in the normal day-to-day operations of the business.

Private piers & boats slips means priers and boat slips that are owned by the person or entity that owns the single family residential lot to which the pier and boat slips are attached, and the private pier and boat slips are used by the lot owner, his family and guest. (99-24)

Projecting sign means any sign affixed to a building or wall in such a manner that its leading edge extends more than six (6) inches beyond the surface of such building or wall.

Rare plants means those plant species listed as "reasons for priority rating" numbers 2, 4, 5, 6, and 7 (pages 1 and 2) in the Basic Inventory and Natural Diversity Summary of the Nags Head Woods, Dare County, North Carolina, 1978.

Recessed or Flush Mounted.. A fixture that is mounted above the ceiling with the opening, lens, or cover of the fixture recessed or level with the ceiling surface, and all light emitted is below the horizontal plane. (02-29)

Residential sign means any sign located in a district zoned for residential uses that contains no commercial message except advertising for goods or services legally offered on the premises where the sign is located, if offering such service at such location conforms with all requirements of this chapter.

Restaurant: See "eating establishment."

Ridge line forest means a type of maritime forest in which the pruning effect of salt mist is moderated by distance from the ocean source. The forest canopy is dominated by various evergreen oaks (live, laurel, water) and an understory subcanopy of dogwood, muscle wood, hop hornbean, and holly. In the Kitty Hawk Woods, the dune ridge maritime forest occupies the highest dunes along a major north-south oriented dune ridge.

Roof overhang means the part of the roof structure that extends beyond the external wall of the building or structure.  Overhang shall include the “eves”.  (99-11)

Roof sign means any sign erected, constructed or placed on or over the roof of a building. (99-11)

Roof sign, integral means any sign erected or constructed as an integral or essentially integral part of a normal roof structure of any design. (99-11)

Salt forest means a plant community that forms a border of irregular width between the forested ridge and the frontal dune system. Characteristic scrub forest vegetation includes loblolly pine, evergreen oaks, northern bayberry and wax myrtle. The area is sparsely to densely vegetated having some areas of bare sand. This plant community is found on USDA soil types of duneland, newland, fine sand, and duneland-newland canopies.

Salt marsh means a flat bed of salt-resistant grasses, sedges and/or rushes that is periodically flooded by salt or brackish water. In Kitty Hawk Woods salt marsh extends in an irregular band along the foot of the westernmost forested dunes westerly to the edge of the estuary. Characteristic plants include cattails, giant cordgrass, and black needle rush.

Setback means the distance between a property line or right-of-way and the building.  It is synonymous with the yard. (also see yards and minimum yard requirements). (99-11)

Sign means any device, fixture, placard or structure that uses any color, form, graphic, illumination, symbol or writing to advertise, announce the purpose of or identify the purpose of a person or entity, or to communicate information of any kind to the public. However, the content or substance of the message or symbol appearing on a sign shall not be subject to or governed by the standards of article VI, division 3, except to define or categorize the sign type subject to regulation.

Sign area means that area of a sign composed in whole or in part of freestanding letters, symbols, devices or sculptured matter not mounted on a measurable surface and shall be construed to be the area of the least square, rectangle or circle that will enclose the letters, devices and/or sculptured matter. The area of a double-faced sign shall be the area of one (1) face of the sign, provided that the two (2) faces are of the same size and are parallel to one another with no more than twenty-four (24) inches between each sign face.

Sill girder means the lowermost structural beam of a building which rests on the foundation wall or pilings and which supports the floor and exterior structure walls.

Site specific development plan means a plan of land development submitted to the town for purposes of obtaining one (1) of the following zoning or land use permits or approvals:

(1) Town council approval of development plan and site plan pursuant to section 20-145(e).

(2) Town council approval of development plan and site plan pursuant to section 20-146(e).

(3) Town council approval of development plan and site plan pursuant to section 20-147(f).

(4) Town council approval of preliminary development plan and commercial site plan for BHMD pursuant to section 20-148.

(5) Town council approval of commercial site plan for BH-2 pursuant to section 20-149(d).

(6) Town council approval of development plan and site plan pursuant to section 20-165(e).

(7) Town council approval of development plan and site plan pursuant to section 20-166(e).

(8) Town council approval of development plan and site plan pursuant to section 20-167(e).

(9) Town council preliminary approval of detailed site or project development and construction plans pursuant to section 20-237(e).

(10) Town council preliminary approval of development plan and site plan pursuant to section 20-268.

(11) Town council preliminary approval of development plan and site plan pursuant to section 20-299.

(12) Town council preliminary approval of subdivision plat pursuant to section 19-47.

(13) Town council approval of mobile home or trailer park pursuant to the provisions of section 20-503.

Notwithstanding the foregoing, neither a variance, a sketch plan or any other document that fails to describe with reasonable certainty the type and intensity of use for a specified parcel or parcels of property shall constitute a site specific development plan.

Skirting area means that area beneath a mobile home from the underside of the floor area to the ground.

Special use district means a general zoning district in which uses which are not permitted as a right may be permitted upon the issuance of a special use permit.

Stable means a building in which horses are sheltered and fed. (00-25)

Street means any of the following:

(1) Any permanently dedicated public right-of-way which has been accepted for maintenance by the state department of transportation or the town.

(2) Any other open area providing the principal means of access for vehicles or pedestrians from a public right-of-way to a building or use of land and which complies with all of the following:

a. Is at least thirty (30) feet in width.

b. Has been approved by the town council as a street.

c. Satisfies the requirements of this chapter.

d. Is covenanted by its owner to remain open and unobstructed throughout the life of any building or use which depends thereon to satisfy any requirement of this chapter.

Street, private means a street that has not been dedicated to public use and for maintenance by the town.

Structure means anything constructed or erected, including parking lots, the use of which requires location on the ground, or attachment to something having location on the ground.

Structure Highlighting. (1) Exposed or channel neon, argon, krypton or similar gas tube lighting, not utilized to illuminate a permitted sign, and directs attention to a building, or structure, (2) Any light source that illuminates a translucent two or three dimensional surface, or object, that is not part of a permitted sign, and directs attention to a building or structure. (02-29)

Subdivision sign; subdivision identification signs means a sign displaying the name of a subdivision, which may be located at each entrance to the subdivision. Only one (1) subdivision identification sign may be located at each entrance road.

Suspended sign means a sign that is suspended from the underside of a horizontal plane surface and is supported by such surface.

Temporary sign means any sign that is used only temporarily and is not approved for permanent installation and use.

Theme overlay district means a district in which additional requirements are imposed on properties within the underlying general use district. The additional requirements may include the regulation and restriction of the erection, construction, reconstruction, alteration, repair or use of buildings, structures or land as well as the creation of architectural themes and standards regulating the general exterior facade, color, and material consistent with the designated architectural theme of the district.

Top plate means the structural member of a building located at the point where the structural wall framing and the structural roof framing join together at the top of the uppermost habitable floor and should the building be designed having top plates at more than one (1) horizontal level, the uppermost plate shall be considered insofar as usage in this chapter is concerned.

Tourist-Oriented Directional Sign (TODS) means a guide sign that displays the business identification of and directional information for tourist-oriented business and tourist-oriented facilities (as defined in GS 136-140.15) or for a class of businesses or facilities that are tourist-oriented.  (03-56)

Townhouse means a single-family dwelling on its own individual lot but connected on two (2) sides, by means of a common wall for at least ten (10) feet of its length, to two (2) other single-family dwellings or an end dwelling of a row of such dwellings. No more than six (6) such dwelling units may be attached in a single group.

Trailer includes any of the following:

(1)Travel trailer, which means a vehicular, portable structure built on a chassis, designed to be used as a temporary dwelling for travel, recreational, and vacation uses, and, when factory-equipped for the roads, it shall have a body width not exceeding eight (8) feet, and a body length not exceeding thirty-two (32) feet.

(2) Pickup coach, which means a structure designed to be mounted on a truck chassis for use as a temporary dwelling for travel, recreation, and vacation.

(3) Motor home, which means a portable, temporary dwelling to be used for travel, recreation and vacation, constructed as an integral part of a self-propelled vehicle.

(4) Camping trailer, which means a folding structure of canvas or other material mounted on wheels and designed for travel, recreation, and vacation use.

Use means any of the following:

(1) Any purpose for which a building or other structure or a tract of land may be designed, arranged, intended, maintained, or occupied.

(2) Any activity, occupation, business, or operation carried on, or intended to be carried on, in a building or other structure or on a tract of land.

Use, conditional means a use that would not be appropriate generally or without restriction throughout a particular zoning district but which, if controlled as to number, area, location or relation to the neighborhood, would preserve the intent of this chapter to promote the public health, safety, morals, and general welfare.

Used or occupied as applied to any land or building includes the words "intended, arranged or designed to be used or occupied."

Variance means a relaxation of the terms of this chapter where such variance will not be contrary to the public interest and where, owing to conditions peculiar to the property and not the result of the actions of the applicant, a literal enforcement of this chapter would result in unnecessary and undue hardship.

Vehicular sign means any message, symbol or design painted on or attached to a vehicle or trailer.

Wall sign means any sign attached parallel to, but within six (6) inches of, a wall, painted on the wall surface of, or erected and confined within the limits of an outside wall of any building or structure, which is supported by such wall or building, and which displays only one (1) sign surface.

Wetland swales means seasonally flooded or water-saturated depressions located between drier adjacent dune ridges. The vegetation cover includes a tree canopy. The general aspect of a swale greatly resembles that of a forested swamp, of which it may be considered a subunit. In the Kitty Hawk Woods dominant wetland swale vegetation includes sweet gum, black gum, red maple, buttonbush, and cattails. Swales may become flooded seasonally or following heavy rainfall.

Window sign means any sign, pictures, symbol or combination thereof, designed to communicate information about an activity, business, commodity, event, sale or service, that is placed inside a window or upon the window panes or glass and is visible from the exterior of the window.

Yard means a required open space other than a court unoccupied and unobstructed by any structure or portion of a structure from thirty (30) inches above the general ground level of the graded lot upward, except that handrails may extend more than thirty (30) inches above the undisturbed ground level provided the area between the support post or piers remains open and unobstructed by pickets, siding or material unnecessary for the structural strength of the handrails. Any pedestrian walkway and handrail must meet applicable CAMA standards and requirements and structural requirements of the North Carolina State Building Code.

Yard, front means a yard extending between side lot lines across the front of a lot adjoining the public street.

Yard, rear means a yard extending across the rear of the lot between side lot lines.

Yard, side means a yard extending from the rear line of the required front yard to the rear yard.

Zoning vested right means a right pursuant to G.S. § 160A-385.1 to undertake and complete the development and use of property under the terms and conditions of an approved site specific development plan.

(Ord. of 8-18-86, §§ 2.01, 2.02; Ord. No. 88-16A, 10-24-88; Ord. No. 89-27, 11-6-89; Ord. No. 90-5, § 6(i), 4-23-90; Ord. No. 90-7, § 2, 5-21-90; Ord. No. 90-17, 12-3-90; Ord. No. 91-14, 10-21-91; Ord. No. 91-19, 12-10-91; Ord. No. 93-2, 1-4-93; Ord. No. 94-2, 1-10-94; Ord. No. 95-17, 9-5-95; Ord. No. 95-18, 9-5-95; Ord. No. 96-7, 4-1-96; Ord. No. 96-11, 5-6-96)

Cross reference(s)--Definitions and rules of construction generally, § 1-2.

### Sec. 20-2.	Purpose.

In accordance with the provisions of G.S. § 160A-387, the town council having designated the planning board as the planning agency to prepare a land use plan showing proposed development patterns and to recommend a zoning ordinance and having received from the planning board a plan taking into consideration the character of each district and its peculiar suitability for particular uses with a view to conserving the value of buildings and encouraging the most appropriate use of land throughout the town, adopts this chapter. This chapter has been prepared in accordance with the land use plan for the development of the town and is designed to lessen congestion in the streets; to secure safety from fire, panic, and other dangers; to promote health and the general welfare; to provide adequate light and air; to prevent the overcrowding of land; to avoid undue concentration of population; to facilitate the adequate provision of transportation, water, sewerage, schools, parks and other public requirements; and to give reasonable consideration to the expansion and development of districts within the town so as to provide for their orderly growth and development.

(Ord. of 8-18-86, § 1.03)

State law reference(s)--Purpose of zoning ordinances, G.S. § 160A-383.

### Sec. 20-3.	Territorial applicability.

The provisions of this chapter shall apply to all lands and structures and uses thereon, within the zoning areas designated on the "Official Zoning Map of the Town of Kitty Hawk, North Carolina."

(Ord. of 8-18-86, § 1.08)

Charter reference(s)--Planning jurisdiction, § 6-1.

### Sec. 20-4.	Compliance.

(a) No building, structure, or land shall be used or occupied, and no structure or part thereof shall hereafter be constructed except in conformity with all of the provisions specified in this chapter for the district in which it is located.

(b) No building or other structure shall hereafter be erected or altered:

(1) To exceed the applicable height or bulk requirements of this chapter.

(2) To accommodate or house a greater number of families than allowed by this chapter.

(3) To occupy a greater percentage of lot area than allowed by this chapter.

(4) To have narrower or smaller rear yards, front yards, side yards, or other open spaces than allowed by this chapter.

(5) In any other manner contrary to the provisions of this chapter.

(Ord. of 8-18-86, § 1.04)

### Sec. 20-5.	Interpretation, purpose and conflict.

In interpreting and applying the provisions of this chapter, the requirements of this chapter shall be held to be the minimum requirements for the promotion of the public safety, health, convenience, property, and general welfare. It is not intended by this chapter to interfere with or abrogate or annul any easements, covenants, or other agreements between parties; provided, however, that where this chapter imposes a greater restriction upon the use of buildings or premises or upon the height of buildings, or requires larger open spaces than are imposed or required by other ordinances, rules, regulations, or by easements, covenants, or agreements, the provisions of this chapter shall govern.

(Ord. of 8-18-86, § 7.01)

State law reference(s)--Conflict of zoning laws with other laws, G.S. § 160A-390.

### Sec. 20-6.	Computation of required spaces.

No part of a yard, or other open space, or off-street parking or loading space required about or in connection with any building for the purpose of complying with this chapter, shall be included as part of a yard, open space, or off-street parking or loading space similarly required for any other building.

The area of a lot or commercial site deemed covered for the purpose of the lot coverage calculation shall consist of all areas of the lot or site utilized in the conduct of the commercial activity on the site or in support thereof, whether improved or unimproved, but shall not include landscaped borders and buffers or buffers upon which there are no commercial activities on or above the ground surface, green areas, open spaces and common areas and facilities approved by the Planning Board on the commercial site, and stormwater retention and disposal areas on site.  Areas approved for outdoor storage and display shall be included as covered area.

All development standards and requirements must be satisfied within the boundaries of the site, tract, parcel or lot submitted for review by the Town except as follows:

In the BC-2 District where a parcel or lot described by deed or map recorded in the Dare County Registry prior to January 1, 1998, has been divided  by the town corporate boundary, and the parcel or lot of land containing land inside and outside of the town boundary limit has been submitted as a commercial site plan for town approval, then the Town may issue a special use permit to allow the applicant to include the area that portion of the commercial site located outside the Town for the purpose of complying with the lot coverage standard set forth in Section 21-146 (d) (5) of the Town Code subject to and provided the following conditions are satisfied:

- The applicant must file a special use permit application with the Town and pay such special use permit fees as may be adopted by the Town Council;

- The applicant must file a copy of the deed or plat recorded in the Dare County Registry with the Town which describes the lot or parcel of land existing on January 1, 1998, which is currently divided by the Town corporate boundary limit;

- That portion of the site lying outside of the corporate limits cannot be separated by a street or vehicular accessway from the commercial site within the Town corporate limits;

- The land area of the site outside of the town limits cannot be subdivided, sold, conveyed or otherwise transferred separate from that portion of the commercial site within the Town if such sale, conveyance, transfer, or subdivision would cause that portion of the site remaining in the Town to violate the coverage standard or any other zoning standard then applicable to commercial lots, and such a sale, conveyance, transfer or subdivision of land shall be a basis for revoking the special use permit and the commercial site approval for the area of  site remaining in the Town as well as the issuance of an injunction by a court which prohibits further use of the site until the site is brought into compliance with the current zoning standards of the Town;

- The area of the site located outside of the corporate limits shall be used only as open space-green area, as access into and out of the site, driveway and for subsurface wastewater drainfield purposes as may be approved by the regulatory authorities.

- The applicant must submit prior to approval by the Town, a  written statement from the appropriate office of the adjoining municipality that the proposed development shown on the site plan within the adjacent municipality complies with all applicable regulations and laws of the adjacent municipality. (98-7)

(Ord. of 8-18-86, § 7.02)

### Sec. 20-7.	Structures excluded from height limitations. (04-18)

The board of adjustment may vary the height limits of these regulations for a church spire, belfry, cupola, and dome or ornamental tower not intended for human occupancy, monument, water tower, smokestack, conveyor radio or television tower, mast or aerial, and parapet wall not extended more than four (4) feet above the roof line of the building, provided such variances will be in harmony with the general purpose and intent of this chapter and will not be injurious to the neighborhood or otherwise detrimental to the public welfare.

(Ord. of 8-18-86, § 7.03)

### Sec. 20-8.	References to certain roads.

All references in any zoning ordinances to U.S. 158 Bypass shall be changed to U.S. 158. All references in any zoning ordinances to U.S. 158 Business shall be changed to N.C. 12.

(Ord. No. 88-15A, 10-24-88)

### Sec. 20-9.	Termination of town approvals.

Unless otherwise provided by the Town Code, any town approval required under chapter 20 as a precondition to undertaking any land use activity, including the construction of improvements to real property or the division or recombination of land, shall expire three hundred sixty-five (365) days following the date of final approval by the town. No action otherwise authorized by such approval shall be undertaken until a subsequent approval has been granted by the town.

(Ord. No. 92-18, 11-2-92)

Secs. 20-10--20-20.  Reserved.

# ARTICLE II.  ADMINISTRATION AND ENFORCEMENT

## DIVISION 1.  GENERALLY

### Sec. 20-21.	Enforcement officer.

The codes enforcement officer or other person designated by the town manager shall administer and enforce this chapter. He may be provided with the assistance of such other persons as the town manager may direct. The codes enforcement officer or such other town employee designated by the Town Manager shall have all necessary authority to administer and enforce this chapter, including the ordering in writing of the remedying of any condition found in violation of this chapter, and bringing of legal action to insure compliance with this chapter, including injunction, abatement, or other appropriate action or proceeding.  (02-21)

(Ord. No. 89-11, 4-17-89)

### Sec. 20-22.	Violations.

If it is found that any of the provisions of this chapter are being violated, the codes enforcement officer or such other town employee designated by the Town Manager shall notify in writing the person responsible for such violations, indicating the nature of the violations, and ordering the action necessary to correct it. He shall order discontinuance of illegal use of land, buildings, or structures; removal of illegal buildings or structures or of additions, alterations, or structural changes thereto; discontinuance of any illegal work being done; or shall take any other action authorized by ordinance to insure compliance with this chapter or to prevent violations of this chapter. (02-21)

(a) While any commercial or multifamily site plan is being reviewed by the town planning board or any zoning request is pending before the planning board, if a zoning or subdivision ordinance violation is located on the site being reviewed or which is the subject of the rezoning request, then the planner shall inform the codes enforcement officer or such other town employee designated by the Town Manager who shall investigate and determine:

(1) Whether there is probable cause to believe that a zoning or subdivision violation exists.

(2) If a violation is found or probable cause to believe a violation has occurred is made by the code enforcement officer or such other town employee designated by the Town Manager then the owner shall be notified of the nature of the violation and the action required to correct the same within the time period specified by the notice sent by the code enforcement officer or such other town employee designated by the Town Manager.

(3) If corrective action is not implemented, then the code enforcement officer or such other town employee designated by the Town Manager shall use any lawful means to terminate the violation. (02-21)

(b) Site plan Violations – The Town Manager or his designee shall investigate any alleged violations of an approved site specific development plan, including but not limited to, any preliminary and final commercial or project site plan, conditions of approval agreed upon by the applicant and contained in the review record during either Planning Board or Council review, and violations of special or conditional use permits.  If a violation is verified by the investigating town official and an administrative approval has not been granted or is not forthwith granted pursuant to the Town Code by the Town Planner or such other town employee designated by the Town Manager, then the Town Manager or his designee shall issue a stop order describing the project work or activity and setting forth the action necessary to cure the violation or steps necessary to bring the project into compliance with the town approvals and permits. (02-21)

Upon notification to the applicant of the stop order by the Town Manager or his designee as provided herein the applicant, its employees, agents, contractors, sub-contractors, independent contractors and any person or entity engaged in the activity or work described in the stop order shall immediately cease work or activity within the geographical area of the project described in the stop order or cease the specific activity or activities described in the stop order.  No project work or activity described in the stop order or in a geographic area described in the stop order shall be continued after notification has been given to the project employee or agent of the applicant, and no work or activity with the scope of the stop order shall be recommenced until the violation has been corrected in accordance with the stop order. (02-21)

A violation of the stop order shall be punishable by a civil penalty not exceeding $500.00  per day for each day or portion thereof that work or activity continues after the issuance of the stop order and notification thereof as provided herein.  Each day shall constitute a separate and distinct violation.  The remedy of a civil action in the nature of a debt to collect the civil penalties shall be in addition to the remedy of injunction for a violation of the stop order or a violation of the town approvals or permits. (02-21)

The scope of the stop order shall be that necessary, as determined by the Town Manager or his designee to effect compliance by the developer and applicant with the approved site specific development plan, permits and conditions of approval by satisfying the requirements to cure or correct the violation as set forth in the stop order; the scope may include the authority to stop the activity or work on the specific object of the violation or within the geographical area of the violation, or with the phase in which the violation has occurred or within the entire project boundary or any portion thereof as well as stopping the specific an entire activity within the project boundary. (02-21)

Until the corrective action has been completed or the violation ceases, the planning board shall make no recommendation of approval, conditional approval or disapproval of the rezoning application or of the site plan or of the multifamily project plan, and any time limits applicable to the review of the site plan or the zoning amendment shall be suspended from the date of the notice of the violation by the Town Manager or his designee until the violation has been corrected, or a court of final jurisdiction determines there is no violation, or a variance has been granted by the appropriate governmental body or agency.  (02-21)

(Ord. of 8-18-86, § 8.04; Ord. No. 89-18, 12-19-88; Ord. No. 89-11, 4-17-89)

State law reference(s)--Remedies for violation of zoning ordinances, G.S. § 160A-389.

### Sec. 20-23.	Fees.

(a) All applications, site plans, planned unit development plans, or other plans required by this chapter to be presented to the town shall be accompanied by a fee. This fee shall be payable to the town. The fee required by this section shall be the amount specified in the regularly adopted fee schedule of the town.

(b) If a preplanning fee is paid, and a preplanning conference is held and the applicant does not present a final site plan for review with the remainder of the review fee within eighteen (18) months, the process is nullified and the initial payment is forfeited.

(c) The property owner or applicant shall be charged the cost of consultants or specialists required by the planning board for the purpose of reviewing the proposed project and the cost of any special meetings of the planning board held upon the request of the applicant. All review costs and fees shall be paid to the town by the property owner or the applicant prior to the final vote of the planning board. The special meeting fee shall be set forth on the town fee schedule as adopted and amended by the town council.

(Ord. of 8-18-86, § 8.06; Ord. No. 94-13, 8-1-94)

Cross reference(s)--Fee schedule, § 5-28 et seq.

### Sec. 20-24.	Amendments.

(a) The town council may, on its own motion or upon motion or upon petition by any person within the zoning jurisdiction of the town, after public notice and hearing, amend, supplement, change, modify or repeal this chapter or the maps which are part of this chapter, subject to the rules prescribed in this chapter. No regulation or map shall be amended, supplemented, changed, modified or repealed until after a public hearing in relation thereto, at which parties in interest and citizens shall have an opportunity to be heard. A notice of such hearing shall be given once a week for two (2) successive calendar weeks in a newspaper of general circulation in the town. The notice shall be published for the first time and for the periods required and in accordance with G.S. § 160A-364 as amended from time to time.

(b) In case of a protest against an amendment, supplement, change, modification, or repeal signed by the owners of twenty (20) percent or more either of the area of the land included in such proposed change, or of the land immediately adjacent thereto extending one hundred (100) feet therefrom, or of the land directly opposite thereto extending one hundred (100) feet from the street frontage of such opposite land, such amendment shall not become effective except by favorable vote of three-fourths of all members of the town council.

(c) Every proposed amendment, supplement, change, modification or repeal to this chapter shall be referred to the planning board for its recommendation and report. The planning board shall have sixty (60) days within which to submit its recommendation to the town council. All petitions for a change in the zoning map shall include a legal description of the property involved and the names and addresses of current abutting property owners.

(d) A fee shall be paid to the town in accordance with the adopted fee schedule for each application for amendment to this chapter to cover the costs of advertising and other administrative expenses involved. (02-24)

(Ord. of 8-18-86, art. X; Ord. No. 89-28, 11-6-89; Ord. No. 90-5, § 6(j), 4-23-90)

State law reference(s)--Amendments, G.S. §§ 160A-384--160A-386.

### Secs. 20-25--20-35.  Reserved.

## DIVISION 2.  APPEALS AND VARIANCES

### Sec. 20-36.	Intent.

It is the intention of this division that all questions arising in connection with the enforcement of the Zoning Ordinance shall be presented first to the Town Planner and that such questions shall be presented to the Board of Adjustment only on appeal from a decision of the Town Planner; and that from the decision of the Board of Adjustment recourse shall be had to the courts, as provided by law.  It is further the intention of this division that appeals from a decision of a building inspector related to the Zoning Ordinance but not pertaining to the State Building Code or State Building laws shall be taken to the Board of Adjustment; and that from the decision of the Board of Adjustment recourse shall be had to the courts, as provided by law.  Except that it shall be the duty of the Town Council to consider any amendments to this chapter, applications for conditional use or special use permits and any appeals or variances related thereto, or site plans as provided by this chapter and any other duty not specifically delegated to the Board of Adjustment herein.  (05-06)

(Ord. of 8-18-86, art. IX)

### Sec. 20-37.	Board of adjustment--Established.

- The Board of Adjustment is established.  The Board shall consist of five (5) members, including the chairman, who shall be appointed by the Town Council.

- Members shall serve terms of three (3) years, with terms staggered to ensure that all members' terms do not expire concurrently.  Vacancies occurring for reasons other than expirations of terms shall be filled as they occur for the period of the unexpired term.  Members may be removed for cause by the Town Council upon presentation of written charges and after providing for a hearing thereon.  The members of the Board may be compensated for expenses incurred in the performance of their duties according to the reimbursement schedule which may be adopted by the Town Council from time to time.  The Board of Adjustment is authorized to expend funds to obtain legal advice or other professional services necessary to discharge their duties.

- In addition to the five (5) regular members of the Board of Adjustment, the Town Council may appoint two (2) alternate members for terms of three (3) years.  Alternate members shall be appointed for the same term, at the same time, and in the same manner as regular members.  An alternate member may sit and deliberate in the place of an absent regular member, and an alternate member sitting on the Board in place of an absent member shall have all the rights, privileges, and duties, including the right and power to vote, as the absent regular member, except that an alternate member sitting for an absent chairperson shall not exercise the duties, rights and privileges of the position of chairman unless the vice-chairman is also absent.  Alternate members may be removed for cause by the Town Council upon presentation of written charges and after providing for a hearing.

- It shall be the duty of any regular Board member to notify the chairman and an alternate of an anticipated absence from any duly called meeting of the Board of Adjustment. (05-06)

(Ord. of 8-18-86, § 9.01)

State law reference(s)--Board of adjustment, § 160A-388.

### Sec. 20-38.	Same--Officers and meetings.

(a) The Town Council shall designate one (1) of the Board of Adjustment members as chairman and another as vice-chairman who shall serve for one (1) year or until a successor is designated. The board may adopt from time to time such rules and regulations as it may deem necessary to carry into effect the provisions of this subsection.

(b) All meetings of the board shall be held at a regular place and shall be open to the public. A quorum of four (4) members shall be present at the designated meeting place before a vote is taken or final disposition of any appeal is made upon which the board is required to pass. The board shall keep minutes of its proceedings, showing the vote of each member upon each question, or if absent or failing to vote, an indication of such fact; and the final disposition of appeals shall be by recorded resolution indicating the reasons of the board therefor all of which shall be a public record.

(Ord. of 8-18-86, §§ 9.02, 9.03)

### Sec. 20-39.	Same--Powers and duties generally.

- The jurisdiction of the Board of Adjustment is limited to questions and issues concerning the Zoning Ordinance, and the Board shall hear administrative appeals and variance requests related thereto.  By a vote of four-fifths of its members, the Board of Adjustment may reverse any order, requirement, decision or determination of any official related to the Zoning Ordinance, or may decide in favor of the applicant a matter upon which the Board is required to pass under the Zoning Ordinance, or may grant a variance from a provision of the Zoning Ordinance.

- The board of adjustment, by a vote of four-fifths of its members, may grant a variance from the requirements of the flood damage prevention provisions outlined in Chapter 8 of the Kitty Hawk Town Code. (06-14)

- Except that the Board of Adjustment may not pass on any question, issue, appeal or variance request related to amendments of this chapter, the issuance of a conditional or special use permit or violations or orders related thereto, or review of site plans, such powers having been specifically reserved by the Town Council and not delegated to the Board.  Decisions rendered by the Town Council are not subject to review by the Board of Adjustment.

- The chairman of the Board of Adjustment or any member temporarily acting as chairman may in his official capacity administer oaths to witnesses in any matter coming before the Board. (05-06)

(Ord. of 8-18-86, § 9.04; Ord. No. 89-11, 4-17-89)

State law reference(s)--Similar provisions, G.S. § 160A-388.

### Sec. 20-40.	Appeals and administrative review.

- Pursuant to its powers and duties, the Board of Adjustment shall hear and decide appeals from and review any order, requirement, decision, or determination made by any official related to the Zoning Ordinance.  Any person aggrieved or any officer, department, board, or bureau of the Town may take an appeal.  Appeals shall be taken within times prescribed by the Board of Adjustment by general rule, by filing with the officer from whom the appeal is taken and with the Board of Adjustment a notice of appeal, specifying the grounds thereof.  The officer from whom the appeal is taken shall forthwith transmit to the Board all the papers constituting the record upon which the action appealed was taken.  An appeal stays all proceedings in furtherance of the action appealed from, unless the officer from whom the appeal is taken certifies to the Board after notice of appeal has been filed with him, that because of facts stated in the certificate would, in his opinion, cause imminent peril to life or property.  In that case, proceedings may not be stayed except by a restraining order, which may be granted by the Board or by a court of record on application, upon notice to the officer from whom appeal is taken and on due cause shown.  The Board shall fix a reasonable time for the hearing of the appeal, give due notice of appeal to the parties, and decide the appeal within a reasonable time.  The Board may reverse or affirm, in whole or in part, or may modify the order, requirement, decision or determination appealed from, and shall make any order, requirement, decision or determination that in its opinion ought to have been made in the circumstances.  To this end the Board shall have all the powers of the officer from whom the appeal is taken. (05-06)

(b) The board shall interpret zoning maps and pass upon disputed questions of lot lines or district boundary lines and similar questions that may arise in the administration of this chapter. The board shall hear and decide all matters referred to it or upon which it is required to pass under this chapter.

(Ord. of 8-18-86, § 9.01(A); Ord. No. 89-11, 4-17-89)

State law reference(s)--Similar provisions, G.S. § 160A-388.

### Sec. 20-41.	Variances.

- Pursuant to its powers and duties, when practical difficulties or unnecessary hardships would result from carrying out the strict letter of the Zoning Ordinance, the Board may, in passing upon appeals, vary or modify any regulation of this chapter, subject to the restrictions enumerated in § 20-41(g), so that the spirit of this chapter is observed, public safety and welfare secured, and substantial justice done.  A variance from the terms of this chapter shall not be granted by the Board until a written application for a variance is submitted demonstrating each of the following: (05-06)

(1) That special conditions and circumstances exist which are peculiar to the land, structure, or building involved and which are not applicable to other lands, structures, or buildings in the same districts.

(2) That literal interpretation of the provisions of this chapter would deprive the applicant of rights commonly enjoyed by other properties in the same district under the terms of this chapter.

(3) That special conditions and circumstances do not result from the actions of the applicant.

(4) That granting the variance requested will not confer on the applicant any special privilege that is denied by this chapter to other lands, structures, or buildings in the same district.

(b) No nonconforming use of neighboring lands, structures, or buildings in the same district, and no permitted use of lands, structures, or buildings in other districts shall be considered grounds for the issuance of a variance.

(c) Notice shall be given at least fifteen (15) days in advance of public hearing at which the board is to consider the variance. The owner of the property for which the variance is sought or his agent shall be notified by mail. Notice of such hearings shall be posted on the property for which the variance is sought at least fifteen (15) days prior to the public hearing. The public hearing shall be held as advertised. Any party may appear in person or be represented by agent or by attorney.

(d) In order for a variance to be granted the board of adjustment must make a finding that all of the following exist:

(1) Requirements of this section have been met by the applicant for a variance.

(2) The reasons set forth in the application justify the granting of the variance, and that the variance is the minimum variance that will make possible the reasonable use of the land, building, or structure.

(3) The granting of the variance will be in harmony with the general purpose and intent of this chapter, and will not be injurious to the neighborhood, or otherwise detrimental to the public welfare.

(e) In granting any variance, the board of adjustment may prescribe appropriate conditions and safeguards in conformity with this chapter. Violation of such conditions and safeguards, when made a part of the terms under which the variance is granted, shall be a violation of this chapter.

(f) Under no circumstances shall the board of adjustment grant a variance to allow a use not permissible under the terms of this chapter in the district involved, or any use expressly or by implication prohibited by the terms of this chapter in the district.

(g) A variance from this chapter is authorized only for deviations in height (not exceeding the maximum building height allowed in this chapter), area, and size of structure, side yard, open space, and building location requirements. The establishment or expansion of the use otherwise prohibited shall not be allowed by variance, nor shall a variance be granted because of the presence of nonconformities in the zoning district or uses in an adjoining zoning district.

## Sec. 20-42.  Variances from flood damage prevention provisions. (06-14)

### (a)	The Board of Adjustment for the Town of Kitty Hawk shall hear and decide requests for variances from the requirements of the flood damage prevention provisions outlined in Chapter 8 of the Kitty Hawk Town Code. (06-14)

### (b)	Any person aggrieved by the decision of the Board of Adjustment may appeal such decision to the Court, as provided in Chapter 7A of the North Carolina General Statutes. (06-14)

### (c)   Variances may be issued for:

#### (1)	the repair or rehabilitation of historic structures upon the determination that the proposed repair or rehabilitation will not preclude the structure's continued designation as a historic structure and that the variance is the minimum necessary to preserve the historic character and design of the structure. (06-14)

#### (2)	functionally dependant facilities if determined to meet the definition as stated in Chapter 8, Article 2 of the Town Code, provided provisions of Chapter 8, Article 4, Section 8-45(i)(2), (3), and (5) have been satisfied, and such facilities are protected by methods that minimize flood damages. (06-14)

#### (3)   any other type of development, provided it meets the requirements stated in this

#### section. (06-14)

### (d)	In passing upon variances, the Board of Adjustment shall consider all technical evaluations, all relevant factors, all standards specified in other sections of this ordinance, and:  (06-14)

#### (1)   the danger that materials may be swept onto other lands to the injury of others;

#### (2)   the danger to life and property due to flooding or erosion damage;

#### (3)	the susceptibility of the proposed facility and its contents to flood damage and the effect of such damage on the individual owner;

#### (4)   the importance of the services provided by the proposed facility to the

#### community;

#### (5)	the necessity to the facility of a waterfront location as defined under Article 2 of this ordinance as a functionally dependant facility, where applicable;

#### (6)   the availability of alternative locations, not subject to flooding or erosion

#### damage, for the proposed use;

#### (7)   the compatibility of the proposed use with existing and anticipated

#### development;

#### (8)   the relationship of the proposed use to the comprehensive plan and floodplain

#### management program for that area;

#### (9)   the safety of access to the property in times of flood for ordinary and

#### emergency vehicles;

#### (10)	the expected heights, velocity, duration, rate of rise, and sediment transport of the floodwaters and the effects of wave action, if applicable, expected at the site; and

#### (11)	the costs of providing governmental services during and after flood conditions including maintenance and repair of public utilities and facilities such as sewer, gas, electrical and water systems, and streets and bridges.

### (e)   A written report addressing each of the above factors shall be submitted with the

### application for a variance. (06-14)

### (f)   Upon consideration of the factors listed above and the purposes of this ordinance, the

### Board of Adjustment may attach such conditions to the granting of variances as it

### deems necessary to further the purposes of this ordinance. (06-14)

### (g)   Any applicant to whom a variance is granted shall be given written notice specifying

### the difference between the Base Flood Elevation (BFE) and the elevation to which

### the structure is to be built and that such construction below the Base Flood Elevation

### increases risks to life and property, and that the issuance of a variance to construct a

### structure below the Base Flood Elevation will result in increased premium rates for

### flood insurance up to $25 per $100 of insurance coverage.  Such notification shall be

### maintained with a record of all variance actions, including justification for their

### issuance. (06-14)

### (h)   The floodplain administrator shall maintain the records of all appeal actions and

### report any variances to the Federal Emergency Management Agency and the State of

### North Carolina upon request. (06-14)

### (i)   Conditions for Variances: (06-14)

#### (1)	  Variances shall not be issued when the variance will make the structure in

#### violation of other Federal, State, or local laws, regulations, or ordinances.

#### (2)	  Variances shall not be issued within any designated floodway or non-

#### encroachment area if the variance would result in any increase in flood levels

#### during the base flood discharge.

#### (3)	  Variances shall only be issued upon a determination that the variance is the

#### minimum necessary, considering the flood hazard, to afford relief.

#### (4)   Variances shall only be issued prior to development permit approval.

#### (5)   Variances shall only be issued upon:

##### a.   a showing of good and sufficient cause;

##### b.   a determination that failure to grant the variance would result in exceptional

##### hardship; and

##### a determination that the granting of a variance will not result in increased

##### flood heights, additional threats to public safety, or extraordinary public

##### expense, create nuisance, cause fraud on or victimization of the public, or

##### conflict with existing local laws or ordinances.”

- A variance may be issued for solid waste disposal facilities, hazardous waste management facilities, salvage yards, and chemical storage facilities that are located in Special Flood Hazard Areas provided that all of the following conditions are met. (06-14)

(1)   The use serves a critical need in the community.

### (2)   No feasible location exists for the use outside the Special Flood Hazard Area.

### (3)   The reference level of any structure is elevated or floodproofed to at least the regulatory flood protection elevation.

### (4)  The use complies with all other applicable Federal, State, and local laws.

(5) The Town of Kitty Hawk has notified the Secretary of the North Carolina Department of Crime Control and Public Safety of its intention to grant a variance at least thirty (30) calendar days prior to granting the variance.

(Ord. of 8-18-86, §§ 2.02, 9.01(B))

State law reference(s)--Variances, G.S. § 160A-388.

### Secs. 20-43--20-50.  Reserved.

## DIVISION 3.  BUILDING PERMITS, CERTIFICATES OF OCCUPANCY, ETC.

### Sec. 20-51.	Building permit generally; occupancy permits generally.

(a) No building or other structure shall be erected or moved, nor shall any existing building or structure hereafter be altered in any manner, and no land-disturbing activity undertaken preliminary to construction of a structure for which a building permit is required, until a building permit therefor has been issued by the building inspector. Each building permit shall expire six (6) months from the date of issuance if work authorized by the permit has not commenced. If, after commencement, the work is discontinued for a period of twelve (12) months, the permit therefor shall immediately expire. The building permit shall expire if less than eighty (80) percent of the work authorized has been completed within twenty-four (24) months from the date the permit was issued. No work authorized by any permit that has expired shall thereafter be performed until a new permit has been secured.

(b) Single family residential structures shall comply with the following requirements: (01-10)

(1) The structure shall not have more than one (1) full kitchen and food preparation area;

(2) The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet;

(3) The structure shall not exceed a total height of thirty-five( 35) feet as provided in the standards of the Zoning District Regulations. (01-10)

(4) Building plans or blueprints and specifications showing utility systems, outlets, and maximum loads of each system shall be filed with the building inspector in addition to the building permit application. The system shall meet or exceed any minimum requirements for the state in addition to the requirements of this chapter. A change in utility system layouts or maximum loads will require the building permit holder or owner to file an amendment to the original building permit describing the change or modification in the utility system. The amendment must be approved in writing by the building inspector.

(5) A building site plan prepared by a licensed surveyor or engineer showing the building foundation footprint (perimeter) and the mean elevation of undisturbed land area at the building location.

(c) All building and occupancy permits shall be conditioned upon continued compliance with the statements set forth in the building permit, and this chapter shall be binding upon the original owner (or building permit applicants if different than the owner), their heirs, successors or assigns. No building shall be occupied or used if the condition of the building permit or occupancy permit has been breached and is not corrected or cured by the owner or otherwise approved by the board of adjustment. The issuance of a building or occupancy permit by the town without approval shall not constitute a waiver of the right of the town to enforce its right to revoke, cancel or terminate the permit due to the violation or breach of any statement contained therein.

(d) The owner of any building to be constructed in the town shall certify on a form approved by the town that all statements, representations, plans and specifications are material representations of fact made to procure the issuance of a building, improvement, or occupancy permit for the building described therein. Any variance therefrom not approved in writing on the permit by the town building inspector or granted by the board of adjustment shall terminate the improvement, building, or occupancy permits and the same shall become null and void.

(e) The town clerk shall maintain a record of all building permits, applications and information required by this section. A separate record shall be maintained by the town clerk, listing those buildings which are in noncompliance with the original building permit or occupancy permit and cause written notice of such noncompliance to be filed with the county register of deeds under the name of the owner of the property.

(Ord. of 8-18-86, § 1.05; Ord. No. 90-5, § 6(k), 4-23-90)

### Sec. 20-52.	Health department approval.

The building inspector shall not approve a building permit for any building for which county health department approval is required, until such approval has been given by the health department.

(Ord. of 8-18-86, § 1.06)

### Sec. 20-53.	Certificate of occupancy.

(a) No land shall be used or occupied and no building structurally altered, erected or moved shall be used or the commercial use of a building changed until a certificate of occupancy has been issued by the building inspector stating that the building and/or the proposed use thereof complies with the provisions of this chapter. A certificate shall be issued for the purpose of renewing, changing, or extending a nonconforming use. A certificate of occupancy, either for the whole or a part of a building, shall be applied for coincident with the application for a building permit and shall be issued within ten (10) days after the erection or structural alterations of such building, or part, shall have been completed in conformity with the provisions of this chapter. A record of all certificates shall be kept on file in the office of the building inspector.

(b) A temporary certificate of occupancy may be issued by the building inspector for a period not exceeding six (6) months during alterations or partial occupancy of a building pending its completion, provided that such temporary certificate may require such conditions and safeguards as will protect the safety of the occupants and the public.

(c) Prior to May 1, 1996, all restaurants operating on the effective date of this amendment (June 5, 1995) shall prepare a restaurant seating plan containing the maximum number of seats approved by the Dare County Board of Health and the approximate location of the seats within the building. The plan shall be signed by the appropriate county health department official to indicate county health department approval, and the original and one (1) signed copy shall be presented to the town planner for his approval which shall be evidenced by the town planner’s signature or such other town employee’s signature as designated by the Town Manager to make such approval.   One (1) copy will be filed with the town planner and the original shall be posted at the restaurant but in no event no more than fifteen (15) feet from either side of the main entrance. The seating plan must be visibly displayed at all times and must be accessible to county or town officials for the purpose of inspecting the seating plan and the premises. (02-21)

(d) The applicant of a commercial restaurant site plan being reviewed by the town shall submit the seating plan as provided by paragraph (c) above to the town planner before an occupancy permit is issued by the town. The original seating plan shall be posted as required by paragraph (c) within ten (10) days of the occupancy permit date. The applicant shall designate on the commercial restaurant site plan the approximate location where the original seating plan will be posted.

(Ord. of 8-18-86, § 1.07; Ord. No. 95-10, 6-5-95)

### Sec. 20-54.	Application for building permit for permitted uses--Single-family detached and two-family residences.

(a) All applications for building permits for single-family detached and two-family residences shall be accompanied by plans in duplicate, drawn to scale, showing:

(1) The actual dimensions and shape of the lot to be built upon.

(2) The application shall include such other information as lawfully may be required by the building inspector including the exact size and location on the lot of existing or proposed buildings or alterations and existing or proposed uses of the buildings and land.

(3) The number of families, housekeeping units, or rental units the building is designed to accommodate.

(4) Conditions existing on the lot.

(5) Any other matters which may be necessary to determine conformance with and provide for the enforcement of this chapter.

(b) One (1) copy of the plans shall be returned to the applicant by the building inspector, after the building inspector has marked the copy as either approved or disapproved and attested to the same by his signature on such copy. The second copy of the plans, similarly marked, shall be retained by the town.

(Ord. of 8-18-86, § 8.02(A); Ord. No. 90-5, § 6(l), 4-23-90)

### Sec. 20-55.  Site Plan Requirements, Site Plan Review, “As built” and Certificate of Occupancy  (03-40)

(a) Where applicable:

1) A site plan review is required for multifamily, and commercial buildings and commercial sites as well as changes of use on existing commercial sites or changes from a residential use to a commercial use of an existing structure.

2) Every applicant seeking review of commercial and multifamily site plans and subdivision plans may have a preplanning board review conference with the town planning staff for the purpose of reviewing and identifying any technical errors, mistakes, or unsatisfied zoning or subdivision ordinance standards shown on the plan or plat under review. Any such errors, mistakes, or unsatisfied standards must be corrected and corrected plats resubmitted to the town planning staff before the application is processed for review by the town.

3) Site plans and modifications to site plans for the following uses when listed as permitted uses or conditional uses within a district must be approved by the planning board and the town council (or the town planning staff as authorized by this chapter) before a building permit for construction may be issued:

a) Multiple-family residential dwellings containing three(3)or more dwelling units.

b) Commercial buildings and use.

c) Apartment buildings and duplexes of two or more buildings.

d) Condominiums.

e) Townhouses.

f) Hotels, motels or motor lodges.

g) Business, commercial or industrial buildings.

h) All other uses similar to but not included above.

(b) Submission requirements:

1) Fifteen (15) copies of a site plan prepared, stamped and endorsed by a registered engineer, surveyor, or other person duly authorized by the state to practice as such shall be submitted no later than thirty (30) days prior to the next regular meeting of the planning board.

Additional copies of each site plan may be requested, as needed, and may be delivered by the planning staff to such town, county and state officials and departments as the planning board or town council may direct.   Each department, agency or staff representing such agency or department shall review the site plan and report any recommendations or comments to the planning staff.

The planning staff shall review the application for compliance with submission requirements, and if the application is complete and all fees paid, schedule a staffing meeting with all participating departments.

The planning staff will notify the applicant of any technical errors, omissions, mistakes or unsatisfied standards which must be corrected before scheduling a review by the planning board.

2) In addition to any other requirements of the town staff or planning board, the site plan shall contain the following information:

a. Property and ownership information.

- The present recorded owner and the map book reference of the site property.

- The owners, lot numbers or map book and page reference of all adjacent properties.

- The boundary of the entire lot by course and distance.

- The width of the existing rights-of-way.

- The nature or purpose, location and size of existing easements.

- Iron pins three-eighths-inch in diameter and thirty-six (36) inches in length at all lot corners, points of tangents, and any angle point along a given court on the lot. Such pins shall be installed at all lot corners.

- The plan shall be drawn to a scale of at least one (1) inch equals one hundred (100) feet and shall show a north arrow.

b. Existing features information.

1) Streets showing the type and width of pavement, curbs and sidewalks.

2)Topographic features of the lot and existing grades for the lot, streets, storm drainage, etc.

3) All other underground utilities and facilities, including gasoline tanks.

4)  Each site plan with wetlands (as defined under the Federal Clean Water Act, as amended from time to time, and the rules and regulations published thereunder) within the site plan boundaries shall have the following certificate of a registered surveyor or engineer printed on the plat:

Wetland Certification

"The site plan contains wetland areas as defined by Federal law and regulated by the Corps of Engineers. The wetland areas as of (date) have been identified based upon the then current Federal rules and regulations as interpreted by the Corps of Engineers. WARNING: The wetland areas as well as the permitted uses of wetland areas may change with subsequent changes in the applicable rules and regulations or the interpretations of them by the Corps of Engineers.”

5) A recordable restrictive covenant containing the following provisions and approved by the town attorney shall be signed by the owners of property subject to commercial site plan review by the planning board prior to final approval by either the town staff or planning board and town council, whichever is appropriate, before commencing the subsequent different commercial use:

"The undersigned, their heirs, successors and assigns, hereby covenant and agree that the property herein described shall be used for the commercial use of (designate commercial use) as allowed by the Kitty Hawk Zoning Ordinance and no other commercial use thereof shall be made without the prior approval of the Town of Kitty Hawk pursuant to the Kitty Hawk Zoning Ordinance. Any changes in the site plan required by the change in commercial use shall be shown on an amended site plan which shall be approved by the Planning Board and Town Council."

c.  Site improvements.

1) The proposed building type (brick or frame)

2) The floor plans and dimensions.

3) The uses and/or narrative of uses within the structure.

4) The proposed first floor elevation.

5) The location and type of all sidewalks and curbs within the site.

6) The location of all wells, as well as size and depth thereof, water lines, water services, fire hydrants, and any other information the planning board may require.

7) The location of sanitary sewer facilities with connection to sewer system or septic tank.

8) The layout and number of parking stalls. The same shall be shown in accordance with article VI, division 2 of this chapter.

9) The finished grades for the entire site.

10) A tabulation of the total number of dwelling units of various types in the project and the overall project density in dwelling units per acre.

d) Before any proposed site plan shall be approved, tentative approval of the proposed sewage treatment and disposal facilities, by the county board of health or the state department having jurisdiction, shall be demonstrated to the planning board along with any dredge and fill permits required by law.

e) The applicant must submit building elevations of each side of the building proposed to be built together with the site plan for the town to review.  The building elevations cannot be changed after town approval of the elevations without subsequent town approval of the revised elevations.  The review of revised elevations may be conducted by the town staff or the planning board in accordance with the policies for review of site plan amendments.

f) Proposed lighting plan in accordance with Sec. 20-412.

g) Right-of-way improvements. Such improvements shall be made in accordance with the policy of the town council.

h) Storm drainage. The same shall be provided in accordance with the policy of the town council.

i) On-site advertising. The location of on-site advertising signs and the distance of the signs from the closest property line.

j) Zoning, etc., information. A statement or certificate of the owner or the owner's agent stating the zoning district classification in which the property is located and, if the property has been previously subdivided, the plat cabinet, and slide number designation assigned by the register of deeds to the recorded subdivision plat.

k)  A reduced site plan on 8-1/2” X 11” that can be shown on an overhead projector.

l) In addition to the building permit covered elsewhere, a permit for any construction within the public right-of-way shall be obtained prior to commencing any work as well as any permits required by the chapter 7, article II and chapter 16.

m) Inspections of sites involving public rights-of-way and inspections of any on-site construction shall be made by the town.

n) Dimensional requirements and development standards shall be in accordance with the district in which the building is to be located, shall be shown on the site plan.

o) Wetlands identification and mapping:

Each applicant seeking planning board review of any proposed subdivision, resubdivision, commercial site plan or multifamily dwelling development shall have the Corps of Engineers verify the location of all wetlands within the property boundaries and the location of the wetland perimeters (404 lines) as flagged by the applicant. Thereafter, three (3) plats prepared by a registered surveyor or engineer shall be filed with the town depicting the boundaries of the wetlands as verified by the Corps or other designated responsible agency along with a written letter or other certification acceptable to the town that the Corps or other designated agency has verified the location of the wetlands as shown on the plats filed with the town.

The 404 or wetland plats shall be filed prior to or at the time the applicant files his application for planning board review. In the event any project contains wetlands and the applicant has not filed the wetlands plat with the town, then the time period for planning board review before town council review shall be automatically suspended and further planning board review forthwith terminated and deferred until the required plats are filed with the town with the appropriate certification. Thereafter, planning board review shall be reinstated without the payment of an additional fee; however, all zoning and subdivision ordinance amendments adopted by the town council during the interim period shall apply in all respects to the applicant's site or development. Any amendments or modifications to the site plan or development plan under review required by zoning or subdivision ordinance amendments shall be incorporated into the site plan or development plan filed with the town before further review by the planning board. If planning board review is terminated under this section and not reinstated within six (6) months of the date of termination, then the applicant's right to reinstate planning board review shall automatically cease and the planning board shall not review the project until the applicant has refiled his application with the town and paid the appropriate fee.

(p) If the site plan depicts the combination of multiple lots, the plat must show the multiple lots as one parcel.  In addition, a covenant that said parcel shall remain permanently combined as one parcel shall be submitted with the site plan.  The plat and covenant shall be recorded in the Dare County Register of Deed’s office and indexed in the name of the current record owner prior to issuance of a building permit. (03-49)

(c) Site plan review process:

All commercial site plans shall be reviewed by the staff in accordance with the administrative procedures established by the town.

(d)  Planning Board Review:

1) The planning board may recommend approval, conditional approval or recommend denial of any proposed site plan. Upon completion of its review, the planning board will transmit its recommendations to the town council.

2) The applicant shall make appropriate revisions to comply with Planning Board recommendations.  The applicant shall submit 15 copies of any revised site plan.

(e) Town Council Approval:

1)  The town council may approve, approve with specific requirements and conditions or disapprove any site plan. A rejected site plan may be resubmitted in accordance with this section when redrafted to meet the specifications of this chapter and upon payment of a filing fee as required in section 20-23.

(f) Application for a building permit.

1) After the Town Council has approved a site plan, the applicant may request a building permit.  The permit shall be granted if it complies with all applicable state building codes and conforms with the approved site plan.

2) The town staff shall conduct a staff review with all the departmental staff who reviewed the site plan and determine if the building permit is in compliance with the approved site plan.

3) The site development shall be in accordance with the site plan and conditions approved by the Town Council.   Any change to the approved site plan shall be submitted to the Planning Board for a recommendation and Town Council for approval.

(g) “As built” approval:

The applicant shall file a final "as built" site plan with the town planning department.  The town staff shall make any inspections necessary to verify the correctness of the site plan. The town planning staff may, utilize the services of an engineer and/or surveyor licensed in the state for the purposes of examining the site plan and comparing the "as built" site plan to the completed site to assure compliance with all applicable zoning, subdivision, soil sedimentation and erosion control and flood ordinances and to assure compliance with the approved site drainage plan as well as any other regulations of the town. The engineer and/or surveyor shall report all noncomplying conditions or standards to the town planning staff.

The “as built” shall contain the following site information, as well as any other information to demonstrate complete in accordance with the approved site plan :

a. The building type, location and the floor dimensions and setbacks.

b. The first floor elevation.

c. The location and type of all sidewalks and curbs within the site.

d. The location of all wells, as well as size and depth thereof, water lines, water services, fire hydrants, and any other information shown on the approved site plan.

e. The location of sanitary sewer facilities with connection to sewer system or septic tank.

f. The layout and number of parking stalls.

g. The finished grades for the entire site along with any stormwater facilities on site.

h. A tabulation of the total number of dwelling units in the project and the overall project density in dwelling units per acre.

i.  Final lot coverage calculations.

The town planning staff shall determine whether the "as built" site plan substantially complies with the project approval (including the approved site plan notwithstanding the deficiencies or nonconformities reported by the engineer and/or surveyor). The town planning staff may:

(1) Deny approval of the "as built" site plan and require modifications and evidence of compliance therewith;

(2) Grant approval; or

(3) Refer the "as built" site plan and engineer's and/or surveyor's report to the planning board for its review and deliberation.

Should the town planning staff deny approval or refer the "as built" site plan to the planning board, then the planning board shall review the "as built" site plan and other information within thirty (30) days of the denial or referral date.

After review by the planning board, it may approve, defer for compliance, or deny approval of the "as built" site plan. If the planning board denies approval of the “as built” site plan, then the applicant may appeal the decision to the town council. The town council may approve, conditionally approve or deny the "as built" site plan.

(h) Issuance of Certificate of Occupancy:

No occupancy permit shall be issued until the "as built" site plan has been approved by either the town planning staff, planning board or town council as provided herein. No final approval shall be granted until all review fees (including engineer's and/or surveyor's fees for services required by this section) have been paid in full to the town by the applicant.

(i) Expiration of site plan approval:

Upon final approval of the site plan, the building inspector may issue a building permit within one hundred eighty (180) days from the date of such approval, provided that all other requirements of this chapter and other applicable town ordinances are met. If a building permit is not secured within one hundred eighty (180) days from the date of final approval of the site plan, the applicant must resubmit the site plan for review by the planning board and the town council.

Prior to the expiration of an approved site plan, the town planning board can grant one extension of the site plan approval for 180 days provided the planning board finds that:

(1) The site plan conforms with all current site plan standards and requirements, and there  have not been any changes in the town code which would make the site plan “non-conforming” on the date of the extension; and

(2)There have been no changes in the site plan.

(j) Changes to an approved site plan

If, following the town council's conditional or final approval, the owner or developer desires to make a change to the approved site plan, the change must be reviewed by the planning board and approved by the town council.

(k) Change of permitted use within a site plan

A change of permitted use of commercial property to another permitted use in the zoning district must be approved by the town prior to commencing the proposed use. If the proposed use is designated as a "permitted use" in the Code for the district, then the town planner may conduct an administrative review and grant town approval of the proposed use and any necessary minor site plan modifications.

The town planning staff may conduct an administrative review and grant town approval under the following circumstances:

(1) The "proposed use" has been designated as a "permitted use" in the Code for the zoning district;

(2) Any site plan modification necessary to conduct the proposed use must be minor and in conformity with all standards and requirements of the Town Code;

(3) An amended site plan depicting the modifications must be filed with the town, and the modifications at the site must be completed, inspected and approved by the town planning staff, the building department and Dare County Health Department, if necessary, before the use is commenced;

(4) After granting town approval pursuant to subparagraph (k), the town planner shall inform the planning board of the approval at the board's next regular meeting following the approval date.

(l) Minor modifications to an approved site plan

Minor modifications to or changes in an approved site plan and in the improved site may be reviewed and administratively approved by the town planning staff provided the modifications or changes to the site plan and the site do not result in any violations of site plan standards and the change or modification must be minor and in conformity with all standards and requirements of the Town Code; and provided further that requirements of section 20-55(k)(3),  and (4) are satisfied. Notwithstanding any request for administrative approval of a change or modification to a site plan and a site, the town planning staff, at their discretion, may require the proposed changes to be reviewed by the town planning board as otherwise provided in the Town Code.

(Ord. of 8-18-86, §§ 1.04, 8.02(B); Ord. No. 87-14, 8-3-87; Ord. No. 88-2, 1-18-88; Ord. No. 88-8a, 8-1-88; Ord. No. 88-13A, 10-24-88; Ord. No. 89-17, 12-19-88; Ord. No. 89-6, 3-20-89; Ord. No. 89-11, 4-17-89; Ord. No. 90-5, § 6(m), 4-23-90; Ord. No. 91-7, 6-17-91; Ord. No. 91-13, § 2, 10-7-91; Ord. No. 95-7, 4-4-95; Ord. No. 95-21, 11-6-95)

### Sec. 20-56.	Application for conditional uses. (3-41)

(a) The town council may approve conditional uses in the zoning districts where such conditional uses are specified by this chapter. Applications for planned unit development shall also be processed under the procedures of this section. The town council may impose such reasonable and appropriate conditions and safeguards upon these conditional use permits as to insure that the spirit and intent of this chapter is preserved and that such conditional use will not adversely affect the public interest.

(b) Commercial uses:

Fifteen(15) copies of a written application and accompanying site plan for a conditional use permit shall be submitted no later than thirty (30) days prior to the next regular meeting of the planning board.  Additional copies of each conditional use application may be requested by the planning staff.

The planning staff shall review the application for compliance with submission requirements.

The conditional use shall meet all the commercial site plan requirements, as well as, provide a list of all adjacent property owners.  In most cases, the conditional use will require a concurrent review of a commercial site plan.   When the staff determines that the application is complete, the staff will schedule the application for the next available planning board meeting.

The planning board shall review the application and shall submit its recommendation as to approval or disapproval along with such conditions as it may deem necessary to the town council.

When it is deemed desirable by the planning board or the town council a public hearing may be held. Notice shall be given at least fifteen (15) days in advance of such public hearing. The owner of the property for which the conditional use is sought or his agent shall be notified by mail fifteen (15) days prior to the public hearing.

The applicant may appear in person, or be represented by an agent, or attorney.

The Town Council shall conduct their review of any conditional use permits as a quasi-judicial body.  Witnesses may be called and facts presented under oath, before the council.

To approve the application, the town council must make a finding that it is empowered as described in the application to grant the permit and that the granting of the conditional use will not adversely affect the public interest.

(c) Residential use in a commercial district:

A conditional use application together with a review fee in accordance with the adopted fee schedule shall be filed with the town for review of residential uses in a commercial district where such is permitted as a conditional use in the zoning district. The town planning staff shall review the application and the town planner will either grant or deny the conditional use requested by the application. If the planner denies the application, then the applicant shall have ten (10) days from the written notification of such denial in order to appeal the town planner's decision to the town council. Upon a timely appeal to the town council the appeal shall be placed on the next regular council agenda for hearing by the town council.

If the town planner or such other town employee designated by the Town Manager grants the conditional residential use in a commercial district, then the planner or such other town employee shall inform the applicant of the following:

(1) A site plan review by the planning board will be necessary in the event the residential use of the dwelling is changed or proposed to be changed to a commercial use.

(2) A commercial use may be commenced on property adjoining the residential use of the applicant.

(d) Conditions and expiration:

In granting a permit for a conditional use, the town council may prescribe appropriate conditions and safeguards in conformity with this chapter. Violation of such conditions and safeguards, when made a part of the terms under which the permit is granted, shall be deemed a violation of this chapter. The town council shall prescribe a time limit within which the action for which the permit is required shall be begun or completed, or both. Failure to begin or complete, or both, such action within the time limit set shall void the permit.

If an approved conditional use, ceases to operate for more than a period of 180 days, the conditional use would expire.  The property owner or subsequent owner/lessee would have to re-apply for the conditional use.  Only permitted uses allowed within the district would be permitted.

(Ord. of 8-18-86, § 8.03; Ord. No. 88-11A, 9-7-88; Ord. No. 95-7, 4-4-95)

Secs. 20-57--20-65.  Reserved.

## DIVISION 4.  NONCONFORMITIES

### Sec. 20-66.	Generally.

(a) Within the districts established by this chapter there exist lots, structures, uses of land and structures, and characteristics of use which were lawful before this chapter was passed or amended, but which would be prohibited, regulated, or restricted under the terms of this Chapter. Except as otherwise specifically provided in this chapter, it is the intent of this chapter to permit these nonconformities to continue until they are removed, but not to encourage their survival. It is further the intent of this chapter that nonconformities shall not be enlarged, expanded, or extended, nor be used as grounds for adding other structures or uses prohibited elsewhere in the same district.

(b) To avoid undue hardship, nothing in this chapter shall be deemed to require a change in the plans, construction, or designated use of any building for which a valid building permit has been issued by the town.

(Ord. of 8-18-86, § 6.01)

### Sec. 20-67.	Nonconforming lots of record.

(a) When a nonconforming lot can be used in conformity with all of the requirements (other than the area or width requirements) applicable to the district in which the lot is located, such a use may be made as of right. Otherwise, the nonconforming lot may be used only in accordance with a special use permit issued by the town council. The town council shall issue such a permit if it finds that the proposed use is one permitted by the regulations applicable to the district in which the property is located, and that the property can be developed as proposed without any significant negative impact on the surrounding property or the public health, safety, or welfare. In issuing the permit the council may allow deviations from applicable dimensional requirements (such as setback lines and yard size minimums) if it finds that no reasonable use of the property can be made without such deviations.

(b) Whenever this chapter creates a nonconforming lot and the owner of the nonconforming lot also owns land adjacent to it, and a portion of this other land may be combined with the nonconforming lot to create a conforming lot (without thereby creating other nonconformities), the owner of the nonconforming lot, or his successors in interest may not take advantage of the provisions of subsection (a).

(c) Notwithstanding the foregoing provisions, on a nonconforming corner lot, the planner may permit the dwelling to be located closer to the side street, provided that the dwelling is not located within the intersection vision clearance area required by section 20-406 or by the state department of transportation.

(d) In any district in which residential multifamily or group development dwellings and/ or projects have been approved by the town planning board and town council prior to August 18, 1986, then the approved multifamily or group development structure may be constructed, erected, repaired, maintained and rebuilt or reconstructed in accordance with the approved project site plans, and the plans and specifications submitted to the building inspector, notwithstanding the limitations by other provisions of this chapter, including but not limited to those pertaining to density, setback and yard area requirements and lot coverage.

(e) In any district in which a motel lawfully existed on June 10, 1981, the motel use may continue. The buildings used for such purpose may be repaired, maintained and rebuilt notwithstanding the limitations imposed by this chapter provided the owner has obtained a special use permit for the motel use and the repairs, maintenance and reconstruction are undertaken and conducted pursuant thereto.

(f) Existing single-family dwellings in commercial zones may be maintained, repaired, and reconstructed.

(g) Any roof overhang, upper floor fireplace or chimney, eave, bay window, or greenhouse window can encroach into the side yard of the BR-1, and BC-1 and BC-2 districts by not more than two (2) feet per side on any residential lot of fifty (50) feet or less in width measured along the front lot line and street right-of-way boundary subject to the review and approval of the same by the building inspector.

(Ord. of 8-18-86, § 6.02(A); Ord. No. 89-11, 4-17-89; Ord. No. 96-2, 1-8-96)

### Sec. 20-68.	Nonconforming uses of land.

Where on August 18, 1986, lawful use of land exists which is not permitted by this chapter, and where such use involves no individual structure with a replacement cost exceeding one thousand dollars ($1,000.00), the use may be continued so long as it remains otherwise lawful, provided:

(1)No such nonconforming use shall be enlarged or increased, nor extended to occupy a greater area of land than was occupied on August 18, 1986 or at the effective date of amendment of this chapter.

(2) No such nonconforming use shall be moved in whole or in part to any portion of the lot or parcel other than that occupied by such use on August 18, 1986 or at the effective date of amendment of this chapter.

(3) If any such nonconforming use of land ceases for any reason for a period of more than thirty (30) days, any subsequent use of land shall conform to this chapter for the district in which such land is located.

(4) No additional structure not conforming to the requirements of this chapter shall be erected in connection with such nonconforming use of land.

(5) The proposed amendment would extend the date by which an owner can replace an existing mobile home with a replacement mobile home from March 21, 1993, through March 21, 2003, or such lesser period of time as may be approved by the town council following the public hearing. The proposed amendment is for the purpose of extending the time of replacement for a specific period of time which would be not less than six (6) months and not more than ten (10) years.

(Ord. of 8-18-86, § 6.02(B); Ord. No. 89-19, 12-19-88; Ord. No. 93-10, 5-3-93)

### Sec. 20-69.	Nonconforming structures.

(a) Where a lawful structure exists on August 18, 1986 or at the effective date of amendment of this chapter that could not be built under the terms of this chapter by reason of restrictions on area, lot coverage, height, yards, its location on the lot, or other requirements concerning the structure, such structure may be continued so long as it remains lawful, subject to the following provisions:

(1) No such nonconforming structure may be enlarged or altered in a way which increases its nonconformity, but any structure or portion thereof may be altered to decrease its nonconformity.

(2) Should such nonconforming structure or nonconforming portion of a structure be destroyed by any means to an extent of more than fifty (50) percent of its replacement cost at time of destruction, it shall not be reconstructed except in conformity with the provisions of this chapter.

(3) Should such structure be moved for any reason for any distance whatever, it shall thereafter conform to the requirements for the district in which it is located after it is moved.

(4) Any such nonconforming structure located on the lots adjacent to the Atlantic Ocean or sound waters may be moved on the same lot provided that such movement does not increase the nonconformity of the structure in any way.

(b) Any outdoor sign which does not comply with this chapter is a nonconforming structure. Notwithstanding any other provisions with regard to nonconforming structures, the continued use of such nonconforming signs as nonconforming structures shall cease and the nonconforming structure be removed (or modified to conform) on January 1, 1994. A nonconforming sign structure which has been destroyed or damaged in excess of fifty (50) percent of its value shall not be repaired or replaced. Such nonconforming sign structure shall be removed by the property owner of the property on which the structure is located.

(c) Prior to January 1, 1987, the planning board shall locate and identify nonconforming signs in the town as well as identify the owner of the sign or the owner of the business or activity advertised on the sign. The codes enforcement officer shall send written notice of the nonconformity to the owner of the sign structure and to the person conducting the activity advertised on the sign, if different from the owner. The notice shall advise such persons that a nonconforming sign structure permit must be obtained from the town codes enforcement officer in order to continue using the nonconforming structure until January 1, 1994. The town codes enforcement officer will issue said permits in the form approved by the town. No fee shall be charged for such permits.

(d) Nothwithstanding any other provision of this chapter to the contrary, any nonconforming structure or structure containing a nonconforming use which was lawful on the adoption date of this chapter can be rebuilt in the event it is damaged or destroyed, whether in whole or in part, by fire, wind, flood or other calamity or catastrophic event. Any such restoration, reconstruction or repair shall be based upon and be substantially similar to the prior structure, and the structure shall not be increased in volume or size, the footprint of the foundation changed or enlarged or decreased, or the building relocated as a result of said restoration, reconstruction or repair. Any such work shall comply with the elerctical, plumbing, heating/air-conditioning and building code in effect at the time of the construction work. In the event a building violated a setback standard of this chapter prior to its destruction by fire, wind, flood or other catastrophic event, then, to the extent possible, said building or portion thereof shall be rebuilt, repaired or restored in compliance with the current setback standards of this chapter. The exterior appearance and materials may be changed with the approval of the planning board.

(e) Should any single-family dwelling meeting the Town’s dimensional requirements as of July 7, 2003 be destroyed, it may be reconstructed with equivalent or reduced dimensions in its current location notwithstanding the setbacks specified herein and the provisions of Kitty Hawk Town Code Section 20-69(a)(2).  (03-44)

(Ord. of 8-18-86, § 6.02(C); Ord. No. 90-6, 5-7-90)

### Sec. 20-70.	Nonconforming uses of buildings or of buildings and premises in combination.

If a lawful use involving buildings with a replacement cost of one thousand dollars ($1,000.00) or more, or of a building and premises in combination, exists on August 18, 1986, or at the effective date of adoption of an amendment to this chapter, that would not be allowed in the district under the terms of this chapter, the lawful use may be continued so long as it remains otherwise lawful, subject to the following provisions:

(1) No existing structure devoted to a use not permitted by this chapter in the district in which it is located shall be enlarged, extended, constructed or moved except in changing the use of the structure to a use permitted in the district in which it is located.

(2) Any nonconforming use may be extended throughout any parts of a building which were manifestly arranged or designed for such use at the time of adoption or amendment of this chapter, but no such use shall be extended to occupy any land outside the building.

(3) If no structural alterations are made, any nonconforming structure or use of structures may be changed to any conforming use, or with the approval of the town council to any use more in character with uses permitted in the district. In permitting such a change, the town council must find that the proposed use is more appropriate or equally as appropriate to the district as the existing nonconforming use and shall require appropriate conditions and safeguards necessary to ensure that the change is in keeping with provisions and spirit of this chapter.

(4) Any structure, or structure and land in combination, in or on which a nonconforming use is superseded by a permitted use, shall thereafter conform to the provisions for the district, and the nonconforming use may not thereafter be resumed.

(5) When a nonconforming use of a structure, structure and premises in combination, or sign is discontinued or abandoned for ten (10) consecutive months (except when government action impedes access to the premises), the structure, or structure and premises in combination, shall not thereafter be used except in conformity with the regulations of the district in which it is located.

(6) Where nonconforming use status applies to a structure and premises in combination, removal of the structure shall eliminate the nonconforming status of the land.

(Ord. of 8-18-86, § 6.02(D); Ord. No. 90-6, 5-7-90)

### Sec. 20-71.	Repairs and maintenance.

(a) In any nonconforming structure or portion of a structure containing a nonconforming use, work may be done in any period of twelve (12) consecutive months for ordinary repairs, or for repair or replacement of nonbearing walls, fixtures, wiring, or plumbing, to an extent not exceeding ten (10) percent of the current replacement cost of the nonconforming structure or nonconforming portion of the structure as the case may be, provided that the cubic content existing when it became nonconforming shall not be increased.

(b) If a nonconforming structure or portion of a structure containing a nonconforming use becomes physically unsafe or unlawful due to lack of repairs and maintenance, and is declared by the building inspector to be unsafe or unlawful by reason of physical condition, it shall not thereafter be restored, repaired or rebuilt except in conformity with the regulations of the district in which it is located.

(c) Nothing in this chapter shall be deemed to prevent the strengthening or restoring to a safe condition of any building or part thereof declared to be unsafe by an official charged with protecting the public safety, upon order of such official.

(Ord. of 8-18-86, § 6.02(E))

### Sec. 20-72.	Conditional uses.

Any use which is permitted as a conditional use in a district under the terms of this chapter shall not be deemed a nonconforming use in such district, but shall without further action be considered a conforming use.

(Ord. of 8-18-86, § 6.02(F))

### Sec. 20-73.	Outstanding building permits.

Nothing in this chapter shall require any change in the plans, construction, size, or designated use of any building, structure, or part thereof for which a building permit has been granted by the building inspector prior to August 18, 1986; provided, however, that where construction is not begun under such outstanding permit within a period of ninety (90) days subsequent to such date or where it has not been prosecuted to completion within six (6) months subsequent to such date, any further construction or use shall be in conformity with the provisions of this chapter.

(Ord. of 8-18-86, § 12.01)

### Sec. 20-74.	Outstanding conditional use and site plan permits.

Any conditional use permit, commercial site plan, subdivision plat, multifamily development, final plat or multifamily site plan which has received final approval (or conditional approval) from the planning board prior to August 18, 1986 shall be allowed to proceed with the commercial or multifamily project or subdivision plan as approved and in accordance with the zoning regulations in effect at the time of the approval and the provisions of this chapter not inconsistent therewith, provided:

(1) All fees owed to the town for the project (or an approved phase if a phased project) and all prior taxes owed by the applicant have been paid.

(2) The building permit for the approved commercial site plan has been issued and construction commenced under the permit by March 18, 1987. Once construction has been commenced it must be continued and be completed within twenty-four (24) months of the date of the building permit.

(3) Final approval from the town council for multistructure projects (multifamily) must be granted and all building permits for the approved project or approved phase must be issued by August 18, 1987 and all construction completed on each structure within twenty-four (24) months of each building permit date. Subsequent phases of a project receiving initial overall approval by the planning board must comply with the requirements of this section.

(Ord. of 8-18-86, § 12.02; Ord. No. 95-3, 3-6-95)

### Secs. 20-75--20-79.  Reserved.

## DIVISION 5.  ZONING VESTED RIGHT

### Sec. 20-80.	Purpose.

The purpose of this division is to implement the provisions of G.S. § 160A-385.1, pursuant to which a statutory zoning vested right is established upon the approval of a site specific development plan.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-81.	Establishment of a zoning vested right.

(a) A zoning vested right shall be deemed established upon the valid final approval or, if required, preliminary approval or conditional preliminary approval, by the town council of a site specific development plan following notice and public hearing. A fee of seventy-five dollars ($75.00) shall be paid to the town at the time the request or application for any zoning vested right permit is filed with the town if a public hearing is required by this chapter.

(b) The approving authority may approve a site specific development plan upon such terms and conditions as may reasonably be necessary to protect the public health, safety and welfare.

(c) Notwithstanding subsections (a) and (b), approval of a site specific development plan with the condition that a variance be obtained shall not confer a zoning vested right unless and until the necessary variance is obtained.

(d) A site specific development plan shall be deemed approved upon the effective date of the approval authority's action or ordinance relating thereto.

(e) The establishment of a zoning vested right shall not preclude the application of overlay zoning that imposes additional requirements which do not affect the allowable type or intensity of use, of ordinances or regulations that are general in nature and are applicable to all property subject to land use regulations by the town including, but not limited to, building, fire, plumbing, electrical and mechanical codes. Otherwise applicable new or amended regulations and standards shall become effective with respect to property that is subject to site specific development plan upon the expiration or termination of the vested right in accordance with this chapter.

(f) A zoning vested right is not a personal right, but shall attach to and run with the applicable property. After approval of a site specific development plan, all successors to the original landowner shall be entitled to exercise such right while applicable.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-82.	Approval procedures and approval authority.

(a) Except as otherwise provided in this section, an application for site specific development plan approval shall be processed in accordance with the procedures established by the Town Code for development and site plan review and shall be considered by the designated approval authority for the specific type of zoning or land use permit or approval for which application is made.

(b) Notwithstanding the provisions of subsection (a), if the authority to issue a particular zoning or land use permit or approval has been delegated by ordinance to a board, committee or administrative official other than the town council, the applicant must request in writing at the time of application that the application be considered and acted on by the town council following notice and a public hearing as provided in G.S. § 160A-364.

(c) In order for a zoning vested right to be established upon approval of a site specific development plan, the applicant must indicate at the time of application, on a form to be provided by the town, that a zoning vested right is being sought (see Appendix 1 [of the ordinance from which this division derives]).

(d) Each map, plat, site plan or other document evidencing a site specific development plan shall contain the following notation: "Approval of this plan establishes a zoning vested right under G.S. § 160A-385.1. Unless terminated at an earlier date, the zoning vested right shall be valid until (date)."

(e) Following approval, preliminary approval or conditional approval of a site specific development plan, nothing in this division shall exempt such a plan from subsequent reviews and approvals to ensure compliance with the terms and conditions of the original approval, provided that such reviews and approvals are not inconsistent with the original approval.

(f) Nothing in this division shall prohibit the revocation of the original approval or other remedies for failure to comply with applicable terms and conditions of the approval or this chapter.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-83.	Duration.

(a) A zoning right that has been vested as provided in this division shall remain vested for a period of two (2) years unless specifically and unambiguously provided otherwise. This vesting shall not be extended by any amendments or modifications to a site specific development plan unless expressly provided by the approval authority at the time the amendment or modification is approved.

(b) Upon issuance of a building permit, the expiration provisions of G.S. § 160A-418 and the revocation provisions of G.S. § 160A-422 shall apply, except that a building permit shall not expire or be revoked because of the running of time while a zoning right under this section is vested.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-84.	Termination.

A zoning right that has vested as provided in this division shall terminate:

(1) At the end of the applicable vesting period with respect to buildings and uses for which no valid building permit applications have been filed;

(2) With the written consent of the affected landowner;

(3) Upon findings by the town council, after notice and a public hearing, that natural or manmade hazards on or in the immediate vicinity of the property, if uncorrected, would pose a serious threat to the public health, safety and welfare if the project were to proceed as contemplated in the site specific development plan;

(4) Upon payment to the affected landowner of compensation for all costs, expenses and other losses incurred by the landowner including, but not limited to, all fees paid in consideration of financing, and all architectural, planning, marketing, legal and other consultants' fees incurred after approval by the town, together with interest thereon at the legal rate until paid. Compensation shall not include any diminution in the value of the property which is caused by such action;

(5) Upon findings by the town council, after notice and a hearing, that the landowner or his representative intentionally supplied inaccurate information or made material misrepresentations which made a difference in the approval by the approval authority of the site specific development plan; or

(6) Upon the enactment or promulgation of a state or federal law or regulation that precludes development as contemplated in the site specific development plan, in which case the approval authority may modify the affected provisions, upon a finding that the change in state or federal law has a fundamental effect on the plan, by town council resolution after notice and a hearing.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-85.	Voluntary annexation.

A petition for annexation filed with the town under G.S. § 160A-31 or G.S. § 160A-58.1 shall contain a signed statement declaring whether or not any zoning vested right with respect to the properties subject to the petition has been established under G.S. § 160A-385.1 or G.S. § 153A-344.1. A statement that declares that no zoning vested right has been established under G.S. § 160A1-385.1 or G.S. § 153A-344.1, or the failure to sign a statement declaring whether or not a zoning vested right has been established, shall be binding on the landowner and any such zoning vested right shall be terminated.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-86.	Limitations.

Nothing in this division is intended or shall be deemed to create any vested right other than those established pursuant to G.S. § 160A-385.1.

(Ord. No. 91-20, 12-10-91)

### Sec. 20-87.	Effective date.

This division shall be effective December 2, 1991, and shall only apply to site specific development plans approved pursuant to the provisions of this division.

(Ord. No. 91-20, 12-10-91)

### Secs. 20-88--20-100.  Reserved.

# ARTICLE III.  ZONING DISTRICTS GENERALLY; ZONING MAP*

### Sec. 20-101.	Official zoning map adopted.

(a)The town is hereby divided into zones or districts within which this chapter shall apply. The districts are shown on a map entitled "Official Zoning Map of the Town of Kitty Hawk, N.C.," which, together with all explanatory matter thereon, is hereby adopted by reference and declared to be a part of this chapter. The official zoning map shall be identified by the signature of the mayor and attested to by the town clerk and bearing the town seal under the following words:

"This is to certify that this is the Official Zoning Map referred to in Article I of the Zoning Ordinance of the Town of Kitty Hawk, N.C.," together with the date of adoption of this ordinance.

(b) Regardless of the existence of purported copies of the official zoning map which may be made or published, the official zoning map which shall be located in the town's administrative building shall be the final authority as to the current zoning status of land and water areas, buildings and other structures in the town.

(c) Extension of boundary. The zoning map of the town is hereby amended so as to extend the zoning boundary for a distance of not more than one (1) mile from the corporate limits, as shown on a zoning map of the extraterritorial zoning area of the town dated May 25, 1994, and subsequently amended, such boundary being described as follows:

(1) Beginning at a point on the mean high water mark of Currituck Sound, said point being the intersection point of the Kitty Hawk Town north boundary line with the mean high water mark of Currituck Sound; thence running the course of the town's north boundary line in a westerly direction for a distance of one (1) mile into the Currituck Sound to a point; thence turning and running in a southerly direction to a point in Currituck Sound (said point being established by extending the southern boundary of the Town of Kitty Hawk in a westerly direction on the same course as said town boundary has on land where it intersects the mean high water mark of Kitty Hawk Bay); thence turning and running from the point thus established in an easterly direction to a point on the mean high water mark of Kitty Hawk Bay (said point being the intersection point of the south boundary of the Town of Kitty Hawk with the mean high water mark of Kitty Hawk Bay); thence running in a general northwesterly direction along the mean high water mark of Kitty Hawk Bay and Currituck Sound to a point on the mean high water mark of Currituck Sound where the Currituck Sound mean high water mark intersects the north boundary of the Town of Kitty Hawk, the point and place of beginning.

(2) Establishment of zones. The areas lying outside of and beyond the corporate limits as shown on the map of the extraterritorial zoning area of the town and as described in subsection (1) are hereby divided into districts in accordance with such map and description, and the provisions of the town zoning ordinance are hereby made applicable in every respect for each of the districts within this extraterritorial area as indicated on the zoning map.

(3) Applicability of ordinances related to zoning, subdivisions, and buildings to extraterritorial area. The zoning ordinance, subdivision ordinance, and building inspection ordinance of the town shall be applicable in every respect within the extraterritorial area in the same manner as they are now applicable within the corporate limits, and the building inspector (or other appropriate official) shall enforce all of the provisions of such ordinances within the extraterritorial area in the same manner as he is authorized to enforce such ordinances within the corporate limits.

(Ord. of 8-18-86, § 1.09; Ord. No. 94-9, 6-27-94)

### Sec. 20-102.	Changes.

If changes are made in district boundaries or other matter portrayed on the official zoning map, such changes shall be promptly entered on the map after the amendment has been approved by the town council.

(Ord. of 8-18-86, § 1.09)

### Sec. 20-103.	Rules for interpretation of district boundaries.

Where uncertainty exists with respect to the boundaries of any of the aforesaid districts as shown on the official zoning map, the following rules shall apply:

(1) Boundaries indicated as approximately following the centerlines of streets, highways, or alleys shall be construed to follow such centerlines.

(2) Boundaries indicated as approximately following platted lot lines shall be construed as following such lot lines.

(3) Boundaries indicated as approximately following corporate limits shall be construed as following such corporate limits.

(4) Boundaries indicated as following shorelines shall be construed to follow such shorelines. In the event of change in the actual shoreline, boundaries indicated as approximately following the centerlines of streams, rivers, canals, lakes, or other bodies of water shall be construed to follow such centerlines.

(5) Boundaries indicated as parallel to or extensions of features indicated in paragraphs (1) through (4) above shall be so construed. Distances not specifically indicated on the official zoning map shall be determined by the scale of the map.

(6) Where physical or cultural features existing on the ground are at variance with those shown on the official zoning map, or in other circumstances not covered by paragraphs (1) through (5) above, the board of adjustment shall interpret the district boundaries.

(7) Where a district boundary lines divides a lot which was in single ownership on August 18, 1986, the town council may permit, as a conditional use, the extension of the regulations for either portion of the lot not to exceed fifty (50) feet beyond the district line into the remaining portion of the lot.

(8) Where the boundaries have been defined by legal description, the legal description shall prevail.

(9) Zoning district boundaries indicated as approximately following platted lot lines and a street, highway or alley right-of-way margin shall be construed as following the centerline of the street, highway or alley.

(Ord. of 8-18-86, § 1.10; Ord. No. 95-15, 8-7-95)

State law reference(s)--Interpretation of boundaries, G.S. § 160A-384.

### Secs. 20-104--20-125.  Reserved.

# ARTICLE IV.  DISTRICT REGULATIONS*

## DIVISION 1.  GENERALLY

### Sec. 20-126.	Kitty Hawk Woods district.

(a) Scope and intent. Unless stated otherwise, this section applies to the Kitty Hawk Woods district. This district is created to encourage development that is compatible with the environmentally sensitive nature of Kitty Hawk Woods and to preserve land in a natural state where such land is considered to be a vital link in the groundwater replenishment cycle of the Outer Banks and where the destruction of natural vegetation would have a harmful effect on the stability of the soil and its resistance to erosion. More specifically, the district is designed to:

(1) Permit low density residential development of those portions of the district suitable for residential use and to encourage open space and limited recreational use of portions not suitable for residential use.

(2) Prohibit commercial and industrial use of the land and any other use not compatible with the ecological carrying capacity of the area.

(3) Preserve the natural features and functions of the area necessary for safe and compatible development on the entire Outer Banks. Such features should include, but are not limited to, the following:

a. The components of the groundwater storage and recharge system which are necessary for the growth and maintenance of the maritime forest vegetation. Such components include ponds, lowlands, marshes, bay forests and wetlands.

b. Vegetation acting as soil stabilizers or which provide significant storm or salt protection value, including the dune ridge plant communities and scrub forest.

(4) Prevent pollution of the estuary and the sound which may adversely affect their biological productivity.

(b) Permitted uses. The following uses are permitted by right:

(1) Single-family dwellings, detached.

(2) Watershed conservation area.

(3) Wildlife and ecological preserves.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches: shall be approved by the town planner and, if required, the town CAMA officer or the town official discharging the duties of the CAMA officer.

(2) Cemeteries with perpetual care: agent shall notify the town planner and, if required, the town CAMA officer or the town official discharging the duties of the CAMA officer before any vegetation is removed.

(3) Hiking trails and special environmental awareness areas of low intensity use.

(4) Commercial kayak guided tours shall be a conditional use subject to the following conditions set forth herein and such other reasonable conditions as may be established by the town: (01-16)

a. Permits – Issuance.  The conditional use must be conducted pursuant to a permit. The town will issue permits for the commercial kayak guided  tours only for applications filed during the month of September each year for the next calendar year.  The permit term will be from January 1 to December 31 of each calendar year unless suspended or revoked by the town.  A permit will be issued to companies that satisfy the following conditions:

1. The permit holder must pay the permit application fee of Three Hundred Dollars ($300.00) to the town and file the completed application and necessary supporting materials during the September of that next year preceding the permit year.

2. Provide evidence of liability insurance coverage of Three Hundred Thousand Dollars ($300,000.00) minimum per occurrence and a total of One Million Dollars ($1,000,000.00).

3. The administrative operations of the company must be located within the commercial districts of the town or outside the boundaries of the town.

b. Permits – Continuance.  Permit holders must satisfy the following requirements at all times in order for a permit to remain active and in good standing during the permit year:

1. A trained tour guide must accompany each tour.

2. The tour guide must have a operational two-way communication device in the possession of the tour guide during the tour.  The device shall be either a cell phone or two-way radio or such other device as approved by the town.

3. Tour guides and tour participants must comply with the rules and regulations established by the town.

4. Tour kayaks must satisfy identification standards established by the town.

5. Kayak operators and participants must obey applicable navigation rules and yield the right of way to larger power boats in narrow channels and shall keep outside the marked navigable channel in curves or bends in the streams or channels.

6. The tour must be conducted during the scheduled time periods for the permit holders and within the areas authorized by the town.

7. Kayak tour participants will not enter upon private property along the streams and canals without permission from the property occupants or owners.

8. Personal floatation devices must be worn by tour participants.

9. Tours south of “Covered Bridge” shall only originate from and shall return to the county boat ramp and shall not extend north of the “Covered Bridge” or onto the tributaries or designated back waters of High Bridge Creek, running from the boat ramp to the sound.

10.  Tours north of “Covered Bridge” shall not extend south of “Covered Bridge” or onto the tributaries or designated back waters of High Bridge Creek or Jeanquite Creek within the boundaries of the town.

11. The permit applicant has not had a permit revoked the prior calendar year.

12.  Kayaks used for research approved by the town shall be exempt from the limits of subparagraph (9) and (10).

c. Permits – Revocation and Suspension

1. The town manager may suspend or revoke a permit for a violation of the rules and regulations by the tour company, its employees, agents or tour participants.  The period of suspension shall be in the discretion of the town.

2.  The town may suspend or revoke the permits for all permit holders at any time the town determines it is in the interest of the public, health, safety and welfare to suspend or revoke the permits.

3. The permit issued by the town is a permissive right to conduct tours as authorized herein and it does not create a vested right to conduct the activity authorized by the permit.

4.  A permit holder may appeal a permit revocation or suspension by the town manager to the town council which shall be heard at the next regular meeting of the town council.  The decision of the council shall be final.

d. Tours

1.  Seven (7) tours per day are authorized north and south of “Covered Bridge” with tour times allocated among permit holders by the town.  Permit holders may exchange allocated tour times with other permit holders after notifying the town of the exchange.

2. Each tour shall consist of one (1) guide and not more than ten (10) kayaks.

3. No tours shall be commenced earlier than one half (1/2) hour before the sunrise and no tour shall end later than one half (1/2) hour after sunset.

e. Penalties.  A person or company violating this ordinance shall be guilty of a misdemeanor upon conviction and punished pursuant to Section 1-5 of the town code.  (01-16)

(5) Family Child Care in a residential structure: (00-34) (04-11)

a. The property owner must be legally operated and obtain a licensed by the NC Department of Health and Human Services – Division of Child Development if required by law, and must be affiliated with the child care resource and referral program in Dare County or a subsequent similar program of the county.

b. All contiguous property owners must give written consent to the use of the property for Family Child Care pursuant to this paragraph.

c. The maximum number of children in the home pursuant to this use shall not exceed the state licensing regulations.

d. Adequate parking and/or “pickup/drop off” space shall be provided on site.

(d) Setback and dimensional requirements. Within the Kitty Hawk Woods district the following dimensional and setback requirements shall be in effect:

(1) The minimum required lot area per dwelling unit is eighty thousand (80,000) square feet of land not including marsh or ponds or wetland swales or streams.

(2) For the purpose of calculating the dwelling density of a tract or parcel to be divided, the proposed Kitty Hawk Woods lots must contain eighty thousand (80,000) square feet of non-wetland area and the lots must have a minimum lot width at the front building setback line of two hundred (200) feet. After the dwelling density has been calculated, a tract may be subdivided into mini conservation lots containing eighty thousand (80,000) square feet of gross area and the subdivision may be approved by the town upon the following conditions:

a. The gross dwelling density of the subdivision shall not exceed the density calculated in paragraph (2) above;

b. The mini conservation lots must comply with all the Kitty Hawk Woods zoning district lot standards and applicable subdivision standards;

c. The remaining portion of the tract or parcel not included within the mini conservation lot subdivision which was included within the original proposed subdivision for the purpose of calculating the dwelling density provided above in paragraph (1) shall be limited to conservation/environmental uses by legal instruments subject to town approval which create and convey perpetual conservation/environmental easements to nonprofit conservation/environmental organizations or to the state (including any appropriate agency or political subdivision thereof) with the underlying fee conveyed to the town, its successors and assigns;

d. The town, in its discretion, may refuse to accept a conveyance of the title to the conservation/environmental tract, and in such event, the town, in its discretion, may approve a conveyance of the title of the conservation/environmental tract subject to the aforesaid easement to an appropriate agency in the state or a nonprofit conservation or environmental organization. In the event the town neither accepts a conveyance of the underlying fee title to the conservation/environmental tract or approves a conveyance of the same to an appropriate state agency or conservation or environmental organization, then the town shall proceed to review the tract under the then current subdivision and zoning regulations with each lot to contain a minimum lot area of eighty thousand (80,000) square feet as specified in paragraph (d)(1) herein.

(3) The minimum required lot width per dwelling unit as measured at the building line is two hundred (200) feet.

(4) Minimum required front yard is fifteen (15) feet, subject to the provisions of paragraph (e)(2) of this section.

(5) Minimum required side yard is ten (10) feet, subject to the provisions of paragraph (e)(2) of this section.

(6) Minimum required rear yard is ten (10) feet, subject to the provisions of paragraph (e)(2) of this section.

(7) Septic tank facilities shall be located in accordance with and shall conform to the requirements of county, state, and federal authorities.

(8) All structures shall be located no closer than the greatest current applicable setback requirement, if any, of the Coastal Area Management Act (or rules and regulations promulgated thereunder) or the Corps of Engineers to any pond, stream, marsh, or any other wetland.

(9) Principal and accessory structures shall meet all requirements of paragraph (e)(2) of this section.

(10) Maximum total height shall not exceed thirty-five (35) feet exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(e)Special development standards, conditions, and requirements. (00-27)

(1) Standards of review. When selecting the building site, the applicant shall adhere to the following standards and to the intent and purpose of this section:

a. Building site selection. The town planner and, if required, the town CAMA officer or town official discharging the duties of the CAMA officer shall review and approve the proposed improvements site plan indicating the proposed building sites before a building permit can be issued. The most suitable building sites are those areas that require the absolute minimum alterations of the natural vegetation, topography, and groundwater systems. Evaluation of site suitability will use the following criteria:

1. Existing unforested land areas shall be considered as the most suitable building sites unless such a selection would threaten the health of the vegetation by stimulating dune migration or cause extensive salt mist intrusion into Kitty Hawk Woods or would involve alterations prohibited elsewhere in this section.

2. Where vegetation must be removed from the building site, the most suitable site will disturb the minimum number of healthy trees and vegetation. (00-28)

In approving the building site, the town planner and, if required, the town CAMA officer or town official discharging the duties of the CAMA officer shall consider the density, etc.

3. The topography of the site will be evaluated for overall development suitability so that all structures can be constructed below the canopy of existing tree cover, yet maintain a setback equal to the greatest current applicable setback requirement, if any, of the state coastal area management agency or the Corps of Engineers from any stream, pond, marsh, or wetland swales.

b. Site design. The following design criteria shall be satisfied, except where an alternative design scheme can be shown to have no adverse effects on the natural features of the district protected under this section:

1. The filling of ponds or marshes or streams or wetland swales is prohibited within the next growing season. The ground stabilization plan shall be approved by the town planner and, if required, the town CAMA officer or town official discharging the duties of the CAMA officer.

2. The dredging of ponds, marshes, streams or wetland swales is prohibited. The developer or his agent shall notify the town planner and, if required, the town CAMA officer or town official discharging the duties of the CAMA officer before any vegetation is removed.

3. The grading or other alteration of dunes with greater than forty (40) percent slope is prohibited.

4. The removal of vegetation which provides storm and salt mist protection or acts to stabilize soil and limit dune migration is to be minimum. Such removal is allowed only for the placement of structures and sewage disposal systems when no other suitable building site is available.

5. The sewage septic tank and other utilities shall be located to the maximum extent feasible on the edges of areas that have been cleared for access and building purposes, and shall satisfy the legal requirements of county, state, and federal authorities. In addition, the use of alternative septic systems that effectively can be located within a forest stand in satisfaction of item 4. of the subparagraph above is suggested and encouraged.

6. The drawdown of groundwater, except for wells, and the alteration of natural drainage patterns is prohibited.

7. The clearing of land to provide access to the physical structures shall be minimized, including the clearing of the forest understory. Driveways shall be the minimum width necessary to allow for easy access by one (1) vehicle and shall follow the natural contour lines of the land insofar as possible.

c. Mitigation requirements. In order to protect and reestablish natural vegetation during and after construction, the following mitigation actions are required:

1. In any areas where vegetation has been removed to expose bare ground, an approved ground stabilization plan shall be in effect within thirty (30) days of the beginning of vegetation removal and suitable native vegetative cover established within the next growing season. (00-29)

2. (00-29) To eliminate pest and disease damage to vegetation, any pruning or damage done to trees during construction should be properly attended to using accepted silvaculture practices.

(Ord. of 8-18-86, § 3.03; Ord. of 3-21-88; Ord. No. 90-3, 3-19-90; Ord. No. 90-5, § 6(n), (o), 4-23-90; Ord. No. 91-15, § 1, 10-21-91; Ord. No. 93-8, 4-5-93)

### Sec. 20-127.	Uses not specifically identified as permitted or conditional uses.

Unless otherwise authorized by an amendment to this Code, any use not specifically identified as a "permitted" or "conditional" use is not allowed.

(Ord. No. 96-19, 9-9-96)

### Secs. 20-128--20-140.  Reserved.

#### DIVISION 2.  KITTY HAWK BEACH

### Sec. 20-141.	Scope.

Except as otherwise provided, this division applies to Kitty Hawk Beach.

### Sec. 20-142.	BR-1 low density beach residential district.

(a) Scope and intent. Unless stated otherwise, this section applies to the BR-1 low density beach residential district. The BR-1 district is intended to encourage the development of low-density residential neighborhoods in Kitty Hawk Beach.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Detached single-family dwellings (not to include mobile homes).

(2) Customary accessory buildings, private swimming pools, tennis courts, private priers and boat slips. (00-3)

(3) Town-owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches.

(2) Fire stations.

(3) Home occupations.

(4) Golf courses and concessions integral thereto, provided that there is no open commercial activity and no sign other than a directional sign is allowed, noncommercial tennis courts and swimming pools associated with residential subdivision amenities, and libraries.

(5) Public utility facilities. Such facilities must provide a vegetated buffer strip at least ten (10) feet in height where the facility abuts a residential lot or use.

(6) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (6) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (6) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(7) Church private schools. A church may have a private school on the church site as an accessory use to the church use upon  the following additional conditions and such other reasonable conditions imposed by the town council:

(a) The church must be the applicant requesting the approval of the private school as an accessory use and the private school must be run, operated and supervised by the church and it must be part of the program of the church.

(b)School activities cannot be scheduled simultaneously with church services or large gatherings.

(c) The number of students cannot exceed the approved seating capacity of the area on the site plan designated for school use.

(d) The church must plant or install and maintain natural or artificial buffers or screens approved by the town where appropriate to screen school activities from adjoining properties and public rights of way.

(e) No increase in exterior sign areas will authorized for the private school accessory use, but the exterior sign area authorized for churches may be divided between the church and the private school accessory use provided the total exterior sign area allowed form churches is not exceeded.  (99-23)

(8) Community swimming pools, tennis courts, and other similar recreational facilities for the use of the project property owners which are owned or operated by a property owner’s association or condominium association and not for commercial use.(00-3)

(9) Community piers and boat slips that are owned or operated by a property owner’s association or condominium association and not for commercial use. (00-3)

(d) Dimensional requirements.  (03-42)

(1) The minimum lot size is fifteen thousand (15,000) square feet. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setback shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials.  The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet. (04-26)

(6) In addition to the minimum number of parking spaces required in sec 20-438, any unit requiring more than four (4) parking spaces, the following shall apply:

a. Drive aisles not less than 10’ wide, shall be provided so that no vehicle will be required to back into the public right-of-way.

b. Required parking spaces and drive aisles shall be graded and paved with asphalt, concrete, or turfstone.

c. Drive aisles shall be a minimum ten (10) feet in width and stacking of vehicles in a drive aisle is prohibited.

d. Stacking of vehicles in areas other than a drive aisle shall be limited to two (2) vehicles per stacked lane.

e. The driveway entrance to a public or private road shall not exceed 20 feet in width.

(7) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with the chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.01(A); Ord. No. 87-1, 1-19-87; Ord. No. 90-3, 3-19-90; Ord. No. 90-21, 9-4-90; Ord. No. 91-15, § 2, 10-21-91; Ord. No. 93-2, 1-4-93)

### Sec. 20-143.	BR-2 medium density beach residential district.

(a) Scope and intent. Unless stated otherwise, this section applies to the BR-2 medium density beach residential district. The BR-2 district is intended to encourage the development of medium density residential neighborhoods in Kitty Hawk Beach.

(b) Permitted uses: The following uses shall be permitted by right:

(1) Detached single-family dwellings not to include mobile homes.

(2) Duplexes.

(3) Multifamily dwellings.

(4) Customary accessory buildings and private swimming pools and tennis courts.

(5) Town-owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches.

(2) Fire stations.

(3) Home occupations.

(4) Golf courses and concessions integral thereto, provided that there is no open commercial activity and no sign other than a directional sign is allowed, noncommercial tennis courts and swimming pools associated with residential subdivision amenities, and libraries.

(5) Funeral homes, provided that:

a. The site is adjacent to or across the street from a commercial zone.

b. The site is contiguous to and has access to a paved public street.

c. The site shall contain no less than the minimum area required for multifamily development.

d. All parking shall be provided onsite or in an adjoining commercial zone at the rate of one (1) parking space for each two and one-half (21/2) chapel seats. The parking may consist of a nine and one-half by eighteen-foot temporary parking lane and drive aisle spaces as delineated on the site plan and approved by the town, provided at least twenty-five (25) percent of the required spaces are the conventional type required by this chapter.

e. A vegetated buffer twenty (20) feet in width shall be provided along any property line adjacent to residential zones. The buffer shall have a nominal screening height of eight (8) feet within two (2) years.

f. Exterior lighting shall be low level and shall be configured to direct the light and glare away from streets and adjacent property. Sodium vapor lights are prohibited.

g. Ancillary sales and services such as, but not limited to, tombstone, memorial tablets and florist shall not be permitted, except in direct association with funeral or burial services provided. Cremation services shall not be permitted.

h. There shall be no promotion of sales or services visible outside the building other than the permitted sign.

i. One (1) sign shall be permitted in accordance with article VI, division 3, except that where the sign is placed along street frontage across from a commercial zoning district, a sign appropriate to that commercial district may be installed. The sign may not be internally illuminated.

j. Lot coverage by principle use and all accessory structures shall not exceed fifty (50) percent.

k. An apartment is permitted on the second floor, provided that applicable state building and fire codes are met and that one (1) parking space per bedroom be reserved for the occupants of the apartment in addition to any other parking spaces that may be required.

l. Such other reasonable conditions as the town council may propose.

(6) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (6) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (6) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(7) Family Child Care in a residential structure: (00-31) (04-11)

a. The property owner must be legally operated and obtain a licensed by the NC Department of Health and Human Services – Division of Child Development if required by law, and must be affiliated with the child care resource and referral program in Dare County or a subsequent similar program of the county.

b. All contiguous property owners must give written consent to the use of the property for Family Child Care pursuant to this paragraph.

c. The maximum number of children in the home pursuant to this use shall not exceed the state licensing regulations.

d. Adequate parking and/or “pickup/drop off” space shall be provided on site.

(d) Dimensional requirements. (03-42)

(1)  The minimum lot size is as follows:

a. For single-family detached residences, fifteen thousand (15,000) square feet.

b. For duplexes, twenty-five thousand (25,000) square feet for each acre or portion thereof.

c. For multifamily dwellings, for each acre or portion thereof, fifteen thousand (15,000) square feet for the first dwelling unit and nine thousand five hundred twenty (9,520) square feet for each additional dwelling unit.

d. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setbacks shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet.(04-26)

(6) In addition to the minimum number of parking spaces required in sec 20-438, any unit requiring more than four (4) parking spaces, the following shall apply:

a. Drive aisles not less than 10’ wide, shall be provided so that no vehicle will be required to back into the public right-of-way.

b. Required parking spaces and drive aisles shall be graded and paved with asphalt, concrete, or turfstone.

c. Drive aisles shall be a minimum ten (10) feet in width and stacking of vehicles in a drive aisle is prohibited.

d. Stacking of vehicles in areas other than a drive aisle shall be limited to two (2) vehicles per stacked lane.

e. The driveway entrance to a public or private road shall not exceed 20 feet in width.

(7) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Density. The maximum permitted density shall be four (4) residential dwelling units per acre.

(f) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.01(B); Ord. No. 89-34, 12-18-89; Ord. No. 90-21, 9-4-90; Ord. No. 93-2, 1-4-93)

### Sec. 20-144.	BR-3 high density beach residential district.

(a) Scope and intent. Unless stated otherwise, this section applies to the BR-3 high density beach residential district. The BR-3 district is established as an area in Kitty Hawk Beach for which the principal use of the land is high density residential and for the development of less intensive residential uses as well as for compatible supporting uses.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Detached single-family dwellings (not to include mobile homes).

(2) Duplexes.

(3) Multifamily dwellings.

(4) Customary accessory buildings including private swimming pools and tennis courts.

(5) Town-owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Home occupations.

(2) Golf courses and concessions integral thereto, provided that there is no open commercial activity and no sign other than a directional sign is allowed, noncommercial tennis courts and swimming pools associated with residential subdivision amenities, and libraries.

(3) Planned unit developments under the provisions of article V of this chapter.

(4) Multifamily developments.

(5) Churches (04-27)

(d) Dimensional requirements. (03-42)

(1)  The minimum lot size is as follows:

a. For single-family detached residences, fifteen thousand (15,000) square feet.

b. For duplexes, twenty-five thousand (25,000) square feet for each acre or portion thereof.

c. For multifamily dwellings, for each acre or portion thereof, fifteen thousand (15,000) square feet for the first dwelling unit and nine thousand five hundred twenty (9,520) square feet for each additional dwelling unit.

d. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setbacks shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet.  (04-26)

(6) In addition to the minimum number of parking spaces required in sec 20-438, any unit requiring more than four (4) parking spaces, the following shall apply:

a. Drive aisles not less than 10’ wide, shall be provided so that no vehicle will be required to back into the public right-of-way.

b. Required parking spaces and drive aisles shall be graded and paved with asphalt, concrete, or turfstone.

c. Drive aisles shall be a minimum ten (10) feet in width and stacking of vehicles in a drive aisle is prohibited.

d. Stacking of vehicles in areas other than a drive aisle shall be limited to two (2) vehicles per stacked lane.

e. The driveway entrance to a public or private road shall not exceed 20 feet in width.

(7) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Density. The maximum permitted density shall be four (4) residential dwelling units per acre.

(f) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.01(C); Ord. No. 90-21, 9-4-90)

### Sec. 20-145.	BC-1 general beach commercial district.

(a) Scope and intent. Unless stated otherwise, this section applies to the BC-1 general beach commercial district. The BC-1 district is established to provide for the proper grouping and development of commercial facilities in Kitty Hawk Beach.

The Kitty Hawk beach commercial-1 district has been established to provide for the commercial needs of the neighborhood and the immediate surrounding geographical area.  The commercial development will be characterized by small to medium size land parcels with commercial development of low intensity.  This district is not intended to be developed for shopping centers, shopping malls, or “Big Box” retail/wholesale businesses. (00-40)

(b) Permitted uses. The following uses shall be permitted by right:

(1) Offices:

a. Business.

b. Financial.

c. Governmental.

d. Professional.

(2) Retail stores:

a. Books.

b. Cameras.

c. Candy.

d. Clothing.

e. Dry goods.

f. Automobile parts, not to include actual repair or service.

g. Drugs.

h. Flowers.

i. Gifts.

j. Hardware, including rental of small equipment (no outside display or storage). (3-36)

k. Hobby goods.

l. Jewelry.

m. Leather goods.

n. Magazines.

o. Music store.

p. Notions.

q. Sporting goods.

r. Toys.

s. Food stores.

t. Antiques.

u. Household appliances.

v. Video rental.

w. Furniture stores (03-02)

x. Bicycle sales and skate board sales, provided there is no outdoor storage or display. (03-05)

y. Medical equipment & supplies- sales and rental. (3-36)

(3) Service establishments, including such uses as:

a. Appliance and electronic equipment sales and repair, provided there is no outdoor storage of appliances or equipment. (94-8)

b. Barber and beauty shops.

c. Dry cleaning and laundry pickup stations including laundromats.

d. Parking lots.

e. Radio and television broadcasting studios.

f. Restaurants (03-19)

g. Shoe repair.

h. Theaters.

(4) Public and private schools.

(5) Town-owned or leased facilities.

(6) Home sales center, provided there is no outdoor display of merchandise. (96-22)

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town as provided in section 20-56:

(1) Single-family dwellings.  A single family dwelling must comply with the dimensional requirements of the BR-1 district. (99-2)

(2) Gas stations or fuel dispensing accessory use facilities, provided that no principal or accessory building shall be located within fifty (50) feet of a residential district and provided that there shall be no storage of wrecked or abandoned cars, and that no portion of a gas station building, equipment or gas pumps shall be nearer than seventy-five (75) feet to any right-of-way. A gas station or fuel dispensing accessory use facility shall be designed and equipped such that no more than eight (8) vehicles can simultaneously receive fuel from the fuel dispensing devices located on the gas station or fuel dispensing accessory use facility site. (94-16)

(3) Motor vehicle service and repair, provided that there shall be no storage of wrecked, junked or abandoned vehicles and that there shall be no fuel dispensing devices.

(4) Conveyor car wash service station is a conditional use upon the following conditions and such other conditions as the town council may reasonably set forth:

a. A pre-wash vehicular lane of sufficient size to accommodate twelve (12) vehicles during the pre-wash phase of the wash operation.

b. A post-wash vehicular lane of sufficient size to accommodate four (4) vehicles being towel dried. The exit from the conveyor belt system shall be the entrance to the post-wash vehicle drying area.

c. The pre- and post-wash vehicle areas must be located separate from and outside of internal traffic circulation lanes and ingress-egress access points with public or private rights-of-way.

d. At the location of the vehicle exit from the building there shall be a water catchment at the post-wash vehicle parking area which shall drain and slope to the water catchment. The water catchment must be large enough to contain all water draining into it from washed vehicles.

e. The water used for washing the vehicles must be recycled on-site through a closed recycle system. The system must be designed, constructed and maintained in order to prevent wash water from entering the subsurface ground area or from flowing upon the ground surface.

f. On-site directional signs must not be illuminated internally. Sign lighting on the site must not interfere with the vision of motor vehicle operators on-site or off-site.

g. Each site shall have restroom facilities open and available to its customers.

h. Trash containers must be located at all vacuum cleaner stations and the area maintained free of trash and rubbish.

i. At least one (1) attendant shall be on duty during the hours of operation.

(5) Public utility facilities.

(6) Seafood markets.

(7) Hospitals and medical centers.

(8) Emergency heliport. The operation of a medical emergency evacuation heliport shall be an accessory use to the primary hospital or medical center conditional use and shall be authorized in addition to the primary hospital or medical center conditional use as a conditional accessory use after the following requirements have been satisfied:

a. Hospitals and medical clinics shall include those facilities which, under normal operating conditions, receive and treat trauma patients.

b. Compliance with and evidence of compliance with any applicable requirements of the department of transportation, division of aviation, the Federal Aviation Administration, and any other federal, state or local agency having [jurisdiction over] the operation of helicopter flights, landings and the environmental impact thereof.

c. Submission of a conditional accessory use application with the town planning department in addition to the following materials:

1. A site plan and map drawn to a scale of 1:100 showing the locations, height and first floor elevations or foundation elevations above mean sea level of all structures, utility and street rights-of-way, existing power lines, towers, undeveloped residential lots, and other similar uses within five hundred (500) feet of the outside edge of the heliport landing pad.

2. Environmental assessment consisting of (i) an analysis of the impact of noise within five hundred (500) feet of the heliport performed by a qualified audio engineer or other appropriate professional; (ii) an analysis of the socioeconomic issues relating to the heliport and the hospital or the clinic; and (iii) viable alternatives. The fees and costs associated with the review of the environmental assessment shall be paid to the town by the applicant prior to the public hearing as part of the application review fee. The analysis is to be submitted prior to the first regular or special meeting of the planning board at which the application is considered.

3. Proposed heliport lighting plan demonstrating the technology and technique for retaining light on the site and prevention of light or light glare from affecting traffic using streets and highways in the area.

4. Proposed approach and departing flight paths shall be shown on the map required in subparagraph 1. above.

5.Proposed protection for the innermost portion of the approach and departure routes.

d. Upon filing the application and all required materials with the town planner, the town council shall set a public hearing on the applicant's request for the conditional accessory use of the medical emergency evacuation heliport. In addition to advertising the public hearing, the town planner shall mail, by certified mail, copies of the notice of hearing to all property owners on the tax records of the town within five hundred (500) feet of the heliport landing pad. The cost of advertising and mailing of the notices shall be borne by the applicant and paid prior to the public hearing date.

e. In the case of a protest against the medical emergency evacuation heliport by the owners of twenty (20) percent or more either of the area of the lots included within five hundred (500) feet of the heliport pad, or of those immediately adjacent thereto either in the rear thereof or on either side thereof or extending five hundred (500) feet therefrom, or of those directly opposite thereto extending five hundred (500) feet from the street frontage of the opposite lots, then a motion to allow the conditional accessory use shall not become effective except by favorable vote of four (4) of the five (5) members of the town council.

f. No protest against the applicant's proposed conditional accessory use shall be valid or effective for the purposes of the above paragraph unless it be in the form of a written petition actually bearing the signatures of the requisite number of property owners and stating that the signers do protest the proposed conditional accessory use and unless it shall have been received by the city clerk within ten (10) days following the public hearing. For the purposes of determining the ten-day period, the date of the public hearing shall be excluded and the tenth day shall be included. Thereafter the town shall determine the sufficiency and accuracy of the petition within two (2) normal work days, excluding Saturdays, Sundays and legal holidays, after the end of the ten-day period for filing protest petitions. The protest petition shall be in a form adopted by the town council. In the event no protest is filed or the protest is inadequate, the favorable vote of three (3) of the five (5) town council members shall be required in order to authorize the conditional accessory use.

g. The minimum lot or parcel size to accommodate a hospital or medical center with a heliport facility shall be three (3) acres or more.

h. The heliport shall be ground based only. No rooftop facility shall be permitted.

i. The heliport shall comply with the latest edition of the FAA regulations in its design, size and use.

j. The heliport shall not be used until FAA approval is obtained and a copy of the approval is submitted to the town.

k. Prior to submitting the FAA permit application for the heliport, a copy of the application shall be forwarded to the town. The town's review of the copy of the application is for informational purposes and not intended as an approval requirement.

l. No fixed base operations or refueling facilities shall be permitted on the hospital-medical center or heliport site.

m. Lighting of the helipad or final approach and takeoff areas shall comply with FAA regulations but shall be ground level based using cutoff or restrictive features to minimize overspill of light from the activity area itself.

n. No takeoff and landing area shall be nearer than two hundred (200) feet at its closest point to a state or federal highway. The entire takeoff and landing area, as defined by the FAA and determined by the size of the helicopter, shall fall within the property lines of the lot. The developer of a heliport shall be required to place signs warning motorists of "Low-Flying Aircraft." These signs shall be placed in accordance with regulations of the state department of transportation.

o. Fire protection and safety measures shall be in accordance with the FAA and NFPA guidelines. The presence of firefighting equipment shall not be required by the town unless requested by the owner. The cost of providing such firefighting equipment coverage shall be the responsibility of the owner.

p. The use of sound buffers, proper facility siting, separation distances or other natural or manmade barriers shall be made a part of the heliport approval process.

q. A protection plan for the innermost portion of the approach and departure routes shall be developed. Protection means may consist of, but not be limited to, acquiring title to the land or leasing the land area, acquisition of air rights, or the written consent of property owners within the approach and departure routes, or other means approved by the town council. The plan shall include an evaluation of viable alternatives. The method of such protection must be approved by the council and consummated by the applicant. The council, acting in its sole discretion, may waive this requirement if the procedure cannot be completed within a reasonable period of time or for reasons beyond the control of the applicant.

(9) Veterinary hospitals and clinics subject to other requirements of this chapter and provided the following conditions are met:

a. No animals shall be kept or boarded outside the principal building and there shall be no structure, runs, or pens used for boarding, holding or restraining animals located upon the site outside of the principal building.

b. The principal building shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will allow no animal noise originating within the building to be heard at any point on the approved site boundary.

c. All animal waste must be disposed of through the facility septic system and the applicant must submit evidence that the appropriate county or state department has approved the proposed septic systems for the disposal of animal waste.

d. No dead animal shall be placed in any outside receptacle located on the premises outside the facility.

e. A vegetated or constructed visual buffer may be required.

f. Exterior lighting shall be of low intensity and shall reflect upon the site in such a manner as not to interfere with traffic on public streets or highways.

(10) Apartments on the second story of compatible commercial uses, provided that applicable state building and fire codes are met and that one (1) parking space per bedroom be reserved for the occupants of the apartment in addition to any other parking spaces that may be required.

(11) Day care centers.

(12) Boat sales and rentals, provided that any outdoor display must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510.  (01-11)

(13) On-site retail sales and production of bakery goods, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of bakery goods must be incidental to and for the purpose of on-site retail sales and not for distribution to wholesale vendors.

b. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the bakery production activity.

c. The owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(14) Craft production and retail sales, subject to any reasonable conditions imposed by the town council and the following conditions:

a. The production of crafts must be incidental to and for the purpose of retail sales and not for distribution to wholesale vendors.

b. The maximum square footage of the area devoted to craft production shall not exceed eight hundred (800) square feet.

c. The building in which crafts are produced shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof used for craft production) if the same is located within a multiunit structure. An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the craft production activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for craft production if the same is located within a multiunit structure).

d. All odors and vapors (including chemical vapors) associated with or generated by the production of crafts shall be contained within the boundaries of the site or within the building (or that portion of the building used for the production of crafts in a multiunit structure). When requested by the planning board, an engineer's certification shall be furnished to the planning board that the design, construction, and proposed methods of odor and vapor control are sufficient to contain all odors and vapors generated by or emanating from the production of crafts within the site boundary (if a freestanding building) or within the interior of that portion of the building used for the production of crafts in a multiunit structure.

e. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the craft production activity.

f. If flammable materials are used in the craft production activity, the owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(15) Print shop. A print shop may be permitted subject to other requirements of this chapter and provided the following conditions are met:

a. No more than one thousand (1,000) square feet may be used for printing purposes, excluding office area.

b. The principal building shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will allow no machine noise originating within the building to be heard at any point on the approved site boundary.

c. In the event any chemicals used or stored on the premises require approval by a governmental agency of the method such chemicals are stored, used or disposed, then written evidence of the approval by the appropriate governmental agency shall be furnished to the town.

d. A list of all chemicals used or stored on the site shall be filed on or before January 10 of each year with the volunteer fire department.

(16) Miniature golf course associated with PCD. A miniature golf course is a course containing all or a significant number of elements of a regulation golf course consisting of some combination of tees, fairways, greens, sand traps, water and vegetation hazards, and varying topography but all of which have been reduced in size to accommodate the project site and designed for the sport of golf. Putt-Putt type golf courses and animated courses are expressly excluded. Miniature golf courses, where authorized in various districts under this chapter, shall meet the following conditions and requirements:

a. The miniature golf course and support facilities must be located within the boundary of the PCD project.

b. The miniature golf course must be compatible with the land uses in the PCD project.

c. The miniature golf course site shall consist of at least thirty thousand (30,000) square feet of land area for an eighteen-hole course with parking and golf course facilities. The total number of golf course holes shall not exceed thirty-six (36) holes of play. In addition, a separate practice putting green may be located within the miniature golf course site boundary.

d. To the extent possible, the course design and layout shall utilize the natural terrain of the site. Alteration of the existing natural topography and the creation of other natural topographical features such as hills, mounds and waterfalls shall require the approval of the planning board and town council. There shall be no animation on or around the course and within the golf course site. The golf course site and course shall utilize only water, earth and natural vegetation as hazards, decorative features and other play-related features and shall not use artificial devices including, but not limited to, miniature buildings, animal, human or vehicular models, signs (other than hole number signs) or billboards. A site plan, including the location and identification of trees larger than six (6) inches in diameter shall be required for miniature golf courses. The PCD submission and review procedure shall apply to the planning review of the golf course site.

f. The course shall be buffered from adjacent parcels outside the PCD by a vegetated buffer; however, it is permissible for the course to be visible from U.S. Highway 158 if the PCD boundary is contiguous to the right-of-way of U.S. Highway 158. The buffer shall be a vegetated buffer with plants, shrubs and trees approved for use by the planning board. The vegetated buffer shall be maintained by the manager or association responsible for maintenance of the planned commercial development.

g. There shall be no neon or flashing lights in the miniature golf course site boundary. Lighting of the course shall be contained on the site and shall not interfere with the reasonable enjoyment of adjacent parcels or traffic on streets or highways.

h. The electrical transmission of sound shall be prohibited.

i. Alcoholic beverages may not be sold or consumed in the miniature golf course area.

j. Restroom facilities shall be located within the golf course area for use by the players.

k. The golf course design shall accommodate stormwater retention and shall comply with the guidelines for soil erosion and sedimentation control.

l. There shall be one (1) parking space for each numbered hole plus one (1) parking space for each employee working in the golf course site. All parking spaces shall be located within four hundred (400) feet of the golf course.

m. Trash receptacles shall be centrally located and maintained within the golf course site.

(17) Cater service (3-38)

(18) Game room subject to the following conditions:

a. A game room conditional use can co-exist with another authorized district use within a freestanding detached building or within the area owned or leased by a person or entity within a multiuse building where the game room utilizes no more than forty-nine (49) percent of the gross (interior) square footage of the floor area excluding designated storage area and areas to which customer access is prohibited.

b. Supervision in the game room area shall be provided during all operating hours. If the owner or lessee of the game room allows the consumption of alcoholic beverages within the game room, then the supervising attendant must be at least eighteen (18) years of age.

c. The game room area shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary. If the game room is located in a structure which houses other nonrelated businesses, the noise within the game room must not be audible beyond the boundaries of the area leased or owned by the game room operator. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will comply with these standards.

d. The maximum number of occupants shall be determined by the building inspector according to the standards of the North Carolina State Fire Code. The maximum number of occupants allowed in the game room shall be posted in a conspicuous place therein and its limits shall not be exceeded.

e. No alcoholic beverages shall be sold in the game room and no alcoholic beverages shall be consumed therein unless permitted by state law and in compliance with state law and the laws of the town.

f. It shall not be permissible to stimulate play by gambling incentives such as (but not limited to) free plays or games, monetary rewards or the exchange of coupons redeemable for play or gifts.

g. The parking requirement shall be calculated on the basis of one (1) space for each two hundred (200) square feet of floor area devoted to game room use.

(19) Natural grass putting golf course. Definition. A natural grass putting golf course is a course containing all or a significant number of elements of a regulation golf course consisting of some combination of tees, fairways, greens, sand traps, water and vegetation hazards and varying topography, but all of which have been reduced in size to accommodate the project site and designed for the sport of golf and limiting play to the use of a putter. Miniature golf courses utilizing artificial playing surfaces and/or with animated features within the golf course site are expressly excluded. All playing surfaces must be natural grass. Playing surfaces may be connected by wooden walkways not to exceed three (3) feet in width and one (1) foot in height. In addition to any other reasonable requirements or conditions imposed by the town council, a natural grass putting golf course shall satisfy the following conditions and requirements:

a. The natural grass putting golf course site shall consist of an area that is at least one (1) acre in size for each eighteen (18) holes of play. The parking area for the natural grass putting golf course site shall not be located within the boundaries of the natural grass putting golf course site, and the parking area cannot be included in the calculation of the required one (1) acre of land area for each eighteen (18) holes of golf. An accessory structure used as a pro shop may be located within the natural grass putting golf course site. In no event shall the total number of golf holes exceed thirty-six (36); however, a separate practice green may be included within the natural grass putting course site in addition to the maximum of thirty-six (36) holes of golf.

b. The natural grass putting golf course shall be designed to utilize the natural terrain of the site as much as possible. The golf course shall contain and consist of only natural materials. A certification must be submitted by the property owner with the site plans stating that there will be no change in natural terrain exceeding two (2) feet in elevation or two (2) feet in excavation of the site. If the existing terrain will be altered by more than a two-foot increase or decrease in elevation, then a topographic plan drawn with elevations at two-foot intervals must be submitted with the site plan.

c. There shall be no artificial moving objects or animated natural objects on or around the course and within the natural grass putting golf course site. Only water, earth and natural vegetation shall be used as hazards or decorative features. Artificial devices including, but not limited to, miniature buildings, animal, human or vehicular models, manmade waterfalls, signs (other than hole number signs) or billboards are prohibited within the golf course site. The site plan shall show the location of and identify trees larger than six (6) inches in diameter within the project site. To the extent possible, trees with diameters larger than six (6) inches shall be preserved.

d. The natural grass course shall include a buffer along its common boundary with adjoining properties. The buffer shall consist of an earthen mound at least ten (10) feet wide at its base and six (6) feet high. Natural vegetation shall be planted on the mound to function as a visual screen. The type of vegetation planted on the buffer shall be approved by the planning board. The screen shall be maintained by the owner or operator of the course. In the absence of a buffer and vegetative screen, there shall be an open space buffer fifty (50) feet in width along the common boundary with adjoining properties and lying within the project site. No golf course playing area shall be located closer than two hundred (200) feet to the right-of-way of any state road or federal highway.

In order to safeguard the public health, safety and welfare, a sufficient vegetative screen shall be provided on property lines adjacent to such state or federal highways. The overall height of the screen shall be at least twelve (12) feet. It shall be opaque to a minimum height of six (6) feet. Intermittent visual obstruction shall be permitted from six (6) feet to a height of at least twelve (12) feet. The opaque portion of the screen is intended to exclude visual contact during all seasons. The intermittent portion shall not contain any unobstructed opening more than fifteen (15) feet wide except at approved access points. The screen shall be composed of a landscaped earthen berm, planted vegetation or existing vegetation. The following planting materials list defines acceptable species which may constitute the screen. A plan showing details of a proposed screen shall be developed and submitted as part of the site plan review procedure. The plan shall be judged sufficient if it satisfies the standards and species defined in this subsection.

Planting Materials

Trees--10' to 40': Black Pine (Pinus thunbergi), Brazilian Butia (Butia capitata), Cabbage Palmetto (Sabal palmetto)*, Cedar (Cedrus deodora), Cluster Pine (Pinus pinaster), Crepe Myrtle (Lagerstroemia indica), Eastern Red Cedar (Juniperus virginiana), Live Oak (Quercus virginiana), Redbay (Perseaborbonia), White Poplar (Populus alba). *Requires special care in colder areas.

Opaque screen--Ground to 6': Bayberry (Myrica pensylvanica), European Privet (Ligustrum vulgare), Japanese Privet (Ligustrum japonicum), Oleander (Nerium oleander), Pampas Grass (Cortaderia selloana et. ac.), Pittosporum (Pittosporum tobira), Russian Olive (Elaeagnus pumgens), Saltcedar (Tamarix gallica), Wax Myrtle (Myrica cerifera), Yaupon Holly (Ilex vomitoria).

e. Lighting within the natural grass putting golf course site boundary shall be shielded and focused on the course in such a manner as not to interfere with the reasonable enjoyment of adjacent parcels of property or traffic on nearby streets or highways. The light shall be contained on the site. Light fixtures shall not exceed ten (10) feet in height. There shall be no neon or flashing lights within the natural grass putting golf course site boundary. A light plan shall be submitted to the planning board as part of the site plan review. The light plan shall demonstrate compliance with the light standards set forth herein.

f. The transmission of voice or musical sound within the natural grass putting golf course site boundary shall be prohibited. Malt beverages, wine or other beverages containing alcohol shall not be sold or consumed within the boundaries of the natural grass putting golf course. Adequate restroom facilities for the use of golf course players and customers shall be located within the golf course area. Trash receptacles approved by the town and/or county shall be located and maintained on the site. The site plan submitted to the planning board shall contain information and design elements demonstrating that stormwater runoff will be managed in a manner which contains the runoff within the boundaries of the site and otherwise complies with the applicable provisions of the soil erosion and sedimentation control ordinance.

g. There shall be a minimum of two (2) parking spaces per hole of golf play and one (1) parking space for each golf course employee at the site. If the natural grass putting golf course is planned and included as part of a restaurant mixed-use development, then the minimum parking requirement may be reduced to one and one-half (11/2) spaces per golf hole within the site and one (1) space for each employee at the site.

Accessory uses. A restaurant center that sells snacks and beverages to the customers may be included as an accessory use within the natural grass putting golf course site.

(20) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (21) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (21) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(21) Clubhouse/pro-shop (00-5) use subject to reasonable conditions established by the town council and the following conditions:

a. The height, intensity and location of the exterior lights must be shown on the site plan and approved by the town.  The maximum height of exterior lights shall not exceed twenty (20) feet, and the exterior lights shall be positioned such that the light is directed down towards the ground surface and shielded to prevent glare from being observed from adjoining properties and town rights of way.  An illustration of the proposed exterior lights must be submitted to the town and approved by the town planner.

(00-5)

b. Cart path surfaces will be maintained in a safe condition by the golf course owner and at the golf course owner’s expense.  The golf course owner must properly repair any cart path within a town street right of way which has been damaged by town, its employees, agents, contractors, or subcontractors performing work for the town or public utilities performing utility work within the street right of way. (00-5)

c. The golf course owner shall maintain liability insurance in an amount and from approved by the town, with the town as an additional insured, against risk causing injury or damage due to the presence and use of the golf paths within the town right of way. (00-5)

d. Warning signs and signals at any town, street, right of way crossing or crossings will be maintained by the applicant (at the applicant’s expense) in good working order in all times that the golf course is open for play.  Failure to maintain signs or signals shall result in a suspension of the conditional use and shall constitute a violation of the this condition with shall be enforceable by the remedy of injunction in addition to, and not in limitation of, other remedies available to the town. (00-5)

e. There shall be a minimum of four (4) parking spaces per hole on the site. (00-5)

A regulation golf course consisting of 18 or more playable holes which has a retail pro shop and/or general administration offices for the golf course use and any ancillary golf course uses may share a maximum of six (6) parking spaces for the golf course use based upon the parking standards for a retail use (one space per 350 square feet) and for an office use (one space per 500 square feet and one parking space per employee) in order to satisfy the total parking standards for the golf course use together with the retail pro shop and general administrative office use. (00-18)

The total number of golf course use shared parking spaces shall not exceed six (6), and the number of shared parking space shall not be increased as a result of an increase in the square footage of the retail pro shop use or the general administrative office use. (00-18)

(22) Sales and installation of floor coverings and materials provided there is no outside storage of materials or products. (00-38)

(23)  Churches (01-7)

(24)  Kayak sales or rental, provided that outdoor display must be completely screened from the public’s view form public and private right-of-way or from the ground level of adjacent properties according to standards in Section 20-510.  (01-13)

(25) Kayak tours, provided that they comply with Section 20-161 (c) (8). (01-13)

(26) Water tank murals.  Water tank murals shall be a conditional use upon the following terms and conditions as well as such other reasonable terms and conditions as the town council may require: (01-31)

a. The water tank must be owned or leased by Dare County and used as part of the Dare County water distribution system;

b. The murals cannot contain words, numbers, and symbols, or commercial, religious, or political themes or content;

c. The murals must not be visible from US 158 right of way or town street rights of way (except for that portion of Sibbern Street west of Croatan Highway); (02-05)

d. A pictorial representation of the murals must be approved by the town planning board and the town council;

e. The murals cannot be altered, changed, modified or repainted without the prior approval of the town planning board and town council.  Any such alterations, changes, modifications or repainting of the mural must be in accordance with the approval given by the town. (01-31)

(27) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(28) Outside storage associated with Furniture stores limited the following: (03-02)

a. All outdoor storage be contained in the maximum of 4% of the site.

b. furniture must be outdoor in type

c. fenced to shield from public view

d. must meet all fire code requirements

e. shall not be located on approved drainfield area.

f. storage area shall not increase the impervious surface.

g. not accessible to the public.

(29) Indoor ramp parks provided the following conditions are met: (03-05)

a. Supervision in the ramp park area shall be provided during all operating hours.

b. The ramp park area shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary. If the ramp park is located in a structure which houses other nonrelated businesses, the noise within the ramp park must not be audible beyond the boundaries of the area leased or owned by the ramp park operator. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will comply with these standards.

c. The maximum number of occupants shall be determined by the building inspector according to the standards of the North Carolina State Fire Code. The maximum number of occupants allowed in the ramp park shall be posted in a conspicuous place therein and its limits shall not be exceeded.

e. No alcoholic beverages shall be sold in the ramp park and no alcoholic beverages shall be consumed therein.

f. It shall not be permissible to stimulate play by gambling incentives such as (but not limited to) free plays or games, monetary rewards or the exchange of coupons redeemable for play or gifts.

g. The parking requirement shall be calculated based on  “retail uses not otherwise listed.”    Parking for food service shall be calculated on “eating establishment.”  Total parking shall be based on the combination of all uses.

h. The owner/ lessee shall execute a “hold harmless” agreement with the town for any accidents or injuries at the ramp park.

i. There will be NO skate boarding outside the building

j. There will be no open air facility.  Any open windows or door will constitute a violation of the noise requirements.

(30) Bicycle rental and repair shop, subject to subject to reasonable conditions established by the Town Council and the following conditions: (03-18)

- No outdoor storage of bicycles and other bicycle accessories.

- No self-propelled or motorized vehicle (i.e. Mopeds) rentals.

- All repairs must be made in the interior of the approved building.

(31) Fitness Center, provided: (03-53)

a. No promotional sales or services shall be visible outside of the building.

b. The building shall be designed so that any noise will be contained within the building.

c. All lighting shall meet Sec 20-414 of the code.

d. Any outdoor uses or facilities shall be setback or buffered so that no noise will go beyond the property.

e. Massage personnel shall be licensed by the state of North Carolina.

(d) Dimensional requirements.

(1) Commercial lots shall be sufficient size to meet requirements of the county health department, to provide adequate siting for structures, and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter; however all commercial lots shall have a minimum lot size of 15,000 square feet.(98-3) In addition, a visual buffer is required where a commercial use or zone abuts a residential district,  in accordance with Article VI, Division 5, Section 20-510. (95-18)

Maximum lot size:  A commercial lot or parcel submitted for review and approval in the BC-1 district shall not exceed four (4) acres in area and no one use of an owner or tenant on the site shall exceed a maximum of twenty five thousand (25,000) square feet of building areas as shown on the commercial site plan. (00-39)

(2) The minimum front yard is fifteen (15) feet.

(3) The minimum side yard is ten (10) feet. No side yard required if commercial building constructed with a common wall. An additional five-foot side yard adjacent to the street is required for a corner lot.

(4) The minimum rear yard is twenty (20) feet.

(5) The maximum allowable lot coverage by principal use and all accessory structures is sixty (60) percent.

(6) Maximum total height shall not exceed thirty-five (35) feet exclusive of chimneys, flagpoles, communication masts and aerials. (01-10) (04-26)

(7) No building or other facility (such as parking areas, trash collection areas, etc.) shall be located nearer than fifty (50) feet to the boundaries of any residential district. The width of a driveway or a road and its right-of-way along such boundary may be included as part, or all, of the fifty-foot separation zone, and subsurface wastewater disposal systems and stormwater runoff collection and disposal systems may be located within the separation zone. The planning board may require vegetation to be planted and maintained in the fifty-foot separation zone. (95-18)

(8) The minimum of 35% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Commercial site plan review. Any person desiring to construct or enlarge a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this article.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain site plan approval from the planning board and an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit except as provided in this section.

(g) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(h) Rental unit nonconforming use and density grandfathering. Any motel, hotel, inn or cottage court in the BC or the BH zones may rebuild structures containing one (1) or more rental units and/or freestanding structures used for rental purposes which were in existence and so used on August 18, 1986; notwithstanding such reconstruction would not comply with the density standards of this chapter. Any expansion of the business by the addition of more rental structures shall comply with all the nonuse standards and density standards of this chapter.

(Ord. of 8-18-86, § 3.01(D); Ord. No. 87-3, 1-19-87; Ord. No. 87-4, 1-19-87; Ord. No. 89-20, 8-21-89; Ord. No. 89-21, 8-21-89; Ord. No. 89-32, 12-4-89; Ord. No. 89-35, 12-18-89; Ord. No. 90-21, 9-4-90; Ord. No. 90-18, 12-3-90; Ord. No. 91-12, § 1, 7-15-91; Ord. No. 91-13, § 3, 10-7-91; Ord. No. 91-18, 12-10-91; Ord. No. 92-1, 3-23-92; Ord. No. 92-8, 7-13-92; Ord. No. 92-12, 8-3-92: Ord. No. 93-1, 1-4-93; Ord. No. 93-2, 1-4-93; Ord. No. 94-8, 6-6-94; Ord. No. 94-16, 9-6-94; Ord. No. 94-17, 9-6-94; Ord. No. 95-18, 9-5-95; Ord. No. 96-22, 10-7-96)

### Sec. 20-146.	BC-2 beach commercial district.

(a) Scope and intent. Unless stated otherwise, this section applies to the BC-2 beach commercial district. The BC-2 district is established to provide for the development of commercial facilities in Kitty Hawk Beach to furnish a broad range of services and commodities to serve the entire community.

The Kitty Hawk beach commercial-2 district has been established to provide for the commercial needs of the town and the immediate surrounding geographical area.  The commercial development with the beach commercial-2 district will be characterized by medium size to large size land parcels with commercial development of medium intensity.  Commercial centers maybe be authorized in this district, but large shopping centers and shopping malls, and “Big Box” retail/wholesale businesses exceed the scale of development planned for this district. (00-41)

(b) Permitted uses. The following uses shall be permitted by right:

(1) All permitted uses allowed within the BC-1 district.

(2) Home center sales, provided there is no outdoor display or merchandise.

(3) Cabinet and woodworking shops.

(4) Town owned or leased facilities.

(5) Marinas.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Single-family dwellings. A single family dwelling must comply with the dimensional requirements of the BR-1 district. (99-3)

(2) Gas stations or fuel dispensing accessory use facilities, provided that no principal or accessory building shall be located within fifty (50) feet of a residential district and provided that there shall be no storage of wrecked or abandoned cars, and that no portion of a gas station building, equipment or gas pumps shall be nearer than seventy-five (75) feet to any right-of-way. A gas station or fuel dispensing accessory use facility shall be designed and equipped such that no more than eight (8) vehicles can simultaneously receive fuel from the fuel dispensing devices located on the gas station or fuel dispensing accessory use facility site. (94-16)

(3) Motor vehicle service and repair, provided that there shall be no storage of wrecked, junked or abandoned vehicles and that there shall be no fuel dispensing devices.

(4) Conveyor car wash service station is a conditional use upon the following conditions and such other conditions as the town council may reasonably set forth:

a. A pre-wash vehicular lane of sufficient size to accommodate twelve (12) vehicles during the pre-wash phase of the wash operation.

b. A post-wash vehicular lane of sufficient size to accommodate four (4) vehicles being towel dried. The exit from the conveyor belt system shall be the entrance to the post-wash vehicle drying area.

c. The pre- and post-wash vehicle areas must be located separate from and outside of internal traffic circulation lanes and ingress-egress access points with public or private rights-of-way.

d. At the location of the vehicle exit from the building there shall be a water catchment at the post-wash vehicle parking area which shall drain and slope to the water catchment. The water catchment must be large enough to contain all water draining into it from washed vehicles.

e. The water used for washing the vehicles must be recycled on-site through a closed recycle system. The system must be designed, constructed and maintained in order to prevent wash water from entering the subsurface ground area or from flowing upon the ground surface.

f. On-site directional signs must not be illuminated internally. Sign lighting on the site must not interfere with the vision of motor vehicle operators on-site or off-site.

g. Each site shall have restroom facilities open and available to its customers.

h. Trash containers must be located at all vacuum cleaner stations and the area maintained free of trash and rubbish.

i. At least one (1) attendant shall be on duty during the hours of operation.

(5) Public utility facilities.

(6) Seafood markets.

(7) Hospitals, clinics and medical offices.

(8) Emergency heliport. The operation of a medical emergency evacuation heliport shall be an accessory use to the primary hospital or medical center conditional use and shall be authorized in addition to the primary hospital or medical center conditional use as a conditional accessory use after the following requirements have been satisfied:

a. Hospitals and medical clinics shall include those facilities which, under normal operating conditions, receive and treat trauma patients.

b. Compliance with and evidence of compliance with any applicable requirements of the department of transportation, division of aviation, the Federal Aviation Administration, and any other federal, state or local agency having [jurisdiction over] the operation of helicopter flights, landings and the environmental impact thereof.

c. Submission of a conditional accessory use application with the town planning department in addition to the following materials:

1. A site plan and map drawn to a scale of 1:100 showing the locations, height and first floor elevations or foundation elevations above mean sea level of all structures, utility and street rights-of-way, existing power lines, towers, undeveloped residential lots, and other similar uses within five hundred (500) feet of the outside edge of the heliport landing pad.

2. Environmental assessment consisting of (i) an analysis of the impact of noise within five hundred (500) feet of the heliport performed by a qualified audio engineer or other appropriate professional; (ii) an analysis of the socioeconomic issues relating to the heliport and the hospital or the clinic; and (iii) viable alternatives. The fees and costs associated with the review of the environmental assessment shall be paid to the town by the applicant prior to the public hearing as part of the application review fee. The analysis is to be submitted prior to the first regular or special meeting of the planning board at which the application is considered.

3. Proposed heliport lighting plan demonstrating the technology and technique for retaining light on the site and prevention of light or light glare from affecting traffic using streets and highways in the area.

4. Proposed approach and departing flight paths shall be shown on the map required in subparagraph 1. above.

5. Proposed protection for the innermost portion of the approach and departure routes.

d. Upon filing the application and all required materials with the town planner, the town council shall set a public hearing on the applicant's request for the conditional accessory use of the medical emergency evacuation heliport. In addition to advertising the public hearing, the town planner shall mail, by certified mail, copies of the notice of hearing to all property owners on the tax records of the town within five hundred (500) feet of the heliport landing pad. The cost of advertising and mailing of the notices shall be borne by the applicant and paid prior to the public hearing date.

e. In the case of a protest against the medical emergency evacuation heliport by the owners of twenty (20) percent or more either of the area of the lots included within five hundred (500) feet of the heliport pad, or of those immediately adjacent thereto either in the rear thereof or on either side thereof or extending five hundred (500) feet therefrom, or of those directly opposite thereto extending five hundred (500) feet from the street frontage of the opposite lots, then a motion to allow the conditional accessory use shall not become effective except by favorable vote of four (4) of the five (5) members of the town council.

f. No protest against the applicant's proposed conditional accessory use shall be valid or effective for the purposes of the above paragraph unless it be in the form of a written petition actually bearing the signatures of the requisite number of property owners and stating that the signers do protest the proposed conditional accessory use and unless it shall have been received by the city clerk within ten (10) days following the public hearing. For the purposes of determining the ten-day period, the date of the public hearing shall be excluded and the tenth day shall be included. Thereafter the town shall determine the sufficiency and accuracy of the petition within two (2) normal work days, excluding Saturdays, Sundays and legal holidays, after the end of the ten-day period for filing protest petitions. The protest petition shall be in a form adopted by the town council. In the event no protest is filed or the protest is inadequate, the favorable vote of three (3) of the five (5) town council members shall be required in order to authorize the conditional accessory use.

g. The minimum lot or parcel size to accommodate a hospital or medical center with a heliport facility shall be three (3) acres or more.

h. The heliport shall be ground based only. No rooftop facility shall be permitted.

i. The heliport shall comply with the latest edition of the FAA regulations in its design, size and use.

j. The heliport shall not be used until FAA approval is obtained and a copy of the approval is submitted to the town.

k. Prior to submitting the FAA permit application for the heliport, a copy of the application shall be forwarded to the town. The town's review of the copy of the application is for informational purposes and not intended as an approval requirement.

l. No fixed base operations or refueling facilities shall be permitted on the hospital-medical center or heliport site.

m. Lighting of the helipad or final approach and takeoff areas shall comply with FAA regulations but shall be ground level based using cutoff or restrictive features to minimize overspill of light from the activity area itself.

n. No takeoff and landing area shall be nearer than two hundred (200) feet at its closest point to a state or federal highway. The entire takeoff and landing area, as defined by the FAA and determined by the size of the helicopter, shall fall within the property lines of the lot. The developer of a heliport shall be required to place signs warning motorists of "Low-Flying Aircraft." These signs shall be placed in accordance with regulations of the state department of transportation.

o. Fire protection and safety measures shall be in accordance with the FAA and NFPA guidelines. The presence of firefighting equipment shall not be required by the town unless requested by the owner. The cost of providing such firefighting equipment coverage shall be the responsibility of the owner.

p. The use of sound buffers, proper facility siting, separation distances or other natural or manmade barriers shall be made a part of the heliport approval process.

q. A protection plan for the innermost portion of the approach and departure routes shall be developed. Protection means may consist of, but not be limited to, acquiring title to the land or leasing the land area, acquisition of air rights, or the written consent of property owners within the approach and departure routes, or other means approved by the town council. The plan shall include an evaluation of viable alternatives. The method of such protection must be approved by the council and consummated by the applicant. The council, acting in its sole discretion, may waive this requirement if the procedure cannot be completed within a reasonable period of time or for reasons beyond the control of the applicant.

(9) Veterinary hospitals and clinics subject to other requirements of this chapter and provided the following conditions are met:

a. No animals shall be kept or boarded outside the principal building and there shall be no structure, runs, or pens used for boarding, holding or restraining animals located upon the site outside of the principal building.

b. The principal building shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will allow no animal noise originating within the building to be heard at any point on the approved site boundary.

c. All animal waste must be disposed of through the facility septic system and the applicant must submit evidence that the appropriate county or state department has approved the proposed septic systems for the disposal of animal waste.

d. No dead animal shall be placed in any outside receptacle located on the premises outside the facility.

e. A vegetated or constructed visual buffer may be required.

f. Exterior lighting shall be of low intensity and shall reflect upon the site in such a manner as not to interfere with traffic on public streets or highways.

(10) Indoor recreation activities.

(11) Apartments on the second story of compatible commercial uses, provided that applicable state building and fire codes are met and that one (1) parking space per bedroom be reserved for the occupants of the apartment in addition to any other parking spaces that may be required.

(12) Day care centers.

(13) Boat sales and rentals, provided that any outdoor display must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510. (01-12)

(14) On-site retail sales and production of bakery goods, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of bakery goods must be incidental to and for the purpose of on-site retail sales and not for distribution to wholesale vendors.

b. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the bakery production activity.

c. The owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(15) Craft production and retail sales, subject to any reasonable conditions imposed by the town council and the following conditions:

a. The production of crafts must be incidental to and for the purpose of retail sales and not for distribution to wholesale vendors.

b. The maximum square footage of the area devoted to craft production shall not exceed eight hundred (800) square feet.

c. The building in which crafts are produced shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof used for craft production) if the same is located within a multiunit structure. An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the craft production activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for craft production if the same is located within a multiunit structure).

d. All odors and vapors (including chemical vapors) associated with or generated by the production of crafts shall be contained within the boundaries of the site or within the building (or that portion of the building used for the production of crafts in a multiunit structure). When requested by the planning board, an engineer's certification shall be furnished to the planning board that the design, construction, and proposed methods of odor and vapor control are sufficient to contain all odors and vapors generated by or emanating from the production of crafts within the site boundary (if a freestanding building) or within the interior of that portion of the building used for the production of crafts in a multiunit structure.

e. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the craft production activity.

f. If flammable materials are used in the craft production activity, the owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(16) A print shop may be permitted subject to other requirements of this chapter and provided the following conditions are met:

a. No more than one thousand (1,000) square feet may be used for printing purposes, excluding office area.

b. The principal building shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will allow no machine noise originating within the building to be heard at any point on the approved site boundary.

c. In the event any chemicals used or stored on the premises require approval by a governmental agency of the method such chemicals are stored, used or disposed, then written evidence of the approval by the appropriate governmental agency shall be furnished to the town.

d. A list of all chemicals used or stored on the site shall be filed on or before January 10 of each year with the volunteer fire department.

(17) Building contractor's offices and storage area, provided that any outdoor storage area must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section     .

(18) Building supply and equipment sales, provided that any outdoor storage area must be completely screened from the public's view from public rights-of-way or the ground level of adjacent properties according to standards in section     .

(19) Plumbing supply and equipment sales, provided that any outdoor storage area must be completely screened from the public's view from public rights-of-way or the ground level of adjacent properties according to standards in section     .

(20) Miniature golf course associated with PCD. A miniature golf course is a course containing all or a significant number of elements of a regulation golf course consisting of some combination of tees, fairways, greens, sand traps, water and vegetation hazards, and varying topography but all of which have been reduced in size to accommodate the project site and designed for the sport of golf. Putt-Putt type golf courses and animated courses are expressly excluded. Miniature golf courses, where authorized in various districts under this chapter, shall meet the following conditions and requirements:

a. The miniature golf course and support facilities must be located within the boundary of the PCD project.

b. The miniature golf course must be compatible with the land uses in the PCD project.

c. The miniature golf course site shall consist of at least thirty thousand (30,000) square feet of land area for an eighteen-hole course with parking and golf course facilities. The total number of golf course holes shall not exceed thirty-six (36) holes of play. In addition, a separate practice putting green may be located within the miniature golf course site boundary.

d. To the extent possible, the course design and layout shall utilize the natural terrain of the site. Alteration of the existing natural topography and the creation of other natural topographical features such as hills, mounds and waterfalls shall require the approval of the planning board and town council. There shall be no animation on or around the course and within the golf course site. The golf course site and course shall utilize only water, earth and natural vegetation as hazards, decorative features and other play-related features and shall not use artificial devices including, but not limited to, miniature buildings, animal, human or vehicular models, signs (other than hole number signs) or billboards. A site plan, including the location and identification of trees larger than six (6) inches in diameter shall be required for miniature golf courses. The PCD submission and review procedure shall apply to the planning review of the golf course site.

f. The course shall be buffered from adjacent parcels outside the PCD by a vegetated buffer; however, it is permissible for the course to be visible from U.S. Highway 158 if the PCD boundary is contiguous to the right-of-way of U.S. Highway 158. The buffer shall be a vegetated buffer with plants, shrubs and trees approved for use by the planning board. The vegetated buffer shall be maintained by the manager or association responsible for maintenance of the planned commercial development.

g. There shall be no neon or flashing lights in the miniature golf course site boundary. Lighting of the course shall be contained on the site and shall not interfere with the reasonable enjoyment of adjacent parcels or traffic on streets or highways.

h. The electrical transmission of sound shall be prohibited.

i. Alcoholic beverages may not be sold or consumed in the miniature golf course area.

j. Restroom facilities shall be located within the golf course area for use by the players.

k. The golf course design shall accommodate stormwater retention and shall comply with the guidelines for soil erosion and sedimentation control.

l. There shall be one (1) parking space for each numbered hole plus one (1) parking space for each employee working in the golf course site. All parking spaces shall be located within four hundred (400) feet of the golf course.

m. Trash receptacles shall be centrally located and maintained within the golf course site.

(21) Cater service (3-38)

(22) Natural grass putting golf course. Definition. A natural grass putting golf course is a course containing all or a significant number of elements of a regulation golf course consisting of some combination of tees, fairways, greens, sand traps, water and vegetation hazards and varying topography, but all of which have been reduced in size to accommodate the project site and designed for the sport of golf and limiting play to the use of a putter. Miniature golf courses utilizing artificial playing surfaces and/or with animated features within the golf course site are expressly excluded. All playing surfaces must be natural grass. Playing surfaces may be connected by wooden walkways not to exceed three (3) feet in width and one (1) foot in height. In addition to any other reasonable requirements or conditions imposed by the town council, a natural grass putting golf course shall satisfy the following conditions and requirements:

a. The natural grass putting golf course site shall consist of an area that is at least one (1) acre in size for each eighteen (18) holes of play. The parking area for the natural grass putting golf course site shall not be located within the boundaries of the natural grass putting golf course site, and the parking area cannot be included in the calculation of the required one (1) acre of land area for each eighteen (18) holes of golf. An accessory structure used as a pro shop may be located within the natural grass putting golf course site. In no event shall the total number of golf holes exceed thirty-six (36); however, a separate practice green may be included within the natural grass putting course site in addition to the maximum of thirty-six (36) holes of golf.

b. The natural grass putting golf course shall be designed to utilize the natural terrain of the site as much as possible. The golf course shall contain and consist of only natural materials. A certification must be submitted by the property owner with the site plans stating that there will be no change in natural terrain exceeding two (2) feet in elevation or two (2) feet in excavation of the site. If the existing terrain will be altered by more than a two-foot increase or decrease in elevation, then a topographic plan drawn with elevations at two-foot intervals must be submitted with the site plan.

c. There shall be no artificial moving objects or animated natural objects on or around the course and within the natural grass putting golf course site. Only water, earth and natural vegetation shall be used as hazards or decorative features. Artificial devices including, but not limited to, miniature buildings, animal, human or vehicular models, manmade waterfalls, signs (other than hole number signs) or billboards are prohibited within the golf course site. The site plan shall show the location of and identify trees larger than six (6) inches in diameter within the project site. To the extent possible, trees with diameters larger than six (6) inches shall be preserved.

d. The natural grass course shall include a buffer along its common boundary with adjoining properties. The buffer shall consist of an earthen mound at least ten (10) feet wide at its base and six (6) feet high. Natural vegetation shall be planted on the mound to function as a visual screen. The type of vegetation planted on the buffer shall be approved by the planning board. The screen shall be maintained by the owner or operator of the course. In the absence of a buffer and vegetative screen, there shall be an open space buffer fifty (50) feet in width along the common boundary with adjoining properties and lying within the project site. No golf course playing area shall be located closer than two hundred (200) feet to the right-of-way of any state road or federal highway.

In order to safeguard the public health, safety and welfare, a sufficient vegetative screen shall be provided on property lines adjacent to such state or federal highways. The overall height of the screen shall be at least twelve (12) feet. It shall be opaque to a minimum height of six (6) feet. Intermittent visual obstruction shall be permitted from six (6) feet to a height of at least twelve (12) feet. The opaque portion of the screen is intended to exclude visual contact during all seasons. The intermittent portion shall not contain any unobstructed opening more than fifteen (15) feet wide except at approved access points. The screen shall be composed of a landscaped earthen berm, planted vegetation or existing vegetation. The following planting materials list defines acceptable species which may constitute the screen. A plan showing details of a proposed screen shall be developed and submitted as part of the site plan review procedure. The plan shall be judged sufficient if it satisfies the standards and species defined in this subsection.

Planting Materials

Trees--10' to 40': Black Pine (Pinus thunbergi), Brazilian Butia (Butia capitata), Cabbage Palmetto (Sabal palmetto)*, Cedar (Cedrus deodora), Cluster Pine (Pinus pinaster), Crepe Myrtle (Lagerstroemia indica), Eastern Red Cedar (Juniperus virginiana), Live Oak (Quercus virginiana), Redbay (Perseaborbonia), White Poplar (Populus alba). *Requires special care in colder areas.

Opaque screen--Ground to 6': Bayberry (Myrica pensylvanica), European Privet (Ligustrum vulgare), Japanese Privet (Ligustrum japonicum), Oleander (Nerium oleander), Pampas Grass (Cortaderia selloana et. ac.), Pittosporum (Pittosporum tobira), Russian Olive (Elaeagnus pumgens), Saltcedar (Tamarix gallica), Wax Myrtle (Myrica cerifera), Yaupon Holly (Ilex vomitoria).

e. Lighting within the natural grass putting golf course site boundary shall be shielded and focused on the course in such a manner as not to interfere with the reasonable enjoyment of adjacent parcels of property or traffic on nearby streets or highways. The light shall be contained on the site. Light fixtures shall not exceed ten (10) feet in height. There shall be no neon or flashing lights within the natural grass putting golf course site boundary. A light plan shall be submitted to the planning board as part of the site plan review. The light plan shall demonstrate compliance with the light standards set forth herein.

f. The transmission of voice or musical sound within the natural grass putting golf course site boundary shall be prohibited. Malt beverages, wine or other beverages containing alcohol shall not be sold or consumed within the boundaries of the natural grass putting golf course. Adequate restroom facilities for the use of golf course players and customers shall be located within the golf course area. Trash receptacles approved by the town and/or county shall be located and maintained on the site. The site plan submitted to the planning board shall contain information and design elements demonstrating that stormwater runoff will be managed in a manner which contains the runoff within the boundaries of the site and otherwise complies with the applicable provisions of the soil erosion and sedimentation control ordinance.

g. There shall be a minimum of two (2) parking spaces per hole of golf play and one (1) parking space for each golf course employee at the site. If the natural grass putting golf course is planned and included as part of a restaurant mixed-use development, then the minimum parking requirement may be reduced to one and one-half (11/2) spaces per golf hole within the site and one (1) space for each employee at the site.

Accessory use:

1. A food preparation center that sells snacks and beverages to the customers and /or food preparation center that prepares “carry out food” to be consumed on site outside of the building or to be delivered off site may be included as an accessory use within the natural grass putting golf course site. (97-12)

2. Restaurant with not more than ten (10) customer seats. (00-23)

(23) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (24) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (24) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(24) Motor Vehicle Dealership:  A motor vehicle dealership may be authorized subject to any reasonable conditions and standards imposed by the town council, the town code and the following conditions:   (00-17)

- The project site must be located and the use conducted west of U.S. Highway 158 within the town;

- The project site must contain a minimum of six (6) acres;

- The project site must have minimum road frontage on U.S. Highway158 of one hundred (100) feet; and a minimum road frontage on a town street intersecting with US 158 of one hundred (100) feet.

- Buildings shall be set back a minimum of seventy five (75) feet from adjoining residential uses or zoning district boundaries;

- Parking spaces and vehicles for sale or display to customers must be set back a minimum of fifty (50) feet from adjoining residential uses or residential zoning boundaries, and a buffer must be installed within those setbacks in accordance with the standards set forth in the town code;

- A building set back of not less than sixty (60) feet shall be maintained from the center line of any town street which intersects with U.S. Highway 158 and is contiguous to the project site;

- A minimum building setback from the right of way margin of U.S. Highway 158 of one hundred seventy five (175) feet;

- Curb cuts, entrances or exit points from the project site to U.S. Highway 158 shall be prohibited.

- One or more curb cuts or driveway access locations may be authorized by the town where the point of access, ingress and egress is located a minimum of one hundred (100) feet from U.S. Highway 158;

- Upon recommendation by the planning board, a traffic study shall be conducted under the supervision of the planning board and at the expense of the applicant, and the applicant will install any traffic control measures recommended by the traffic study at the intersection of U.S. Highway 158 by the town street which is contiguous to the project site;

- No motor vehicles shall be placed, parked or displayed within, the town rights of way or the right of way of U.S. Highway 158. (00-17)

(25) Sales& installation of floor coverings and materials provided there is no outside storage of materials and products.  (00-42)

(26) Churches (01-8)

(27) Kayak sales and rental, provided that outdoor display must be completely screened from the public’s view from public and private right-of-way or from the ground level of adjacent properties according to standards in Section 20-510. (01-14)

(28)  Kayak tours, provide that they comply with Section 20-161 (c) (8). (01-14)

(29) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(30) Outside storage associated with Furniture stores limited the following: (03-02)

a. All outdoor storage be contained in the maximum of 4% of the site.

b. furniture must be outdoor in type

c. fenced to shield from public view

d. must meet all fire code requirements

e. shall not be located on approved drainfield area.

f. storage area shall not increase the impervious surface.

g. not accessible to the public.

(31) Fitness Center, provided: (03-53)

a. No promotional sales or services shall be visible outside of the building.

b. The building shall be designed so that any noise will be contained within the building.

c. All lighting shall meet Sec 20-414 of the code.

d. Any outdoor uses or facilities shall be setback or buffered so that no noise will go beyond the property.

e. Massage personnel shall be licensed by the state of North Carolina.

(32) The retail sale of fruits, vegetables, flowers, containerized house or bedding plants, herbs and other agricultural food products, such as jam, jelly, vinegar, eggs, cheese or honey subject to the following terms and conditions: (05-03)

- No produce, plants or related products shall be located within the Town or State right-of way or in designated parking spaces or fire lanes;

- The activity may not utilize on-site required parking spaces to conduct retail activities or storage;

- No sales shall be conducted after 10:00 p.m. and prior to 7:00 a.m.;

- No music shall be produced on the site which music is audible beyond the property lines of the lot on which the sales are being conducted;

- Any signs must meet current Town code standards and applicable sign permit requirements;

- No additional lighting shall be allowed on the site;

- Trailers or vehicles used to transport produce and materials to the site shall be concealed from observation from Town streets and State rights-of-way;

- Prior to issuance of a conditional use permit the applicant shall present a scaled site plan illustrating the location of all vendor display and storage area, pedestrian circulation areas and ways, and location of required parking;

- Any structures must meet wind construction standards and tie down standards per North Carolina Building Code;

- One (1) paved off street parking stall shall be required for each 300 square feet of occupied retail space. For purposes of this section, retail space shall be that area occupied for the display, sale or storage of goods;

- The required parking shall not be allowed in public or private rights-of-way, fire lanes or travel lanes designated on approved site plan;

- Any tent occupying over 200 square feet of the site shall require approval from the Fire Chief;

- Sanitary facilities shall be required in manner consistent with the International Building Code for a similarly sized retail space and shall be located on the subject property and within 200 feet of the market;

- On-site garbage or trash receptacles must be provided and properly maintained at all times.

(d) Dimensional requirements.

(1) Commercial lots shall be sufficient size to meet requirements of the county health department, to provide adequate siting for structures, and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter; however all commercial lots shall have a minimum lot size of 15,000 square feet.(98-3) In addition, a visual buffer is required where a commercial use or zone abuts a residential district,  in accordance with Article VI, Division 5, Section 20-510. (95-18)

Maximum lot size:  A commercial lot or parcel submitted for review and approval in the BC-2 district shall not exceed seven (7) acres in area and no one use of an owner or tenant on the site shall exceed a maximum of forty thousand (40,000) square feet of building areas as shown on the commercial site plan. (00-43)

(2) The minimum front yard is fifteen (15) feet.

(3) The minimum side yard is ten (10) feet. No side yard required if commercial building constructed with a common wall. An additional five-foot side yard adjacent to the street is required for a corner lot.

(4) The minimum rear yard is twenty (20) feet.

(5) The maximum allowable lot coverage by principal use and all accessory structures is sixty (60) percent.

(6) Maximum total height shall not exceed thirty-five (35) feet exclusive of chimneys, flagpoles, communication masts and aerials. (01-10) (04-26)

(7) No building or other facility (such as parking areas, trash collection areas, etc.) shall be located nearer than fifty (50) feet to the boundaries of any residential district. The width of a driveway or a road and its right-of-way along such boundary may be included as part, or all, of the fifty-foot separation zone, and subsurface wastewater disposal systems and stormwater runoff collection and disposal systems may be located within the separation zone. The planning board may require vegetation to be planted and maintained in the fifty-foot separation zone. (95-18)

(8) The minimum of 35% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Commercial site plan review. Any person desiring to construct a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this chapter.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit.

(g) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(h) Rental unit nonconforming use and density grandfathering. Any motel, hotel, inn or cottage court in BC or BH zones may rebuild structures containing one (1) or more rental units and/or freestanding structures used for rental purposes which were in existence and so used on August 18, 1986; notwithstanding such reconstruction would not comply with the nonuse and density standards of this chapter. Any expansion of the business by the addition of more rental structures shall comply with all the nonuse standards and density standards of this chapter.

(Ord. of 8-18-86, § 3.01(E); Ord. No. 87-1, 1-19-87; Ord. No. 87-3, 1-19-87; Ord. No. 87-4, 1-19-87; Ord. No. 89-20, 8-21-89; Ord. No. 89-32, 12-4-89; Ord. No. 90-3, 3-19-90; Ord. No. 90-21, 9-4-90; Ord. No. 90-19, 12-3-90; Ord. No. 91-12, § 2, 7-15-91; Ord. No. 91-15, § 3, 10-21-91; Ord. No. 92-1, 3-23-92; Ord. No. 92-8, 7-13-92; Ord. No. 92-13, 8-3-92: Ord. No. 93-1, 1-4-93; Ord. No. 93-2, 1-4-93; Ord. No. 94-16, 9-6-94; Ord. No. 95-18, 9-5-95)

Sec. 20-147. BC-3 community shopping mall district; community shopping center district.

(a) Intent. The BC-3 district is established to provide for the proper grouping and development of commercial facilities and to provide a means for protecting the public from dangerous arrangement of vehicle and pedestrian ways within shopping malls and shopping centers.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Community mall consisting of any of, or any combination of, the following:

a. Any of the following offices including such uses as:

1. Business.

2. Financial.

3. Governmental.

4. Medical and professional.

b. Any of the following primary retail stores including such uses as:

1. Books.

2. Cameras.

3. Candy.

4. Clothing.

5. Craft goods.

6. Dry goods.

7. Drugs.

8. Flowers.

9. Gifts.

10. Hardware.

11. Jewelry.

12. Leather goods.

13. Magazines.

14. Musical instruments.

15. Notions.

16. Sporting goods.

17. Toys.

18. Food stores and supermarkets.

19. Household appliances.

20. Furniture and furnishings.

21. Card stores.

c. Department stores.

d. Service establishments such as:

1. Beauty and barber shops.

2. Cafeterias.

3. Dry cleaning and laundry pickup stations.

4. Restaurants.

5. Shoe repairs.

6. Theaters.

7. Health and physical fitness centers.

e. Town, county, state, or federal government owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Public utility facilities.

(2) Seafood market.

(3) Medical centers or clinics.

(4) Optometrist facilities.

(5) Banks.

(6) Childcare facilities.

(7) Broadcasting facilities excluding antenna tower structures.

(8) Bakery retail and bakery products produced to be sold on premises.

(9) Chiropractor offices.

(10) Gas station or fuel dispensing accessory use facility, provided that there shall be no storage of wrecked or abandoned cars and that no portion of a gas station building, equipment or gas pumps shall be nearer than seventy-five (75) feet to any right-of-way. A gas station or fuel dispensing accessory use facility shall be designed and equipped such that no more than eight (8) vehicles can simultaneously receive fuel from the fuel dispensing devices located on the gas station or fuel dispensing accessory use facility site. (94-16)

(11) Cater service (3-38)

(12) Sport facilities, real estate (time share) sales, community association facilities, and building maintenance facilities pursuant to the application and approval procedures of the planned commercial development regulations. (94-7)

(13) In a PCD overlay district, the permitted and conditional uses under the PCD overlay shall be established by the town council as a condition of the PCD overlay approval and shall be set forth in the special use permit as amended by the council from time to time. (94-7)

(14) Home Improvement Warehouse as a PCD overlay zone.  (00-44)

(15) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(16) Churches (04-27)

(d) Prohibited uses. Residential uses, whether permanent or transitory, shall not be permitted in the BC-3 district.

(e) Standards and requirements.

(1) Community mall standard requirements. The following provisions apply to community malls:

a. Minimum lot size: A shopping mall shall be located on a parcel having an area of at least thirty (30) acres. A shopping mall site shall not lie on two (2) sides of a public or private street.

b. Minimum front yard: Each mall shall have a building setback from all public or private street rights-of-way of at least one hundred (100) feet. A strip at least fifty (50) feet deep shall be maintained as a buffer planted with shrubs and trees as well as grass and/or a ground cover along the frontage. No parking or other structure shall be permitted in this buffer. A freestanding sign identifying the center may be located in the setback area. If a shopping mall has more than four hundred thousand (400,000) square feet of gross leasable area, the vegetated buffer strip shall be increased to seventy-five (75) feet.

c. Minimum side yard: Each mall parcel shall have a minimum side yard at least fifty (50) feet in width. A strip at least twenty (20) feet in width shall be maintained as a vegetated buffer.

d. Minimum rear yard: Each mall parcel shall have a minimum rear yard at least fifty (50) feet in depth. A strip at least twenty (20) feet in width shall be maintained as a vegetated buffer.

e. Other yard requirements: The front yard dimensional requirements shall apply to any mall parcel site or rear yard which adjoins a satellite business lot or is separated from a satellite business lot by a vehicular area for parking or access.

f. Maximum lot coverage: The ground area occupied by all principal buildings together with all accessory buildings shall not exceed twenty-five (25) percent of the total area of the mall or shopping center parcel. The maximum lot coverage by principal and accessory structures and other impervious surfaces including parking areas, drive aisles and loading zone accessways (but excluding the primary access to the parcel) shall be sixty (60) percent.

g. Separation from residential districts: No building or other facility (such as parking areas, trash collection areas, etc.) shall be located closer than one hundred (100) feet to the boundaries of a residential district. The width of a driveway or a road right-of-way along such boundary may be included as part of the separation zone. Vegetation shall be planted and maintained in the separation zone.

A fence or other visual barrier constructed out of manmade or natural material approved by the planning board shall be constructed and maintained upon the mall district property fifty (50) feet from any boundaries separating the mall parcel or a satellite business lot from an existing residential use or residential zoning district.

h. Height limitation. Maximum total height shall not exceed thirty-five (35) feet from existing grade (04-26) , exclusive of chimneys, flagpoles, communications masts and aerials.

i. Traffic control and traffic flow:

1. There should be one (1) primary accessway from the mall district property to a public or private right-of-way. One (1) or more secondary accessways may be permitted by the town if additional access points are recommended by the town's traffic engineer or traffic consultant in order to insure orderly and safe traffic movement within the district and between the district and adjoining areas. The recommendation of the town traffic engineer or traffic consultant must be approved by the town planning board.

2. The applicant shall install such traffic-control devices as recommended by the traffic engineer, traffic consultant or the state highway engineer both within the district and at any places where mall traffic, arteries, roads, streets or ways intersect a public or private right-of-way. The cost of installing the traffic-control devices shall be paid by the applicant. The mall shall maintain all traffic-control devices within the district which are located on the mall property.

3. There shall be no parking areas contiguous to the right-of-way margin or any area used for vehicular traffic such that a vehicle is required to back or maneuver into the mall traffic system while entering or leaving a parking space.

4. The applicant must submit a mall district internal traffic flow plan and an internal parking area traffic flow plan to the planning board for its approval.

5. Accessways shall be not less than thirty (30) feet nor more than forty (40) feet in width at their intersection with the property line. Accessways shall be installed with curbs of concrete. Curb returns shall have a minimum radius of thirty (30) feet. At its intersection with a property line, an accessway shall not be less than one hundred (100) feet from another accessway or fifty (50) feet from the corner of a property.

6. The location and design of accessways shall be such that traffic congestion is reduced as much as possible and traffic hazards are eliminated.

j. Offstreet parking.

1. Offstreet parking shall be provided according to the following requirements:

A. Not less than four (4) spaces per one thousand (1,000) square feet of gross leasable area for malls having twenty-five thousand (25,000) to four hundred thousand (400,000) square feet G.L.A.

B. Not less than five (5) spaces per one thousand (1,000) square feet of gross leasable area for malls having more than four hundred thousand (400,000) square feet G.L.A.

2. Handicapped ramps shall be provided near handicapped parking areas.

3. Each shop or store shall have access to a rear or side entrance that is within a reasonable distance and is accessible to a loading area and service drive. Service drives shall be a minimum of twenty-six (26) feet in width and shall be in addition to and not a part of the drives or circulation system used by vehicles of shoppers. Loading and delivery zones shall be clearly marked.

4. Bicycle parking devices shall be installed to provide one (1) bicycle parking space for each fifty (50) automobile parking spaces. Design of parking devices can be obtained from the bicycle program of the state department of transportation.

k. Lighting: All parking areas and accessways shall be floodlighted in accordance with the standards of the town. All outside lighting shall be arranged and shielded to prevent glare or reflection, nuisance, inconvenience or hazardous interference of any kind on adjoining lots or residential areas or area traveled by motor vehicles and shall provide security for mall customers.

l. Fire hydrants:

1. Fire hydrants shall be required at locations recommended by the Fire Chief and approved by the planning board. No portion of a building shall be farther than three hundred (300) feet nor closer than fifty (50) feet to a fire hydrant and not more than 500 feet in hose lay distance from the hydrant. Hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department. The mall shall mark and maintain fire lanes. Parking or storage of merchandise shall not be permitted in the fire lanes. (97-6)

2. Fire lanes. A paved fire lane with a width no greater than allowed by the North Carolina Fire Code and which width may be reduced by the Town Council upon recommendation of the Fire Chief shall encircle the mall buildings.  A fire lane may be required along the rear or side of a mall satellite building based upon recommendation of the fire chief and approval of the Town Council.  Code violations of this requirement shall be misdemeanor. (97-6)

m. Solid waste: Containers for garbage and refuse shall be provided in accordance with the requirements of the town and, if applicable, the county department of public works. Areas for screened dumpsters shall be provided so as to be out of the traffic flow, accessible to garbage trucks at all times and of adequate size to meet the needs of all uses at the center. Garbage pickup and commercial deliveries shall be during the hours between 7:00 a.m. and 9:00 p.m.

n. Stormwater management: It shall be required that an adequate method for managing stormwater runoff shall be developed. Whenever possible, stormwater management systems, parking layout and the location of curbs and gutters should be planned simultaneously. Any enclosed portion of a system should be designed to manage stormwater not just to dispose of it or disperse it. No stormwater runoff shall be allowed within pedestrian ways or pedestrian easement areas.

o. Public restrooms: Public restrooms shall be required on each mall level opened to the public and located such as to be convenient to the public and shall be maintained by the mall operator in a clean and sanitary condition. The location of all restrooms shall be clearly marked and convenient to mall customers.

p. Community mall signage: A uniform community mall signage criteria plan shall be submitted to the town for approval. The purpose of this plan is to define and specify all exterior signage for the community mall. This plan will set forth guidelines and design concepts for a coordinated and complementary graphic image for the entire community mall.

1. Community mall site identification sign. One (1) freestanding community mall site identification sign shall be permitted at the access or entranceway designated as the primary access or entranceway. The mall identification site sign shall not exceed sixty-four (64) square feet. The sign and the supporting structure shall not exceed twenty (20) feet in height from the undisturbed ground area.

2. Secondary accessway or entranceway mall identification signs. A secondary access or entranceway mall site identification sign may be permitted with the approval of the planning board. The secondary mall site identification sign shall not exceed ten (10) square feet, and the sign and structure supporting it shall not exceed ten (10) feet from the undisturbed ground surface.

3. Location of identification signs. Both the primary and secondary mall site identification signs must be located such as not to block or interfere with the ability of motor vehicle operators to observe the traffic while the operators are entering or exiting the mall area.

4. Lighting of mall identification signs. All mall site identification signs shall be indirectly lighted such that the light is retained on the site and does not interfere with traffic or neighboring property owners. No internal illumination shall be allowed, and no neon or animated signs shall be allowed.

5. Directory locator sign. Directory locator signs may be located at the entrances to parking lots. The businesses contained on the directory locator sign should be those businesses whose mall entrances are closest to the parking lot where the specific directory locator is located. These signs should be no larger than necessary to allow them to be read from a slowly moving vehicle. These signs shall not exceed fifteen (15) feet in height.

6. Directional signs. Directional signs for pedestrian and vehicle traffic shall be located as needed within the mall development subject to planning board approval. No directional sign shall exceed six (6) square feet in size.

7. Exterior wall signs. An exterior wall sign may be permitted on anchor stores with more than twenty-five thousand (25,000) square feet of gross leasable area. Such signs may be up to ten (10) percent of the facade occupied by the anchor store, but in no case shall the display exceed ninety (90) square feet.

q. Satellite business lots standards and requirements:

1. Dimensional standards. At least one (1) acre of gross area within the lot lines is required.

2. Signs. One (1) freestanding sign shall be permitted per lot, which sign shall not exceed thirty-two (32) square feet in area. The maximum height for both the structure and the sign shall be eight (8) feet from the undisturbed ground surface.

3. Lot coverage. The maximum lot coverage per parcel is sixty (60) percent, which shall include parking and driveway areas.

4. Number of buildings. One (1) freestanding building is allowed per lot. Except in theaters, no party or common walls are allowed.

5. Parking. Parking requirements for each use shall be those as set forth in this chapter. There shall be no shared parking with the mall parcel, except that theaters may share parking areas with the mall parcel with the approval of the planning board.

6. Architectural control and review. The exterior of all buildings and signs must be compatible with a central mall architectural theme.

7. Dumpsters and deliveries. Dumpster location and deliveries shall be approved by the planning board. Deliveries shall only occur between 7:00 a.m. and 7:00 p.m.

8. Lighting standards. All exterior lighting shall shine within the site and be directed such that it does not interfere with neighboring businesses and traffic.

9. Height limitation.  Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. (04-26)

r. Screening and buffers. A vegetated buffer of one hundred (100) feet shall extend around the outer perimeter of the mall development and upon the mall property. Fifty (50) feet in from the outer perimeter of the mall property, a visual screen or barrier of manmade or natural material approved by the planning board shall be constructed and maintained wherever residential districts or uses border the mall perimeter or boundary. The mall buffer of one hundred (100) feet shall not be used for traffic lanes or streets. It shall remain open space and shall be maintained with trees and shrubs. Buffer zones may be used for septic tank repair areas and for the location of subsurface drain lines and utilities.

s. Administrative expenses. The reasonable expenses and/or fees of any specialists or consultants required by the planning board for the purpose of reviewing the mall development plan or any portions or facets thereof shall be charged as additional review fees and paid by the property owner to the town.

(2) Community shopping center districts (CSCD).

a. Minimum lot size: A CSCD shall be located on a parcel having an area of at least twenty (20) acres. A CSCD site shall not lie on two (2) sides of a public or private street.

b. Minimum front yard: Each CSCD shall have a building setback from all public or private street rights-of-way of at least one hundred (100) feet. A strip at least thirty-five (35) feet deep shall be maintained as a buffer planted with shrubs and trees as well as grass and/or a ground cover along the frontage. A freestanding sign identifying the center may be located in the setback area.

c. Minimum side yard: Each CSCD parcel shall have a minimum side yard at least thirty-five (35) feet in width. A strip at least twenty (20) feet in width shall be maintained as a vegetated buffer.

d. Minimum rear yard: Each CSCD parcel shall have a minimum rear yard at least fifty (50) feet in depth. A strip at least twenty (20) feet in width shall be maintained as a vegetated buffer.

e. Other yard requirements: The front yard dimensional requirements shall apply to any CSCD parcel site or rear yard which adjoins a satellite business lot or is separated from a satellite business lot by a vehicular area for parking or access.

f. Maximum lot coverage: The ground area occupied by all principal buildings together with all accessory buildings shall not exceed twenty-five (25) percent of the total area of the mall or shopping center parcel. The maximum lot coverage by principal and accessory structures and other impervious surfaces including parking areas, drive aisles and loading zone accessways (but excluding the primary access to the parcel) shall be sixty (60) percent.

g. Separation from residential districts: No building or other facility (such as parking areas, trash collection areas, etc.) shall be located closer than one hundred (100) feet to the boundaries of a residential district. The width of a driveway or a road right-of-way along such boundary may be included as part of the separation zone. Vegetation shall be planted and maintained in the separation zone. A fence or other visual barrier constructed out of manmade or natural material approved by the planning board shall be constructed and maintained upon the CSCD property fifty (50) feet from any boundaries separating the CSCD parcel or a satellite business lot from an existing residential use or residential zoning district.

h. Height limitation. Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials.  Structures may include a cupola, dome, or ornamental tower provided the cupola, doeme or tower is not intended for human occupancy and does not extend more than five (5) feet above the thirty-five foot height limit.  (01-10) (04-26)  (see minutes of 9/13/04)

i. Traffic control and traffic flow:

1. There should be one (1) primary accessway from the CSCD district property to a public right-of-way. One (1) or more secondary accessways may be permitted by the town if additional access points are recommended by the town's traffic engineer or traffic consultant in order to insure orderly and safe traffic movement within the district and between the district and adjoining areas. The recommendation of the town traffic engineer or traffic consultant must be approved by the town planning board.

2. The applicant shall install such traffic-control devices as recommended by the traffic engineer, traffic consultant or the state highway engineer both within the district and at any places where CSCD traffic, arteries, roads, streets or ways intersect a public or private right-of-way. The cost of installing the traffic-control devices shall be paid by the applicant. The CSCD shall maintain all traffic-control devices within the district which are located on the CSCD property.

3. There shall be no parking areas contiguous to the right-of-way margin or any area used for vehicular traffic such that a vehicle is required to back or maneuver into the CSCD traffic system while entering or leaving a parking space.

4. The applicant must submit a CSCD internal traffic flow plan and an internal parking area traffic flow plan to the planning board for its approval.

5. Accessways shall be not less than thirty (30) feet nor more than forty (40) feet in width at their intersection with the property line. Accessways shall be installed with curbs of concrete. Curb returns shall have a minimum radius of thirty (30) feet. At its intersection with a property line, an accessway shall not be less than one hundred (100) feet from another accessway or fifty (50) feet from the corner of a property.

6. The location and design of accessways shall be such that traffic congestion is reduced as much as possible and traffic hazards are eliminated.

j. Offstreet parking:

1. Offstreet parking shall be provided according to the following requirements:

(a) Not less than four (4) spaces per one thousand (1,000) square feet of gross leasable area for CSCDs having twenty-five thousand (25,000) to four hundred thousand (400,000) square feet G.L.A.

(b) Not less than five (5) spaces per one thousand (1,000) square feet of gross leasable area for CSCDs having more than four hundred thousand (400,000) square feet G.L.A.

2. Handicapped ramps shall be provided near handicapped parking areas.

3. Each shop or store in excess of fifty (50) feet deep shall have access to a rear or side entrance that is within a reasonable distance and is accessible to a loading area and service drive. Service drives shall be a minimum of twenty-six (26) feet in width and shall be in addition to and not a part of the drives or circulation system used by vehicles of shoppers. Loading and delivery zones shall be clearly marked. Shops or stores less than fifty (50) feet deep are not required to have access to a rear or side entrance, but they must be protected by the installation of a sprinkler system design to comply with installations approved by the state insurance commission. Shops less than fifty (50) feet deep may not constitute more than thirty (30) percent of the leasable area of a CSCD.

4. Bicycle parking devices shall be installed to provide one (1) bicycle parking space for each fifty (50) automobile parking spaces. Design of parking devices can be obtained from the bicycle program of the state department of transportation.

k. Lighting: All parking areas and accessways shall be floodlighted in accordance with the standards of the town. All outside lighting shall be arranged and shielded to prevent glare or reflection, nuisance, inconvenience or hazardous interference of any kind on adjoining lots or residential areas or area traveled by motor vehicles and shall provide security for CSCD customers.

l. Fire hydrants: Fire hydrants shall be required at locations approved by the planning board and fire chief. No portion of a building shall be farther than two hundred fifty (250) feet nor closer than fifty (50) feet to a fire hydrant. Hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department. The CSCD shall mark and maintain fire lanes. Parking shall not be permitted in the fire lanes. A paved fire lane twelve (12) feet in width shall encircle the CSCD buildings. The fire lane may be permitted along the rear or side of a CSCD satellite building upon recommendation of the fire chief and approval of the planning board. Upon recommendation of the fire chief and the town planner with the approval of the planning board, other alternative means of providing emergency vehicle access may be authorized, or, upon request of the applicant, the planning board may waive the requirement of the twelve-foot paved vehicular fire lane around the buildings provided the applicant, in lieu thereof, agrees to and does install in the CSCD buildings a sprinkler system in accordance with the National Fire Protection Association's standards and the state insurance commission's standards for commercial structures. In addition, the sprinkler system shall be approved by the town building inspector and/or fire chief.

m. Solid waste: Containers for garbage and refuse shall be provided in accordance with the requirements of the town and, if applicable, the county department of public works. Areas for screened dumpsters shall be provided so as to be out of the traffic flow, accessible to garbage trucks at all times and of adequate size to meet the needs of all uses at the center. Garbage pickup and commercial deliveries shall be during the hours between 7:00 a.m. and 9:00 p.m.

n. Stormwater management: It shall be required that an adequate method for managing stormwater runoff shall be developed. Whenever possible, stormwater management systems, parking layout and the location of curbs and gutters should be planned simultaneously. Any enclosed portion of a system should be designed to manage stormwater not just to dispose of it or disperse it. No stormwater runoff shall be allowed within pedestrian ways or pedestrian easement areas.

o. Public restrooms: Any CSCD shop or store occupying more than ten thousand (10,000) square feet shall provide restroom facilities available to the general public. These restroom facilities shall be clearly marked and shall be maintained by the shop or store.

p. Shopping center signage: A uniform shopping center signage criteria plan shall be submitted to the town for approval. The purpose of this plan is to define and specify all exterior signage for the shopping center. This plan will set forth guidelines and design concepts for a coordinated and complementary graphic image for the entire shopping center.

1. Shopping center site identification sign. One (1) freestanding shopping center site identification sign shall be permitted at the access or entranceway. The shopping center identification site sign shall not exceed sixty-four (64) square feet. The sign and the supporting structure shall not exceed twenty (20) feet in height from the undisturbed ground area.

2. Secondary accessway or entranceway shopping center identification signs. A secondary access or entranceway shopping center site identification sign may be permitted with the approval of the planning board. The secondary shopping center site identification sign shall not exceed ten (10) square feet, and the sign and structure supporting it shall not exceed ten (10) feet from the undisturbed ground surface.

3. Location of identification signs. Both the primary and secondary CSCD site identification signs must be located such as not to block or interfere with the ability of motor vehicle operators to observe the traffic while the operators are entering or exiting the CSCD area.

4. Lighting of CSCD identification signs. All CSCD site identification signs shall be indirectly lighted such that the light is retained on the site and does not interfere with traffic or neighboring property owners. No internal illumination shall be allowed, and no neon or animated signs shall be allowed.

5. Directory locator sign. Directory locator signs may be located at the entrances to parking lots. The businesses contained on the directory locator sign should be those businesses whose CSCD entrances are closest to the parking lot where the specific directory locator is located. These signs should be no larger than necessary to allow them to be read from a slowly moving vehicle. These signs shall not exceed fifteen (15) feet in height.

6. Directional signs. Directional signs for pedestrian and vehicle traffic shall be located as needed within the CSCD development subject to planning board approval. No directional sign shall exceed six (6) square feet in size.

7. Exterior wall signs. Exterior wall signs may be permitted within limits of a uniform shopping center signage criteria plan. Such signs may occupy up to ten (10) percent of the display area of the facade of individual shops, but in no case shall an individual store's sign display exceed ninety (90) square feet.

q. Satellite business lots standards and requirements:

1. Dimensional standards. At least one (1) acre of gross area within the lot lines is required.

2. Signs. One (1) freestanding sign shall be permitted per lot, which sign shall not exceed thirty-two (32) square feet in area. The maximum height for both the structure and the sign shall be eight (8) feet from the undisturbed ground surface.

3. Lot coverage. The maximum lot coverage per parcel is sixty (60) percent, which shall include parking and driveway areas.

4. Number of buildings. One (1) freestanding building is allowed per lot. Except for theaters, no party or common walls are allowed.

5. Parking. Parking requirements for each use shall be those as set forth in this chapter. There shall be no shared parking with the shopping center parcel, except that theaters may share parking areas with the CSCD parcel with the approval of the planning board.

6. Architectural control and review. The exterior of all buildings and signs must be compatible with the shopping center architectural theme.

7. Dumpsters and deliveries. Dumpster location and deliveries shall be approved by the planning board. Deliveries shall only occur between 7:00 a.m. and 7:00 p.m.

8. Lighting standards. All exterior lighting shall shine within the site and be directed such that it does not interfere with neighboring businesses and traffic.

9. Minimum front yard. Each satellite business lot shall have a minimum front yard of thirty-five (35) feet. This area shall be maintained as a buffer planted with ground cover along this frontage.

10. Minimum side yard. Each lot shall have a minimum side yard of fifteen (15) feet. A strip of at least ten (10) feet shall be maintained as a vegetated buffer.

11. Minimum rear yard. Each lot shall have a minimum rear yard of fifteen (15) feet. A strip of at least ten (10) feet shall be maintained as a vegetated buffer.

12. Height limitation.  Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. (04-26)

r. Separation from residential districts: No building or other facility (such as parking areas, trash collection areas, etc.) shall be located closer than one hundred (100) feet to the boundaries of a residential district. The width of a driveway or a road right-of-way along such boundary may be included as part of the separation zone. Vegetation shall be planted and maintained in the separation zone.

s. Reserved.

t. Internal lot lines. The planning board may approve subdivision plans which do not provide for minimum yards or building setbacks along interior lot lines within the community shopping center development; however, all building setback lines, buffer and yard requirements along the perimeter or outside boundaries of the project (and boundaries in common with satellite business lots) shall meet the building setback, minimum yard and buffer requirements as required herein.

(f) Commercial site plan review. Any person desiring to construct or enlarge a structure for a commercial use shall first submit a site plan to the planning board for its review and the town council for its approval. The applicant shall provide the following information:

(1) The proposed name of the shopping mall/shopping center.

(2) Location by legal description showing boundary line and the total acreage encompassed thereby.

(3) Name(s) and address(es) of the owner(s) and the applicant if different from owner(s). Name and address of designer of the plan.

(4) Scale of plan shall be one (1) inch to one hundred (100) feet.

(5) Date.

(6) North arrow and vicinity map.

(7) Contours at two-foot intervals.

(8) Location, widths, and names of all existing or prior platted streets, utility easements and rights-of-way, buildings and structures, and municipal boundary lines within five hundred (500) feet.

(9) Existing water mains, culverts and other underground facilities within the tract, indicating pipe sizes, grades, manholes and locations.

(10) Location, arrangement and dimensions of automobile parking space, width of aisles, width of bays, angle of parking and the location of handicapped spaces.

(11) Location, arrangement and dimensions of truck loading and unloading spaces and doors.

(12) Locations and dimensions of vehicular drives, entrances, exits, acceleration and deceleration lanes, fire lanes and service lanes.

(13) Location and dimensions of pedestrian entrances, exits and walkways.

(14) Drainage system, stormwater management plan and septic facilities.

(15) Location, height, and materials of walls, fences and screen plantings, buffer zones.

(16) Ground cover, finished grades, slopes, banks and ditches.

(17) Architectural sketches of proposed buildings and their locations, and all buildings shall be planned with a common architectural theme for the entire project.

(18) Location, size, and orientation of all signs.

(19)	Market analysis. A market analysis shall be prepared and signed by an independent market analyst containing the following information:

a. Trade area of proposed shopping mall/shopping center.

b. Population of the trade area, present and projected.

c. Effective buying power in the trade area, present and projected.

d. Net potential customer buying power for stores in the proposed shopping mall/shopping center and, on the basis of such buying power, the recommended store types and store floor area.

e. Residual buying power and how it may be expected to be expended in existing business areas serving the proposed trade area.

f. The mall/shopping center market analysis shall be furnished to the town planner. The town planner shall certify to the planning board that the required market analysis has been completed, and the town planner shall then return the market analysis materials to the developer along with comments, if any. The contents shall be kept confidential.

(20) Elevations of all buildings proposed to be built within the project.

(21) Restrictive covenants as may be required to enforce the architectural theme on satellite business lots.

(22) Evidence of compliance with the following:

a. Chapter 7, article II.

b. Chapter 16.

c. Natural features site plan.

(23) A natural features site plan.

(24) The rules, regulations and procedures of the appropriate regulatory agency or agencies must be satisfied with respect to wastewater treatment systems and any necessary permits therefor obtained.

(g) Phased development and controls. After the overall project (including satellite business lots) has been approved, then the project may be constructed in phases provided the following requirements are met:

(1) Each phase must meet all the requirements of this chapter.

(2) Each phase must be completed before the next approved phase is commenced.

(3) Time period for commencing and completing the first phase shall be two (2) years and one (1) year for each successive phase thereafter.

(4) Development must be submitted and approved in its entirety; construction only is allowed in phases.

(5) After preliminary approval of the mall/shopping center project, the construction of the infrastructure necessary for a particular phase may be commenced as well as approved building construction in that phase. After the infrastructure and streets have been completed, then the evidence thereof required by the planning board shall be submitted to the town planner with the final plat. The final plat of the phase shall be approved by the planning board.

(Ord. of 8-18-86, § 3.01(H); Ord. No. 88-10, 6-6-88; Ord. No. 90-12, 7-2-90; Ord. No. 90-21, 9-4-90; Ord. No. 91-12, § 3, 7-15-91; Ord. No. 91-13, §§ 4--6, 10-7-91; Ord. No. 91-16, 12-10-91; Ord. No. 91-17, 12-10-91; Ord. No. 91-21, § A, 12-10-91; Ord. No. 92-10, 8-3-92; Ord. No. 94-7, 3-7-94; Ord. No. 94-14, 8-1-94; Ord. No. 94-16, 9-6-94)

### Sec. 20-148.	BH-1 beach hotel district.

(a) Intent. The beach hotel district is established to provide the proper grouping and development of hotel facilities in the town. The hotel use is the most intense land use permitted by this Chapter. The problems resulting from the hotel use require standards as well as regulations which mitigate the impact of the intense use of the land upon the neighboring land and surrounding community. Notwithstanding the intensity of the hotel use itself, the town believes that the hotel use can be combined with other commercial uses in a compatible mix of uses, provided the project is planned and built in accordance with a unified plan.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Hotels and motels provided the following standards are met:

(a) The maximum height of a structure shall be thirty-five (35) feet.

(b) A minimum building setback of seventy-five (75) feet is required from the right-of-way margin of U.S. 158 or N.C. 12, excluding parking lots as provided in subsection (b)(4).

(c) A minimum setback for any structure from a property line other than a property line along a state highway right-of-way shall be thirty (30) feet.

(d) Setbacks shall be left as open space. However, subsurface sewage systems, repair areas and parking lots are allowed in the required setbacks. Parking lots within the building setback area shall be located no closer than twenty-five (25) feet to the right-of-way margin, and the vehicles parked in the setback area must be screened from the contiguous right-of-way by means of an earthen mound or a landscaped vegetative buffer which must be maintained by the property owner or the property owner's association if applicable. The obligation to maintain the mound or landscaped vegetative buffer such that the vehicles parked therein are screened from the view of the public on the right-of-way shall be a continuing obligation for beach hotel district permit validity, and the failure to maintain the mound or buffer shall be grounds for beach hotel district permit and building occupancy permit revocation. The vegetation to cover the mound or used as the screen within the buffer shall be selected from the list of vegetation approved by the planner and planning board and on file with the clerk.

(e) The density of hotel units shall not exceed twenty-four (24) hotel units or hotel efficiency units per acre.

(f) Hotel units shall be at least three hundred (300) square feet in area.

(g)The total area of impervious surfaces within the development shall not exceed an amount equal to sixty (60) percent of the area of the development site.

(h) Offstreet loading and unloading areas shall be provided in accordance with article VI, division 2 of this chapter and shall be marked so as to be distinguished from driveways and parking areas. Required loading areas shall be designed so as not to block the access by fire and rescue vehicles.

(i) Containers for garbage and refuse shall be provided in accordance with the requirements of the town and, if applicable, the county department of public works. Areas for screened dumpsters shall be provided so as to be out of the traffic flow and accessible to garbage trucks at all times.

(j) Handicap ramps shall be provided near handicapped parking areas.

(k) No portion of any building shall be further than two hundred fifty (250) feet nor closer than fifty (50) feet to a fire hydrant.

(l) Fire hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department.

(m) Emergency electric generators or approved and maintained battery packs, to provide lighting in hallways and stairwells during periods of utility power outages, shall be installed and tested annually at the expense of the developer.

(n) In order to provide emergency vehicular access to all sides of each building and to provide a flat stable surface upon which emergency operations can be conducted, a paved vehicle access shall be paved around the building no closer than ten (10) feet or further than twenty (20) feet from the building. Upon recommendation of the fire chief and the town planner, with the approval of the planning board, other alternative means of providing emergency vehicle access may be authorized, or, upon the request of the applicant, the planning board may waive the requirement of the ten-foot paved vehicular access around the building provided the applicant, in lieu thereof, agrees to and does install in the hotel building a sprinkler system in accordance with the National Fire Protection Association's standards and the state insurance commission's standards for buildings in excess of thirty-five (35) feet, notwithstanding the hotel or motel uses conducted in a structure of thirty-five (35) feet or less. In addition, the sprinkler system shall be approved by the town building inspector and/or fire chief.

(o) Each hotel site with its building, parking lots and driveways shall be physically separated from each adjoining street or other lots by a curb or other suitable barrier to prevent unchannelled vehicular ingress or egress.

(p) A minimum of six hundred (600) feet of frontage along a street shall be required before two (2) accessways are permitted to the same street. A minimum of nine hundred (900) feet of frontage shall be required before three (3) accessways are permitted. No hotel development shall have more than three (3) accessways to one (1) street. The accessways shall comply with the following standards:

1. Accessways shall not be less than thirty (30) feet nor more than forty (40) in width at their intersection with the property line and shall be installed with curbs of asphalt or concrete at street connections to prevent damage to roadways.

2. The principal accessways shall have an exit lane for left turns where permitted and an exit lane for right turns and one (1) entrance lane. The lanes shall be appropriately marked to indicate exists and entrances.

3. At its intersection with the property line, an accessway shall not be less than one hundred (100) feet from another accessway or fifty (50) feet from a corner of the property; however, at least one (1) accessway will be allowed for each hotel development.

4. Notwithstanding the foregoing, private driveways providing the sole access to two (2) or more parcels jointly, positioned either within fifty (50) feet of a property line or straddling said property line, are permitted and encouraged as long as proper cross access agreements are provided.

(q) If the hotel development is to be constructed in progressive stages, no occupancy permit will be granted for any one (1) stage until all site improvements and conditions assigned to the stage being constructed are completed.

(r) Lighting in accordance with the requirements of the town lighting standards shall be installed in all parking and service areas.

(s) Except as may be hereinafter provided, hotels may have as an accessory use retail shops or offices which offer goods and services for the benefit of their occupants and guests, provided that all such establishments shall be designed to meet the requirements of occupants and guests of the hotels in which such establishments are located and further provided that there be no sign or evidence of such establishments outside of the hotels.

(t) For hotel uses only, the structure may have three (3) habitable floors, provided the maximum height of thirty-five (35) feet is not exceeded.

(u) Hotel parking shall meet the requirements of section 20-438. (99-17)

(v) The term hotel shall include motel and motor inn for the purposes of this section.

(w) The developer of a hotel shall provide evidence of the existence of a private or public beach fronting on the Atlantic Ocean for a distance of at least one hundred (100) feet in width and extending landward from the mean high water mark line to the eastern right-of-way margin of N.C. Highway 12 and readily accessible for use and enjoyment by the guests of the hotel. The area shall be accessible by a public accessway extending from N.C. Highway 12 to the Atlantic Ocean. In the absence of such a public beach area, the hotel developer shall acquire a fee simple or by perpetual easement such an area and dedicate the area as a public beach. As an alternative to acquiring the prescribed beach area, the developer may pay to the town, prior to the issuance of an occupancy permit, a beach access impact fee in such amount as may be specified in the regularly adopted beach access impact fee schedule of the town. Any such fees shall be held by the town to be earmarked for land used in carrying out a program of acquiring beach front properties for public use or to be used by the town to provide alternative means of accessing the beaches through a public transportation system or other means.

(x) Ground stabilization plan. A plan [shall be provided] that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 16 and chapter 7, article II.

(2)  Offices:  (99-8)

a. Business

b. Financial

c. Governmental

d. Medical and professional

(3)  Primary retail uses: (99-8)

a. Antiques

b. Books

c. Cameras

d. Clothing

e. Craft goods

f. Dry goods

g. Flowers

h. Gifts

i. Hobby goods

j. Jewelry

k. Leather goods

l. Magazines

m. Musical instruments

n. Notions

o. Pharmaceutical and drugs

p. Sporting goods

q. Toys

r. Video sales and rentals

(4). Recreation: (99-8)

a. Indoor recreation

b. Marinas and commercial fishing piers

c. Theaters

d. Sailboat and sailboard rentals for use off premises

(5). Service establishments: (99-8)

a. Barber and beauty shops

b. Dry cleaning and laundry pickup stations, including Laundromats

c. Parking lots

d. Restaurants

e. Shoe repairs

f. Child care facilities for hotel quests

(6)  Town owned or leased facilities (99-8)

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Residential, single-family dwellings, provided the height from ground elevation to the top of top plate is twenty-seven (27) feet, the height of habitable floors does not exceed nineteen (19) feet and the total height shall not exceed thirty-five (35) feet exclusive of chimneys, flag poles, communication masts and aerials.

(2) Rental recreational vessels powered by fuel, wind or human effort upon the following conditions and such other reasonable conditions as may be imposed by the town council:

a. The business activity associated with the rental of jet ski type recreational vessels shall be open to the public only from the hours of 10:00 a.m. until 6:00 p.m. of each day.

b. There shall be one (1) parking space per on-site employee and one (1) parking space for each vessel available for rent by the public.

c. Separate male and female restroom facilities or port-o-johns shall be maintained on the site at a location convenient for the business customers.

d. Sound shall not be transmitted over a public address system or other sound system designed for use outdoors.

e. No alcoholic beverages shall be consumed on the premises.

f. The owner shall provide a safe and convenient area for launching and disembarking from vessels and watercraft, and such area (as well as any areas intended to be used for the activity in close proximity to the shoreline) shall be free of subsurface objects which are or could constitute a hazard to vessels or occupants. Any underwater or subsurface objects or debris which could constitute a safety hazard shall be removed by the owners from the area of operation, and a safe, marked and unobstructed channel to deep water shall be established and maintained for those vessels intended or capable of being operated on the open waters of the sound.

g. During that portion of the year in which the business activity is opened to the public, the owner shall obtain and maintain public liability insurance in an amount of not less than fifty thousand dollars ($50,000.00) and with a deductible not exceeding one thousand dollars ($1,000.00).

h. The on-site manager and all employees shall be trained in the use of the vessels and craft available for rent and shall possess the skills necessary to effect marine rescues utilizing watercraft.

i. A power boat which is not available for rent shall be maintained in an operable condition by the site manager at all times for rescue and emergency purposes.

j. There shall be no fuel storage facility on-site.

k. Trash receptacles of appropriate size and approved by the county shall be maintained on-site. The trash receptacle area shall be maintained in a litter-free manner.

(3) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (3) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (3) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(4) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(5) Churches (04-27)

(d) Dimensional requirements. (99-18)

- Commercial lots shall be a minimum of 15,000 square feet and of sufficient size to meet the requirements of the county health department to provide adequate siting for structures, and to provide parking, loading, and maneuvering space for vehicles as required by Article VI, Division 2 of this chapter.  In addition, buffers are required in accordance with Article VI, Division 5, Section 20-510 for hotel projects.

- The minimum front yard is fifteen (15) feet.

- The minimum side yard is ten (10) feet.  No side yard required if commercial building is constructed with common wall.  An additional five-foot yard adjacent to the street is required for a corner lot.

- The minimum rear yard is twenty (20) feet.

- The maximum allowable lot coverage by principal use and all accessory structures is sixty (60) percent.

- Maximum height limitations are as follows:

a. Thirty-five (35) feet total height from existing grade, exclusive of chimneys, flagpoles, communication masts and aerials. (04-26)

(7) No building or other facility (such as parking areas, trash collection areas, etc.) shall be located nearer than fifty (50) feet to the boundaries of any residential district.  The width of a driveway or a road and its right-of-way along such boundary may be included as part, or all, of the fifty-foot separation zone, and subsurface wastewater disposal systems and stormwater runoff collection and disposal systems may be located within the separation zone.  The planning board may require vegetation to be planted and maintained in the fifty-foot separation zone. (99-18)

(e) Commercial site plan review.  Any person desiring to construct or enlarge a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of any existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this article. (99-18)

(f) Existing structures; use change; building code.  Any building constructed in a  commercial zone for a commercial use must comply with the applicable standards of the North Carolina Building Code.  Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain site plan approval from the planning board and an occupancy permit before making a commercial use of the building.  The building must satisfy all North Carolina Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit except as provided in this section. (99-18)

The town council may approve an access from US158 or NC12 onto a commercial site which also abuts another dedicated public right-of-way upon satisfaction of the following:

Upon the request of the applicant and at the applicant’s expense, the town will engage a traffic engineer to study the site plan submitted by the applicant and the proposed traffic movements utilizing the side street only for access and the side street together with an access (or limited access) from US158 or NC12.  If the traffic engineer concluded that it would be safe for the public for the applicant’s site to be accessed from NC12 or US158 as well as the abutting street, then the town council may consider approving the access onto US158 or NC12.  In the event the traffic engineer recommends improvements to the right-of-way or restrictions on the use of the access on US158 or NC12, then the applicant must construct the improvements at this expense before the occupancy permit will be issued.  The use of the accessway can be restricted based upon the recommendation of the traffic engineer and the approval of the town council. The applicant must receive the consent of the Department of Transportation of the State of North Carolina before commencing any right-of-way improvements required by the town council. (99-18)

(g) Ground stabilization plan.  A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with Chapter 7, Article II, and Chapter 16 is required.  (99-18)

(h) Rental unit nonconforming use and density grandfathering.   Any motel, hotel, inn or cottage court in the BC or the BH zones may rebuild structures containing one (1) or more rental purposes which were in existence and so used on August 18, 1986; notwithstanding such reconstruction would not comply with the density standards of this chapter.  Any expansion of the business by the addition of more rental structures shall comply with all the use standards and density standards of this chapter. (99-18)

(i) Miscellaneous regulations.

(1) The proposed commercial buildings and structures on the BH-1 site plan shall all be designed with the Coastal Coastguard-Lifesaving Station architectural themes.  Each building or structure must incorporate at least the minimum number of design elements set forth in and selected from the schedule of design specifications of the Coastal Coastguard-Lifesaving Station architectural themes as adopted from time to time by the town council. (99-18)

- The project must be constructed in accordance with the approved plans and specifications.

(3) Traffic control and traffic flow.  There should be one (1) primary accessway from the beach hotel district site to a public or private right-of-way.  If the beach hotel district site fronts on US158 or NC12, then it shall have one access to said highway.   The planning board may approve one (1) or more secondary accessways for hotel sites if additional access points are needed in order to ensure orderly and safe traffic movement within the hotel project site and between the hotel site and adjoining areas.  The beach hotel district project shall maintain all traffic control devices which are located on the development property.  There shall be no parking areas contiguous to the right-of-way margin or any used for thru vehicular traffic such that a vehicle is required to back or maneuver into the project through traffic system while entering or leaving a parking space.   This requirement shall not apply to internal traffic lanes within parking areas or lots. (99-18)

(4) Lighting.  All parking areas and accessways shall be lighted in accordance with the standards of the town.   All outside lighting shall be arranged and shielded to prevent glare or reflection or nuisance or hazardous interference of any kind with or on adjoining lots or residential areas or areas traveled by motor vehicles and shall provide security for beach hotel district customers. (99-18)

(5) Fire hydrants.  Fire hydrants shall be of the type and at locations approved by the planning board and the fire chief.  No portion of a building shall be farther than two hundred fifty (250) feet from nor closer than fifty (50) feet to a fire hydrant.  Hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department.  The beach hotel project owner shall mark and maintain fire lanes.  Parking shall not be permitted in fire lanes. (99-18)

(6) Garbage and refuse containers.  Containers for garbage and refuse shall be provided in accordance with the requirements of the town and, if applicable, the county department of public works.  Areas for screened dumpsters shall be provided so as to be out of the traffic flow, accessible to garbage trucks at all times and of adequate size to meet the needs of all uses at the center.  Garbage pickup and commercial deliveries shall be during the hours between 7:00 a.m. and 9:00 p.m.  (99-18)

(7) Stormwater management.  It shall be required that an adequate method for managing stormwater runoff shall be developed.  Whenever possible, stormwater management systems, parking layout and the location of curbs and gutters should be planned simultaneously.  Any enclosed portion of a system should be designed to manage stormwater not just to dispose of it or disperse it.  No stormwater runoff shall be allowed within pedestrian ways or pedestrian easement areas, and stormwater runoff shall be retained with the project boundaries. (99-18)

(8) Public restrooms.  Public restrooms shall be required in each commercial retail building.  The public restroom shall be located such as to be convenient to the public and shall be maintained by the operator or applicant in a clean and sanitary condition.  The location of all restrooms shall be clearly marked.  This requirement shall be satisfied if each store, office and commercial unit contains restrooms available to the public and clearly designated as “public restrooms” in a manner visible to store customers.  Building used solely for offices or offices and a residential use shall not be required to contain public restrooms. (99-18)

(9) Directional signs.  Directional signs for pedestrian and vehicle traffic shall be located as needed within the beach hotel district, subject to planning board approval and compliance with the approved master sign plan.  No directional sign shall exceed six (6) square feet in size. (99-18)

(10) Sign standards for all uses except hotel use.  The signs for a commercial use in this district which does not include a hotel use on the approved site shall comply with the sign regulations and standards of the BC-2 district. (00-13)

(Ord. of 8-18-86, § 3.01(F); Ord. No. 88-15A, 10-24-88; Ord. No. 89-15, 6-19-89; Ord. No. 89-22, 8-21-89; Ord. No. 91-21, § B, 12-10-91; Ord. No. 92-4, 6-29-92; Ord. No. 93-2, 1-4-93; Ord. No. 93-5, 2-1-93; Ord. No. 94-14, 8-1-94)

### Sec. 20-149.	BH-2 beach hotel district.

(a) Generally. Unless stated otherwise, this section applies to the BH-2 beach hotel district. In this section "hotel" includes a motel or motor inn. The BH-2 district is established to provide the proper location and development of hotel, motel, and motor inn facilities in Kitty Hawk Beach. Due to the unique problems associated with hotel development by reason of its increased density and need for supporting services, the town has established the beach hotel zones in which hotels are authorized provided the requirements and conditions of this section are met.

(b) Structures thirty-five or less feet in height. The following provisions apply to structures that are thirty-five (35) feet or less in height:

(1) Lot width. A hotel lot shall have a minimum width of two hundred (200) feet measured at the front setback line.

(2) Setback from roads. A minimum setback of seventy-five (75) feet is required from the right-of-way line of N.C. 12 or U.S. 158.

(3) Other setbacks. The minimum setback for any structure from a property line other than a property line along a state highway right-of-way shall be thirty (30) feet.

(4) Maximum height. The maximum height of the structure shall be a thirty-five (35) feet except as otherwise provided.

(5) Open space. Setbacks shall be left as open space; however, storm water retention basins, subsurface sewage systems,  repair areas and motor vehicles accesses are allowed in the required setbacks. (98-30)

(6) Density. The density of hotel units shall not exceed twenty-four (24) hotel units or hotel efficiency units per acre.

(7) Area of units. Hotel units shall be at least three hundred (300) square feet in area. Hotel efficiency units shall be at least four hundred (400) square feet in area but not greater than six hundred (600) square feet in area.

(8) Floor area of structures. The total floor area of all structures within the development shall not exceed an amount equal to thirty-three (33) percent of the total land area of the development site.

(9) Vehicular access areas. A twelve (12) wide paved vehicular access along all four (4) sides of principal structures shall be provided which must be suitable for fire-fighting and rescue equipment. The edge of the paved access closest to the structure shall be no closer than ten (10) feet, nor farther than twenty (20) feet, from the sides of the structure. (98-30)

Upon request of the applicant, review by the planning board, and approval of the council, the Town may approve a cellular confinement system for the construction of the vehicular access required in this paragraph instead of concrete or asphalt.  The proposed cellular confinement system must be approved by the fire chief.  The applicant must submit to the planning board a certification from a licensed engineer that the proposed cellular confinement system will support the weight of the heaviest town fire fighting vehicle if it is constructed or installed at the site as proposed by the applicant. (98-30)

Upon the request of the applicant and a favorable recommendation of the planning board, the Town Council may approve alternative means of emergency vehicle access in lieu of a twelve (12) foot wide vehicular access provided the approved alternative will maintain or improve the ISO rating of the Town. (98-30)

(10) Off-street loading. Off-street loading and unloading areas shall be provided in accordance with this chapter and shall be marked so as to be distinguished from driveways and parking areas. Required loading areas shall be designed so as to not block the access by fire and rescue vehicles.

(11) Solid waste. Containers for garbage and refuse shall be provided in accordance with the requirements of the town, and if applicable, the county department of public works. Areas for screened dumpsters shall be provided so as to be out of the traffic flow and accessible to garbage trucks at all times.

(12) Handicapped ramps. Handicapped ramps shall be provided near handicapped parking areas.

(13) Sidewalks. A sidewalk or boardwalk constructed to provide a grade separation from vehicular traffic of at least six (6) inches shall connect all principal buildings on the site except where passageways cross traffic lanes. Separate buildings shall be connected with pedestrian passageways that are striped when crossing traffic lanes.

(14) Proximity to fire hydrants. No portion of any building shall be farther than two hundred fifty (250) feet or closer than fifty (50) feet to a fire hydrant.

(15) Protection of fire hydrants. Fire hydrants shall be protected from traffic in accordance with requirements of the town and shall be marked and painted as required by the town and the fire department.

(16) Fire hydrant performance standards. If the building is not to be sprinkler protected in accordance with National Fire Protection Association standards, the fire flow of the fire hydrants at or nearest the site shall be tested by an independent testing laboratory at the expense of the developer. If the fire flow is found to be deficient according to Insurance Service Office (ISO) standards applicable to the requirements of the town, the fire flow shall be brought up to ISO standards at the developer's expense. The decision to use a sprinkler system or to adjust a fire flow shall be made prior to issuance of a building permit. The fire flow test shall be made during the period of peak water demand as determined from water consumption data maintained by the county water department. Where sprinkler systems are not required, a standpipe system shall be installed which meets ISO fire flow standards.

(17) Emergency electrical service. Emergency electric generators or approved and maintained battery packs, to provide lighting in hallways and stairwells during periods of public utility power outages, shall be installed and tested regularly at the expense of the developer.

(18) Street frontage, etc. A minimum of six hundred (600) feet of frontage along a street shall be required before two (2) accessways are permitted to the same street. A minimum of nine hundred (900) feet of frontage shall be required before three (3) accessways are permitted. No hotel development shall have more than three (3) accessways to one (1) street. The accessways shall comply with the following standards:

a. Accessways shall not be less than thirty (30) feet or more than forty (40) feet in width at their intersection with the property line and shall be installed with curbs of asphalt or concrete at street connections to prevent damage to roadways.

b. The principal accessways shall have an exit lane for left turns where permitted and an exit lane for right turns and one (1) entrance lane. The lanes shall be appropriately marked to indicate exit and entrance.

c. At its intersection with the property line, an accessway shall not be less than one hundred (100) feet from another accessway or fifty (50) feet from a corner of the property. Notwithstanding, at least one (1) accessway will be allowed for each hotel development.

(19) Curbs. Each hotel site with its building, parking lots and driveways shall be physically separated from each adjoining street by a curb or other suitable barrier to prevent unchanneled vehicular ingress or egress.

(20) Occupancy permits. If the hotel development is to be constructed in progressive stages, no occupancy permit will be granted for any one (1) stage until all site improvements and conditions assigned to the stage being constructed are completed.

(21)  Lighting. Lighting in accordance with the requirements of the town's lighting standards shall be installed in all parking and service areas.

(22) Structure separation. There shall be a minimum separation of forty (40) feet between detached principal structures.

(23) Dimensions. To the extent not inconsistent with the specific provisions of this subsection, the dimensional requirements specified in section 20-55, subsection (f) shall govern a hotel development.

(24) Retail shops. Hotels may have as accessory uses retail shops or offices which offer goods and services for the benefit of their occupants and guests; provided that all such establishments shall be designed to meet only the requirements of occupants and guests of the hotel in which such establishment is located and provided further that there be no sign or evidence of such establishment outside of the hotel.

(25) Preapplication conference. A preapplication conference shall be held between the planning board and the applicant. The applicant shall present at least a survey and conceptual site plan and one-half of the planning conference fee.

(26) Additional requirements. Additional requirements and conditions may be imposed in accordance with section 20-55.

(27) Beach front. The developer of a hotel shall afford evidence of the existence of a private or public beach fronting on the Atlantic Ocean for a distance of at least one hundred (100) feet in width and extending landward from the mean high water line to the eastern right-of-way margin of the N.C. 12, and readily accessible for use and enjoyment by the guests of the hotel. The area shall be accessible by public accessway extending from N.C. 12 to the Atlantic Ocean. In the absence of such a public beach area, the hotel developer shall acquire in fee simple or by perpetual easement such an area and dedicate the area as a public beach. As an alternative to acquiring the prescribed beach area, the developer may pay to the town prior to the issuance of an occupancy permit, a beach access impact fee in such amount as may be specified in the regularly adopted beach access impact fee schedule of the town to be earmarked for land used by the town in carrying out a program of acquiring beach front properties for public use.

(28) Maximum floors. For hotel uses only in the BH-2 zones, the structure may have three (3) habitable floors provided the maximum height of thirty-five (35) feet is not exceeded.

(29) Parking spaces. Parking spaces shall be allocated as specified in Section 20-438 for each hotel unit. (98-30)  These spaces shall be in addition to the spaces required for any accessory or secondary uses in connection with the hotel project. Commercial uses within the hotel complex shall meet the parking requirements for the same or comparable use as required by this chapter. Parking under wood frame hotel structures is prohibited.

(30) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with the chapter 7, article II and chapter 16 is required.

(31) Building height measurement. If the building site is below flood grade, then the building height will be measured from the lesser of the Town flood plain level or the finished grade level. (98-30)

(32) Lot coverage.  The total coverage by the principal use and all accessory uses on the lot shall not exceed 60% of the lot area.  Only site area within the town limits shall be considered for the purpose of this calculation.  Natural areas and landscaped areas shall not be included as areas of coverage. (98-30)

(33) Building elevations.  The applicant must submit detailed elevations of each side of the structure for approval by the Town.  The structure must have a multi-pitched, shingled roof structure.  Flat roofs are prohibited.  Exterior corridors connecting the room, the grown floor and other floors are prohibited (except for a fire escape system if required by an appropriate regulatory governmental agency). (98-30)

Changes to the approved elevations plans shall not be made without the Town Council approval unless the modification or change is subject to approval by the town planner as provided in the code. (98-30)

“(c) Structures taller than thirty-five feet. As a conditional use in this zone, hotel and motel uses may be constructed to a maximum of five habitable floors with a mean roof height not to exceed 62 feet, with a maximum of 55 feet at the top of the fifth floor, and a maximum total height of  76 feet(06-09), excluding the cupola and provided a special use permit to construct and use a hotel or motel structure with a maximum height that meets the aforesaid standards has been issued by the Town Council.  The applicant for the special use permit must satisfy the following additional conditions and standards as well as such other reasonable conditions and standards as may be imposed by the town and set forth in the special use permit approved by the applicant: (04-02)

(1) The tract or parcel of land being developed for a hotel use and upon which a structure in excess of thirty-five (35) feet is planned shall equal or exceed five (5) acres. The permitted commercial uses of the BC-1 zone may be authorized as conditional uses secondary to and supporting the primary hotel use of the site provided the requirements of this ordinance and reasonable requirements of the planning board imposed thereon are satisfied.

(2) A hotel use permitted within the zone may be constructed to a total of five habitable floors with a mean roof height not to exceed 62 feet, with a maximum of 55 feet at the top of the fifth floor, and a maximum total height of 76 feet(06-09), excluding the cupola and (04-02)  provided each of the front and side setback requirements shall be increased by an additional one foot for each additional foot by which the building exceeds forty-five (45) feet in height. The additional setback area shall be measured from the interior setback lines which satisfy the standard setback requirements of this chapter. Only one (1) structure on any given site plan shall be permitted to exceed the height of thirty-five (35) feet.  The additional required setback area and distance maybe satisfied on either side of the structure provided that the minimum distance from the building is not less than 30 feet and the building not less than fifty (50) feet from any existing structures on adjoining properties. The measurement must be made from a point on the building foundation which is the point closest to the side line or existing adjacent structure under consideration and should be the shortest distance from the point on the foundation to the side line or the existing adjacent structure. (98-30)

(3) The setback area shall be open space in which pedestrian easements shall be permitted. The Town Council may approve motor vehicle and/or subsurface utility easements, as well as storm water retention basins within the setback area.  In the event the project parcel or lot boundary is located outside of the municipal jurisdiction of the town, then the setback may be measured from the boundary line of the project site provided no structures are built or located between the structure or building and the project site boundary located outside the corporate limits of the town. (98-30) Coverage within the 75 feet front setback may not exceed 30 percent of the setback area. (04-02)

(4) The special use permit shall be in a form complying with the requirements for recording instruments in the Office of the Register of Deeds of Dare County and the from approved by the Town Council. (98-30)

(5) The special use permit shall be issued by the Town after all necessary zoning amendments and project permits have been granted and the Town has approved the special use permit conditions and standards.  The conditions and standards of the special use permit are continuing in nature, and a failure to comply with the special use permit standards and requirements after the permit has been issued shall be a basis for revoking the permit.  Upon revocation of the special use permit by the Town, the permit holder shall cease using that portion of the hotel structure above the height of thirty-five (35) feet for the hotel use or any other commercial use until the special use permit has been reinstated by the Town. (98-30)

(6) The special use permit is transferable and the right and burdens set forth therein shall run with the land described in the permit.  Upon a transfer of the property described in the special use permit, the transferee shall obtain a reissued special use permit in the name of the transferee within thirty (30) days of the transfer date.  The reissued special use permit shall be filed in the Office of the Register of Deeds in Dare County by the Town. The transferee shall pay the recording costs. (98-30)

(7) The Town Council shall utilize the procedure used by the board of adjustments when the council issues or denies a special use permit except that no vote greater than a majority vote shall be required for the Town Council to issue the special use permit, and every decision of the Town Council shall be subject to review by the Superior County by proceedings in the nature of certiorari. (98-30)

The appeal to the Superior County from the decision of the Town Council shall be filed with the Clerk of Superior County within thirty (30) days after the Town Council has files its decision with the Town Clerk, or after the written copy thereof has been delivered or mailed to the address set forth in a written request by every aggrieved party with the clerk at the time of the hearing of the special use permit, whichever is later. The decision of the Town Council may be delivered to the aggrieved party either by personal delivery or by register or certified mail return receipt requested. (98-30)

(8)All development standards and requirements must be satisfied within the boundaries of the site, tract, or lot submitted for review by the Town except as follows:

In the BH-2 district where a parcel or lot described by deed or map recorded by Dare County Registry prior to January 1, 1998, has been divided by the town corporate boundary, and the parcel or lot of land containing land inside and outside of the town boundary limit has been submitted as a commercial site plan for town approval, then the Town may issue a special use permit to allow the applicant to include in the areas that portion of the commercial site plan located outside of the Town for the purpose of complying with the parking standard set forth in Section 20-149 (b) (29) of the Town Code subject to and provided the following conditions are satisfied:

- The applicant must file a special use permit application with the Town and pay such special use permit fees as may be adopted by the Town Council;

- The applicant must file a copy of the deed or plat recorded in the Dare County Registry with the Town which describes the lot or parcel of land existing on January 1, 1998, which is currently divided by the Town corporate limits;

- That portion of the site lying outside of the corporate limits cannot be separated by a street or vehicular accessway from the commercial site within the Town corporate limits;

- The land area of the site outside of the town limits cannot be subdivided, sold or conveyed or otherwise transferred separate from that portion of the commercial site within the Town if such sale, conveyance, transfer, or subdivision would cause that portion of the site remaining in the Town to violate the coverage standard or any other zoning standard then applicable to commercial lots, and such a sale, conveyance, transfer or subdivision of land shall be a basis for revoking the special use permit and the commercial site  approval for the area of the site remaining in the Town as well as the issuance of an injunction by a court which prohibits further use of the site until the site is brought into compliance with the current zoning standards of the town;

- The area of the site located outside of the corporate limits shall be used only as open space-green area, as access into and out of the site, driveway and for subsurface wastewater drainfield purposes as may be approved by the appropriate regulatory authorities, or for the parking of motor vehicles on paved parking spaces in accordance with the appropriate town parking space standards.

- The applicant must submit prior to approval by the Town, a written statement from the appropriate office of the adjoining municipality that proposed development shown on the site plan within the adjacent municipality complies with all applicable regulations and laws of the adjacent municipality.(00-1)

(9) The density of hotel units shall not exceed 26 units or hotel efficiency units per acre.(04-02)

(10) The total floor area of all structures within the development shall not exceed an amount equal to 40 percent of the total land area of the development site. (04-02)

(d) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(e) Commercial site plan review. Any person desiring to construct a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this chapter.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit.

(g) Rental unit nonconforming use and density grandfathering. Any motel, hotel, inn or cottage court in BC or BH zone may rebuild structures containing one (1) or more rental units and/or freestanding structures used for rental purposes which were in existence and so used on August 18, 1981 notwithstanding such reconstruction would not comply with the nonuse and density standards of this chapter. Any expansion of the business by the addition of more rental structures shall comply with all the nonuse standards and density standards of this chapter.

(Ord. of 8-18-86, § 3.01(G); Ord. No. 88-15A, 10-24-88; Ord. No. 90-21, 9-4-90)

### Secs. 20-150--20-160.  Reserved.

## DIVISION 3.  KITTY HAWK VILLAGE

### Sec. 20-161.	Scope.

Except as otherwise provided, this division applies to Kitty Hawk Village.

### Sec. 20-162.	VR-1 low density village residential district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VR-1 low density village residential district. The VR-1 district is intended to encourage the development of low density residential neighborhoods in Kitty Hawk Village.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Detached single-family dwellings, not to include mobile homes.

(2) Customary accessory buildings, private swimming pools, tennis courts, private piers and boat slips. (00-4)

(3) Town-owned or leased facilities.

(c) Conditional uses permitted. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches and cemeteries.

(2) Private schools.(05-21)

(3) Home occupations.

(4) Golf courses and concessions integral thereto, provided that there is no open commercial activity and no sign other than a directional sign is allowed, noncommercial tennis courts and swimming pools associated with residential subdivision amenities, and libraries.

(5) Except for the subdivision known as Kitty Hawk Landing Subdivision and that portion of the Kitty Hawk Bay Soundfront running from Hay Pointe to the point of intersection of the Beach Residential (BR-1) zone district with the Village Residential -1 (VR-1) zone district in the vicinity of Penney Toler ditch, commercial crabbing and fishing operations conducted by land owners (as well as their family members and employees as permitted herein) who own land fronting on and adjoining the navigable canals and waterways in the town shall be conditionally authorized to conduct commercial crabbing and fishing operations on their property fronting and adjoining said navigable canals and waterways (including tributaries thereto) including maintaining and using shedding boxes, upon the following conditions and such other reasonable conditions as the town may impose: (01-1)

a. Seafood shall not be sold, processed or distributed on the premises. (01-1)

b. The cleaning of fish or other seafood for commercial purposes is prohibited.

c. Any exterior or outside lighting used in the commercial crabbing and fishing operation shall be low profile, shielded and directed toward the operations, and to the extent practical, away from adjoining neighbors.

d. The maximum number of crab shedding boxes permitted on the premises shall be thirty-six (36) and the shedding boxes shall not exceed a size of four (4) feet by eight (8) feet.

e. While engaging in the conditional use authorized by this paragraph, no radio, musical, loudspeaker, or other such sound shall be audible beyond the property line of the landowner engaging in the authorized conditional use.

f. No more than four (4) people shall be involved in or conducting the authorized conditional use on the property at any one  time. (01-1)

g. Only boats owned and operated by the landowner or his family may be loaded and unloaded at the property site, and not more than four (4) boats shall be docked at the property or loaded and unloaded as authorized by this section.

h. The equipment and machinery used for commercial crabbing or fishing purposes as authorized herein shall be screened as the town council, in its discretion, may require in order to minimize the commercial impact upon adjoining residential property owners.

i. Storage of crab pots and crab shedding boxes shall be screened from adjacent residential properties; (01-1)

j.  A CAMA permit shall be obtained if applicable. (01-1)

(6) Noncommercial boat docks and boat basins owned or operated by any governmental agency upon such reasonable conditions as the town may impose as well as the following conditions:

a. The site shall contain one and one-half (11/2) spaces per boat slip and a minimum of fifteen (15) trailer-car spaces per boat ramp. The trailer-car parking spaces shall be no less than ten (10) feet in width and forty-five (45) feet in length.

b. A dumpster pad shall be located on the site plan and a dumpster placed on the site with screening of the dumpster in a manner and with materials approved by the planning board.

c. Exterior lighting shall be adequate to utilize the facility at night and shall be of low intensity and shall reflect upon the site in such a manner as not to interfere with traffic on public streets or highways and with adjoining property owners.

(7) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (7) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (7) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(8) Commercial kayak guided tours shall be a conditional use subject to the following conditions set forth herein and such other reasonable conditions as may be established by the town: (01-15)

a. Permits – Issuance.  The conditional use must be conducted pursuant to a permit. The town will issue permits for the commercial kayak guided  tours only for applications filed during the month of September each year for the next calendar year.  The permit term will be from January 1 to December 31 of each calendar year unless suspended or revoked by the town.  A permit will be issued to companies that satisfy the following conditions:

1. The permit holder must pay the permit application fee of Three Hundred Dollars ($300.00) to the town and file the completed application and necessary supporting materials during the September of that next year preceding the permit year.

2. Provide evidence of liability insurance coverage of Three Hundred Thousand Dollars ($300,000.00) minimum per occurrence and a total of One Million Dollars ($1,000,000.00).

3. The administrative operations of the company must be located within the commercial districts of the town or outside the boundaries of the town.

b. Permits – Continuance.  Permit holders must satisfy the following requirements at all times in order for a permit to remain active and in good standing during the permit year:

1. A trained tour guide must accompany each tour.

2. The tour guide must have a operational two-way communication device in the possession of the tour guide during the tour.  The device shall be either a cell phone or two-way radio or such other device as approved by the town.

3. Tour guides and tour participants must comply with the rules and regulations established by the town.

4. Tour kayaks must satisfy identification standards established by the town.

5. Kayak operators and participants must obey applicable navigation rules and yield the right of way to larger power boats in narrow channels and shall keep outside the marked navigable channel in curves or bends in the streams or channels.

6. The tour must be conducted during the scheduled time periods for the permit holders and within the areas authorized by the town.

7. Kayak tour participants will not enter upon private property along the streams and canals without permission from the property occupants or owners.

8. Personal floatation devices must be worn by tour participants.

9. Tours south of “Covered Bridge” shall only originate from and shall return to the county boat ramp and shall not extend north of the “Covered Bridge” or onto the tributaries or designated back waters of High Bridge Creek.

10.  Tours north of “Covered Bridge” shall not extend south of “Covered Bridge” or onto the tributaries or designated back waters of High Bridge Creek or Jeanquite Creek within the boundaries of the town.

11. The permit applicant has not had a permit revoked the prior calendar year.

12.  Kayaks used for research approved by the town shall be exempt from the limits of subparagraph (9) and (10).

c. Permits – Revocation and Suspension

1. The town manager may suspend or revoke a permit for a violation of the rules and regulations by the tour company, its employees, agents or tour participants.  The period of suspension shall be in the discretion of the town.

2.  The town may suspend or revoke the permits for all permit holders at any time the town determines it is in the interest of the public, health, safety and welfare to suspend or revoke the permits.

3. The permit issued by the town is a permissive right to conduct tours as authorized herein and it does not create a vested right to conduct the activity authorized by the permit.

4.  A permit holder may appeal a permit revocation or suspension by the town manager to the town council which shall be heard at the next regular meeting of the town council.  The decision of the council shall be final.

d. Tours

1.  Seven (7) tours per day are authorized north and south of “Covered Bridge”  with tour times allocated among permit holders by the town.  Permit holders may exchange allocated tour times with other permit holders after notifying the town of the exchange.

2. Each tour shall consist of one (1) guide and not more than ten (10) kayaks.

3. No tours shall be commenced earlier than one half (1/2) hour before the sunrise and no tour shall end later than one half (1/2) hour after sunset.

e. Penalties.  A  person or company violating this ordinance shall be guilty of a misdemeanor upon conviction and punished pursuant to Section 1.5 of the town code.  (01-15)

(9) Public utility facility subject to town site plan approval and compliance with site plan conditions imposed by the town council. (99-20)

(10)Community swimming pools, tennis courts and other similar recreational facilities for the use of the project property owners and which are owned or operated by a property owner’s association or condominium association and not for commercial use.(00-4)

(11) Community docks and piers that are owned or operated by a property owner’s association or condominium association and not for commercial use. (00-4)

(12) Family Child Care in residential structure (00-33) (04-11)

a. The property owner must be legally operated and obtain a licensed by the NC Department of Health and Human Services – Division of Child Development if required by law, and must be affiliated with the child care resource and referral program in Dare County or a subsequent similar program of the county.

b. All contiguous property owners must give written consent to the use of the property for Family Child Care pursuant to this paragraph.

c. The maximum number of children in the home pursuant to this use shall not exceed the state licensing regulations.

d. Adequate parking and/or “pickup/drop off” space shall be provided on site.

(13) Fire Stations subject to the following conditions: (05-21)

- The maximum allowable lot coverage by principal use and all accessory structures is fifty (50) percent, provided that the fire station employs stormwater reclamation and reuse techniques and the site is engineered to limit stormwater runoff to an amount less than that discharged from a similarly situated site having a lot coverage of thirty (30) percent. Otherwise, the maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

- Open space on site shall be no less than forty (40) percent.

- Driveway widths shall be no greater than one hundred (100) feet in width at the property line provided the drive is at least two hundred (200) feet from the nearest street corner.

- Required parking for the fire station shall be one (1) parking space for each 500 square feet of usable gross floor space.  For purposes of this requirement, parking bays for fire trucks and space used for storage of fire-fighting equipment shall not be included.

- In addition to parking surfaces approved by Section 20-436(e), parking for the fire station may also be constructed of porous geotextile fabrics, geogrids, geocells, and other forms of separation, containment and reinforcement used in combination creating on-site storage volume for rainfall during rain events and a porous pavement structure that sits atop an engineered porous base course and filled with a high-permeability wear course, such as washed stone. In no instance shall the base course or wear course be an impervious material.  The depth of the courses for this parking surface shall be determined by evaluation of the design load to be supported, and the load capacity of the subsoil. The pavement structure must be designed to support the vehicular traffic expected for this use.

(d) Dimensional requirements. (03-42)

(1) The minimum lot size is fifteen thousand (15,000) square feet. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setbacks shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials.(01-10) The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet.(04-26)

(6) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Density. The maximum permitted density shall be two (2) residential single-family dwellings per acre.

(f) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(A); Ord. No. 88-1, 1-18-88; Ord. No. 90-3, 3-19-90; Ord. No. 90-21, 9-4-90; Ord. No. 91-15, § 4, 10-21-91; Ord. No. 93-2, 1-4-93; Ord. No. 96-18, 9-9-96; Ord. No. 96-21, 10-7-96)

### Sec. 20-163.	VR-2 medium density village residential district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VR-2 medium density village residential district. The VR-2 district is intended to encourage the development of medium density residential neighborhoods in Kitty Hawk Village.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Detached single-family dwellings.

(2) Duplexes.

(3) Customary accessory buildings and private swimming pools and tennis courts.

(4) Mobile homes, provided:  (02-15)

a. Skirting shall be placed under all mobile homes brought into the Town:

1.When a mobile home is placed on pilings, acceptable skirting can include lattice or 1X4 slats (1 space with a slat and one space without) completely enclosing the underneath with the exception of access doors, parking areas, vents, or other opening required by the Town or State Building Code.

2. Other acceptable foundations or skirting also include brick, cinderblock and/or other similar decorative features.

3. Skirting shall hide the chassis of the mobile home.

4. All skirting shall be constructed within 30 days of occupancy.

b. Mobile homes shall be placed on the lot parallel to the adjacent road whenever possible.

c. The tongue, transporting lights, and removable towing apparatus must be removed after placement on the lot and before final occupancy.

d. Mobile homes shall not be used as a storage facility.

- Permanent living quarters or additions shall not be constructed to adjoin any newly installed mobile homes with the exception of decks and/or screened porches.  Existing mobile homes in the Town prior to the adoption date of this ordinance shall be allowed to have additions that adjoin their trailer.  (building code allows additions, however they must be freestanding)

- Two or more single wide mobile homes shall not be connected together as one dwelling nor may a mobile home be attached to any accessory dwelling with the exception of a garage or a carport. (building code allows two units)

- The exterior siding must consist of wood, hardboard, vinyl or aluminum.

- Mobile homes shall be occupied as single family dwelling only.  Home occupations may be permitted pursuant to Chapter 20 of the Kitty Hawk Town Code.

- Only those mobile homes constructed after July 13, 1994 (wind zone II) can be installed in the Town and only if those mobile homes are being relocated from another location within Dare County.  All new mobile homes that are brought into the Town from outside the County must be HUD approved, wind zone III structures.

(5) Town-owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches and cemeteries.

(2) Fire stations and private schools.

(3) Home occupations.

(4) Golf courses and concessions integral thereto, provided that there is no open commercial activity and no sign other than a directional sign is allowed, noncommercial tennis courts and swimming pools associated with residential subdivision amenities, and libraries.

(5) Multifamily dwellings.

(6) Except for the subdivision known as Kitty Hawk Landing Subdivision and that portion of the Kitty Hawk Bay Soundfront running from Hay Pointe to the point of intersection of the Beach Residential (BR-1) zone district with the Village Residential -1 (VR-1) zone district in the vicinity of Penney Toler ditch, commercial crabbing and fishing operations conducted by land owners (as well as their family members and employees as permitted herein) who own land fronting on and adjoining the navigable canals and waterways in the town shall be conditionally authorized to conduct commercial crabbing and fishing operations on their property fronting and adjoining said navigable canals and waterways (including tributaries thereto) including maintaining and using shedding boxes, upon the following conditions and such other reasonable conditions as the town may impose: (01-1)

a. Seafood shall not be sold, processed or distributed on the premises. (01-1)

b. The cleaning of fish or other seafood for commercial purposes is prohibited.

c. Any exterior or outside lighting used in the commercial crabbing and fishing operation shall be low profile, shielded and directed toward the operations, and to the extent practical, away from adjoining neighbors.

d. The maximum number of crab shedding boxes permitted on the premises shall be thirty-six (36) and the shedding boxes shall not exceed a size of four (4) feet by eight (8) feet.

e. While engaging in the conditional use authorized by this paragraph, no radio, musical, loudspeaker, or other such sound shall be audible beyond the property line of the landowner engaging in the authorized conditional use.

f. No more than four (4) people shall be involved in or conducting the authorized conditional use on the property at any one  time. (01-1)

g. Only boats owned and operated by the landowner or his family may be loaded and unloaded at the property site, and not more than four (4) boats shall be docked at the property or loaded and unloaded as authorized by this section.

h. The equipment and machinery used for commercial crabbing or fishing purposes as authorized herein shall be screened as the town council, in its discretion, may require in order to minimize the commercial impact upon adjoining residential property owners.

i. Storage of crab pots and crab shedding boxes shall be screened from adjacent residential properties; (01-1)

j.  A CAMA permit shall be obtained if applicable. (01-1)

(7) Bed and breakfast facility:

a. The bed and breakfast facility shall be compatible in scale of structure and scale of development with residential structures in the neighborhood in which it is located. The principal use of such dwelling structures is residential. The exterior of the structure shall be harmonious with surrounding property. The bed and breakfast use shall be operated in a primary, single-family residential structure and not in any accessory structure.

b. The operation shall be conducted by persons who own and reside within the dwelling unit. It shall be permissible to employ the equivalent of one (1) full-time person to assist in the operation.

c. The structure must contain at least one (1) full bathroom for the exclusive use of the owner and other members of the immediate household, plus one (1) private bathroom for each guest bedroom. Each full, discrete bathroom must include a minimum of a water closet, a lavatory and a bath or shower and meet current building code requirements.

d. The rental of rooms shall be on a daily or weekly basis to tourists, vacationers or similar transients. The rental period shall not exceed fourteen (14) consecutive days in any thirty-day period.

e. There shall be no cooking facilities or kitchen-type appliances in the rental dwelling rooms. A breakfast meal may be provided. It is intended by this subsection (7) that meals may be provided only to registered guests of the facility.

f. Smoke alarms shall be installed in all rental rooms and in common areas.

g. The total rented dwelling rooms shall not exceed four (4) and the total occupancy, including the owner(s), shall not exceed ten (10) persons.

h. Parking shall be provided on the basis of one (1) space per rental dwelling room in addition to two (2) spaces for the owner(s). Parking spaces for rental dwelling rooms may be grass, geowebb or gravel. Such parking areas shall be maintained in a dust-free, rut-free condition and shall be visually screened with a vegetated buffer if adjacent to an existing residential structure or a vacant lot on which a residential structure can be built.

i. Only signs which comply with standards for display in the zoning district in which the structure is located may be permitted.

j. No dwelling may be used as a bed and breakfast unless and until it shall have been permitted by the county environmental health department.

k. Lighting of the premises shall be harmonious with surrounding property. Lighting shall not create glare or interfere with the reasonable enjoyment of adjacent properties.

l. Noise generated within the structure shall not exceed a volume normally associated with residential occupancy. Between 9:00 p.m. and 6:00 a.m., noise originating within the structure shall not be audible beyond the property lines.

m. The town shall issue a permit if all the requirements of this subsection (7) and other applicable provisions of the Town Code have been met. The permit shall remain in effect for a period of one (1) year and may be suspended or revoked if a violation of standards is found. The permit shall be renewed each year, upon request, by the codes enforcement officer unless it is determined that the use of the dwelling is not in compliance with the Town Code. The use of the building as a bed and breakfast shall cease if the permit is not kept in effect.

(8)  Family Child Care in residential structure (00-33) (04-11)

a. The property owner must be legally operated and obtain a licensed by the NC Department of Health and Human Services – Division of Child Development if required by law, and must be affiliated with the child care resource and referral program in Dare County or a subsequent similar program of the county.

b. All contiguous property owners must give written consent to the use of the property for Family Child Care pursuant to this paragraph.

c. The maximum number of children in the home pursuant to this use shall not exceed the state licensing regulations.

d. Adequate parking and/or “pickup/drop off” space shall be provided on site.

(d) Dimensional requirements. (03-42)

(1)  The minimum lot size is as follows:

a. For single-family detached residences, fifteen thousand (15,000) square feet.

b. For duplexes, twenty-five thousand (25,000) square feet for each acre or portion thereof.

c. For multifamily dwellings, for each acre or portion thereof, fifteen thousand (15,000) square feet for the first dwelling unit and nine thousand five hundred twenty (9,520) square feet for each additional dwelling unit.

d. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setbacks shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet.(04-26)

(6) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Density. The maximum permitted density shall be four (4) residential dwelling units per acre. (01-29)(02-10)

(f) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(B); Ord. No. 90-21, 9-4-90; Ord. No. 93-2, 1-4-93; Ord. No. 96-18, 9-9-96)

### Sec. 20-164.	VR-3 high density residential village district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VR-3 high density residential village district. The VR-3 district is established as an area in Kitty HawkVillage in which the principal use of the land is for high density residential purposes, the development of less intensive residential uses as well as for compatible supporting uses.

(b) Permitted uses. The following uses shall be permitted by right:

(1) Detached single-family dwellings.

(2) Duplexes.

(3)Multifamily dwellings.

(4) Customary accessory buildings including private swimming pools and tennis courts.

(5) Mobile homes, provided: (02-16)

a. Skirting shall be placed under all mobile homes brought into the Town:

1.When a mobile home is placed on pilings, acceptable skirting can include lattice or 1X4 slats (1 space with a slat and one space without) completely enclosing the underneath with the exception of access doors, parking areas, vents, or other opening required by the Town or State Building Code.

2. Other acceptable foundations or skirting also include brick, cinderblock and/or other similar decorative features.

3. Skirting shall hide the chassis of the mobile home.

4. All skirting shall be constructed within 30 days of occupancy.

b. Mobile homes shall be placed on the lot parallel to the adjacent road whenever possible.

c. The tongue, transporting lights, and removable towing apparatus must be removed after placement on the lot and before final occupancy.

d. Mobile homes shall not be used as a storage facility.

- Permanent living quarters or additions shall not be constructed to adjoin any newly installed mobile homes with the exception of decks and/or screened porches.  Existing mobile homes in the Town prior to the adoption date of this ordinance shall be allowed to have additions that adjoin their trailer.  (building code allows additions, however they must be freestanding)

- Two or more single wide mobile homes shall not be connected together as one dwelling nor may a mobile home be attached to any accessory dwelling with the exception of a garage or a carport. (building code allows two units)

- The exterior siding must consist of wood, hardboard, vinyl or aluminum.

- Mobile homes shall be occupied as single family dwelling only.  Home occupations may be permitted pursuant to Chapter 20 of the Kitty Hawk Town Code.

- Only those mobile homes constructed after July 13, 1994 (wind zone II) can be installed in the Town and only if those mobile homes are being relocated from another location within Dare County.  All new mobile homes that are brought into the Town from outside the County must be HUD approved, wind zone III structures.

(6) Town-owned or leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Churches.

(2) Cemeteries.

(3) Public parks and playgrounds.

(4) Golf courses.

(5) Home occupations.

(6) Nursing homes.

(7) Planned unit developments under the provisions of article IV.

(8) Mobile home parks under the provisions of article VI, division 4 of this chapter.

(9) Hostel – Tent camping shall be an accessory conditional use to the hostel use subject to the following conditions: (98-26)

a. The land area of the hostel site shall be a minimum of four (4) acres excluding any wetland areas contained therein. (98-26)

b. The maximum number of tents per acre shall be eighteen (18).  Land area of the hostel site great than one-half acre but less than an acre shall have a maximum density of nine tents and a portion of the site area less than one-half acre shall not be included in the density calculations for the tent camping use. (98-26)

c. All applicable health department requirements must be satisfied and evidence of department approval and/or necessary permits shall be furnished to the town prior to town approval. (98-26)

d. One parking space per tent shall be provided on site and shown on the site plan. (98-26)

e. The tent camping use shall be screened from adjoining streets and properties with an opaque barrier no less than six feet in height or , upon recommendation of the planning board, a barrier consisting of bushes and trees.  The hostel owner shall maintain the barrier around the tent camping use as long as the use is conducted on the site.  The standard of maintenance is the degree of maintenance necessary to prevent the tent camping use from being visible from a height of six feet above the surface of the adjoining properties or street. (98-26)

f. Campfires at campsite are prohibited; however, one open fire at a location designated on the site plan and approved by the town is authorized provided the fire is maintained in accordance with the Kitty Hawk Fire Department regulations and pursuant to a current permit for the open fire issued by the Kitty Hawk Fire Department. (99-26)

g. A surface water drainage plan designed to contain surface water run off on the site shall be submitted to the town for review and town approval. (98-26)

h. A central garbage collection area must be shown on the site plan.  It must be adequate in container type and size to contain the garbage and trash generated by the tent camping use. (98-26)

i. Trailers, campers, and motor homes or any type of over night accommodation on wheels shall be prohibited. (98-26)

(d) Dimensional requirements. (03-42)

(1)  The minimum lot size is as follows:

a. For single-family detached residences, fifteen thousand (15,000) square feet.

b. For duplexes, twenty-five thousand (25,000) square feet for each acre or portion thereof.

c. For multifamily dwellings, for each acre or portion thereof, fifteen thousand (15,000) square feet for the first dwelling unit and nine thousand five hundred twenty (9,520) square feet for each additional dwelling unit.

d. Marsh and wetland areas, as determined by CAMA and/or CRC regulations, which are contiguous with estuarine waters, sounds and bays, and areas waterward of the oceanfront vegetation line, as determined by CAMA regulations, may not be used for the minimum lot size. (04-01)

(2) The minimum lot width is seventy-five (75) feet. The minimum lot width shall be measured at the minimum front building set back line as set forth in each zoning district in the code.

(3) The minimum yard setbacks shall be in accord with the following chart:

Front yard	Side yard	Rear yard

house size:		required:	required:	required:

≤ 2500			30 ft.		10 ft.		30 ft.

2501-3000		30 ft.		12.5 ft.		30 ft.

3001-3500		35 ft.		15 ft.		35 ft.

3501-4000		35 ft.		17.5 ft.		35 ft.

4001-5000		40 ft.		20 ft.		40 ft.

5001+			40 ft.		25 ft.		40 ft.

(4) The maximum allowable lot coverage by principal use and all accessory structures is thirty (30) percent.

(5) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials. The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet. (04-26)

(6) The minimum of 65% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Density.

1. The maximum density shall be four (4) residential dwelling units per acre.

2. Planned Unit Developments (PUD’s) may be granted with a maximum density of five (5) residential units per acre when an on-site, community wastewater system is provided and the number of bedrooms per residential unit does not exceed four (4) bedrooms. (04-03)

(f) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(C); Ord. No. 87-1, 1-19-87; Ord. No. 90-3, 3-19-90; Ord. No. 90-21, 9-4-90; Ord. No. 91-15, § 5, 10-21-91)

### Sec. 20-165.	VC-1 village commercial district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VC-1 village commercial district. The VC-1 district is established to provide an area where commercial uses are permitted as accessory uses to residential uses, which are to be the primary use. The accessory commercial use must be of and by a person residing upon the lot or parcel upon which the accessory use is made. The residential occupant must be the owner and on premises supervisor of the commercial use. Employees may reside off the premises. (99-4)

(b) Permitted uses. The following uses shall be permitted by right as an accessory use to a primary residential use:

(1) Offices:

a. Business.

b. Financial.

c. Professional.

(2) Retail stores:

a. Books.

b. Cameras.

c. Candy.

d. Flowers.

e. Gifts.

f. Hobby goods.

g. Jewelry.

h. Leather goods.

i. Sporting goods.

j. Toys.

k. Antiques/used furniture.

l. Household appliances.

(3) Service establishments:

a. Barber and beauty shops.

b. Churches.

c. Shoe repair.

d. Small engine and appliance repair.

(4) Detached single-family dwellings and duplexes in accordance with the dimensional requirements in this section. Mobile homes are permitted provided they are skirted, placed on a foundation and anchored according to the North Carolina State Building Code for mobile homes in hurricane areas and in accordance with the standards established by the federal Mobile Home Construction Safety Standards Act of 1974 for single family manufactured homes or the Standards for Installation of Mobile Homes adopted by the state commissioner of insurance, whichever is applicable. Single family dwellings shall comply with the dimensional requirements of the VR-1 District. (03-35)

(5) Town-owned and leased facilities.

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Print shops may be permitted subject to other requirements of this chapter and provided the following conditions are met:

a. No more than eight hundred (800) square feet may be used for printing purposes excluding office area.

b. The principal building shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line. An architect, engineer or other qualified professional must certify on the site plan that the proposed design and materials will allow no machine noise originating within the building to be heard at any point on the approved site boundary.

c. In the event any chemicals used or stored on the premises require approval by a governmental agency of the method such chemicals are stored, used or disposed, then written evidence of the approval by the appropriate governmental agency shall be furnished to the town.

d. A list of all chemicals used or stored on the site shall be filed on or before January 10 of each year with the volunteer fire department.

(2) Medical office.

(3) On-site retail sales and production of bakery goods, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of bakery goods must be incidental to and for the purpose of on-site retail sales and not for distribution to wholesale vendors.

b. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the bakery production activity.

c. The owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(4) Craft production and retail sales, subject to any reasonable conditions imposed by the town council and the following conditions:

a. The production of crafts must be incidental to and for the purpose of retail sales and not for distribution to wholesale vendors.

b. The maximum square footage of the area devoted to craft production shall not exceed eight hundred (800) square feet.

c. The building in which crafts are produced shall be constructed in such manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof used for craft production) if the same is located within a multiunit structure. An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the craft production activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for craft production if the same is located within a multiunit structure).

(5) The retail sale of fruits, vegetables, flowers, containerized house or bedding plants, herbs and other agricultural food products, such as jam, jelly, vinegar, eggs, cheese or honey subject to the following terms and conditions: (05-03)

- No produce, plants or related products shall be located within the Town or State right-of way or in designated parking spaces or fire lanes;

- The activity may not utilize on-site required parking spaces to conduct retail activities or storage;

- No sales shall be conducted after 10:00 p.m. and prior to 7:00 a.m.;

- No music shall be produced on the site which music is audible beyond the property lines of the lot on which the sales are being conducted;

- Any signs must meet current Town code standards and applicable sign permit requirements;

- No additional lighting shall be allowed on the site;

- Trailers or vehicles used to transport produce and materials to the site shall be concealed from observation from Town streets and State rights-of-way;

- Prior to issuance of a conditional use permit the applicant shall present a scaled site plan illustrating the location of all vendor display and storage area, pedestrian circulation areas and ways, and location of required parking;

- Any structures must meet wind construction standards and tie down standards per North Carolina Building Code;

- One (1) paved off street parking stall shall be required for each 300 square feet of occupied retail space. For purposes of this section, retail space shall be that area occupied for the display, sale or storage of goods;

- The required parking shall not be allowed in public or private rights-of-way, fire lanes or travel lanes designated on approved site plan;

- Any tent occupying over 200 square feet of the site shall require approval from the Fire Chief;

- Sanitary facilities shall be required in manner consistent with the International Building Code for a similarly sized retail space and shall be located on the subject property and within 200 feet of the market;

- On-site garbage or trash receptacles must be provided and properly maintained at all times.

(6) Minor home repair and maintenance.

(7) Home occupation (subject to the provisions of section 20-421).

(8) Family Child Care in residential structure (00-33) (04-11)

a. The property owner must be legally operated and obtain a licensed by the NC Department of Health and Human Services – Division of Child Development if required by law, and must be affiliated with the child care resource and referral program in Dare County or a subsequent similar program of the county.

b. The maximum number of children in the home pursuant to this use shall not exceed the state licensing regulations.

c. Adequate parking and/or “pickup/drop off” space shall be provided on site.

(9) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(10) Except for the subdivision known as Kitty Hawk Landing Subdivision and that portion of the Kitty Hawk Bay Soundfront running from Hay Pointe to the point of intersection of the Beach Residential (BR-1) zone district with the Village Residential -1 (VR-1) zone district in the vicinity of Penney Toler ditch, commercial crabbing and fishing operations conducted by land owners (as well as their family members and employees as permitted herein) who own land fronting on and adjoining the navigable canals and waterways in the town shall be conditionally authorized to conduct commercial crabbing and fishing operations on their property fronting and adjoining said navigable canals and waterways (including tributaries thereto) including maintaining and using shedding boxes, upon the following conditions and such other reasonable conditions as the town may impose: (01-1)

a. Seafood shall not be sold, processed or distributed on the premises. (01-1)

b. The cleaning of fish or other seafood for commercial purposes is prohibited.

c. Any exterior or outside lighting used in the commercial crabbing and fishing operation shall be low profile, shielded and directed toward the operations, and to the extent practical, away from adjoining neighbors.

d. The maximum number of crab shedding boxes permitted on the premises shall be thirty-six (36) and the shedding boxes shall not exceed a size of four (4) feet by eight (8) feet.

e. While engaging in the conditional use authorized by this paragraph, no radio, musical, loudspeaker, or other such sound shall be audible beyond the property line of the landowner engaging in the authorized conditional use.

f. No more than four (4) people shall be involved in or conducting the authorized conditional use on the property at any one  time. (01-1)

g. Only boats owned and operated by the landowner or his family may be loaded and unloaded at the property site, and not more than four (4) boats shall be docked at the property or loaded and unloaded as authorized by this section.

h. The equipment and machinery used for commercial crabbing or fishing purposes as authorized herein shall be screened as the town council, in its discretion, may require in order to minimize the commercial impact upon adjoining residential property owners.

i. Storage of crab pots and crab shedding boxes shall be screened from adjacent residential properties; (01-1)

j.  A CAMA permit shall be obtained if applicable. (01-1)

(11) Churches (04-27)

(d) Dimensional requirements.

(1) Lots shall be of sufficient size to meet requirements of the county health department, to provide adequate siting of structures, and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter; however all commercial lots shall have a minimum lot size of 15,000 square feet. (98-3) In addition, a ten-foot visual buffer consisting of trees and/or shrubs with a growth rate of approximately ten (10) feet within five (5) years shall be required where a commercial use abuts a residential use or district in accordance with article VI, division 5, section 20-510. (95-18)

(2) On any lot with an area of two (2) acres or less, on which there exists a residential dwelling, only one (1) accessory structure shall be permitted for commercial use.

(3) Lots for single-family detached residences shall be at least fifteen thousand (15,000) square feet.

(4) Lots for duplexes shall be at least twenty-five thousand (25,000) square feet.

(5) The minimum front yard is fifteen (15) feet.(06-12)

(6) The minimum side yard is ten (10) feet. No side yard is required if an accessory building is constructed with a common wall. (99-1)

(7) The minimum rear yard is twenty (20) feet.

(8) The maximum allowable lot coverage by the principal use and accessory structure is sixty (60) percent.

(9) Maximum total height shall not exceed thirty-five (35) feet from existing grade(04-26) exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(10) The secondary accessory commercial use shall not exceed an enclosed area greater than eight hundred (800) square feet.

(11) The minimum of 35% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Commercial site plan review. Any person desiring to construct or enlarge a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this chapter.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit.

(g) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(D); Ord. No. 89-21, 8-21-89; Ord. No. 89-25, 11-21-88; Ord. No. 90-5, § 6(p), 4-23-90; Ord. No. 90-21, 9-4-90; Ord. No. 91-13, § 7, 10-7-91; Ord. No. 95-18, 9-5-95)

### Sec. 20-166.	VC-2 commercial district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VC-2 commercial district. The VC-2 district is established to provide for the development of commercial facilities in Kitty Hawk Village to furnish a broad range of services and commodities to serve the entire community.

(b) Permitted uses. The following uses shall be permitted by right:

(1) All permitted uses allowed within the VC-1 village commercial district. Single family dwellings shall comply with the dimensional requirements of the VR-1 District. (3-35)

(2) Cabinet and woodworking shops.

(3) Town-owned or leased facilities.

(4) Hardware, including rental of small equipment (no outdoor display or storage). (3-36)

(5) Medical equipment & supplies – sales and rental. (3-36)

(6) Rental of household/cottage furnishings & equipment and beach equipment. (3-36)

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Home occupations.

(2) Public utility facilities.

(3) Greenhouses and plant nursery.

(4) Mini-storage facility: In the event that more than one building on the site or lot is to be used for mini-storage use, then all separate buildings on site must be connected as one principal building before a different use can be conducted on the site, and an amended site plan must be approved by the town. (02-02)

(5) Private schools.

(6) Boat sales, provided that any outdoor display must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510.

(7) Marinas and docks.

(8) On-site retail sales and production of bakery goods, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of bakery goods must be incidental to and for the purpose of on-site retail sales and not for distribution to wholesale vendors.

b. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the bakery production activity.

c. The owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(9) Craft production and retail sales thereof, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of crafts must be incidental to and for the purpose of retail sales and not for distribution to wholesale vendors.

b. The maximum square footage of the area devoted to craft production shall not exceed eight hundred (800) square feet.

c. The building in which crafts are produced shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof used for craft production) if the same is located within a multiunit structure. An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the craft production activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for craft production if the same is located within a multiunit structure).

d. All odors and vapors (including chemical vapors) associated with or generated by the production of crafts shall be contained within the boundaries of the site or within the building (or that portion of the building used for the production of crafts in a multiunit structure). When requested by the planning board, an engineer's certification shall be furnished to the planning board that the design, construction, and proposed methods of odor and vapor control are sufficient to contain all odors and vapors generated by or emanating from the production of crafts within the site boundary (if a freestanding building) or within the interior of that portion of the building used for the production of crafts in a multiunit structure.

e. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the craft production activity.

f. If flammable materials are used in the craft production activity, the owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(10) Building contractor's offices and storage area, provided that any outdoor storage area is completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510.

(11) Commercial crabbing and fishing operations conducted by land owners (as well as their family members or employees as permitted herein) who own land fronting on or adjoining the navigable canals and waterways in the town shall be conditionally authorized to conduct commercial crabbing and fishing operations on their property fronting and adjoining said navigable canals and waterways, including maintaining and using shedding boxes, upon the following conditions and such other reasonable conditions as the town may impose: (01-2)

a.  The cleaning of fish or other seafood for commercial purposes is prohibited. (01-2)

b. Any exterior or outside lighting used in the commercial crabbing and fishing operation shall be low profile, shielded and directed toward the operations, and to the extent practical, away from adjoining neighbors. (01-2)

c. The maximum number of crab shedding boxes permitted on the premises shall be thirty-six (36) and the shedding boxes shall not exceed a size of four (4) feet by eight (8) feet. (01-2)

d. While engaging in the conditional use authorized by this paragraph, no radio, music, loudspeaker, or other such sound shall be audible beyond the property line of the landowner engaging in the authorized conditional use. (01-2)

e. Only boats owned and operated by the landowner or his family may be loaded and unloaded at the property site, and not more than four (4) boats shall be docked at the property or loaded and unloaded as authorized by this section. (01-2)

f. The equipment and machinery used for commercial crabbing or fishing purposes as authorized herein shall be screened as the town council, in its discretion, may require in order to minimize the commercial impact upon adjoining residential property owners.  (01-2)

g. Storage of crab pots and crab shedding boxes shall be screened from adjacent residential owners. (01-2)

h. A CAMA permit shall be obtained if applicable. (01-2)

(12) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(13) Major Communication Tower: (02-04)

a.  Each freestanding town shall have a minimum setback from all property lines of a distance equal to twenty percent (20%) of the total height of the tower and the antenna thereon.  Minimum setbacks from all property lines for guy wire towers shall be thirty (30) feet measured in a straight line from the property lines of the guy wire anchor.

b.  A security fence at least ten (10) feet in height shall be installed to encompass the base of the tower and accessory building.

c.  The applicant shall submit an engineering report and site plan that contains at least the height of the tower and antenna and the building materials to be used in the tower, the number of proposed antenna and locations.  The report shall also include certification from a structural engineer verifying that the tower structure will withstand one-hundred ten (110) mile per hour winds and will support the proposed number of antennas.

d.  Written verification that the proposed tower complies with federal regulations administrated by the Federal Communications Commission (FCC) and the Federal Aviation Administration (FAA) shall be provided to the town before construction commences. (02-04)

(14) Churches (04-27)

(15) Apartments on the second story of compatible commercial uses, provided that applicable state building and fire codes are met and that one (1) parking space per bedroom be reserved for the occupants of the apartment in addition to any other parking spaces that may be required. (05-20)

(d) Dimensional requirements.

(1) Commercial lots shall be of sufficient size to meet requirements of the county health department, to provide adequate siting of structures, and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter; however all commercial lots shall have a minimum lot size of 15,000 square feet. (98-3) In addition a visual buffer is required where a commercial use or zone abuts a residential district in accordance with article VI, division 5, section 20-510. (95-18)

(2) The minimum front yard is fifteen (15) feet.

(3) The minimum side yard is ten (10) feet, except that zero lot lines are permitted where access is provided to the rear of each commercial building or use with a zero lot line from a public street or highway. This exception shall not apply to corner lots. Corner lots shall have an additional five-foot side yard adjacent to the street.

(4) The minimum rear yard is twenty (20) feet.

(5) The maximum allowable lot coverage by the principal use and all accessory structures is sixty (60) percent.

(6) Maximum total height shall not exceed thirty-five (35) feet from existing grade (04-26) exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(7) The minimum of 35% of the site shall remain in natural or man-made landscaped open space. (03-26)

(e) Commercial site plan review. Any person desiring to construct or enlarge a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this chapter.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina State Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit.

(g) Ground stabilization plan. A plan that will assure stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II, and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(E); Ord. No. 89-25, 11-21-88; Ord. No. 90-21, 9-4-91; Ord. No. 95-18, 9-5-95; Ord. No. 96-18, 9-9-96)

### Sec. 20-167.	VC-3 village commercial district.

(a) Scope and intent. Unless stated otherwise, this section applies to the VC-3 village commercial district. The VC-3 district is established to provide for the development of commercial and light industrial facilities and uses in Kitty Hawk Village to better furnish a broad range of services and commodities to serve the entire community including but not limited to such facilities as commercial laundry, building supply facilities, construction equipment storage and servicing, manufacture, production and marketing of concrete and concrete products and other similar uses.

(b) Permitted uses. The following uses shall be permitted by right:

(1) All permitted uses in a VC-2 commercial district.

(2) Commercial dry cleaning and laundry.

(3) Public utility facilities.

(4) Town-owned or leased facilities.

(5) Mobile homes as provided for in the VR-2 district. Single family dwellings shall comply with the dimensional requirements of the VR-1 District. (3-35)

(c) Conditional uses. The following uses are permitted subject to the requirements of this district and additional regulations and requirements imposed by the town council as provided in section 20-56:

(1) Docks and marinas.

(2) On-site retail sales and production of bakery goods, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of bakery goods must be incidental to and for the purpose of on-site retail sales and not for distribution to wholesale vendors.

b. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the bakery production activity.

c. The owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(3) Craft production and retail sales thereof, subject to any reasonable conditions imposed by the town council and the following specific conditions:

a. The production of crafts must be incidental to and for the purpose of retail sales and not for distribution to wholesale vendors.

b. The maximum square footage of the area devoted to craft production shall not exceed eight hundred (800) square feet.

c. The building in which crafts are produced shall be constructed in such a manner and with such materials as to prevent any noise originating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof used for craft production if the same is located within a multiunit structure). An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the craft production activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for craft production if the same is located within a multiunit structure).

d. All odors and vapors (including chemical vapors) associated with or generated by the production of crafts shall be contained within the boundaries of the site or within the building (or that portion of the building used for the production of crafts in a multiunit structure). When requested by the planning board, an engineer's certification shall be furnished to the planning board that the design, construction, and proposed methods of odor and vapor control are sufficient to contain all odors and vapors generated by or emanating from the production of crafts within the site boundary (if a freestanding building) or within the interior of that portion of the building used for the production of crafts in a multiunit structure.

e. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the craft production activity.

f. If flammable materials are used in the craft production activity, the owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(4) Contractor's supplies and storage areas, provided that any outdoor storage must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510.

(5) Plumbing, heating and mechanical contractor's supplies and sales, provided that any outdoor storage must be completely screened from the public's view from public and private rights-of-way or the ground level of adjacent properties according to standards in section 20-510.

(6) Marketing of concrete and concrete products and wood products (to exclude salt treating, chemically treating woods, creosote treatment of wood). (03-04)

(7) Storage of frozen seafood products. (94-18)

(8) Vehicle paint shop and body shop to VC-3 district, subject to any reasonable conditions imposed by the town council and the following specific conditions: (95-2)

a. The building in which the activity is taking place shall be constructed in such a manner and with such materials as to prevent any noise generating within the facility from being heard beyond the approved site boundary line if the same is a freestanding building and to prevent any noise emanating from within the building (or portion thereof for body repair/painting if the same is located within a multiunit structure). An architect, engineer or qualified professional must certify on the site plan that the proposed design and materials will allow no noise from the shop activity within the building to be heard at any point on the approved site boundary or outside of the building (or that portion of the building used for shop activities if the same is located within a multiunit structure).

b. All odors and vapors (including chemical vapors) associated with or generated by the shop activities shall be contained within the boundaries of the site or within the building (or portion of the building used for the shop activities in a multiunit structure). When requested by the planning board, an engineer's certification shall be furnished to the planning board that the design, construction, and proposed methods of odor and vapor control are sufficient to contain all odors and vapors generated by or emanating from the shop activities within the site boundary (if a freestanding building) or within the interior portion of the building used for the production of crafts in a multiunit structure.

c. The property owner must provide a waste container of sufficient size to accommodate all of the waste generated by the shop activity.

d. If flammable materials are used in the shop activity, the owner shall obtain and maintain firefighting apparatus or fire extinguishing apparatus in accordance with the requirements of the planning board or the volunteer fire department.

(9)  Min-warehouses subject to the following specific conditions and such other reasonable conditions as may be imposed by the town council: (99-27)

a.  A visual buffer consisting of plant species approved by the town or materials approved by the town.  The visual buffer shall be maintained by the owner such that the warehouses are not visible from adjoining properties or rights of way.

b.  The project site will be surrounded by  a metal security fence no less than six (6) feet in height with one or more gates for access to the site.

c.  Exterior lights shall be designed and erected to retain the light within the site boundary while providing sufficient light to use the premises at night and provide security for the customers.  Exterior lights shall be shielded and directed downward and into the site.  Exterior lights shall not interfere with motor vehicle operators on streets or highways. (99-27)

(10) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(11) Churches (04-27)

(d) Dimensional and development requirements.

(1) All uses within a VC-3 district which are permitted uses in the VC-2 village commercial district shall conform to the dimensional requirements set out for the VC-2 village commercial district. Maximum total height shall not exceed thirty-five (35) feet from existing grade (04-26) exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(2) Except as provided in paragraph (1) of this subsection, permitted and conditional uses within a VC-3 district shall meet the following standards:

a. No portion of a building or open storage or processing area shall be closer than seventy-five (75) feet to a residential district boundary.

b. Individual lot sizes for the VC-3 district shall not be less than one (1) acre in size.

c. No use shall be permitted in a VC-3 district which has noxious, harmful or deleterious effect on other development.

d. The maximum allowable lot coverage by principal use and all accessory structures is sixty (60) percent.

e. The off-street parking requirements in article VI, division 2 of this chapter shall apply except that no off-street parking or loading space shall be located closer than fifty (50) feet to a residential district boundary.

f. Ingress and egress shall be only from and to a state-maintained road.

g. Buffer shall be required in accordance with article VI, division 5, section 20-510.

h. The minimum of 35% of the site shall remain in natural or man-made landscaped open space. (03-26)

(3) On tracts and parcels of land which are 15 acres or greater in areas and which are planned, reviewed and approved to be developed as one complete project, then the following standards and conditions must be satisfied for approval as a multiple building site in addition to the standards provided by paragraphs a. and b. below: (99-5)

a. More than one principle building per project or development site is authorized under paragraph (3) hereof provided the maximum number of buildings shall not exceed one building per acre.  A portion of an acres shall not be used in calculating the building density. (99-5)

Accessory buildings shall not be allowed except upon recommendation of the Planning Board and approval of the Town Council. (99-5)

In multiple building development sites, the following additional standards shall apply:

1)In addition to the VC-3 building setbacks, building setbacks within a multiple building development site must have a minimum separation distance of twenty (20) feet from each building within the development site. (99-5)

2) Where the boundaries of a multiple building development site are contiguous with the boundaries of two or more publicly maintained rights of way, then the development site may have a free standing sign on two of the publicly maintained rights of way provide not more than two said signs shall be authorized.  The free standing sign standards must be satisfied by each free standing sign and the free standing sign must be a minimum distance of 500 feet apart. (99-5)

3) No structure or piece of equipment shall exceed the height of thirty-five feet (35)from existing grade. (99-5) (04-26)

4) Each building shall have a user identification sign affixed to the building and shall be visible to the parking area. (99-5)

5) The screening and berm standard adopted by the Town must be satisfied or any screening or berm standards imposed as a condition of approval by the Town Council must be satisfied. (99-5)

6) In the event of a conflict between a VC-3 standard or other standard of this ordinance and specific standard in this sub-paragraph (3) then the standard in this sub-paragraph shall apply. (99-5)

(e) Commercial site plan review. Any person desiring to construct or enlarge a structure for a commercial use or to support a commercial use on the same site and any person desiring to change the authorized commercial use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this chapter.

(f) Existing structures; use change; building code. Any building constructed in a commercial zone for a commercial use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from a noncommercial to a commercial use the owner must obtain an occupancy permit before making a commercial use of the building. The structure must satisfy all North Carolina State Commercial Building Code standards and requirements applicable to commercial structures for the commercial use proposed at the time of the application of the occupancy permit.

(g) Ground stabilization plan. A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. of 8-18-86, § 3.02(F); Ord. No. 87-3, 1-19-87; Ord. No. 87-4, 1-19-87; Ord. No. 89-25, 11-21-88; Ord. No. 90-3, 3-19-90; Ord. No. 90-21, 9-4-90; Ord. No. 94-18, 9-6-94; Ord. No. 95-2, 2-6-95; Ord. No. 95-18, 9-5-95)

### Secs. 20-168--20-180.  Reserved.

## DIVISION 4.  REMEDIES

### Sec. 20-181.	Remedies.

- Purpose.  The purpose of this code provision is to enumerate those powers conferred to the Town by G.S. § 160A-175 and § 160A-389 for violation of any ordinance found in Chapter 20 of the Town Code.  (02-14)

- Remedies.  If a building or structure is erected, reconstructed, altered, repaired, converted or maintained, or any building or structure or land is used in violation of this chapter or of any ordinance or other regulation made under authority conferred thereby, for the purpose of preventing the unlawful erection, construction, reconstruction, alteration, repair, conversion, maintenance or use, to restrain, correct or abate the violation, to prevent occupancy of the building, structure or land, or to prevent any illegal act, conduct or business use in or about the premises, the town may institute the following action or actions: (02-14)

  - Civil Penalties.   A violation of a zoning ordinance shall be punishable by a civil penalty not exceeding five hundred dollars ($500.00) per violation for each day or portion thereof that the violation continues after the issuance of notice of the violation has been given.  Notice of a violation shall be given pursuant to § 20-181 (c) of this provision.  Each day shall constitute a separate and distinct violation.  Unless otherwise provided, the following penalties shall be assessed:

    - Upon failure of an offending party to obey the warning citation after receipt of an official citation giving notice of a violation of an ordinance under this chapter, a penalty of one hundred dollars ($100.00) per day, per violation for ten (10) successive days shall be assessed.

(ii)	In the further event that the initial penalties are disregarded and the offending party fails to make corrective measures to remedy the violation and / or fails to remit any penalties enumerated under (i) above, the penalty shall increase to five hundred dollars ($500.00) per day, per violation.  (02-14)

- Notwithstanding provision (i) and (ii) above, violations of an order to vacate a structure based on an order of condemnation and revocation of an occupancy permit shall automatically subject the offending party to a civil penalty in the amount of five hundred dollars ($500.00) per day, per violation. (02-14)

The civil penalty assessed for a violation of an ordinance in this chapter shall be recovered by the town in a civil action in the nature of a debt if the offender fails to pay the penalty within a proscribed period of time after the issuance of a civil citation. (02-14)

  - Injunctions and Orders of Abatement.   The town may enforce the zoning code by injunction and/or order of abatement, and the general court of justice shall have jurisdiction to issue such orders.  The town may apply to the appropriate division of the General Court of Justice for a mandatory or prohibitory injunction and order of abatement commanding the offending party to correct the unlawful condition or cease the unlawful use of the property.  The action shall be governed by the laws and rules governing civil proceedings, including the Rules of Civil Procedure.  The court may enter an order of abatement as a part of judgment and may command that:

    - Buildings or structures on the property be closed, demolished or removed;

    - Fixtures, furniture or other movable property may be removed from buildings on the property;

    - Grass and weeds be cut;

    - Improvements or repairs be made; or

    - Any other action be taken that is necessary to bring the property into Compliance with this Code or such ordinance.

If the defendant fails or refuses to comply with an injunction or order of abatement within the time specified by the court, he may be cited for contempt, and the town may execute the order of abatement.   The town shall have a lien on the property for the cost of executing an order of abatement in the nature of a mechanic’s and materialman’s lien.  The defendant may secure cancellation of an order of abatement by paying all costs of the proceedings and posting a bond for compliance with the order.  The bond shall be given before whom the matter is heard and shall be conditioned on the defendant’s full compliance with the terms of the order of abatement within a time fixed by the judge.  Cancellation of an order of abatement shall not suspend or cancel an injunction issued in conjunction therewith.

  - Criminal Penalties.  Except as otherwise provided for in this section, any person in violation of any ordinance provided for in Chapter 20 of this Code shall be guilty of a Class 3 misdemeanor pursuant to N.C.G.S.§ 14-4(a) and shall be fined not more than five hundred dollars ($500.00).  Violation of a stop work order shall constitute a Class 1 misdemeanor pursuant to N.C.G.S. §160A-421 (d).

- Determination of a Violation.   All violations shall be determined by the Town’s Code Enforcement Officer or other applicable official designated by the town pursuant to the provisions provided in §§ 20-21 and 20-22 of the town code.

- Notice. Upon determination of a violation of any section of this Code, the penalty for which is a civil penalty, a warning citation shall be issued by the Codes Enforcement Officer or any other person designated by the town council to perform such duties.  The warning citation shall set out the nature of the violation, the date of the violation, and an order to immediately cease the violation.  If the violation is in the nature of an infraction for which an order of abatement would be appropriate for a civil proceeding, the warning citation shall state a reasonable period of time in which the violation must be abated.  Any warning citation shall specify that a second citation shall incur a civil penalty.

Upon failure of the offending party to obey the warning citation, a civil citation shall be issued by the appropriate official of the town and served directly on the violator or his duly designated agent, or registered agent if a corporation, in person or posted in the United States mail service by first class mail addressed to the last known address of the violator as contained in the records of the town, or obtained from the offending party at the time the warning citation was issued.  The offending party shall be deemed to have served with notice upon the mailing of the civil citation.   The civil citation shall direct the offending party to appear before the town clerk located in the Town of Kitty Hawk Municipal Center at 101 Veteran’s Memorial Drive, Kitty Hawk, North Carolina within ten (10) days of the date of the citation or alternatively to pay the citation by mail.  The violation for which the citation was issued must have been corrected by the time the citation is paid, and the offending party shall be informed that further citations shall be issued until such violations are corrected.  Citations may be issued for each day the offending violation continues until the prohibited activity ceases or abates.

If the violator fails to respond to a citation within ten days (10) of its issuance and pay the penalty prescribed therein, the town may institute a civil action in the nature of a debt in the appropriate division of the North Carolina Court of General Justice.

- Appeals.   Any person charged with a violation of those ordinances enumerated in Chapter 20 of the town code shall have as a matter of right to appeal any decision by the Code Enforcement Officer to the Board of Adjustment by those processes enumerated in N.C.G.S. § 160A-338. (02-14)

### Secs. 20-182--20-200.  Reserved.

# ARTICLE V.  SPECIAL DISTRICTS, PUDS, OVERLAY DISTRICTS, ETC.

## DIVISION 1.  GENERALLY

### Secs. 20-201--20-215.  Reserved.

## DIVISION 2.  EXTRATERRITORIAL AREA*

### Sec. 20-216.	Sound waters district.

(a) Intent. The sound waters district is established to provide the proper use of sound waters, including islands, that adjoin the Town of Kitty Hawk to assure the continued scenic, conservation, environmental and recreational value that these waters provide to the town, its residents, visitors and the surrounding area.

(b) Permitted uses. The following are permitted uses in this district:

(1) Swimming, boating, sailing, fishing, crabbing, shrimping, hunting and eel fishing. (99-25)

(2) Marine bulkheads, provided all appropriate government permits have been obtained. (99-25)

(c) Conditional uses.

(1) Commercial recreational activities which are approved as conditional use as hereinafter set forth provided that the use is authorized in the zoning district in which it is located.  The following are authorized commercial recreational uses: (99-25)

(TO BE LISTED)

(2) Private, community or commercial piers and boat slips subject to continuing compliance with the following requirements: (99-25)

a.  That all appropriate governmental and regulatory agency permits have been issued for the facility and use;

b. That land activities of the marine use must be an authorized use within the zoning district where the land activities associated with the marine use will be conducted;

c.  A site plan of the land and water areas to be utilized for the use or uses depicting the location of all proposed and existing improvements must be reviewed and approved by the town.  The site plan must conform with the general site plan standards of the town code together with the additional standards of this section.

d.  The pier or marine improvements shall not extend further waterward of the mean high water mark of the bay or sound than provided as follows:

Private Pier and Boat Slips		100 feet

Community Pier and Boat Slips	200 feet

Commercial Pier and Boat Slips	200 feet

e.  Light fixtures and lighting. Private and community piers, and commercial piers on Kitty Hawk Bay as well as boat slips used with the piers shall have and maintain lights or reflectors to warn watercraft operators of the length and location of the pier and boat slips.  A minimum length of fifteen (15) feet shall be maintained between lights or reflectors along the sides of the pier and not more than two lights shall be located at either end of the pier.

Except as provided above lights shall be attached to or on the pier or boat slips.

f. Devices used to create noise or amplified sound shall not be attached to or used on the pier.

g. Group social or party activities shall be prohibited on the pier between the hours of 8:00 p.m. in the evening through and including 8:00 a.m.

h. Covered boat slips are prohibited.

i.  Community piers shall be used by the property owners in the multifamily project or subdivision, their guest, or vacation tenants during the rental period of their lease, and community and private piers shall not be open to the general public.

j.  No transit boats shall be left at a community pier for more than forty-eight (48 ) consecutive hours.

k. A boat dock locker may be located at each boat slip provided the locker does not exceed two (2) feet in height measured from the pier deck.

l.  No fuel dispensing facility shall be located on a pier.

m. Hunting and igniting fireworks shall be prohibited from the pier.

n. The owner of the pier shall place into effect and maintain casualty insurance in an amount equal to eighty (80) percent of the replacement cost of the pier with evidence thereof furnished to the town upon request.

o.  The pier and boat slips must be maintained in good and safe condition at all times.

p.  Damage from storms or other casualties must be repaired within six (6) months of the occurrence of the damage.  Abandoned piers or piers damaged to the extent of fifty (50) percent or more of its fair market value and which have been unrepaired for a period of nine (9) consecutive months, may be removed by the town and the cost of the removal charged to and assessed against the property owners or their association.

q. There shall be only one pier pre parcel or lot. (99-25)

(d) Prohibited uses. The following are prohibited uses in this district:

(1) Outdoor advertisement displays or signs.

(e) Nonconformities; exception. Piers exceeding one hundred (100) feet waterward of the mean high water mark of Kitty Hawk Bay or the sounds on the effective date of this ordinance, can be maintained, repaired, and replaced as needed notwithstanding any provision in the town code to the contrary.  The pier length, width, and height cannot be increased without prior town council and planning board approval.  A nonconforming pier authorized under this paragraph shall install and maintain lights or reflectors in accordance with Section (c) (2) e. of this section of the code. (99-25)

(Ord. No. 94-9, 6-27-94)

### Sec. 20-217.	Marine habitat and estuarine water district.

(a) Areas within the extraterritorial jurisdiction which have been identified as habitat for marine mammals, migratory fowl and wading bird species or used as nesting areas by shore and wading birds and migratory fowl; and

(b) Areas within the extraterritorial jurisdiction which have been identified as reproduction and nursery areas for fin fish and shellfish.

(c) Permitted uses. The following are permitted uses in this district:

(1) Research, conservation and educational activities;

(2) Fishing (both fin fish and shellfish) and crabbing;

(3) Hunting and trapping as provided by state law;

(4) Observation of wildlife and marine life.

(d) Prohibited uses. The following are prohibited uses in this district:

(1) Any use (whether recreational or commercial) involving the operation of a motor propelled watercraft at a speed higher than "no wake speed" within the district. Persons found guilty of violating this use prohibition shall be guilty of a misdemeanor and punishable as provided by section 1-5 of the Town Code.

(2) Outdoor advertising signs or displays.

(Ord. No. 94-9, 6-27-94)

### Sec. 20-218.	Recreational beach and swimming district.

(a) Permitted uses. The following are permitted uses in this district:

(1) Wading, swimming and other similar water activities;

(2) Launching and beaching of human powered and wind powered watercraft;

(3) Fishing.

(b)Prohibited uses. The following are prohibited uses in this district:

(1) Operation of motor propelled watercraft;

(2) Hunting and trapping;

(3) Outdoor advertising displays or signs.

(Ord. No. 94-9, 6-27-94)

### Secs. 20-219--20-230.  Reserved.

## DIVISION 3.  PLANNED UNIT DEVELOPMENT

### Sec. 20-231.	Generally.

(a) In this division "planned unit development" means the complete development of land which is under central control, or for which central control mechanisms have been established.

(b) The plan will be in accordance with such guides and objectives as may be established by the planning board and the town council.

(c) Except as otherwise provided, this division applies to planned unit developments and to issuance of building permits required for same.

(d) This division is intended to provide developers with an option by which they can achieve flexibility of design, the integration of mutually compatible uses, and optimum land planning with greater efficiency, convenience, and amenity than may be permitted as of right under the other parts of this requirement. A planned unit development is a privilege, not a right.

(e) Within specified districts created by this chapter it is intended to permit, on application and on approval of detailed site, use, building and development plan, establishment of planned unit developments in areas which are suitable with respect to location, size, and physical character for development as units. Suitability of such tracts for the planned unit development proposed shall be determined primarily by reference to the goals and objectives to the land use plan, and this ordinance, by the physical characteristics of the site and area, and by the nature of the surrounding development. This division is intended to accomplish the purpose of zoning and subdivision control to the same degree as those regulations are intended to control development on a lot-by-lot rather than unified basis, and to promote economical and efficient land use, a higher level of amenities, appropriate and harmonious variety in physical development, creative design, and an improved living and working environment.

(f) In view of the substantial public advantages of planned unit development, it is the intent of this division to promote and encourage development in this form where appropriate in location and character.

(Ord. of 8-18-86, §§ 4.02, 4.02(A), 4.02(B), 4.02(C))

### Sec. 20-232.	Relationship of division to chapter and other ordinances.

Where there are conflicts between this division and other parts of this chapter, or chapter 19, this division shall apply unless the town council shall find in the particular case:

- That the provisions of this division do not serve public purposes to a degree at least equivalent to such general zoning, subdivision or other regulations or requirements; or

- That actions, designs or solutions proposed by the applicant, although literally in accord with these special regulations, do not satisfy public purposes to at least an equivalent degree as the general regulations; provided, however, that where dwelling unit density, floor area and similar ratios have been established by other parts of this chapter the town shall not alter such ratios in a planned unit development.

(Ord. of 8-18-86, § 4.02(C))

### Sec. 20-233.	Where permitted.

Planned unit developments may be established as a conditional use where tracts suitable in location and character for the uses and structures proposed are to be planned and developed as units, according to the requirements and procedures set forth in this division. Planned unit developments shall be appropriately located with respect to intended functions, to the pattern and timing of development existing or proposed in the land use plan, and to public and private facilities, existing or clearly to be available by the time development reaches the stage where they will be needed.

(Ord. of 8-18-86, § 4.02(D))

### Sec. 20-234.	Approval procedures generally.

An application for planned unit development approval shall be submitted as for other conditional uses. Material submitted with the application or on request by the planning board or the town council shall include all plans, maps, studies, and reports which may reasonably be required to make the determination called for in the particular case, with sufficient copies for necessary referrals and records. All of the following shall be required:

(1) A map showing the proposed development in relation to its surrounding area defining the relative size and location of streets, utilities, schools, and commercial facilities expected to serve the area.

(2) A survey report covering soil conditions, drainage, topography, location and character of surface water, flora and fauna and other such information as may be required to determine if the site is suitable for planned unit development without hazards to occupants or adjoining properties.

(3) An overall preliminary development plan, which plan shall show:

a. Proposed major vehicular and pedestrian circulation systems.

b. Proposed land uses including residential densities and nonresidential uses.

c. Proposed plans and regulations for major reservations of land for parks, parkways, playgrounds, school sites and other public uses or facilities.

d. The relationship to existing land uses in the surrounding areas.

(4) Common areas. An area designated on the site development plan as a common area and on the subdivision plan as an area to be held in separate ownership for the use and benefit of residents occupying specified lots shown on such subdivision plan may be approved as part of such subdivision plan, provided that it meets the following requirements:

a. It shall be conveniently accessible to all residents of the development.

b. It shall be made available in its improved state as set forth on the site development plan in accordance with an approved time schedule.

c. It shall be maintained in accordance with an approved maintenance plan specifying what such maintenance shall consist of, whose responsibility it shall be, and assuring satisfactory execution of maintenance.

d. Provisions to insure its continuing availability shall be included in the deed to each parcel to be served by such common area.

e. Such other information as may be required by the planning board or the town council to determine the impact of the proposed development on the town.

(Ord. of 8-18-86, § 4.02(E))

### Sec. 20-235.	Site development plan.

(a) No building permits shall be issued for a development until the planning board and town council have approved site development plans and reports for the development as a whole. No structure or use other than as indicated in approved site development plans and reports shall be permitted.

(1) After the town council has approved the entire preliminary development plan the development maybe constructed in phases provided the following requirements are met:

a.  Each phase must meet all the requirements of the ordinance and the approved preliminary development plan, and no construction shall commence on any phase until all final development plans within the phase have been approved by the town council.

b.  The infrastructure improvements, including but not limited to, roads, utilities and wastewater treatment systems and all amenities and common facilities in the phase must be completed, inspected and satisfactory test results submitted to the town before the next approved phase of construction can commence.

c.  The time period for commencing and completing each construction phase shall be one (1) calendar year.

d.  All phases must be constructed in sequence unless the planning board approves a modification to the approved phase construction schedule.

(2) After construction of the infrastructure within the project or within a phase of the project , the applicant shall submit the following to the town:

a.  Test results confirming that all roads, driveways, or vehicular ways within the development have been constructed and paved to town street standards.

b.  Written evidence that the appropriate governmental agency has approved the design and construction plans and, upon completion of construction, certification from an engineer licensed in the state that the septic, water, or sewer system or utility system has been constructed in accordance with the approved plans and specifications.  Making a false certification under this paragraph shall be a misdemeanor and conviction of same shall be reported to the appropriate licensing board regulating the profession of the person so convicted.

(b) Approval of site development plans and reports shall be based on either of the following:

(1) Compliance with the provision applying at the time the land was proposed for the use, including applicable provisions of chapter 19, unless in conflict with this chapter, and related capital improvement requirements of the town as regards construction of physical improvements and bonding thereof and such specific modifications as were made by the town council in the approved action; or

(2) At the option of the applicant, with provisions applying currently. Upon approval of site development plans and reports, building permits shall be issued in the same manner as for building permits generally, provided that any requirements concerning the order or location in which building permits are to be issued in the particular development shall be observed. Except as provided in this division, final plans and reports approved shall be binding on the applicant and any successors in interest so long as planned unit development status applies to the land.

(c) After review of the site plan application by the planning board, the town council may permit changes when requested by the developer, but only on a finding that such changes are in accord with all regulations in effect at the time the site development plan or plat was approved, and in accord with the general interest and purposes of the comprehensive plan in effect at such time, provided that the applicant may elect to proceed in accord with the regulations and land use plan currently in effect. Changes other than as indicated above shall be made only by a new planned unit development application.

(d) Action in connection with approval of site development plans or changes in approved plans not requiring ordinance amendment are administrative, and do not require public notice and hearing but the planning board and town council may hold such hearings as deemed desirable in connection with such action.

(Ord. of 8-18-86, § 4.02(F)--(H))

### Sec. 20-236.	Expiration of time limits.

(a) If action required in a planned unit development is not taken within time limits set, the planning board shall review the circumstances and recommend to the town council one (1) of the following:

(1) Planned unit development approval for the entire area be continued with revised time limits.

(2) Planned unit development approval be continued for part of the area with revised time limits, and the remainder returned to conventional zoning control.

(3) Planned unit development approval be removed from the entire project.

(b) Such recommendations shall include proposals for appropriate action with respect to any legal instruments, dedications, contributions or guarantees in the case.

(Ord. of 8-18-86, § 4.02(I))

### Sec. 20-237.	Development plan review procedure.

(a) Generally. All development plans for a planned unit development will proceed as provided by ordinance for processing conditional use permits, with additional steps as outlined below:

(1) Preapplication conference (sketch plan proposal).

(2) Consideration and recommendation of the preliminary development plan by the planning board.

(3) Public hearing and approval by the town council as required by the council.

(4) Final approval of all detailed project development and construction plans for the project as a whole or a phased portion thereof by the planning board and the town council.

(5) Permits for the phase or phases of the planned unit development for which final development and construction plans have been approved by the town council shall be issued by the building inspector.

(b) Preapplication conference (sketch plan proposal).

(1) On request by applicants and upon payment of the appropriate fee, members of the planning board and the planner shall meet with the applicants to review the original application, including the developer's report, if submitted, and the sketch plan of the proposed planned unit development. The purpose of the preapplication conference shall be to assist in bringing the report, if submitted, and the sketch plan as nearly as possible into conformity with these or other land development regulations applying in the case, and to define special variations from application of provisions which would otherwise apply which seem justified in view of equivalent services or the public purposes of such provisions.

(2) In the course of a preapplication conference, any recommendation for changes shall be recorded in writing, and shall become part of the record in the case. All such recommendations shall be supported by stated reasons for the proposal for change. Applicants may, in writing, indicate their agreement to such recommendations, or their disagreement. If there is disagreement, applicants shall, in writing, indicate their reasons therefor. Responses by applicants shall also be included in the record.

(3) Sketch plans may be drawn in such manner as to minimize initial expense and encourage sufficient design flexibility to accommodate required changes, without undue hardship to the developer. All plans submitted shall be drawn to appropriate scale and shall show the locations of all lots, streets, drives, off-street parking areas and other pertinent features, together with building locations, if appropriate.

(4) A developer may submit preliminary plats in lieu of sketch development plans.

(c) Consideration and recommendation by planning board. Following the preapplication conference, the planning board shall review the application for planned unit development, which shall include preliminary development plan and reports or preliminary subdivision plans if submitted in lieu thereof.  The preliminary development plan shall consist of:

- The proposed name of the planned unit development.

- The location by legal description and survey showing boundary line and the total acreage encompassed thereby.

- The name and address of the owner and the applicant if different from the owner, together with the name and address of plan engineer, architect, or designer of the plan.

- A scaled plan at least one(1) inch to one hundred (100) feet, and including the date, north arrow and a vicinity map

- A detailed topographical map.

- A tabulation of the various land uses proposed, showing the total site area and the total floor area for each use, including the maximum proposed density for the preliminary plan as well as each phase plan.

- A tabulation of all natural or landscaped open areas shown on the plot plan including the square footage of each type.

- The location, widths, and names of all existing or prior platted streets, utility easements and right-of-way, buildings and structures, and municipal boundary lines within five hundred (500) feet.

- Existing water mains, culverts, and other underground facilities within the tract, indicating pipe sizes, grades, manholes, and location.

- A planned unit development internal traffic flow and pedestrian walkway plan and an internal parking area traffic flow plan.

- The location and arrangement of automobile parking spaces, aisles, bays, angle of parking, and location of handicapped spaces and ramps.

- The methods of separating vehicular and pedestrian circulation patterns and location of pedestrian access patterns to various pedestrian-oriented areas of the planned unit development from parking areas and public transportation stops or terminals.  Pedestrian ways shall be designed to accommodate bicycle use or, in the alternative, bike paths may be included as part of the vehicular and pedestrian flow plan.

- The location of vehicular drives, entrances, exists, acceleration and deceleration lanes, fire lanes and service lanes.

- The location of pedestrian entrances, exists and walkways.

- The drainage and stormwater management system, and sanitary disposal facilities.

- The location, height, and materials of walls, fences and screen plantings and buffer zones.

- The size, location and orientation of all signs.

- Where applicable, the applicant shall comply with all of the following:

  - Chapter 19, article II

  - A natural features site plan.  The natural features site plan shall identify and locate significant natural features within the project as well as demonstrate how those features will be utilized, preserved, or protected within the project site.  Significant natural features shall include, but not be limited to, dunes, ridge lines, swales, large trees, ditches, banks and rare or endangered plant and tree species indigenous to the area and identified on the schedule filed with the town.

  - CAMA rules and regulations.

  - Chapter 8.

- Where the planned unit development is to be phased, then a phased construction plan shall be filed indicating the boundaries of each construction phase and the sequence of the phased construction.

The planning board may waive the requirement that one (1) or more of the above requirements be depicted or shown on the development project plans if the planning board deems depiction of the item on the plans is unnecessary in order to assure satisfaction of the particular standard.

The planning board, or its representatives, when appropriate, shall seek the advice of the county health department, state department of transportation or other agencies as necessary, to accomplish a complete review of any development plans. Whenever the planning board determines that the characteristics of a proposed development should be modified to protect the occupants of such development, or the public interest, the board may recommend modifications in building location, driveway location or design, location of recreation areas or open spaces, lot sizes or other essential elements of any development plan. The planning board will recommend approval, or denial, of planned unit development applications. In its action, the planning board will reflect its views upon issues left unresolved in the preapplication conference. As required by this chapter, the planning board shall forward its recommendations to the town council in accordance with established procedures for conditional use permits.

(d) Action by town council. A public hearing advertised, as provided by ordinance for rezoning hearings, may be held before the town council, for any planned unit development proposed to be established in any appropriate zoning district. The town council may approve such application in accordance with all requirements; may include specific modifications of all requirements, as recommended by the planning board; may return the application to the planning board for further consideration of specific suggested changes; or may deny the application. Upon approval of the town council, the developer is required to submit final detailed project development and construction plans for the proposed PUD to the planning board, as provided in this section.

(e)Final approval of detailed plan by planning board and town council. Following approval of a proposed planned unit development preliminary development plan and the approval of all required rezoning actions, if any, by the town council, detailed plans for the planned unit development  as a whole or for phases thereof shall be submitted to the planning board by the developer; (provided, however, that a preliminary subdivision plat may be submitted in lieu of detailed plan proposals, if necessary information is submitted with such plat to show proposed building locations, or other features).  The final detailed development plan for the whole project or a phase shall consist of:

(1) The approved preliminary development plan.

(2) The width, grades and construction details of streets and roadways within the tract.

(3) Construction details of any proposed vehicular bridges.

(4) The location, arrangement and specific details of water lines, culverts, sewer lines and other underground facilities within the site indicating pipe sizes, grades manholes, etc.

(5) Location, arrangement and dimensions of automobile parking spaces, width of aisles, width and angle of parking spaces and handicapped ramps.

(6) Location, arrangement and details of service and delivery areas including the dimensions of truck loading and unloading spaces and doors.

(7) The ground cover, finished grades, slopes, banks and ditches.

(8) Architectural elevation sketches of proposed buildings, structures and signage.

(9) Buildings shall be planned with a common architectural theme or compatible themes for the entire project.

(10) Restrictive covenants.

(11)  A Soil Erosion, Sediment Control Plan.

(12) The rules, regulations and procedures of the appropriate regulatory agency or agencies must be satisfied with respect to wastewater treatment systems and any necessary permits therefore must be obtained.

Review of detailed plan proposals, or preliminary subdivision plats if submitted in lieu thereof, shall follow procedures established in chapter 19 for all developments, including those proposing private streets, private common open spaces or private lot access easements. All final detailed plans, or preliminary subdivision plans if submitted in lieu thereof, must be approved by the town council.

(f) Issuance of building permits. When a detailed development plan or final subdivision plat has been approved by the planning board and approved by the town council, it shall be so certified to the building inspector. Building permits shall be issued only for improvements and development on a planned unit development site which conforms to the approved  final phase or PUD development plan or subdivision plat and only after all other plans have been approved and any permits required by applicable town, state or federal law, rules, or regulations have been issued.

(g) Amendment to the preliminary development plan or final development plan. An amendment to the preliminary development plan or final development plan shall be required for any significant deviation in the construction and implementation of the project plans. The town planner shall determine whether a deviation is significant within the meaning of this paragraph; however, nay deviation which itself fails to meet the standards of this chapter or which results in a standard becoming unsatisfied as well as changes in the locational  layout of infrastructure, uses, additional structures, or additions to existing structures and changes in location of commercial sites other than minor adjustments of the building footprint shall be deemed to be significant deviations or modification requiring planning board and town council approval.

(Ord. of 8-18-86, § 4.02(J); Ord. No. 89-11, 4-17-89)

### Sec. 20-238.	Standards and requirements.

All planned unit developments shall conform to the following standards and requirements in addition to any other requirements of the town council and county, state, and federal regulations applicable to the planned unit development:

(1) Minimum size of site. A planned unit development site or each phase in a phased planned unit development shall not be less than ten (10) acres in size.

(2) Maximum unit density. The maximum density permitted in any planned unit development  or each phase in a phased planned unit development shall be the same as for the district in which it is located. If the planned unit development is in more than one (1) district, the district requirements shall apply to the portion of the planned unit development within that district.  Land not suitable for building shall not be included in calculating density.

(3) Number of buildings. More than one (1) principal building may be permitted on a lot in a planned unit development, provided each principal building is designed and arranged to conform with a development plan approved by the planning board and the town council

(4) Minimum buffer required. No structure, parking area or other use, except open spaces for recreational or decorative purposes, shall be erected or established within a distance of fifty (50) feet from any planned unit development project exterior boundary line or perimeter line. This restriction shall not apply to any interior development line or phase line, or to any exterior boundary line or perimeter line which abuts a nonresidential zoning district.

(5) Off-street parking and loading facilities. All off-street parking and loading facilities established as a part of a planned unit development shall conform to the requirements and design standards established by this chapter.

(6) Preservation of noteworthy features. In all planned unit developments, to the maximum degree reasonably practicable, efforts shall be made to preserve the natural environment, historic sites, scenic points, large trees and other desirable natural growths, watercourses and other water areas, and other features worthy of preservation, either as portions of public sites and open spaces, or in such other forms as to provide amenity to the neighborhood.

(7) Land-disturbing activities. All areas in which land-disturbing activity will be undertaken in the course of the development of the project shall be designated on the detail plan. The developer or owner shall submit a plan for the stabilization of the disturbed area, for the prevention or control of erosion and the protection of sand dunes located within the land-disturbing activity area. The developer and/or owner must comply with chapter 7, article II and chapter 16.

The removal of any tree with a caliper (diameter) greater than sixteen (16) inches measured at one (1) foot above the ground or any tree deemed to be a significant species as noted on the schedule on file with the town by its location on the Outer Banks is prohibited except for those trees:

- Within a building site or within twenty (20) foot perimeter around the building site;

- Within a required accessway,  parking lots, driveway, utility installation site or five (5) foot perimeter around these areas (accessory buildings, hiking trails or other similar such non-required site entities shall not be considered sufficient cause to remove this sized tree);

- Within the septic nitrification field and an area around such field as may be determined by the Dare County Department of Environmental Health as to ensure proper functioning of the septic system; or

- Diseased or severely decayed trees which are in danger of imminent collapse.

- The planner may administratively authorize the removal of any tree which poses a substantial risk to persons or property by reason of its location relative to buildings or areas used by people.

(8)Fire Hydrants.   Fire hydrants shall be required at locations approved by the planning board and the fire chief.  No portion of a building shall be farther than two hundred fifty (250) feet nor closer than fifty (50) feet to a fire hydrant.  Hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department.  The planned unit development shall mark and maintain fire lanes.  Parking shall not be permitted in fire lanes.

(9)Garbage and refuse containers.  Containers for garbage and refuse shall be provided in accordance with the requirements of the town, and if applicable, the county department of public works.  Such areas shall be screened and shall be located so as to be out of the traffic flow, accessible to garbage trucks at all times and of adequate size to meet the needs of all uses at the development.  Garbage pickup and commercial deliveries shall be during between 7:00 a.m. and 9:00 p.m.

(10) Stormwater management.  An adequate method of managing stormwater runoff shall be developed, and stormwater management systems, parking layout and the location of curbs and gutters should be planned simultaneously.  Any enclosed portion of a system should be designed to manage stormwater not just to dispose of it or dispense it.  No stormwater runoff shall be allowed within pedestrian ways or pedestrian easement areas.

(11) Outdoor lighting.  The location of all outside lights shall be shown on the preliminary and final development plans, and with the final development plan, a schedule shall be furnished to the town relating the outside light locations on the plans with the following information:  the type of light fixture proposed, the wattage or intensity of the bulb to be used in the fixture and the number of bulbs if more than one, and the height of the light fixture above the finished ground surface.   Light fixtures shall be located, arranged and, if the town deems it necessary, shielded in order to prevent the observation of glare or the direct observation of the lights bulbs by persons on adjoining properties or traveling on streets and roads within close proximity to the project site.

(12)height.  Maximum total height shall not exceed thirty-five (35) feet from existing grade (04-26) exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(Ord. of 8-18-86, § 4.02(K))

### Sec. 20-239.	Uses permitted.

(a) Uses permitted in a planned unit development shall be those residential uses permitted in the zoning district in which it is located, except that:

(1) In developments comprising one hundred (100) or more dwelling units, convenience commercial establishments may be permitted to be established to provide the following services and facilities for residents of the development and their guests:

a. Food stores.

b. Drugstores.

c. Barber or beauty shops.

d. Restaurants.

e. Professional offices.

f. Similar or related uses.

(2) The total maximum floor area of all convenience commercial uses established as part of any planned unit development shall not exceed five (5) percent of the total floor area of the project, or twenty-five thousand (25,000) square feet, whichever is less.

(3) Off-street parking areas shall be provided for each use as required by this chapter.

(4) Uses established shall be designed and scaled to meet only the needs of residents of the development and their guests.

(5) One (1) non-illuminated sign shall be permitted per use established. The maximum sign area shall be ten (10) square feet.

(6) No commercial use, or sign established therewith, shall be visible from any adjacent street outside of the planned unit development.

(7) In addition to the uses permitted by the underlying zoning district, the following conditional uses and amenities may be shown in the preliminary site plan and the final site plan subject to Town approval and any reasonable conditions imposed by the Town: (98-16)

(a) Gazebos, shelters, benches, decks, platforms and rest areas and meeting places;

(b) Natural grass putting and practice greens;

(c) Nature and exercise trails, walkways, and paths;

(d) Swimming and wading pools, including associated decks, but excluding bumper boats, water rides and vehicles;

(e) Playground equipment including but not limited to swings, slides, ropes, ladders, teeter-totters, non-motorized merry-go-rounds, and jungle-gyms;

(f) court games including but not limited to basket ball, volleyball, badminton, tennis (including serving machines), croquet and shuffleboard;

(g) Field  games and areas including horseshoes, baseball (including pitching machines), softball, football, and soccer, running and jumping areas;

(h) Picnic areas;

(i) Climbing walls with appropriate supervision;

(8) All amenities provided within the PUD shall be for the use and enjoyment of the residents, their guests, and buyer prospects within the PUD.  Amenity site shall not be used for commercial recreational uses open to the general public.(98-16)

(9) All recreational amenity uses shall be subject to review and approval by the Planning Board and Town Council in accordance with the requirements of a Preliminary Plan and Final Plan review procedure. (98-16)

(b) Building permits shall be issued for convenience commercial facilities only after permits have been obtained by the developer for the minimum number of dwelling units required as a prerequisite for such facilities.

(c) Business licenses shall be issued for convenience commercial facilities only after at least fifty (50) percent of construction has been completed on all the minimum required dwelling units.

SCHEDULE OF SIGNIFICANT PLANT SPECIES

PLANNED UNIT DEVELOPMENT ORDINANCE

- American Elm – Ulmus Americana

- Bald Cypress – Taxodium distichum

- Beech – Fagus grandifolia

- Black Cherry – Purnus serotina

- Dogwood – Cornus florida

- Hickory – Carya sp.

- Hop Hornbeam – Ostrya virginiana

- Sweet gum – Liquidambar styraciflua

(Ord. of 8-18-86, § 4.02(L))

### Secs. 20-240--20-260.  Reserved.

## DIVISION 4.  PLANNED COMMERCIAL DEVELOPMENT

### Sec. 20-261. Generally.

(a)In this division "planned commercial development" means the complete development of land which is under central control, or for which central control mechanisms have been established and is a predominantly commercial project designed and improved in accordance with a comprehensive project plan. It may be constructed in a series of phases which reflect the anticipated needs of the projected population growth in the service area of the project. The plan will be in accordance with such guides and objectives as may be established by the planning board and town council.

Cross reference(s)--Definitions and rules of construction generally, § 1-2.

(b) This division applies to planned commercial developments.

(c) This division is intended to provide developers with an option by which they can achieve flexibility of design, the integration of mutually compatible uses, and optimum land planning with greater efficiency, convenience, and amenity than may be permitted as of right under the other parts of this requirement. A planned commercial development is a privilege, not a right.

(d) Within specified districts created by this chapter it is intended to permit, on application and on approval of detailed site, use, building and development plans, and any other plans required by this chapter, establishment of planned commercial developments in areas which are suitable with respect to location, size, and physical character for development as units. Suitability of such tracts for the planned commercial development proposed shall be determined primarily by reference to the goals and objectives to the land use plan, and this chapter, by the physical characteristics of the site and by the nature of the surrounding development. This division is intended to accomplish the purpose and zoning and subdivision control as a unified basis, and to promote economical and efficient land use, a higher level of amenities, appropriate and harmonious variety in physical development, creative design, and an improved living and working environment. In view of the substantial public advantages of planned commercial development, it is the intent of this division to promote and encourage development in this form where appropriate in location and character within the BC-1, BC-2, BC-3 and BH-1 zoning districts of the town. The planned commercial development is established to provide for the proper grouping and development of commercial facilities and to provide a means for protecting the public from dangerous arrangement of vehicle and pedestrian ways within planned commercial development.

(Ord. of 8-18-86, §§ 3.04, 3.04(A)--(C); Ord. No. 88-21A, 10-13-88; Ord. No. 94-7, 3-7-94)

### Sec. 20-262.  Relationship of division to chapter and other ordinances.

(a) Where there are conflicts between this division and other parts of this chapter or chapter 19, this division shall apply unless the town council shall find in the particular case:

(1) That the provisions of this division do not serve public purposes to a degree at least equivalent to such general zoning, subdivision, or other regulations or requirements.

(2) That actions, designs or solutions proposed by the applicant, although literally in accord with these special regulations, do not satisfy public purposes to at least an equivalent degree as the general regulations. Except as may be otherwise provided in this division, where density, floor area and similar ratios have been established by other parts of this ordinance, the town shall not alter such ratios in a planned commercial development.

(3) Except as indicated in this section, notwithstanding procedures and requirements generally in effect, procedures and requirements set forth herein shall apply to all planned commercial developments and to issuance of any building permits required for them.

(4)Should there be a conflict between any planned commercial development standards and the standards elsewhere in this chapter or in chapter 19, then the planned commercial development standards shall apply.

(Ord. of 8-18-86, § 3.04(C); Ord. No. 88-21A, 10-13-88)

### Sec. 20-263.  Where permitted.

Planned commercial developments may be established as a conditional use where tracts suitable in location and character for the uses and structures proposed are to be planned and developed as units, according to the requirements and procedures set forth in this division.

(Ord. of 8-18-86, § 3.04(D); Ord. No. 88-21A, 10-13-88)

### Sec. 20-264.  Permitted and conditional uses.

(a) Generally. The permitted and conditional uses of the underlying zoning districts within the proposed planned commercial development at the time the planned commercial development is located on the zoning district map by the town shall be incorporated in and be the permitted and conditional uses within the planned commercial development, except that uses may be deleted, or compatible uses may be added by the town at the time of approval as a condition of approval of the PCD, or from to time upon approval of the use amendments to the special use permit by the town council. All uses must be designated in the PCD plan and in the project documentation.

(b) Specific conditional uses.

(1) Miniature golf course. A miniature golf course is a course containing all or a significant number of elements of a regulation golf course consisting of some combination of tees, fairways, greens, sand traps, water and vegetation hazards, and varying topography, but all of which have been reduced in size to accommodate the project site and designed for the sport of golf. Putt-putt type golf courses and animated courses are expressly excluded. Miniature golf courses, where authorized in various districts under this chapter, shall meet the following conditions and requirements:

a. The miniature golf course and support facilities must be located within the boundary of the PCD project.

b. The miniature golf course must be compatible with the land uses in the PCD project.

c. The miniature golf course site shall consist of at least thirty thousand (30,000) square feet of land area for an eighteen-hole course with parking and golf course facilities. The total number of golf course holes shall not exceed thirty-six (36) holes of play. In addition, a separate practice putting green may be located within the miniature golf course site boundary.

d. To the extent possible, the course design and layout shall utilize the natural terrain of the site. Alteration of the existing natural topography and the creation of other natural topographical features such as hills, mounds and waterfalls shall require the approval of the planning board and town council. There shall be no animation on or around the course and within the golf course site. The golf course site and course shall utilize only water, earth and natural vegetation as hazards, decorative features and other play-related features and shall not use artificial devices including, but not limited to, miniature buildings, animal, human or vehicular models, signs (other than hole number signs) or billboards. A site plan, including the location and identification of trees larger than six (6) inches in diameter, shall be required for miniature golf courses. The PCD submission and review procedure shall apply to the planning review of the golf course site.

e. The course shall be buffered from adjacent parcels outside the PCD by a vegetated buffer; however, it is permissible for the course to be visible from U.S. Highway 158 if the PCD boundary is contiguous to the right-of-way of U.S. 158. The buffer shall be a vegetated buffer with plants, shrubs and trees approved for such use by the planning board. The vegetated buffer shall be maintained by the manager or association responsible for maintenance of the planned commercial development.

f. There shall be no neon or flashing lights in the miniature golf course site boundary. Lighting of the course shall be contained on the site and shall not interfere with the reasonable enjoyment of adjacent parcels or traffic on streets or highways.

g. The electrical transmission of sound shall be prohibited.

h. Alcoholic beverages may not be sold or consumed in the miniature golf course area.

i. Restroom facilities shall be located within the golf course area for use by the players.

j. The golf course design shall accommodate stormwater retention and shall comply with chapter 16.

k. There shall be one (1) space of parking for each numbered hole plus one (1) parking space for each employee working in the golf course site. All parking spaces shall be located within four hundred (400) feet of the golf course.

l. Trash receptacles shall be centrally located and maintained within the golf course site.

(2) Retail sale of Christmas trees and wreaths subject to the regulations set forth in Section 20-401 (3). (97-13)

(Ord. of 8-18-86, § 3.04(E); Ord. No. 88-21A, 10-13-88; Ord. No. 89-14, 7-17-89; Ord. No. 94-7, 3-7-94)

### Sec. 20-265. Application procedures generally.

(a) The four (4) review phases of planned commercial development shall be as follows:

(1) Preapplication conference.

(2) Conceptual review.

(3) Preliminary development plan and site plan approval.

(4) Final development and site plan approval.

(b) The developer or landowners may waive the preapplication conference and proceed with the conceptual review phase; however, developers and owners are encouraged to request a preapplication conference with the planning board.

(Ord. of 8-18-86, § 3.04(F); Ord. No. 88-21A, 10-13-88)

### Sec. 20-266. Preapplication conference review phase.

(a) Preapplication fee. Any person seeking a preapplication conference with the planning board shall submit a written request for the conference with the town planner and pay to the town the sum of five hundred twenty-five dollars ($525.00) as a preapplication conference fee. The planner shall schedule a preapplication conference with the planning board not less than fifteen (15) days and not more than twenty-five (25) days after the written request has been filed with the town planner.

(b) Purposes. The purpose of the preapplication conference shall include, but not be limited to:

(1) Discussion and explanation by the planning board of the procedure for reviewing and approving a planned commercial development as well as materials and information required for such review.

(2) Discussion by the planning board of the applicant's development concept and proposal including proposed, permitted, conditional, and unspecified uses within the planned commercial development boundaries.

(3) Discussion by the planning board of any environmental issues raised or identified by the owner, developer, planning board or town staff.

(4) Discussion of the proposed uses with respect to the surrounding uses and zoning districts as well as general land use plan considerations.

(c) Sketch plan. Sketch plans drawn to appropriate scale and in such manner as to minimize initial expense and encourage sufficient design flexibility to accommodate required changes without undue hardship to the developer may be submitted, and in such event, shall be reviewed by the planning board.

(Ord. of 8-18-86, § 3.04(F)(a); Ord. No. 88-21A, 10-13-88)

### Sec. 20-267. Conceptual phase review.

(a) Generally. The conceptual plan shall demonstrate a project concept in which various general land uses and developments have been integrated into a harmonious plan concerning such matters as vehicular and pedestrian corridors, recreational areas, open space and green areas, parking areas, general proposed uses, utility areas and corridors, and special environmental areas.

(b) Formal application. Any person or entity seeking planning board review of a proposed planned commercial development shall file a written application in the form provided by the town. In addition, the applicant must file a wetland map delineating any wetland areas within the project boundary with a certification by a licensed engineer or surveyor that the wetlands delineated thereon have been flagged on the ground surface, reviewed, and approved by the Corps of Engineers, and accurately depicted on the wetland map. If there are no wetland areas within the project boundary, then a certification to that effect shall be signed and filed with the town planner at the time the application is filed.

(c) Conceptual plan submission requirement. Any applicant seeking conceptual plan review by the planning board shall file the following with the town prior to being scheduled for planning board review:

(1) A written application on the form provided by the town, signed by the applicant and all property owners within the proposed planned commercial district, including their addresses and telephone numbers. The application shall include the zoning amendment application to locate the planned commercial development within an existing zone authorized for planned commercial districts.

(2) Payment of the planned commercial district review fee which shall be one-third of the planned commercial district review fee.

(3) The conceptual plan prepared in conformity with this chapter.

(4) A written legal description prepared by an attorney licensed in the state based on a boundary survey of the property within the planned commercial development.

(5) A boundary survey of the property within the planned commercial development prepared by a surveyor licensed in the state or an engineer licensed in the state.

(d) Conceptual review. The town planner shall compute the planned commercial development review fee for the project and the applicant shall pay one-third of the total planned commercial development review fee computed by the town planner as the conceptual phase review fee. Such fee shall be paid when the application is filed with the town. No conceptual review shall commence until the applicant has completed procedures and requirments outlined in subsections (b) and (c).

(e) Purposes of conceptual phase review. The planning board shall review the conceptual plan of the applicant to obtain as nearly as possible conformity with these regulations and assure:

(1) The proposed land uses are allowed by the underlying zones of the proposed planned commercial development project area.

(2) The grouping of land uses within the planned commercial development are appropriate to each other and compatible with or minimally impact adjoining property uses.

(3) The proposed pedestrian and vehicular circulation patterns within the project will be compatible and the impact of existing adjoining trafficways will be minimized.

(4) The open spaces, parking, circulation patterns, land use types, pedestrian easements and amenities, and architectural styles and themes are well integrated, and are related to the natural features of the site.

Specific site and zone elements, standards, and specifications will not be reviewed during the conceptual phase review and the applicant should not submit such detailed information with the materials filed for conceptual phase review.

(f) Conceptual phase review procedure. After all required materials have been filed with the town and the conceptual review phase fee has been paid, the town planner shall review the materials to assure their completeness and compliance with this chapter. If the application and materials are approved by the town planner then the development shall be placed on the planning board agenda for review by the board. During conceptual review the planning board shall make any recommendations or changes and board recommendations shall be recorded in writing. All such recommendations shall be supported by stated reasons for the proposal for change. Applicants may, in writing, indicate their agreement to such recommendations or their disagreement. If the applicant disagrees with the planning board, then the applicant shall, in writing, indicate the reasons for the disagreement. The written response of the application shall be included in the town project file.

The planning board shall approve, disapprove, or approve with recommended modifications the conceptual plan as well as approve or deny the applicant's zoning amendment request to locate a planned commercial development district at the location set forth in the application and the necessary zoning changes in the town zoning map. Thereafter, the recommendation of the planning board shall be placed on a regular council meeting agenda at which the town council shall set a public hearing on the applicant's zoning amendment request. Following the public hearing the town council may adopt or reject the zoning amendment amending the town zoning map. If the zoning amendment is adopted, then within sixty (60) days thereof the applicant shall pay the balance of the planned commercial development review fee and proceed with the preliminary plan review phase as set forth in this division. If the applicant does not pay the balance of the planned commercial development fee and file the required plans and documents within sixty (60) days then the town council may by zoning amendment repeal the planned commercial development district amendment to the zoning map.

The planning board will recommend approval, denial, or approval with recommendations of the planned commercial development application and in its recommendation, the planning board shall state its position with respect to issues left unresolved by the conceptual phase review.

(Ord. of 8-18-86, § 3.04(F)(b); Ord. No. 88-21A, 10-13-88)

### Sec. 20-268. Preliminary development plan and site plan review.

(a) Purpose. The detailed project plan and site plan review is to establish that the plans are in compliance with the project standards, specifications, and conditions or the ordinance and consistent with the conceptual plan approval. Project plats should be sufficiently detailed to permit the location of all improvements, common areas, rights-of-way, and open spaces or common properties, structures, signs, utilities, lighting systems, and any other information required by this chapter on the surface of the ground as shown on the project plat.

(b) Plan submission requirements. The applicant shall submit ten (10) copies of the preliminary development plan and separate site plans for each proposed use within the development. The planning board may waive the requirement of submitting one (1) or more of the separate site plans. The applicant shall pay the balance of the planned commercial development review fee. The material submitted with the application or requested by the planning board or the town council shall include such plans, maps, studies, and reports which may reasonably be required to conduct the review in accordance with this chapter. Ten (10) copies of each map, plan, plat, or other material shall be submitted to the town unless the planning board permits less than ten (10) or requires more than ten (10) of any such item. In addition to any other reasonable specifications deemed by the planning board necessary to assure compliance and consistency of the preliminary plan with the conceptual plan, the preliminary development plan and commercial site plan shall include and comply with the following:

(1) The proposed name of the planned commercial development.

(2) The location by legal description and survey showing boundary line and the total acreage encompassed thereby.

(3) The name and address of the owner and the applicant if different from owner, together with the name and address of plan engineer, architect, or designer of the plan.

(4) The scale of plan shall be one (1) inch to one hundred (100) feet, and the date, north arrow and a vicinity map shall be included.

(5) A detailed topographical map.

(6) A tabulation of the various land uses proposed, showing the total site area and the total floor area for each use.

(7) A tabulation of all natural or landscaped open areas shown on the plot plan including the square footage of each type.

(8) The location, widths, and names of all existing or prior platted streets, utility easements and rights-of-way, buildings and structures, and municipal boundary lines within five hundred (500) feet.

(9) Existing water mains, culverts, and other underground facilities within the tract, indicating pipe sizes, grades, manholes, and location.

(10) A planned commercial development internal traffic flow and pedestrian walkway plan and an internal parking area traffic flow plan. Accessways shall not be less than thirty (30) feet or not more than forty (40) feet in width at their intersection with the project boundary line. Accessways shall be installed with curbs of concrete if required by the planning board. In such event, curb returns shall have a minimum radius of thirty (30) feet. No accessway entering the project shall be located less than one hundred (100) feet from another accessway or fifty (50) feet from the adjoining property owner. The location and design of accessways shall be such that traffic congestion is reduced as much as possible and traffic hazards are eliminated.

(11) The location, arrangement, and dimensions of automobile parking spaces, width of aisles, width of bays, angle of parking, and location of handicapped spaces and ramps.

(12) The separation of service and delivery areas from customer and resident parking areas as well as from other vehicular and pedestrian circulation patterns.

(13) The methods of separating vehicular and pedestrian circulation patterns and location of pedestrian access patterns to various pedestrian-oriented areas of the planned commercial development from parking areas and public transportation stops or terminals. Pedestrian ways shall be designed to accommodate bicycle use or, in the alternative, bike paths may be included as part of the vehicular and pedestrian flow plan.

(14) The location, arrangement, and dimensions of truck loading and unloading spaces and doors.

(15) The location and dimensions of vehicular drives, entrances, exits, acceleration and deceleration lanes, fire lanes and service lanes.

(16) The location and dimensions of pedestrian entrances, exits, and walkways.

(17) The drainage system, stormwater management plan, and septic facilities.

(18) The location, height, and materials of walls, fences and screen plantings and buffer zones.

(19) The ground cover, finished grades, slopes, banks and ditches.

(20) Architectural sketches of proposed buildings, structures, and signs.

(21) The size, location, and orientation of all signs.

(22) Buildings shall be planned with a common architectural theme or compatible themes for the entire project.

(23) The elevations of all buildings proposed to be built within the project.

(24) Restrictive covenants as may be required.

(25) Where applicable, the applicant shall comply with all of the following:

a. Chapter 19, article II.

b. Chapter 16.

c. A natural features site plan. The natural features site plan shall identify and locate significant natural features within the project as well as demonstrate how those features will be utilized, preserved, or protected within the project site. Significant natural features shall include, but not be limited to, dunes, ridge lines, swales, large trees, ditches, banks and rare or endangered plant species indigenous to the area and identified on the schedule of rare and endangered plants filed with the town.

d. CAMA rules and regulations.

e. Chapter 8.

(26) The rules, regulations, and procedures of the appropriate regulatory agency or agencies must be satisfied with respect to wastewater treatment systems and any necessary permits therefor obtained.

(27) Where the planned commercial development is to be phased, then a phased construction plan shall be filed indicating the boundaries of each construction phase and the sequence of the phased construction.

The planning board may waive the requirement that one (1) or more of the above requirements be depicted or shown on the preliminary project plans if the planning board deems depiction of the item on the plans is unnecessary in order to assure satisfaction of the particular standard.

(c) Procedure.

(1) The town planner shall review the preliminary development plan and site plans to assure the plans are complete and fulfill the technical requirements of this ordinance. If acceptable, the town planner shall schedule the preliminary review at the next regular planning board meeting. The planning board may continue its review at subsequent meetings, whether regular or special, until the review has been completed.

(2) After completing its review, the planning board shall make its recommendation to the town council which shall be either approval, disapproval, or conditional approval of the preliminary development plan and site plans.

(3) The town council shall review the preliminary development plan and the commercial site plans and shall grant approval, disapproval, or conditional approval of the preliminary development plan and any commercial site plans. The town council, in its discretion and at its option, may approve the preliminary development plan and disapprove or conditionally approve one (1) or more commercial site plans within the project. In such event, no construction work shall commence within the area of the disapproved commercial site plan or conditionally approved site plan until the conditions have been satisfied and verified by the town planner or the disapproved commercial site plan has been modified and subsequently approved by the town council.

(4) An amendment to the preliminary plan or commercial site plan shall be required for any significant deviation in the construction and implementation of the project plans. The town planner shall determine whether a deviation is significant within the meaning of this paragraph; however, any deviation which itself fails to meet the standards of this chapter or which results in a standard becoming unsatisfied as well as changes in the locational layout of infrastructure, uses, additional structures, or additions to existing structures and change in location of commercial sites other than minor adjustments of the building footprint shall be deemed to be significant deviations or modifications requiring planning board and town council approval.

(5) The planning board shall complete its plan review and submit its recommendation to the town council within one hundred twenty (120) days of the initial planning board meeting for preliminary development phase review of the project unless the town council extends the planning board review period for an additional period not to exceed ninety (90) days. Should the town review process be interrupted for any reason beyond the control of the town, then the provisions of this paragraph shall be suspended until the town review process can be recommenced.

(6) After the town council has approved the preliminary development review plan, the applicant may be issued a land-disturbing activity permit and commence construction of the infrastructure, notwithstanding the disapproval or conditional approval of one (1) or more commercial site plans within the planned commercial development.

(7) No land-disturbing activity or construction of buildings shall occur on a commercial site within the planned commercial development until the commercial site plan for the proposed building and improvements has been approved by the town council. Thereafter, land-disturbing activity permits for the construction of utilities and infrastructure improvements within the commercial site and building permits for the construction of structures to be utilized for the approved uses of the commercial site shall be issued upon the application therefor by the owner or applicant.

(8) After the town council has approved the entire preliminary development plan the development may be constructed in phases provided the following requirements are met:

a. Each phase must meet all the requirements of the ordinance and the approved preliminary plan, and no construction shall commence on any phase until all commercial site plans within the phase have been approved by the town council.

b. Each phase must be completed, inspected, and satisfactory test results submitted to the town before the next approved phase of construction can commence.

c. Time period for commencing and completing the first phase shall be two (2) years and one (1) year for each successive phase thereafter.

(9) After construction of the infrastructure within the project or within a phase of the project, the applicant shall submit the following to the town:

a. Test results confirming that all roads, driveways, or vehicular ways within the development have been constructed and paved to town street standards.

b. Written evidence that the appropriate governmental agency has approved the design and construction plans and, upon completion of construction, certification from an engineer licensed in the state that the septic, water, or sewer system or utility system has been constructed in accordance with the approved plans and specifications. Making a false certification under this paragraph shall be a misdemeanor and conviction of same shall be reported to the appropriate licensing board regulating the profession of the person so convicted.

(Ord. of 8-18-86, § 3.04(F)(c); Ord. No. 88-21A, 10-13-88; Ord. No. 91-21, § C, 12-10-91)

### Sec. 20-269.	Standards and requirements.

All planned commercial developments shall conform to following standards and requirements in addition to any other requirements of the town council, and county, state, and federal regulations applicable to the planned commercial development:

(1) Minimum size of site. A planned commercial development site shall not be less than ten (10) contiguous acres in size except that portions of the site may be separated by public or private rights-of-way not more than sixty (60) feet in width. A planned commercial development shall not be divided or traversed by U.S. 158 and NC12 . (00-45)

(2) Number of buildings. More than one (1) principal building may be permitted on a lot in a planned commercial development, provided each principal building is designed and arranged to conform with a development plan approved by the planning board and the town council.

(3) Minimum buffer required. No structure, parking area, or other facility such as trash collection areas or other use, except open spaces for recreational or decorative purposes and subsurface water, wastewater, and septic systems and underground utilities, shall be erected or established within a distance of fifty (50) feet from any planned commercial development project exterior boundary or perimeter line, except that where a PCD abuts a planned unit development (PUD), a landscaped buffer acceptable to and approved by the town of thirty (30) feet in width shall be maintained, not less than fifteen (15) feet of which shall be on the PCD site. Vegetation shall be planted and maintained in the buffer zone. (94-7)

(4) Off-street parking and loading facilities.

a. All off-street parking and loading facilities established as a part of a planned commercial development shall conform to the requirements and design standards established by this chapter.

b. Handicapped ramps shall be provided near handicapped parking areas.

c. Each shop or store shall have access to a rear or side entrance that is within a reasonable distance and is accessible to a loading area and service drive. Service drives shall be of such width and arrangement to provide adequate access to site facilities, as approved by the town, and shall be in addition to and not a part of the drives or circulation systems used by vehicles or shoppers. Loading and delivery zones shall be clearly marked.

d. Bicycle parking devices shall be installed to provide one (1) bicycle parking space for each fifty (50) auto parking spaces. Design of parking devices can be obtained from the bicycle program of the state department of transportation and shall be built in accordance therewith.

(5) Preservation of noteworthy features. In all planned commercial developments, to the maximum degree reasonably practicable, efforts shall be made to preserve the natural environment, historic sites, scenic points, large trees and other desirable natural growths, watercourses and other water areas, and other features worthy of preservation, either as portions of public sites and open spaces, or in such other form as to provide amenity to the neighborhood. Large trees or other desirable natural growth located in public or private right-of-ways or public or private easements shall not be removed unless such removal is necessary for the installation of utilities or drainage structures or for other purposes in the public interest. Such removal may be prohibited if the amenity of adjacent property, or the amenity of the general neighborhoods, is adversely affected.

(6) Maximum lot coverage. The total ground area occupied by all principal buildings together with all accessory buildings and parking lots shall not exceed fifty (50) percent of the total area of the planned commercial development, but driveways, walkways, and streets shall not be included for the purpose of the maximum lot coverage calculation.

(7) Height limitations. Maximum total height shall not exceed thirty-five (35) feet  from existing grade (04-26) exclusive of chimneys, flagpoles, communication masts and aerials. (01-10)

(8) Traffic control and traffic flow. There should be one (1) primary accessway from the planned commercial development property to a public or private right-of-way. If the planned commercial development fronts on U.S. 158, then it shall have one (1) access to said highway. The planning board may approve one (1) or more secondary accessways if additional access points are needed in order to insure orderly and safe traffic movement within the district and between the district and adjoining areas. The applicant shall install such traffic-control devices as recommended by a traffic engineer, traffic consultant or the state highway engineer and approved by the planning board both within the district and at any places where planned commercial development traffic arteries, roads, streets or ways intersect a public or private right-of-way. The cost of installing the traffic-control devices shall be paid by the applicant. The planned commercial development shall maintain all traffic-control devices within the district which are located on the development property.

There shall be no parking areas contiguous to the right-of-way margin or any area used for through vehicular traffic such that a vehicle is required to back or maneuver into the planned commercial development traffic system while entering or leaving a parking space.

(9) Lighting. All parking areas and accessways shall be floodlighted in accordance with the standards of the town. All outside lighting shall be arranged and shielded to prevent glare or reflection, nuisance, inconvenience or hazardous interference of any kind on adjoining lots or residential areas or area traveled by motor vehicles, and shall provide security for planned commercial development customers.

(10) Fire hydrants. Fire hydrants shall be required at locations approved by the planning board and the fire chief. No portion of a building shall be farther than two hundred fifty (250) feet nor closer than fifty (50) feet to a fire hydrant. Hydrants shall be protected from traffic in accordance with the requirements of the town and shall be marked and painted as required by the town and the fire department. The planned commercial development shall mark and maintain fire lanes. Parking shall not be permitted in fire lanes.

(11) Garbage and refuse containers. Containers for garbage and refuse shall be provided in accordance with the requirements of the town, and if applicable, the county department of public works. Areas for screened dumpsters shall be provided so as to be out of the traffic flow, accessible to garbage trucks at all times and of adequate size to meet the needs of all uses at the center. Garbage pickup and commercial deliveries shall be during between 7:00 a.m. and 9:00 p.m.

(12) Stormwater management. It shall be required that an adequate method for managing stormwater runoff be developed. Whenever possible, stormwater management systems, parking layout and the location of curbs and gutters should be planned simultaneously. Any enclosed portion of a system should be designed to manage stormwater not just to dispose of it or disperse it. No stormwater runoff shall be allowed within pedestrian ways or pedestrian easement areas.

(13) Public restrooms. Public restrooms shall be required on each commercial retail building. The public restroom shall be located such as to be convenient to the public and shall be maintained by the operator or applicant in a clean and sanitary condition. The location of all restrooms shall be clearly marked. Buildings used solely for offices or offices and residential use shall not be required to contain public restrooms.

(14) Signs.

a. Identification sign. One (1) freestanding planned commercial development site identification sign shall be permitted at the access or entranceway designated as the primary accessway or entranceway. The planned commercial development identification site sign shall not exceed forty-eight (48) square feet, except that for a PCD overlay on a BC-3 zoning district, the sign shall not exceed sixty-four (64) square feet.

b. Secondary accessway or entrance way identification signs. A secondary accessway or entranceway planned commercial development site identification sign may be permitted with the approval of the planning board. The secondary planned commercial development site identification sign shall not exceed ten (10) square feet and the sign and structure supporting it shall not exceed ten (10) feet from the undisturbed ground surface. Both the primary and secondary planned commercial development site identification signs must be located so as not to block or interfere with the ability of motor vehicle operators to observe the traffic while the operators are entering or exiting the planned commercial development area. All planned commercial development site identification signs shall be indirectly lighted so that the light is retained on the site and does not interfere with traffic or neighboring property owners. No internal illumination shall be allowed and no neon or animated signs shall be allowed.

c. Directory locator sign. Directory locator signs may be located at the entrances to parking lots. The businesses contained on the directory locator sign should be those businesses whose planned commercial development entrance is closest to the parking lot where the specific directory locator is located. These signs should be no larger than necessary to allow them to be read from a slowly moving vehicle. These signs shall not exceed seven (7) feet in height.

d. Directional signs. Directional signs for pedestrian and vehicle traffic shall be located as needed within the planned commercial development subject to planning board approval. No directional sign shall exceed six (6) square feet in size.

(15) Hotels. In this paragraph "hotel" includes any motel or motor inn. The following provisions apply to hotels:

a. The maximum height of a structure shall be thirty-five (35) feet, exclusive of chimneys, communications masts and aerials, and observation towers.

b. The density of hotel units shall not exceed twenty-four (24) hotel units or hotel efficiency units per acre.

c. Hotel units shall be at least three hundred (300) square feet in area.

d. A ten-foot-wide paved vehicular access along all four (4) sides of principal structures shall be provided suitable for firefighting and rescue equipment. The edge of the paved access nearer the structure shall be no closer than ten (10) feet nor farther than twenty (20) feet, from the sides of the structure.

e. Fire hydrants shall be protected from traffic in accordance with requirements of the town and shall be marked and painted as required by the town and the fire department.

f. If the building is not to be sprinkler protected in accordance with National Fire Protection Association standards, the fire flow of the fire hydrants at or nearest the site shall be tested by an independent testing laboratory at the expense of the developer. If the fire flow is found to be deficient according to Insurance Service Office (ISO) standards applicable to the requirements of the town the fire flow shall be brought up to ISO standards at the developer's expense. The decisions to use a sprinkler system or to adjust a fire flow shall be made prior to issuance of a building permit. The fire flow test shall be made during the period of peak water demand as determined from water consumption data maintained by the county water department. Where sprinkler systems are not required, a standpipe system shall be installed which meets ISO fire flow standards.

g. Emergency electric generators or approved and maintained battery packs, to provide lighting in hallways and stairwells during period of public utility power outages, shall be installed and tested regularly at the expense of the developer.

h. To the extent not inconsistent with the specific provisions of this paragraph, the dimensional requirements specified in section 20-55 shall govern a hotel development.

i. Hotels may have as accessory uses retail shops or offices which offer goods and services for the benefit of their occupants and guests provided that all such establishments shall be designed to meet the requirements of occupants and guests of the hotel in which such establishment is located and further provided that there be no sign or evidence of such establishment outside of the hotel.

j. For hotel uses only in the BH-1 zones of the town, the structure may have three (3) habitable floors provided the maximum height of thirty-five (35) feet is not exceeded.

k. One and one-half (11/2) parking spaces shall be allocated for each hotel unit. Parking under wood frame hotel structures is prohibited.

l. The developer of a hotel shall afford evidence of the existence of a private or public beach fronting on the Atlantic Ocean for a distance of at least one hundred (100) feet in width and extending landward from the mean high water line to the eastern right-of-way margin of N.C. 12, and readily accessible for use and enjoyment by the guests of the hotel. The area shall be accessible by public accessway extending from N.C. 12 to the Atlantic Ocean. In the absence of such a public beach area, the hotel developer shall acquire in fee simple or by perpetual easement such an area and dedicate the area as a public beach. As an alternative to acquiring the prescribed beach area, the developer may pay to the town prior to the issuance of an occupancy permit, an annual beach access impact fee in such amount as may be specified in the regularly adopted beach access impact fee schedule of the town to be earmarked for land used by the town in carrying out a program of acquiring beach front properties for public use.

(Ord. of 8-18-86, § 3.04(G); Ord. No. 88-21A, 10-13-88; Ord. No. 94-7, 3-7-94)

### Sec. 20-270.  Reserved.

Editor's note--Ord. No. 94-14, adopted Aug. 1, 1994, repealed § 20-270, which pertained to review fees and expenses and derived from § 3.04(H)(1) of an ordinance adopted Aug. 18, 1986, as amended by Ord. No. 88-21A, adopted Oct. 13, 1988.

### Sec. 20-271. Covenants and restrictions.

The developer or owner shall submit a declaration of covenants and restrictions to the town for its review and approval. The covenants and restrictions shall provide for the regulation, maintenance, and operation of the planned commercial development common areas, open spaces, recreational areas, and, if appropriate, the maintenance of rights-of-way within the site.

(Ord. of 8-18-86, § 3.04(H)(2); Ord. No. 88-21A, 10-13-88)

### Sec. 20-272. Final approval of project and site plans.

The planning board shall conduct the final review of the commercial development and the commercial site plans after completion of the construction of the development or each phase if constructed in stages or phases. Prior to final review by the planning board, the applicant shall submit any necessary infrastructure test results and a certification from the project engineer and/or architect that the project "as built" conforms with the plans and specifications filed with the town. In addition, the town building inspector and town zoning officer shall submit to the planning board an "as built" certification that based upon their examination of the project, it complies with the preliminary development plan and commercial site plans. Final review by the town council shall not be required.

(Ord. of 8-18-86, § 3.04(I); Ord. No. 88-21A, 10-13-88)

### Sec. 20-273. Permit.

(a) After final approval by the planning board of the final plat or the final plat of a specific phase of the project, the applicant shall deliver a copy of the plat acceptable for recording with the office of the register of deeds of the county, and the applicable recording fee to the town planner within ten (10) days of the final planning board approval.

(b) Upon receiving the recording fee and plat, the town planner shall issue a permit to the applicant stating that the phase (if appropriate) or the project development has been finally approved and buildings within the approved phase or within the project may be occupied for the authorized uses in accordance with the project approval. Thereafter, the building inspector may issue certificates of occupancy for the buildings within the approved phase provided other building requirements have been satisfied.

(c) After final approval of a phase or the project, there shall be no deviation in or change in the use of any building or any area of the approved commercial development without first obtaining planning board and/or town council approval.

(Ord. of 8-18-86, § 3.04(J), (K); Ord. No. 88-21A, 10-13-88)

### Secs. 20-274--20-290.  Reserved.

## DIVISION 5.  EMERGENCY AND GOVERNMENTAL SERVICES DISTRICT*

### Sec. 20-291. Scope and intent.

Unless stated otherwise, this division applies to the emergency and governmental services district. This district is established to provide for the proper grouping and development of medical services, medical offices, emergency and governmental service facilities in the Town of Kitty Hawk.

(Ord. No. 96-9, 5-6-96)

### Sec. 20-292. Permitted uses.

The following are permitted uses in this district:

(1) Hospitals and medical centers.

(2) Town-owned or leased facilities.

(3) State-owned facilities.

(Ord. No. 96-9, 5-6-96)

### Sec. 20-293. Conditional uses.

The following are conditional uses in this district:

(1) Licensed physician offices.

(2) Chiropractor offices.

(3) Optometrist offices.

(4) Fire stations.

(5) Emergency heliport, pursuant to the standards set forth in section 20-145(c)(8).

(6) Emergency medical station.

(Ord. No. 96-9, 5-6-96)

### Sec. 20-294.	Dimensional requirements.

(a) Lots shall be sufficient size to meet the requirements of the county health department, to provide adequate siting for structures and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter.

(b) The minimum front yard is fifteen (15) feet.

(c) The minimum side yard is ten (10) feet. No side yard is required if the building is constructed with a common wall. An additional five (5) feet side yard adjacent to the street is required for a corner lot.

(d) The minimum rear yard is twenty (20) feet.

(e) The maximum allowable lot coverage by the principal use and all accessory structures is sixty (60) percent.

(f) Maximum height limitations are as follows:

(1) Thirty-five (35) feet total height from existing grade, exclusive of chimneys, flagpoles, communications masts and aerials. (04-26)

(Ord. No. 96-9, 5-6-96)

### Sec. 20-295. Site plan review.

Any person desiring to construct or enlarge a structure for a use or to support a use on the same site and any person desiring to change the authorized use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this article.

(Ord. No. 96-9, 5-6-96)

### Sec. 20-296. Existing structures; use change; building code.

Any building constructed in the zone for a use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from an existing use, the owner must obtain site plan approval from the planning board and an occupancy permit before making a different use of the building. The structure must satisfy all North Carolina Building Code standards and requirements applicable to structures for the use proposed at the time of the application of the occupancy permit.

(Ord. No. 96-9, 5-6-96)

### Sec. 20-297. Ground stabilization.

A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

(Ord. No. 96-9, 5-6-96)

### Secs. 20-298--20-299.  Reserved.

## DIVISION 6.  OPEN SPACE & RECREATION  DISTRICT (0S) (04-05)

### Sec. 20-300. Scope and intent.

Unless stated otherwise, this division applies to the Open Space and Recreation district. This district is established to provide for the proper grouping and development of public parks and open space, nature preserves and maritime forest areas, public recreational areas and certain private recreational uses in the Town of Kitty Hawk.

### Sec. 20-301. Permitted uses.

The following are permitted uses in this district:

(1) Public parks and recreational facilities

(2) Public beach and sound accesses

(3) Maritime Forest preserves

(4) Multi-purpose trails

(5) Town-owned or leased facilities.

(6) State-owned facilities.

(7) Public water passenger shuttle service

### Sec. 20-302. Conditional uses.

The following are conditional uses in this district:

(1) Public & private golf courses.

(2) Public swimming pools and recreational facilities.

(3) Public playground equipment.

(4) Public Information centers.

Sec. 20-303.  Specific uses not allowed.

This district is for the provision of traditional recreational facilities but not recreational uses such as ,but not limited to, the following:

- Public or private amusement parks

- Public or private carnivals

- Public or private go-carts, motorcycle or terrain vehicle tracks.

- Public or private putt-putts, mini-golf, etc.

- Public or private horse back rides or carriage rides.

### Sec. 20-304. Dimensional requirements.

(a) Lots shall be sufficient size to meet the requirements of the county health department, to provide adequate siting for structures and to provide parking, loading, and maneuvering space for vehicles as required by article VI, division 2 of this chapter.

(b) The minimum front yard is fifteen (20) feet.

(c) The minimum side yard is ten (10) feet.

(d) The minimum rear yard is twenty (20) feet.

(e) The maximum allowable lot coverage by the principal use and all accessory structures is thirty (30) percent, not including pedestrian trails or walkways.

(f) Maximum total height shall not exceed thirty-five (35) feet from existing grade exclusive of chimneys, flagpoles, communication masts and aerials.  The structure shall have a minimum roof pitch of three (3) feet by twelve (12) feet. (04-26)

### Sec. 20-305. Site plan review.

Any person desiring to construct or enlarge a structure for a use or to support a use on the same site and any person desiring to change the authorized use of an existing structure to another permitted or conditional use shall first submit site plans to the planning board for review and its approval as provided in this article.

### Sec. 20-306. Existing structures; use change; building code.

Any building constructed in the zone for a use must comply with the applicable standards of the North Carolina State Building Code. Where the use of an existing building is changed from an existing use, the owner must obtain site plan approval from the planning board and an occupancy permit before making a different use of the building. The structure must satisfy all North Carolina Building Code standards and requirements applicable to structures for the use proposed at the time of the application of the occupancy permit.

### Sec. 20-307. Ground stabilization.

A plan that will assure the stabilization and subsequent revegetation of all areas that have been disturbed in accordance with chapter 7, article II and chapter 16 is required.

### Secs. 20-310—20-400.  Reserved.

# ARTICLE VI.  SUPPLEMENTAL REGULATIONS

## DIVISION 1.  GENERALLY

### Sec. 20-401. Temporary uses.

The town council may approve as conditional uses under the application provisions of section 20-56 the following temporary uses notwithstanding other restrictions of this chapter but including such additional conditions and safeguards as may be required by the council as a provision of such approval:

(1) Temporary real estate sales offices may be permitted in any residential district for onsite sales of land or residences located within the subdivision within which such office is located. Any such temporary use must be terminated no more than thirty (30) days from the date that sixty (60) percent of the lots or residences within that subdivision are sold or one (1) year from the date it was authorized, whichever occurs first.

(2)Temporary construction offices may be permitted in any district to provide on-site quarters for the management and security of construction projects only. Any such temporary use must be terminated no more than thirty (30) days from the date that construction is substantially completed or one (1) year from the date it was authorized, whichever occurs first. The town council may delegate the administrative approval of temporary construction offices to the town manager and/or his designee. (02-08)

(3)The sale of Christmas trees and wreaths (whether natural or artificial) during the period beginning November 1 and ending December 31 of each year subject to the following terms and conditions:  (97-13)

- A temporary use permit must be obtained from the Town Planning Department.  The Town Manager and planning staff members designated by the Town Manager shall be authorized to issue temporary use permits. If a temporary use permit application is denied by the Planning Department, the applicant may appeal the denial to the Town Council at its next regular meeting by filing a written request to be on the Town Council agenda with the Town Clerk at least seven (7) days prior to the next regular meeting of the Town Council and by also informing the Town Planner of the appeal to the next regular meeting. The Town Council will either approve, deny or defer action on the application until a subsequent meeting;

- No trees or wreaths shall be located within the Town or State right-of way or in sidewalks or fire lanes;

- The temporary use activity may utilize not more than twenty percent (20%) of the required parking on-site as well as the open space areas and excess parking areas;

- No Christmas tree sales shall be conducted after 11:00 p.m. and prior to 8:00 a.m.;

- No music shall be produced on the site with the temporary use which music is audible beyond the property lines of the lot on which the Christmas tree sales are being conducted;

- Any signs used in connection with the temporary use activity must meet current Town code standards and applicable sign permit requirements;

- All exterior light generated in connection with the temporary use activity must be contained on the site and exterior lights must comply with current Town standards;

- The temporary use can occur only on commercially zoned property, and the issuance of a temporary use permit to an applicant does not create a vested right for approval of temporary use permits in subsequent years.  The Town may deny a temporary use permit who had temporary use permit violations in the prior year which were not corrected as required by the Town Planner;

- An application must be submitted for each calendar year, and a deposit of $100.00 must be made with the Town Planner before the issuance of the temporary use permit.  The applicant shall remove all Christmas trees and other materials in connection with the temporary use with ten (10) days following the expiration of the permit.  If the trees and materials are not removed, then the Town may remove the material and trees and deduct the cost of such removal from the $100.00 deposit held by the Town.  The deposit or balance remaining from the deposit after deducting the aforesaid cost will be refunded to the applicant.

- Trailers or vehicles used to transport trees and materials to the site of the temporary use shall be concealed from observation from Town streets and State rights-of-way.

- In the event of a conflict between provisions of this section and Chapter 9, then the provisions of this section shall control.

(4) Temporary sales of lawn and garden supplies. (04-08) (05-13)

a.  Sales of lawn and garden supplies and equipment may be allowed as a temporary conditional use only in the BC-3 zoning district subject to the standards and requirements set forth herein. (06-08)

b. The applicant shall complete the temporary conditional use application setting forth the square footage of the site being designated as the temporary conditional use of garden and lawn center sales, the period of time that the applicant desires to conduct the temporary conditional use, and such other information contained on the application submitted for the temporary conditional use permit and information deemed necessary by Town staff in order to complete the application. (06-08)

c. The applicant must designate the location where the proposed temporary conditional use will be conducted on the approved site plan. (06-08)

d.  The duration of a temporary conditional use permit issued pursuant to this subsection shall not exceed 120 days. (06-08)

e.  No additional lighting shall be allowed on the site for the temporary conditional use.

f.  All signs, banners and advertising displays must comply with the requirements of the Town Code.  The maximum sign area authorized by the code shall not be exceeded. (06-08)

g.  Parking for the temporary use shall not be allowed in public or private rights-of-way, fire lanes or travel lanes designated on approved site plan. (06-08)

h. Only the parking spaces in excess of the minimum required shall be used for the temporary sales of lawn and garden supplies. (06-08)

(5) Charitable fund raising activities. (98-11)

a.  Intent, Scope and Definition.  To provide uniform standards in beach and village commercial, planned commercial and hotel zone districts for individuals to conduct charitable fund raising activities and sales and local public benefit activities as a temporary conditional use outside the building on an approved site. This ordinance shall apply to parking lots, as well as open spaces and green areas on approved site.

b. Definition of local Public Benefit Activity.  An event conducted as a part of or in furtherance of public purpose germane to the Outer Banks or for the benefit of one or more of its communities.

c. Temporary Conditional Use Permit.  A written permit must be issued for  the temporary conditional use of charitable fund raising activities and sales and local benefit activities within an area designated on the site plan for the 48 hour period specified in the permit.  The permit shall be issued to the property owner and the individual or organization conducting the fund raising event.  The temporary conditional use permit and submission of application shall require the information approved by the planning board based on the recommendation of the town planning staff.

d.  Non-profit Organization Charitable Fund Raising Activities and Sales and Local Benefit Activity.  Charitable fund raising activities and sales and local public benefit activities may be conducted on approved site plans of churches and public institutions in residential zones of the Town and on approved commercial site plans outside the building shown thereon in the beach and village commercial, planned commercial, and hotel zoning districts as a temporary conditional use pursuant to the following conditions:

1.  Charitable fund raising activities and sales and local public benefit activities conducted on open space and green areas as well as parking areas in excess of the number of the parking spaces required by the Town code for the primary use approved for the site.  The temporary conditional use area of the parking lot must provide a means of separating vehicular traffic from pedestrian traffic.  If the proposed activity is scheduled for the day after Labor Day the day before Memorial Day inclusive, then the temporary conditional use of required parking spaces may be authorized by the Town Manager or his designee provided the applicant demonstrates there is adequate parking for the regular approved commercial uses on the site and the applicant has a means of separating vehicular traffic from the temporary conditional use area.

2.  Sidewalks and fire lanes shall not be blocked or pedestrian traffic impeded by the participants of the conditional or temporary use of charitable fund raising activities and sales or local public benefit activities.

3. Not more than six (6) charitable fund raising activities and sales or local public benefit activities shall be conducted on any commercial site during each calendar year.  The temporary conditional use shall commence and terminate at the times set forth in the temporary conditional use permit, and the term of the permit shall not exceed 48 hours unless otherwise approved by the Town Manager, but in no event more than seven (7) days.

4.  No land disturbing activities shall be undertaken in connection with the temporary conditional use on the site during the period of time set forth in the temporary conditional use permit.

5. The owner or legal occupant of the premises shall provide security for the items for sale and the activities conditional on the property.

6. In the event of a conflict between this section and Chapter 9, then the provisions of this section shall control.

7. The applicant and the owner shall provide reasonable access to sanitary facilities for the public during the period specified in the conditional use permit.  The location of the sanitary facilities must be designated on the site submission or designated in the application.   If the Town Manager determines that the restroom facilities are inadequate to provide service for the number of people present or anticipated to be present, then the Town Manager may suspend a permit already issued until adequate sanitation facilities have been provided or placed on the site.

8.  The applicant shall complete the temporary conditional use permit application form and agree to donate 100% of the proceeds of the charitable event after expenses to the designated charity.  (02-22)

9.  No activity not otherwise authorized by this Code shall be conducted pursuant to this paragraph including, but not limited to, animal or mechanized rides, but further provided that games of skill or chance may be conducted in a designated area on the site pursuant to applicable local, state and federal laws.

10.  Food and beverages to be consumed within the temporary use site area and in the site area may be sold and consumed in accordance with applicable county and state health and sanitary regulations.

11.  Temporary sold waste containers approved by the Town Planner shall be furnished during the term of the permit and removed within 24 hours after the expiration or termination of the permit.  The disposal of solid waste shall comply with applicable local and state regulations.

12.  No music or amplified noise shall be audible beyond the property lines of the approved commercial site on which the charitable fund raising activities and sales are being conducted except as provided herein.

On any commercial site on which music or amplified noise is proposed as part of the  non-profit charitable activity, the applicant has obtained the written consent of each owner of improved property contiguous to the subject site of the charitable fund raising activity, then this restriction shall not apply.  The consent of each owner of improved contiguous property shall be required for each temporary conditional use permit sought by the applicant and shall expire with the expiration of the temporary conditional use permit.  A consent of property owners who have contiguous improved property must be files with the Town not less than 48 hours prior to the temporary conditional use permit issuance date.

13. All artificial lighting used in the conduct of the charitable fund raising activities and sales shall be contained on the site and shall not interfere with the operation of motor vehicles on adjoining rights-of-way or interfere with the use of adjoining properties.

14. Any signs used in connection with the charitable fund raising activities and sales or local public benefit activities must comply with the Town Code.

15. The issuance of a temporary conditional use permit does not create a vested right to the issuance of a subsequent permit for the same or similar activity from year-to-year, and the failure of the permittee or property owner to comply with the standards and conditions of the permit and the Town Code shall be a ground for denying a subsequent permit application.

16. The Town Manager and town staff designated by the Town Manager are authorized to issue permits pursuant to this paragraph; however, the denial of a permit may be appealed within ten (10) days of the date of denial to the Planning Board and the Town Council.  (98-11)

(Ord. of 8-18-86, § 5.14)

### Sec. 20-402.  Use of habitable and nonhabitable floors.

(a) For the purposes of this chapter, area or space above the top plate of a habitable floor which is open, unrestricted and unobstructed such as used in an A-frame, cathedral ceiling or opposed shed roof type construction shall not be considered as a part of such habitable floor. The height of habitable floors shall be measured vertically from the bottom of the sill girder to the top of the top plate. It is the purpose and intent of this subsection to allow the construction of buildings within the town consisting of two (2) habitable floors or levels and one (1) floor or level which is capable of some, but not all, of the primary functions set forth in the definition of habitable floors and levels in section 20-1 in addition to multiple secondary uses or functions.

(b) The enclosed area above the top plate of a structure shall not be inhabited by any person or used for any purpose other than the storage of personal effects or property.

(c) Only habitable floors or levels shall be occupied or used for more than two (2) of the primary functions specified in the definition of habitable floors and levels in section 20-1. A nonhabitable floor or level may be used for two (2) of such functions in addition to other secondary uses, such as, but not limited to, recreational, artistic, storage, entertainment, business, professional, garage, or any other use allowed or permitted under this chapter.

(Ord. of 8-18-86, § 2.02)

### Sec. 20-403.	Yards generally.

(a) Fences, walls, poles, posts, and other customary yard accessories, ornaments, and furniture may be permitted in any yard subject to height limitations and requirements limiting obstruction of visibility or any other requirements of this chapter. Accessory buildings shall comply with setbacks in accordance with Sec 20-423. (03-25)

(b) The depth of the required front yard shall be measured at right angles to a straight line joining the foremost points of the side lot lines. In the case of rounded property corners at street intersections, the foremost point of the side lot line shall be assumed to be the point at which the side and front lot lines would have met without such rounding. Front and rear yard lines shall be essentially parallel.

(c) The depth of a required rear yard shall be measured in such a manner that the yard established is a strip of the minimum width required by district regulations with its inner edge parallel with the rear lot line.

(d) The width of the required side yard shall be measured in such a manner that the yard established is a strip of the minimum width required by district regulations with its inner edge parallel with the side lot line.

(Ord. of 8-18-86, § 2.02)

### Sec. 20-404.  Reduction of lots or areas below minimum.

(a) No yard or lot existing on August 18, 1986 shall be reduced in dimension or area below the minimum requirements set forth in this chapter.

(b) Yards or lots created after August 18, 1986 shall meet at least the minimum requirements established by this chapter.

(Ord. of 8-18-86, § 5.01)

### Sec. 20-405.  Lot access requirements.

No structure requiring a building permit shall be erected on any lot which:

(1) Does not abut either a public right-of-way or a private street or easement at least thirty (30) feet in width which has been approved in accordance with the provisions of this chapter and recorded by the register of deeds of the county; or

(2) Does not have a cartroad, lane or other access to a public street or highway which access is described in an instrument recorded in the office of the register of deeds of the county prior to adoption of this ordinance or described and sworn or affirmed to by affidavits presented to the planner setting forth the fact that the same existed prior to August 18, 1986.

(Ord. of 8-18-86, § 5.03; Ord. No. 89-11, 4-17-89)

### Sec. 20-406.  Vision clearance at intersections.

On a corner lot which abuts a state-maintained right-of-way in any district no planting, structure, fence, wall or other obstruction to vision more than three (3) feet in height shall be placed or maintained within the triangular area formed by the intersecting street right-of-way lines and a straight line connecting points on said street lines each of which is twenty-five (25) feet in distance from the point of intersection. On all right-of-ways proper setback for vision clearance shall be determined by the planner, but in no case shall the requirement exceed that stated above.

(Ord. of 8-18-86, § 5.04; Ord. No. 89-11, 4-17-89)

### Sec. 20-407.  Walls and fences.

(a) Notwithstanding any other provision of this Code, a fence or retaining wall which meets the requirements of this section may be placed, constructed or erected within the front, rear or side yard established by the setback requirements of the Town Code. Within the residential districts of the town and along the common boundaries between residential and commercial districts, no wall or fence shall exceed six (6) feet in height, except that a fence surrounding or partially enclosing a tennis court may not exceed twelve (12) feet in height. No fence or retaining wall within a commercial district shall exceed ten (10) feet in height.

(b) Swimming pools shall be surrounded by a fence or wall. The fence or wall shall be constructed out of materials and to a height which is sufficient to prevent small children from entering the enclosure without the knowledge and supervision of an older, responsible child or adult. Access to the pool through the fence shall be by means of a gate equipped with a latch or other device which maintains the gate in a closed position when it is not open for the purpose of providing ingress or egress to the pool.

(Ord. of 8-18-86, § 5.05; Ord. No. 90-11, 7-16-90)

### Sec. 20-408.  Buffer strips.

Where a buffer strip is required by the provisions of this chapter, a plan for each such buffer strip shall be approved by the planning board and upon completion the buffer strip must be approved by the zoning administrator.

(Ord. of 8-18-86, § 2.02)

### Sec. 20-409.  Reduction of front yard setback requirements.

In any residential district, where the average setback distance for existing buildings on all lots located wholly or partly within two hundred (200) feet of any lot, and within the same zoning district and fronting on the same side of the same street as such lot, is less than the minimum setback required in the zoning district, the setback on the lot may be less than the required setback, but not less than the existing average setback distance for all lots within the two hundred (200) feet and in no instance shall the setback be reduced to less than fifteen (15) feet. When lots within such two hundred (200) feet are vacant, the vacant lots shall be considered as having the minimum required setback for the purpose of computing an average setback distance.

(Ord. of 8-18-86, § 5.06)

### Sec. 20-410. Maximum number of principal buildings on lot.

Only one (1) principal building and its customary accessory buildings may hereafter be erected on any lot except as authorized in this chapter for planned unit or planned commercial development, multi-family developments (04-25) and mini-storage facilities. (97-5c)

(Ord. of 8-18-86, § 5.02)

### Sec. 20-411.  Aboveground utility service wires and lines.

All aboveground cable TV and utility wires and lines running from the boundary line of a lot or tract used for commercial, multifamily or group development purposes are declared to be a nonconforming use and shall not be repaired or replaced if service is interrupted due to damage or destruction of the wire or line. In such event, the line or wire shall be placed underground from the lot line to the building as soon as the utility can install the underground service.

(Ord. of 8-18-86, § 5.07)

### Sec. 20-412. Private utilities as a conditional use in all zoning districts.

(a) Private utilities or privately owned facilities treating, processing, transmitting, or furnishing water, sewage, and/or wastewater are permitted in all zoning districts, subject to the following conditions and such other reasonable conditions as may be imposed by the town council:

(1) The private utility or privately owned facility must be a part of and located within the boundaries of a subdivision or planned unit development.

(2) The private utility or privately owned facility shall be used and serve only the occupants of the subdivision, planned unit development, multifamily development project, or commercial development for which it was constructed and approved by the town.

(3) The proposed schedule of all use fees and charges shall be submitted prior to preliminary approval by the planning board as well as a certification from a licensed practicing engineer that the proposed schedule is sufficient (including the necessary capital reserves) to operate, repair, and maintain the private utility or facility in good condition.

(4) All odors associated with or generated by the operation of the private utility or privately owned facility (including odors from the materials treated or processed thereat) shall be contained within the boundaries of the site and, when requested by the planning board, an engineer's certification shall be furnished to the planning board that the private utilities or privately owned facilities design, construction, and proposed methods of odor control are sufficient to contain all odors within the site boundaries.

(5) The developer shall post a surety bond in the amount of one hundred twenty-five (125) percent of the repair, maintenance, replacement and renovation costs to maintain the private utility or facility in accordance with applicable state standards and regulations for a period of fifteen (15) years from the date of town approval of the project. The licensed practicing engineer preparing the schedule of user fees and charges shall include the maintenance, repair, replacement, and renovation costs which shall determine the amount of the bond or security required by this paragraph. The town, acting in its sole discretion, may accept a bond or other form of indebtedness secured by a first lien on real estate with an appraised value of one hundred forty (140) percent of the amount of the secured obligation, or the town, in its discretion, may accept a cash escrow arrangement in which a portion of the obligation is paid pursuant to an agreement from the sale proceeds of each unit or time share sale (or any combination of the aforegoing means of securing the projected maintenance, repair, replacement, and renovation costs). The town shall have the absolute right to assign its interest in any bond, security agreement, deed of trust, escrow agreement, or any other agreement or undertaking delivered to the town in satisfaction of this paragraph to the property owners association of property owners in the project, and any such transfer shall terminate the obligations, responsibilities, or duties of the town with respect to the bond, security agreement, escrow deposit, or other arrangements securing the obligations of the developer set forth in this paragraph.

(6) The use of any of the sound or surface waters, including swamps, streams, canals, or waterways which connect to the sounds or to the ocean for the purpose of discharging wastewater or treated effluent from a sewage system shall be prohibited. All wastewater and treated effluent shall be land applied on the project site served by the private utility or private sewage facility and shall comply in all respects with applicable state and county health standards and regulations.

(7) The wastewater treatment facility shall be designed and constructed based upon the capacity calculated in gallons of flow per day (GPD) utilizing applicable state or county health department standards and regulations. The maximum GPD of capacity shall be that amount necessary to provide sewage and wastewater treatment of the waste originating within the subdivision, PUD, PCD, or commercial project site approved by the town and for which it is proposed to be built. The maximum capacity shall include any GPD flow load margins required by the state or local government regulations.

(8) The wastewater treatment facility shall include all forms of treatment (other than septic tanks) of human waste and sewage as well as wastewater generated by the occupants within the subdivision, PCD, PUD, or within the commercial project site.

(b) This section shall not be applicable to individual septic tanks.

(Ord. of 8-18-86, § 5.10; Ord. No. 94-20, 11-7-94)

### Sec. 20-413.	Water supply and sewage disposal requirements.

(a) Approval. Each application for site plan review, a building permit, or a certificate of occupancy shall be accompanied with plans of the proposed methods of water supply and sewage disposal approved by the appropriate authorizing agency. The planning board, however, may grant conditional approval pending receipt of the appropriate permit upon submission of a letter from the authorizing agency stating that the application has been submitted and is being reviewed. The site plan shall not be approved until approval of the required permits has been verified by the planning board. The plans submitted shall be of sufficient detail to show the exact location of all subsurface facilities including but not limited to, all wells, piping, laterals, and disposal areas.

(b) Method to be specified. Any such application shall specify the method or methods to be used and shall describe any special conditions to be met. Such methods, and the approvals required, include, but are not necessarily limited to, the following:

(1) For a connection to public sewer or water systems operated by a municipality, sanitary district, or other governmental agency, connection approval by an authorized officer of such systems.

(2) For a connection to community sewer or water systems operated by a responsible person, other than a governmental agency, connection approval by an authorized officer of such systems.

(3) For installation of other than public or community sewer systems, design approval by the county health department.

(c) System approval. The sewer and water systems to which connections are to be made shall be authorized as follows:

(1) Water supply.

a. For an individual water supply system, approval by the county health department is required.

b. For water supply systems serving ten (10) to twenty-five (25) dwelling units, approval by the state department of human resources is required.

c. For water supply systems serving twenty-five (25) or more dwelling units, approval by the state department of human resources and the state utilities commission, if appropriate, is required.

(2) Sewage collection and treatment.

a. For a septic tank, approval by the county health department is required.

b. For a sewerage system serving facilities regulated by the state department of human resources, i.e., institutions, restaurants, motels, etc., approval by the county health department is required.

c. For a sewer system serving all other uses, i.e., industry, commerce, communities, etc., approval by the state department of human resources and the state department of natural resources and community development is required.

The person operating a community water system for twenty-five (25) or more customers or a sewer system for which a rate is charged shall hold a certificate of public convenience and necessity from the state utilities commission, and there shall be recorded with a plat of the property the written affidavit of a registered engineer, engaged in the independent practice of civil engineering, that water and sewer mains and laterals comply with pertinent standards of the state department of human resources, as existing or as may hereafter be amended, and that such mains and laterals are installed and approved in writing by the state department of human resources; and a bond or trust instrument, or other form or written assurance, satisfactory to the town council, assuring the continuous proper maintenance and operation of such sewer and water systems. Where a community water system or a sewerage system furnishing services for twenty-five (25) or more customers within a multifamily or group development project (including planned unit or planned commercial developments) is charged an assessment by a property owners' association and a portion of the assessment is used for the purpose of paying the cost and expense of operating, maintaining, and repairing the community water system or the community sewerage system, then all the requirements of this paragraph must be satisfied.

(Ord. of 8-18-86, § 5.09)

### Sec. 20-414.	Outdoor lighting.  (02-31)

(a) Intent. Outdoor lighting standards are hereby established to provide safe lighting levels at proper intensities so as to adequately serve their intended uses and not unreasonably interfere with the use and enjoyment of neighboring properties. These standards are not intended to regulate single family and duplex security lighting, provided such lighting does not create a nuisance to neighboring properties. Regulations are intended to preserve the visual integrity of the nighttime environment by reducing glare, discouraging unnecessary illumination, and prohibiting the use of structure highlighting without decreasing safety, utility, and security.  To satisfy this intent, and for the purposes of this ordinance, interior lighting shall be subject to these regulations provided interior lighting contributes to the excessive illumination of an outdoor area, or is a source of glare that is visible from the property line or any off-site location. All outdoor lighting shall be erected, altered and maintained in accordance with the following provisions

(b) Purpose. The Purpose of this ordinance is to regulate artificial illuminating devices utilized for general illumination or advertisement. Regulations are designed to provide a uniform distribution of light that minimizes light trespass and controls glare on and off the property. The following general and specific standards encourage lighting that favorably contributes to visual performance, safety, and aesthetics from properly shielded light sources for the following lighting applications: security, parking lots, recreational facilities, buildings and structures, landscaping, open canopies, and signs.

(c) Inspections. All outdoor light fixtures are subject to inspection at any time by the Town of Kitty Hawk employee designated by the town manager to assure that the light fixtures are in compliance with these provisions. When a light fixture fails to comply, the Town employee shall give written notice to the owner and tenant of the site on which the light fixtures are located stating that the light fixtures shall be brought into compliance or removed at the owner’s expense. The remedies set out in Sec 20-468 and Sec 20-469 of this chapter shall also apply to this section.

(d) Lighting Prohibited.

- Light fixtures that imitate an official highway or traffic control light or sign are prohibited.

- Light fixtures in the direct line of vision with any traffic control light or sign are prohibited.

- Light fixtures that have a flashing or intermittent pattern of illumination, are prohibited.

- Privately owned light fixtures located in the public right-of-way are prohibited.

- Light fixtures that are a source of glare by their design, orientation or intensity are prohibited.

- Searchlights are prohibited except when used by federal, state or local authorities in the operation of their official business.

- Light fixtures that violate any law of the State of North Carolina relative to outdoor lighting are prohibited.

- General purpose area and sports floodlights are prohibited except to illuminate building facades and outdoor recreational uses as provided in subsections (h) (4) b. and (h) (5) a.

- Illumination of the public beach and estuarine waters from uses that are not water dependent shall be prohibited.

- Uplighting of windsocks, fountains or flags is prohibited, excepting the United States flag, North Carolina flag, official flag of the Town of Kitty Hawk  or permitted flag sign.

- Unshielded open vertical light fixtures are prohibited.

- Outdoor miniature lights on strings are prohibited except from mid November until mid January, provided such lighting does not create glare or is a nuisance to adjacent properties.

(e) General Provisions.

  - Light fixtures shall be located on the site and designed, shielded, or oriented in such a manner as to minimize light spill across property lines and prevent glare at any location on or off the property.

  - All wiring to light fixtures not located on a building shall be placed underground.

  - Principal buildings shall provide security lighting.

  - Light fixtures and supporting structures shall be designed and constructed to comply with North Carolina State Building Code requirements.

  - No light fixture, including signs, shall exceed thirty (30) feet in height.

(f) Lighting Plan Required.

(1)All proposed development for which site plan approval is required, shall require a lighting plan produced by the lighting manufacturer, registered architect, or engineer and shall be submitted as part of the site plan review process. To enable the Town to determine that applicable provisions of this ordinance will be satisfied, the lighting plan shall include the following:

a.	A site plan, drawn to a scale of at least one (1) inch equal one hundred (100) feet, showing proposed features to be illuminated, including but not limited to: building, landscaping, signs, parking and loading areas, and location of all proposed exterior light fixtures;

b.	Specifications and descriptions for all proposed light fixtures including photometric data;

c.	Proposed mounting height of all exterior light fixtures;

d.	Analyses and illuminance level plans in the form of an iso foot-candle or grid diagram drawn to a scale referenced above. The submitted grid plan shall plot lighting levels at ten-foot intervals or less. The iso foot-candle diagram shall plot foot-candles in increments of 0.5 foot-candle or less. Either plan shall clearly account for lighting levels produced by all exterior light sources. The outdoor lighting plan shall account for cumulative lighting levels of combined indoor and outdoor light sources if any wall of the proposed building meets either subsection (f) (1). f 1 or (f) (1).f 2 below.

- Drawings of all proposed building elevations showing the location and type of light fixtures, the portions of the walls to be illuminated, the illuminance levels at the wall surface, and the aiming points for any floodlight fixture.

- In addition to the above, an indoor lighting plan that details the location, specifications, and descriptions of proposed interior light fixtures and proposed average foot-candles of interior customer areas shall be required for approval when submitted building elevations demonstrate:

- a wall surface area is comprised of fifty (50) percent glass or greater, or

- the location of any glass area or architectural feature creates a potential for off-site glare.

- Any proposed modification or alteration to a previously approved building elevation, site plan or lighting plan may be approved administratively by the Town employee designated by the town manager provided the proposed change is considered minor and does not violate the standards noted herein.

(g)	General Performance Standards. The illumination level from any light source or combination of light sources shall not exceed the following horizontal foot-candle level.

- For sites with cut-off fixtures, the maximum maintained foot-candle level at any point on a common property line of improved residential property shall not exceed one and one-half (1.5) foot-candles.

- For sites with non cut-off fixtures, the maximum maintained foot-candle level at any point on a common property line of improved residential property shall not exceed four-tenths( 0.4) foot-candle.

(h)	Specific Lighting Application Standards.

- Parking Lots. Illumination standards for parking lots are based on anticipated vehicular and pedestrian activity. The standards are designed to provide the minimum lighting necessary to ensure adequate vision and safety in parking areas while at the same time minimizing glare. Uses with medium and low levels of vehicular activity have the option of providing cut-off or non cut-off fixtures.

- High levels of activity include, but are not limited to the following uses: Automobile Service Station, Convenience Store, Outdoor Recreational Uses, Home improvement center, Auto Dealership, Video Store, Attended Car Wash, Financial Institution, Food Store, Theater, and Restaurant.

- Medium levels of activity include, but are not limited to the following uses: , Retail, Indoor Recreational Use, Retail Shopping Center, Hotel, Medical and Health Clinics, Drug Store, Home Center, Child Day Care Center, Indoor Assembly, Hospital.

- Low levels of activity include, but are not limited to the following uses: Professional Office, Religious Complex, Townhouse and Multi-family dwellings, School, Marina, Wholesale, Warehouse, Self Storage Complex, Furniture or Appliance Store, Private Club.

- Parking lot lighting shall meet one (1) of the following standards:

- When all light fixtures are cut-off fixtures, the following standards shall apply:

| Level of Activity | Maximum Maintained Foot-Candles | Minimum Maintained Foot-Candles |
| --- | --- | --- |
| High | 10.0 fc | 0.7 fc |
| Medium | 7.0 fc | 0.5 fc |
| Low | 3.0 fc | 0.2fc |

- When light fixtures are not all cut-off fixtures, the following standards shall apply:

| Level of Activity | Maximum Maintained Foot-Candles | Minimum Maintained Foot-Candles |
| --- | --- | --- |
| Medium | 3.5 fc | 0.5 fc |
| Low | 1.5 fc | 0.2 fc |

- Other uses

| Cut-off fixtures required | Maximum Maintained Foot-Candles | Minimum Maintained Foot-Candles |
| --- | --- | --- |
| Sexually Oriented Businesses | 10.0 fc | 1.5 fc |

(2) Canopy Lighting. Open canopies located over automobile service station gas pumps and drive-through banking facilities shall comply with the following standards:

- Only the area directly below canopies may be illuminated such that illumination levels shall not exceed fifteen (15) maximum maintained foot-candles and shall not be less than two (2) minimum maintained foot-candles.

- Light fixtures shall be mounted only under the canopy and shall be either recessed, or cut-off fixtures.

- Light fixtures shall not be located elsewhere on the canopy and the sides of the canopy shall not be internally or externally illuminated.

- Parking spaces not located directly beneath the canopy shall be illuminated in accordance with the requirements for parking areas.

- Security Lighting. Security lighting may be utilized for the protection of people and property by illuminating areas of building ingress, walkways, and open storage areas. Security lighting may include vertical illumination, provided illumination above the doorway, entry or window elevation is minimal. Security lighting shall not exceed one (1) average maintained foot-candle.

- Architectural and Interior Lighting.

- Light fixtures mounted on the building shall be recessed, fully shielded or directed to the wall surface.

- Floodlights may be utilized to up-light opaque wall surfaces from the ground, provided the light fixture is located a horizontal distance no greater than fifteen (15) feet from the wall, shall not be mounted greater than 30 inches in height, and illumination levels measured at the vertical plane shall not be exceeded.

- The maximum vertical illumination level of an entrance façade shall not exceed (5) average maintained foot-candles. The maximum illumination level shall not exceed (1) average maintained foot-candle for other surfaces.

- Walkways covered by a supported roof canopy may be illuminated up to twelve (12) average foot-candles.

e.	When a single wall surface area of a building is fifty (50) percent glass or greater, or interior light fixtures are visible from the property line or off-site location, the following requirements shall apply:

- Interior light fixtures shall be fully shielded or shielded to any cut-off angle required to prevent line of sight contact with the light source, lens or opening.

- Average interior illumination level shall be the minimum average IESNA recommended level of illumination for that particular use.

- Lighting of Outdoor Recreational Facilities.

a.	General purpose area and sports lighting floodlight fixtures may be utilized provided all such light fixtures are located a minimum of two hundred (200) feet from the nearest right-of-way, and shall be hooded, directed, or shielded such that the light source cannot be directly viewed from adjacent properties and streets.

- Areas intended solely for pedestrian circulation, such as walkways connecting accessory buildings and viewing stands, shall be provided with security level illumination.

- Light fixtures for the recreational use shall be turned off no later than 11:30 p.m., unless otherwise specified. Only security level lighting shall be utilized to facilitate cleanup and nighttime maintenance.

- A single flashing navigational light may be used in conjunction with an approved boat rental establishment subject to the following provisions:

1.	The boat rental establishment shall provide documentation to the Town of Kitty Hawk staff that the light fixture and its location have been approved by the Commander, Fifth Coast Guard District, Portsmouth, Virginia.

2.	The light shall be used only when the business is open and between the hours of sunrise to one-half (1/2) hour after sunset.

3.	The light source shall not be visible from adjacent properties and rights-of-way.

- Navigational lights shall not be used in conjunction with any permitted sign.

e.	The following illumination levels shall not be exceeded:

| Recreational Use | Average Maintained Foot-Candles |
| --- | --- |
| Driving range | 15 |
| Basketball court | 15 |
| Miniature Golf | 10 |
| Tennis/handball | 15 |

- Illuminated Signs.

a.	Internally illuminated wall or freestanding signs that utilize fluorescent tubes as a light source shall be designed and constructed such that tubes are spaced at least twelve (12) inches on center, and mounted at least three and one-half (3.5) inches from the translucent sign surface.  Internal illumination may be permitted provided the sign surface has an opaque background and the light from illumination is only visible through letters, numbers or symbols on the sign surface.

b.	Spot or floodlights may be utilized to externally illuminate wall and freestanding signs, provided the light fixtures are designed, located, aimed, and shielded so that light is directed only onto the sign area. Light fixtures shall not be aimed toward adjacent streets, or properties. Floodlights designed to illuminate signs from the ground, shall not be mounted at a height greater than thirty inches, or located a horizontal distance greater than three-quarters the height of the sign or fifteen feet, whichever is less. The average level of illumination shall not exceed five (5) foot-candles.

(i)	Measurements.

- Lighting levels shall be measured in foot-candles with a direct-reading, portable light meter. The meter shall have a color and cosine-corrected sensor with multiple scales and shall read within an accuracy of plus or minus five (5) percent. It shall have been tested, calibrated, and certified by an independent commercial photometric laboratory or the manufacturer within one year of the date of its use.

- Unless otherwise specified, the meter sensor shall be mounted not more than six (6) inches above ground level in a horizontal position to measure horizontal illumination. Vertical illumination shall be measured at a height of five feet with the meter sensor mounted not more than six (6) inches from the wall surface and the meter sensor in the vertical position. Readings shall be taken by only after the cell has been exposed long enough to provide a constant reading.

- Measurements of newly installed high intensity discharge light fixtures shall account for the light loss factor figured into the calculation of a point-by-point printout. A light loss factor of 0.75 is normal for metal halide and 0.80 is normal for high-pressure sodium when adjusting for lamp lumen depreciation and luminaire dirt depreciation as the system ages. As an example, a calculation with a light loss factor of 0.7 would suggest a meter reading of one (1) initial foot-candle for a printout calculation of 0.7 maintained foot-candles.

(j) Non-conformities.

(1) This chapter shall be effective immediately upon adoption by the Town Council.

(2) Any lighting not in conformity with the requirements of this chapter and lawfully in place prior to the effective date of this chapter shall be removed or rendered in compliance with this chapter within five (5) years of its effective date.

(3) After the effective date of this chapter, any lighting system that replaces a lighting system not in conformity, or any lighting system that is moved, must meet the standards of this chapter.

(Ord. of 8-18-86, § 5.12; Ord. No. 93-3, 1-4-93)

### Sec. 20-415.	Access to U.S. 158 and N.C. 12.

Due to the limited amount of land available within the town for major thoroughfare rights-of-way and the traffic hazard involved in frequent entrances and exits from a major thoroughfare, it is the intent of this chapter to keep driveways and street intersections along U.S. 158 and N.C. 12 to the minimum possible. In any district established by this ordinance where a lot abutting either U.S. 158 or N.C. 12 also abuts any other dedicated public right-of-way, such right-of-way shall be used for access rather than the highway.

The town council may approve an access from US 158 or NC 12 onto a commercial site which also abuts another dedicated public right-of-way upon satisfaction of the following: Upon the request of the applicant and at the applicant's expense, the town will engage a traffic engineer to study the site plan submitted by the applicant and the proposed traffic movements utilizing the side street only for access and the side street together with an access (or limited access) from US 158 or NC 12. If the traffic engineer concludes that it would be safe for the public for the applicant's site to be accessed from NC 12 or US 158 as well as the abutting street, then the town council may consider approving the access onto US 158 or NC 12. In the event the traffic engineer recommends improvements to the right-of-way or restrictions on the use of the access on US 158 or NC 12, then the applicant must construct the improvements at his expense before the occupancy permit will be issued. The use of the accessway can be restricted based upon the recommendations of the traffic engineer and the approval of the town council. The applicant must receive the consent of the Department of Transportation of the State of North Carolina before commencing any right-of-way improvements required by the town council. (94-17)

(Ord. of 8-18-86, § 5.15; Ord. No. 88-15A, 10-24-88)

### Sec. 20-416.	Streets in certain developments.

When a multifamily, group development project, group housing project or commercial site is subject to review under article II, division 2 of this chapter and the project or site plan contains a street within the site or project (or crossing the same), then the street shall be constructed in accordance with the design and construction standards set forth in chapter 19, article IV as the same is amended from time to time and which standards are incorporated herein by reference. A way of ingress, egress and regress providing traffic circulation within the site or project shall be considered a street for the purposes of this section, but the planning board may recommend and the town council may approve a right-of-way of less than sixty (60) feet but not less than thirty (30) feet if it finds that the volume of traffic, the traffic circulation pattern and the utility layout do not necessitate a right-of-way width of sixty (60) feet.

(Ord. of 8-18-86, § 5.17; Ord. No. 87-2, 1-19-87)

### Sec. 20-417.	Private streets.

(a) Private streets shall be designed and constructed to the town's street standards and the developer shall submit the required test results to the town in order for the town to verify that the street construction standards have been satisfied.

(b) Private streets shall be permitted only in those subdivisions in which the developer and the developer's successors in interest have implemented and maintained a means of controlling and restricting access, ingress and egress over and upon the subdivision streets, such as guards, gates, chains, pass keys or cards.

(c) Subdivisions with private streets shall have the following certificate placed on the subdivision plat filed with the county register of deeds:

"The streets in this subdivision are private. The property owners are responsible for maintaining and repairing the streets as well as paying the costs thereof. Municipal and other governmental services may be restricted or not be furnished to the property of owners using the private streets for access. With a few exceptions, the traffic laws of the State of North Carolina and the Town of Kitty Hawk are not enforceable on private streets."

(Ord. of 8-18-86, § 2.02; Ord. No. 88-16A, 10-24-88)

### Sec. 20-418.  Parking spaces.

Each parking space shall be located outside any dedicated right-of-way or private right-of-way or access easement. Parking spaces located along streets, roads, access easements or ways of ingress and egress within subdivisions, multifamily housing projects, planned unit or commercial developments, or commercial sites, shall provide sufficient distance between the right-of-way and the parking space for motor vehicles to maneuver into and out of the spaces without backing onto the right-of-way.

(Ord. of 8-18-86, § 2.02)

### Sec. 20-419. Parking, storage, or use of major recreational equipment.

(a) In this section, "major recreational equipment" includes any boat, boat trailer, pickup camper or coach (designed to be mounted on an automotive vehicle); motorized dwelling, tent trailer, and the like, and any case or box used for transporting recreational equipment, whether occupied by such equipment or not.

(b) Major recreational equipment parked or stored on residential premises or in any location other than an approved travel trailer park shall not be used for living, sleeping, or housekeeping purposes.

(Ord. of 8-18-86, § 5.13)

### Sec. 20-420. Fire hydrants.

(a) Fire hydrant locations shall be shown on all multifamily, commercial and shopping mall and any non-residential site plans submitted to the town for review. A fire hydrant shall be located no more than three hundred (300) feet from each building shown on the site plan and not more than 500 feet of hose lay distance from a fire hydrant to the building, and a fire hydrant shall be located within 150 feet of the fire department connection as the fire truck travels along access roadways with all-weather driving surfaces. The fire hydrant shall be installed at the expense of the property owner and shall comply with the standards and specifications of the town and/or the state.  No hydrant shall be closer than fifty (50) feet to a building.  Fire hydrant locations must be approved by the Town Council based upon the recommendation of the fire department. (97-6)

(b) The fire hydrant shall be either a Centurion model manufactured by Mueller Co., Inc., or a Waterous Hydrant manufactured by Waterous Co., Inc.

(c) The property owner shall install fire hydrant location markers pursuant to the standards of the volunteer fire department and paint the fire hydrant red with a reflectorized white top using paint approved by the planning board or the county water department.

(Ord. No. 89-4, 3-20-89)

### Sec. 20-421.  Home occupations.

(a) The purpose and intent of a home occupation is to allow a low intensity business use of a small portion of residential property that does not alter the character of the structure in which the use takes place or alter the residential character of the neighborhood in which the residential structure is located.

(b) There shall be two (2) levels of home occupations. Level I shall be a use that does not require any more than a phone and/or a computer within the home to conduct the business, and no significant additional traffic is generated to or from the home as a result of the business being conducted, either by clients or deliveries. A level I home occupation may be administratively approved by the planner; however, the planner may refer the proposed use to the planning board for a recommendation and to the council for final approval.

(c) Where home occupations have been designated as either a permitted or conditional use in a zoning district of the town, the level I home occupation use shall comply with the following conditions and standards as well as any other reasonable conditions imposed by the town planner or town council:

(1) No merchandise or commodity shall be produced on the site or sold from the site, only a service shall be provided;

(2) Not more than twenty-five (25) percent of the total actual floor area of the dwelling shall be used for the home occupation;

(3) Only those residents living on the lot shall engage in the home occupation activity. Not more than two (2) residents living on the premises shall engage in the home occupation. The home occupation shall not employ persons living off the premises;

(4) Any commercial vehicle utilized in connection with the home occupation shall be licensed in the name of the owner and resident of the premises who is a participant in the home occupation activity and which is used on a regular basis by the residents of the premises for personal reasons as well as commercial uses incident to the home occupation;

(5) In addition to the required residential parking, two (2) on-site parking spaces shall be provided for the home occupation use, if the customers or clients need to come to the site. (This regulation may be waived either administratively or by the council if deemed unnecessary relative to the use being proposed.);

(6) The home occupation shall create no noise which is audible beyond the boundary line of the home occupation site;

(7) The service shall not generate more than three (3) deliveries per week to the site of the home occupation.

(d)A level II home occupation shall be other uses that meet the criteria listed below and/or may need additional conditions to meet the purpose and intent of the Code, and protect the integrity of the residential neighborhood. A level II home occupation shall require the review and recommendation of the planning board and the approval of the town council.

(e) Where home occupations have been designated as either a permitted or conditional use in a zoning district of the town, the home occupation use shall comply with the following conditions and standards as well as any other reasonable conditions imposed by the town council:

(1) No merchandise or commodity other than that produced on the premises shall be sold on the premises;

(2) Not more than twenty-five (25) percent of the total actual floor area of any dwelling shall be used for home occupation. If an accessory building is used for home occupations, no more than three hundred (300) square feet shall be used for the home occupation. In such event, the dwelling shall not be used for the home occupation;

(3) Only those residents living on the lot shall engage in the home occupation activity. Not more than two (2) residents living on the premises shall engage in the home occupation. The home occupation shall not employ persons living off the premises;

(4) Any commercial vehicle utilized in connection with the home occupation shall be licensed in the name of the owner and resident of the premises who is a participant in the home occupation activity and which is used on a regular basis by the residents of the premises for personal reasons as well as commercial uses incident to the home occupation;

(5) No home occupation or activity shall require the presence of the customer or client in order for the home occupation or activity to be performed or accomplished;

(6) In addition to the required residential parking, two (2) on-site parking spaces shall be provided for the home occupation use. (This regulation may be waived by the council if the council deems the parking standard unnecessary relative to the use being proposed and the specific site utilized. If the parking standard is waived by the council, then the council shall enter the reason for waiving the standard upon the minutes of the meeting at which the action was taken.);

(7) If the home occupation creates noise which is audible beyond the boundary line of the home occupation site, then the home occupation shall not be conducted before 8:00 a.m. and after 6:00 p.m.;

(8) No person shall use or store hazardous or dangerous materials in such quantities as to create a danger to adjoining residents or property. No disposal of hazardous or dangerous material shall be made on the site;

(9) Any odors generated by the home occupation shall be contained and retained on the site;

(10) No physical alterations to the home or improvements to the property shall be required or necessary to conduct the business, such as changes to the septic/sewage system, water supply, entrances, etc.;

(11) In any district in which a home occupation use lawfully existed on May 7, 1990, the home occupation may continue notwithstanding its nonconformity with the standards set forth above. The buildings used for such purpose may be repaired, maintained and rebuilt, provided the owner has obtained a conditional use permit for said home occupation and the repairs, maintenance and reconstruction are undertaken and conducted pursuant thereto.

(Ord. No. 90-7, § 1, 5-21-90; Ord. No. 96-6, 4-1-96)

### Sec. 20-422.	Prohibited recreational uses.

(a) The town supports the provision of traditional recreational facilities within its borders, but does not support, promote or encourage, outdoor uses such as (but not limited to) amusement parks, carnivals and carnival rides, water rides, and vehicle rides, or any go-cart, motorcycle, or terrain vehicle tracks, or any other similar use. Outdoor uses should be of low intensity and shall utilize the natural terrain as much as possible without the addition of tracks, mechanized facilities or structures for the conduct of the use.

(b) The following outdoor recreational uses have either been prohibited or subjected to substantial restrictions in the conduct thereof:

(1) Putt-putt type golf course and animated courses: section 20-264(b)(1).

(2) Horseback riding: section 3-6.

(3) Surfer leashes: section 4-1(d).

(4) Jet-propelled personal watercraft: section 4-2.

(5) Vehicles on beaches: section 4-16.

(6) Fires on beaches: section 4-17.

(7) Prohibition of firearms: section 10-6.

(8) Dogs hunting deer: section 1 of Chapter 585, House Bill 1633.

(Ord. No. 96-1, 1-8-96)

### Sec. 20-423.  Minimum yard requirements

Minimum yard requirements for all zoning district shall be open and unobstructed from thirty (30) inches above the general ground level of the graded lot upward, except for the following intrusions:  roof eaves or overhangs, firewalls as required by North Carolina Building Code, elevated bay windows, elevated fireplace projections, and light fixtures, may project into the minimum yard not more than two (2) feet.  (99-12)

### Sec. 20-424.  Metal Buildings (03-24)

Metal buildings which are visible from Croatan Highway (U.S. Highway 158) or

N.C. Highway 12 (North Virginia Dare Trail) will be approved only if the front, sides and back are architecturally treated with non-metal facades.  Existing or proposed vegetated or material buffers, screens, or fences which obstruct the visibility of the building from adjacent property owners shall not be an acceptable alternative to providing a non-metal façade.  The Planning Board must approve the method in which a building is architecturally treated.

The Town may approve metal buildings, which are not visible from Croatan Highway or N.C. Highway 12. (03-24)

### Secs. 20-425--20-435.  Reserved.

## DIVISION 2.  OFF-STREET PARKING AND LOADING

### Sec. 20-436.	Generally.

(a) At the time of erection of any building or structure, or at the time any main building or structure is enlarged or increased in capacity by adding dwelling units, guest rooms, seats or floor area, or before conversion to another zoning use or occupancy, permanent off-street parking space shall be provided according to the amounts and specifications provided by this division.

(b) Each parking space shall have a minimum length of eighteen (18) feet and a minimum width of nine (9) feet six (6) inches with the addition of an unpaved open area two (2) feet in length beyond the concrete or wooden curb installed at the end of each parking space. It shall have vehicular access to a publicly dedicated street or alley, except as authorized in this chapter for planned unit development.

(c) Sufficient maneuvering space shall be provided so that no vehicle will be required to back into the public right-of-way or into the accessway or private street used for the purposes of access, ingress and egress by the public or other property owners within the specific project.

(d) No parking spaces for residential use, except for single-family or two-family use, shall be located in the required front yard.

(e)  Approved surfaces: (02-30)

(1) Required commercial parking spaces, loading areas and driveways shall be graded and improved with surfaces approved by the town. Such parking surfaces are concrete, asphalt or turfstone. Grass, however, may be used as a parking surface, in commercial districts, if the town council approves it as a conditional use. Driveways shall be constructed of concrete, asphalt, or turfstone. (02-30) Parking surface and driveways within Ocean Hazard Areas of Environmental Concern maybe be constructed of alternative materials as approved by the Coastal Area Management Act. (06-03)All parking surfaces must be maintained in a manner that promotes safe and convenient use in all weather conditions. (02-30)

(2) Required residential parking spaces and driveways shall be graded and improved with surfaces approved by the town.  Such parking surfaces are concrete, asphalt, or  turfstone.  Grass or gravel, however, maybe used as a parking surface in the VR-1 and VR-2 districts. (02-30)

(3) Notwithstanding any lot coverage standard and parking surface standard herein to the contrary, a church or cemetery site plan in residential districts of the town can utilize a reinforced grass parking surface for that portion of the site plan which exceeds thirty (30) percent lot coverage but does not exceed fifty (50) percent lot coverage subject to the following conditions:  (99-22)

a.  The parking lot area exceeding thirty (30) percent but less than fifty (50) percent lot coverage must be improved with a pervious material which will support the weight of the motor vehicle that the impervious parking area was designed to support and which will allow grass to grow within the parking area exceeding thirty (30) percent lot coverage.

b.  All other parking lot standards and requirements not inconsistent with this paragraph must be satisfied.

c. The reinforced grass surface parking area shall not exceed twenty (20) percent of the total site square footage and the site plan notes must include the reinforced grass parking surface requirements of this subparagraph (2).

d. The boundaries of the reinforced grass parking surface area must be clearly delineated on the site plan, and that areas shall not be covered with any impervious material, except for motor vehicles temporarily parked on the reinforced grass parking surface area.

e.  The owner or the occupant of the premises shall establish and maintain a natural grass surface on the reinforced grass parking area designated on the site plan.  (99-22)

f.  (05-01)

1. As an alternative to clay drive lanes in reinforced grass parking lots, the parking lot area, both drive aisles and parking spaces, exceeding thirty (30) percent but less than fifty (50) percent lot coverage maybe be improved with a compact three-quarter (3/4) inch to one (1) inch washed gravel surface with a compacted base material of Geo-grid/web or Sub-grade fabric that will support the weight of the motor vehicles that the impervious parking area was designed to support.  (05-01)

2. Pea gravel for the purpose of this section is defined as three-quarter (3/4) inch to one (1) inch, round washed gravel. (05-01)

3. The drive aisle must be able to support the weight of the fire apparatus required for fire protection. (05-01)

4. The pea gravel must be maintained for material continuity, i.e.- pea gravel must not be rutted and deformed from traffic useage. (05-01)

5. The pea gravel must be maintained in order to keep intersections with Town and State roads clean of  the material. (05-01)

(f) Required off-street parking spaces are permanent areas and shall not be used for any other aboveground purpose.

(g) For uses not specifically mentioned, off-street parking requirements shall be applied by the manager or his designee based upon requirements in this division for similar uses. (02-30)

(h) All space requirements which are based upon employment shall be computed on the basis of the greatest number of persons on duty at any one (1) period during the day or night. In instances where calculations indicate a portion of one (1) space is required, an additional full space shall be provided.

(i) Each application for a building permit or certification of occupancy submitted to the manager or his designee shall include information as to the location and dimensions of off-street parking and loading space and the means of ingress and egress to such space. This information shall be in sufficient detail to enable the manager or his designee to determine whether or not the requirements of this division are met. (02-30)

(j) The required parking space for any number of separate uses may be combined in one (1) lot but the required space assigned to one (1) use may not be assigned to another use, except that one-half of the parking space required for churches, theaters, or assembly halls whose peak attendance will be at night or on Sundays may be assigned to a use which will be closed at night and on Sundays.

(k) Reserved.

(l) Any off-street parking space required by any use permitted in any residential or commercial district shall be provided on the same lot with the use by which it is required. Off-street parking space in conjunction with commercial uses in other districts shall not be permitted in a single-family residential district. (02-30)

(m) Where off-street parking is provided between the building line and the property line for any business use a buffer strip of at least five (5) feet in width shall be provided adjacent to such property line. Curb cuts through such buffer strips shall be located no closer than thirty (30) feet to other curb cuts unless otherwise approved by the planning board.

(n) Any parking spaces required by this chapter shall be located within the corporate limits of Kitty Hawk.

(Ord. of 8-18-86, §§ 5.08, 5.08(A); Ord. No. 89-11, 4-17-89; Ord. No. 89-22, 8-21-89; Ord. No. 96-4, 2-7-96)

### Sec. 20-437.   Parking lots.

Where parking space for five (5) or more cars is permitted or required, there shall be compliance with the following provisions:

(1) Yards. No parking lot shall be located closer than ten (10) feet to a public right-of-way. The area between the parking lot and street right-of-way shall be planted and maintained in lawn or other appropriate planting, or shall be improved otherwise as approved in site plan review.

(2) Curb bumpers. The required front and side yards shall be set off from the parking area by a fixed curb of either concrete or chemically treated wood, not less than eight (8) inches or more than two (2) feet high.

(3) Lighting. Any lighting shall be arranged so as to direct the light and glare away from streets and adjacent property.

(4) Surface. All parking lots shall be provided with a town-approved surface with adequate controlled drainage facilities.

(5) Drainage. Parking lots shall not drain onto or across public sidewalks, or into adjacent property except into a natural watercourse or a drainage easement.

(6) Markings. Each parking space shall be marked off and maintained so as to be distinguishable.

(7) Entrances. On all corner lots, no vehicular openings shall be located at closer than thirty (30) feet from the point of intersection of the established street right-of-way lines. No entrances or exits, whether or not on a corner lot, shall exceed thirty (30) feet in width at the property line, or forty (40) feet at the curbline. There shall be a minimum distance between one-way driveways of twenty-five (25) feet measured along the curbline.

(8) Internal circulation. The internal circulation plan of parking lots shall be approved by the planning board.

(9) Solid waste container requirements. Sufficient space shall be provided on the premises for the location of a solid waste container or containers as required by ordinance which regulates the collection and disposal of trash and garbage. Such solid waste container location may be in a required parking lot; provided, however, that such location does not occupy a required parking space or maneuvering space and further provided that such solid waste container location shall provide convenient and safe access to the servicing vehicle.

(Ord. of 8-18-86, § 5.08(B); Ord. No. 90-5, § 6(q), 4-23-90; Ord. No. 93-3, 1-4-93)

### Sec. 20-438. Minimum parking requirements.

(a) The number of off-street parking spaces required below shall be provided on the same lot with the principal use except as provided in section 20-436, subsection (k), and the required number of off-street parking spaces specified for each use shall be considered as the absolute minimum. Where a fraction of a space is required by this ordinance the next whole number shall be provided. In addition, a developer shall evaluate his own needs to determine if they are greater than the minimum specified by this section.

Residential and Related Uses				Required Parking

“Single-family detached				1 space per 600 sq. ft. of total heated                  dwelling units				space.”  (03-43)

Townhouses						1.0 spaces per bedroom (02-28)

Multifamily development				1 parking space on the same

projects					lot for each bedroom or loft.

Hotels, tourist homes,					1.5 parking spaces for each

motels, motor courts,				room to be rented, plus 1

rooming or boarding houses			additional parking space for

each 3 employees .

Professional offices in own				3 parking spaces per office

residences					or home occupation in

addition to residence

requirements.

Hotels and Motels					Required Parking

Hotel and motel					1.15 spaces per rental room.

Hotel with restaurant in				1 parking space per 3

hotel structure					restaurant seats for 50

percent of the seating

capacity.

Hotel with restaurant					1 parking space per 3

located in separate,				restaurant seats.

freestanding building

Hotel with retail space				1 parking space per 350

within the hotel structure			square feet of the total

enclosed leasable or

occupiable retail space less

500 square feet of such

space. In no event shall

there be less than 1 parking

space.

Hotel with retail space				1 parking space for each

located in a separate				350 square feet of enclosed

freestanding building				leasable or occupiable retail

space.

Hotel with a lounge in hotel				1 parking space per 3

structure					lounge seats for 50 percent

of the seating capacity.

Hotel with a lounge					1 parking space per 3

providing live entertainment			lounge seats.

within the hotel structure

Hotel with a lounge located				1 parking space per 3

in separate, freestanding			lounge seats.

building

Hotel with meeting rooms				1 parking space per 3

located within the hotel			meeting room seats for 50

structure					percent of the seating

capacity.

Public and Semipublic Uses				Required Parking

Hospitals						1 parking space for each 2

beds intended for patient

use, 1 space for each

resident physician and 1

space for each 2 employees.

Clinics							10 parking spaces minimum

plus 5 parking spaces for

each doctor assigned plus 1

parking space for each

employee except where no

more than 2 doctors or 2

dentists are practicing in the

same building, the number

of parking spaces per doctor

or dentist need not be more

than 5 spaces and not more

than 1 space per staff

employee.

Nursing homes					1 parking space for each 5

beds intended for patient

use, and 1 space for each 3

employees .

Churches						1 parking space for each 2.5

Seats in the sanctuary  (03-03)

Elementary schools and				25 spaces in addition to 1

junior high schools				parking space for each

classroom and

administrative office.

Senior high schools					1 parking space for each 5

students for which the

building was designed plus

1 parking space for each

classroom and

administrative office.

Stadiums						1 parking space for each 2

spectator seats.

Auditoriums						1 parking space for each 2

seats in the largest assembly

room.

Public or private clubs				1 parking space for each

200 square feet of gross

floor space.

Public utilities						1 parking space for each

employee. With customer

service facilities, a

minimum of 5 additional

spaces. 1 space required for

unmanned facilities.

Retail and Office Uses				Required Parking

General or professional				1 parking space for each

offices and banks, see clinic			300 square feet of gross

requirements					floor space plus 1 space for

each 2 employees

Restaurants, cafes or public				1 parking space per 55 square feet

eating places				of customer service area,

Or:

1 parking space per 100 square feet

of gross floor area,

Drive-in restaurant or					For delivery type food

refreshment stands				establishments where there

is no customer seating, 8

parking spaces per 1,000

square feet of leasable area.

Grocery stores						1 parking space for each

200 square feet of gross

floor area.

Furniture and appliance				1 parking space for each

stores						300 square feet of gross

floor area.

Fishing piers						A minimum of 50 parking

spaces with 1 additional

parking space for each 250

square feet of gross floor

space enclosed within a building. (00-2)

Fitness Center					1 space / 150 sq.ft. gross floor area  (03-53)

Indoor entertainment					1 parking space for each

facilities including bowling			100 square feet of gross

alleys, dance halls, skating			floor area and 1 additional

rinks, amusement arcades			space for each 2 employees.

and similar facilities

Retail and Office Uses				Required Parking

Theaters						1 parking space for each 3

seats in the auditorium.

Funeral homes						1 parking space for each 2.5

chapel seats. Not less than

25 percent of the required

parking may be 9.5 by 18

feet stacking lane and drive

aisle spaces as delineated

on the site plan approved by

the town.

Retail uses not otherwise				1 parking space for each

listed						350 square feet of gross

floor space in addition to

the required number of

spaces for employee

parking. Gross floor space

may exclude floor space

used solely for storage and

which is inaccessible to the

public; however, if floor

area not to be open to the

public is excluded, a

recordable instrument

(approved as to form by the

town) must be filed with the

county register of deeds

limiting the excluded area

to either storage or other

uses supportive of the

primary business use and

which prohibits public

access to the excluded area.

Services						Required Parking

Conveyor car wash					1 parking space for each 1.5

employees during any shift

and 1 parking space for

each enclosed bay. The

enclosed space within the

bay does not count as a

parking space for these

purposes.

Wholesale Uses and Warehouses			Required Parking

Wholesale uses					1 parking space for each

employee on the largest

shift.

Warehouses						2 parking spaces for each 3

employees on the largest

shift.

(b) The following shall apply to all commercial establishments:

(1) All commercial establishments must have a minimum of five (5) customer parking spaces in addition to the employee parking spaces.

(2) Town-approved side markers are required to designate parking spaces.

(c)The following shall apply to all parking requirements for the hotel and hotel accessory uses: The seating capacity or a number of seats to be used in calculating the required parking spaces shall be determined by the planning board after consultation with the applicant and, in the discretion of the planning board, with a consulting fire marshall in order to determine the maximum number of seats from the standpoint of fire safety rather than designed capacity.

(Ord. of 8-18-86, § 5.08(C); Ord. No. 89-10, 4-17-89; Ord. No. 89-22, 8-21-89; Ord. No. 89-32, 12-4-89; Ord. No. 89-33, 12-4-89; Ord. No. 89-34, 12-18-89; Ord. No. 89-35, 12-18-89; Ord. No. 89-36, 12-18-89; Ord. No. 91-13, §§ 10--12, 10-7-91; Ord. No. 92-14, 8-3-92; Ord. No. 96-8, 4-1-96)

### Sec. 20-439.	Off-street loading.

Where off-street loading space is required under the dimensional requirements of a particular district, one (1) or more loading berths or other space shall be provided for standing, loading and unloading operations either inside or outside a building and on the same or adjoining premises with every building or structure erected after the enactment of this ordinance all to be in accordance with the requirements of the following table. A loading berth shall have a minimum plan dimension of twelve (12) feet by twenty-five (25) feet and a fourteen-foot overhead clearance. A loading space need not be necessarily a full berth but shall be sufficient to allow normal loading and unloading operations of a kind and magnitude appropriate to the property served thereby. The planner shall determine the sufficiency of loading space but in no case shall the use of such space hinder the free movement of vehicles and pedestrians over a street, sidewalk or alley. Specific requirements are as follows:

Use Classification					Space Requirements

Retail operation, including				1 loading berth for every

restaurant and dining				20,000 square feet of floor area

facilities within hotels and

office buildings, with a total

usable floor area of 20,000

square feet or more devoted

to that purpose

Retail operations, and all				A loading space (not

first floor non-residential			necessarily a full berth)

uses, with a gross floor area

of less than 20,000 square

feet, and all wholesale

operations with a gross

floor area of less than

10,000 square feet

Office buildings and hotels				1 loading berth for every

with a total usable area of			100,000 feet of floor area

100,000 square feet or more

devoted to such purposes

Wholesale operations with a				Minimum number of

gross floor area of 10,000			loading berths required:

square feet or over and as

follows (sq. feet):

10,000--40,000				1

40,000--100,000				2

100,000--160,000				3

160,000--240,000				4

240,000--320,000				5

320,000--400,000				6

Each 90,000 sq. ft. above			1

400,000 sq. ft.

(Ord. of 8-18-86, § 5.08(D); Ord. No. 88-12A, 10-24-88; Ord. No. 89-11, 4-17-89)

### Secs. 20-440--20-450.  Reserved.

## DIVISION 3.  SIGNS*

### Sec. 20-451.	Intent.

It is the intent of this division to: encourage the effective use of signs as a means of communication in the town; maintain and enhance the aesthetic environment and the town's ability to attract sources of economic development and growth; be appropriate to and improve pedestrian and traffic safety; minimize the possible adverse effect of signs on nearby public and private property; enable the fair and consistent enforcement of these sign restrictions.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-452.	Applicability; effect.

A sign may be erected, placed, established, painted, created or maintained in the town only in conformance with the standards, procedures, exemptions and other requirements of this division. The effect of this division as more specifically set forth herein is:

(1) To establish a permit system to allow a variety of types of signs in commercial zones and a limited variety of signs in other zones, subject to the standards and the permit procedures of this division;

(2) To allow certain signs that are small, unobtrusive and incidental to the principal use of the respective lots on which they are located, subject to the substantive requirements of this division, but without a requirement for permits;

(3) To provide for temporary signs without commercial messages in limited circumstances in the public right-of-way;

(4) To prohibit all signs not expressly permitted by this division; and

(5) To provide for the enforcement of the provisions of this division.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-453.	Permits required.

(a) If a sign requiring a permit under the provisions of this division is to be placed, constructed, erected or modified on a lot, the owner of the lot shall secure a sign permit prior to the construction, placement, erection or modification of such a sign in accordance with the requirements of section 20-463. Furthermore, the property owner shall maintain in force, at all times, a sign permit for such sign in accordance with section 20-464.

(b) No signs shall be erected in the public right-of-way except in accordance with section 20-459.

(c) No sign permit other than that noted in section 20-466(3) shall be issued for an existing or proposed sign unless such sign is consistent with the requirements of this division (including those protecting existing signs) in every respect and with the master signage plan or common signage plan in effect for the property.

(Ord. No. 91-14, 10-21-91; Ord. No. 94-3, 2-7-94)

### Sec. 20-454.	Design, construction and maintenance.

All signs shall be designed, constructed and maintained in accordance with the following standards:

(1) All signs shall comply with applicable provisions of the North Carolina State Building Code at all times.

(2) Except where specifically exempted by this division, all signs, including the supports, frames and embellishments thereto, shall not be located within any public right-of-way, nor shall any sign be attached, affixed or painted on any utility pole, light standard, telephone or telegraph pole, any tree, rock or other natural object.

(3) Except for flags, window signs and regulated temporary signs conforming in all respects with the requirements of this division, all signs shall be constructed of permanent materials and shall be permanently attached to the ground, a building or another structure by direct attachment to a rigid wall, frame or structure.

(4) All signs shall be maintained in good structural condition, in compliance with all building and electrical codes and in conformance with this Code at all times.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-455. Signs allowed on private property with and without permits.

(a) Signs shall be allowed on private property in the town only in accordance with Table I. If the letter "A" appears for a sign type in a column, such sign is allowed without prior permit approval in the zoning districts represented by that column. If the letter "P" appears for a sign type in a column, such sign is allowed only with prior permit approval in the zoning districts represented by that column. Special conditions may apply in some cases. If the letter "N" appears for a sign type in a column, such a sign is not allowed in the zoning districts represented by that column under any circumstances.

(b) Although permitted under the previous paragraph, a sign designated by a "P" or "A" in Table I shall be allowed only if:

(1) The sum of the area of all building and freestanding signs on the lot conforms with the maximum permitted sign area as determined by the formula for the zoning district in which the lot is located as specified in Table II.

(2) The size, location and number of signs on the lot conform with the requirements of Tables III and IV, which establish permitted sign dimensions by sign type, and with any additional limitations listed in Table I.

(3) The characteristics of the sign conform with the limitations of Table V, Permitted Sign Characteristics, and with any additional limitations on characteristics listed in Table I.

(c) Campaign signs advertising candidates for political office or communicating an opinion concerning any measure on a public ballot shall be considered temporary and shall be allowed without permit provided such signs do not exceed six (6) square feet in area per display surface, do not exceed forty-two inches (42”) in height, are not erected prior to ninety (90) days before the appropriate election or measure on a public ballot, and are removed within fourteen (14) days after the election or public vote on a measure.  Signs must be erected a minimum of eight feet (8’) from the edge of pavement or eight feet (8’) from the centerline of an unpaved street.  Signs erected in any other fashion shall be subject to the provisions of this division. (06-07)

(d) Yard or garage sale signs announcing yard or garage sales shall be considered temporary and allowed without permit provided such signs do not exceed one (1) sign per site of such sale, six (6) square feet in area per display surface and are removed within seven (7) days of erection.

(e) Public event announcements by public or nonprofit organizations of special events or activities of interest to the general public shall be considered temporary signs subject to permit at no fee. Such signs shall not exceed one (1) sign placed at the site of such events or activities and not to exceed sixteen (16) square feet in area per display surface. Such signs shall be removed within five (5) days following the event but shall not be displayed for a period of time to exceed thirty (30) days.

(f) Real estate signs advertising the sale or rental availability of a property shall be considered temporary signs allowed without a permit provided such signs comply with the following standards:

(1) Residential zones:

a. One (1) freestanding sign advertising the sale of a residential lot or structure shall be allowed. Such sign shall not exceed six (6) square feet in area per display surface.

b. One (1) sign advertising the rental of a residential structure shall be allowed. Such sign shall be affixed to the front wall of the residence and shall not exceed six (6) square feet in area.

(2) Commercial zones:

a. One (1) freestanding sign advertising the sale or lease of a commercial lot or structure shall be allowed. Such sign shall not exceed sixteen (16) square feet in area per display surface. The sign shall be removed no longer than twenty (20) days following the completion of the sale of the subject lot or structure.

(g) Freestanding stationary directional signs for public or private accredited educational institutions or churches within the town upon the following conditions:

(1) Not more than two (2) signs per institution or church;

(2) The content of the sign must be limited to directional information and the hours of operation of the activity at the site;

(3) The sign shall not exceed thirty-two (32) square feet and shall not be located closer than fifty (50) feet to any existing lawful sign;

(4) A sign permit must be obtained from the town before a sign is erected or located within the town pursuant to this subparagraph (g);

(5) The sign owner must authorize the town in writing to remove and dispose of any sign permitted pursuant to this subparagraph (g) which does not have a current permit or which does not comply with all of the applicable conditions and regulations of this subparagraph (g);

(6) The erection, location or maintenance of an off-site, portable sign or stationary directional sign contrary to the provisions of this subparagraph (g) is prohibited and is a violation of the Kitty Hawk Town Code punishable as otherwise provided by this article;

(7) Any sign owner who applies for a permit to erect and maintain a sign pursuant to this section must furnish with the application submitted to the town written consent of either the private property owner or the North Carolina Department of Transportation;

(8) Internal or external lighting of a sign erected pursuant to this section is prohibited.

(h) Civic service club display signs. One (1) display structure may be erected and maintained at one (1) location in the town approved by the town council upon which civic service clubs with an active membership on the Outer Banks may display the club's service club identification symbol. The size of the club identification symbol displayed must be similar in size to other displayed symbols and its display must be approved by the town.

(i) In the BC-1, BC-2, BH-1 and BH-2 zones of the town, two (2) on-site freestanding signs may be used on one (1) commercial site provided the following conditions are satisfied:

(1) The commercial site must be contiguous to U.S. Highway 158 Bypass (Croatan Highway) and North Carolina Highway 12;

(2) The project site must have a minimum of seventy-five (75) continuous linear feet of frontage on both U.S. Highway 158 Bypass (Croatan Highway) and North Carolina Highway 12;

(3) The second freestanding sign must not exceed forty-eight (48) square feet in size and the total sign display area on the site must not exceed one hundred (100) square feet as allowed by the Town Code; the second freestanding on-site sign must be used in lieu of any wall signs except that a sign indicating the business is "open" may be displayed provided the displayed "open" sign has been approved by the town planner with respect to its display location and size. The "open" sign shall not exceed two (2) square feet in size;

(4) Both on-site freestanding signs must have an area surrounding them landscaped with plants and vegetation approved by the planner, and the owner of the signs shall maintain the landscaped area around each sign.

(j) A sign attached to or painted on a motor vehicle or trailer may be displayed in accordance with the following requirements:

(1) The vehicular sign must be affixed to the vehicle or trailer;

(2) The vehicle or trailer upon which the vehicular sign is affixed must be used in the routine conduct of the business advertised on the sign and the vehicle or trailer must have a current safety inspection sticker and current license displayed thereon in accordance with applicable state law. A principal use of the vehicle or trailer cannot be to advertise or display the vehicular sign, and parking spaces on commercial property cannot be used by off-site businesses and their vehicles for advertising purposes;

(3) On commercial sites, the vehicle or trailer must be correctly parked within a designated parking space or loading zone. No vehicle or trailer with a vehicular sign can park within a street right-of-way except for temporary (less than one-half hour) emergency parking;

(4) For illustration but not limitation, banners, flags, inflatable objects, "sandwich board signs" and frame signs placed within the bed of a pickup truck are expressly prohibited;

(5) Commercial buses and taxicabs may display vehicular signs which advertise businesses other than the bus company in accordance with the following requirements:

a. The display of vehicular signs must be incidental to the operation of the bus or taxicab as a mode of transportation;

b. Except for temporary (less than one-half hour) emergency parking, the bus or taxicab displaying vehicular signs shall not be parked in parking lots, public access areas or on private property with the displayed vehicular signs visible from a public right-of-way.

(k) Religious signs and symbols:  (00-7)

(1) An on-site freestanding sign is permitted for churches in residential districts of the town provided the maximum sign area does not exceed sixteen (16) square feet and the maximum height of the sign and its supporting structure does not exceed five (5) feet from the finished grade level of the site.   The grade level cannot be increase above the grade level approved on the site plan in order to increase the height of the sign unless the town approves an amendment to the site plan authorizing the change in grade elevation.  The sign cannot be internally illuminated or illuminated after ten o’clock p.m., and in the residential districts of the town unless a service is being conducted after ten o’clock p.m. and in that event the light must be turned off after the service has been concluded. (00-7)

(2) In addition to a freestanding sign, one sign in the form or shape of a religious symbol shall be permitted on the church site. The religious symbol sign may be freestanding, attached to the building facade or incorporated into the building facade. The longest vertical element of the symbol shall not exceed a length of 20 feet, and the highest point of the symbol shall not exceed 35 feet in height from the town approved finished grade. Illumination of the religious sign must not interfere with motorists on adjacent streets or public rights of way. The lights used for illuminating the religious sign must be low intensity lights, and the light must be directed away from the adjacent properties. (03-27)

(l) Temporary Constructions signs (03-39)

Construction signs located on private property shall be considered temporary signs allowed without a permit provided such signs comply with the following standards:

(1) Residential zones:

- One freestanding sign advertising the contractor or developer of a residential lot or structure shall be allowed.  Such sign shall not exceed six square feet in area per display surface.

(2) Commercial zones:

- Two freestanding signs advertising the contractor, developer, or financial institution of a commercial lot or structure shall be allowed. Such signs shall not exceed 16 square feet in area per display surface.

These signs shall be removed no longer than 10 days after a certificate of occupancy is issued.

(Ord. No. 91-14, 10-21-91; Ord. No. 94-1, 1-10-94; Ord. No. 94-2, 1-10-94; Ord. No. 94-4, 2-7-94; Ord. No. 95-17, 9-5-95)

TABLE I. PERMITTED SIGNS BY TYPE AND ZONING DISTRICT

______________________________________________________________________________

Sign Type	All	All

BR 	VR	 VC-1	VC-2	VC-3	BC-1	BC-2	BC-3	BH	BH-2	PCD

______________________________________________________________________________

Freestanding

Residential	P	P	P	N	N	N	N	N	N	N	N

Other		N	N	P	P	P	P	P	P	P	P	P

Incidental	N	N	P	P	P	A	A	A	A	A	A

Building

Banner		N	N	N	N	N	N	N	N	N	N	N

Building	A	A	A	A	A	A	A	A	A	A	A

marker

Canopy	N	N	N	N	N	P	P	P	P	P	P

Identification	A	A	A	A	A	A	A	A	A	A	A

Incidental	N	N	A	A	A	A	A	A	A	A	A

Projecting	N	N	N	P	P	P	P	P	N	N	N

Residential	A	A	A	N	N	N	N	N	N	N	N

Roof		N	N	N	N	N	N	N	N	N	N	N

Roof, integral	N	N	N	N	N	N	N	N	N	N	N

Suspended	N	N	N	N	N	P	P	P	P	P	P

Temporary	P	P	P	P	P	P	P	P	N	N	N

Wall	N	N	P	P	P	P	P	P	P	P	P

Window	N	N	N	N	N	P	P	P	N	N	P

Miscellaneous

Banner		N	N	N	N	N	P	P	P	N	N	N

Flag		A	A	A	A	A	A	A	A	A	A	A

Portable	N	N	N	N	N	N	N	N	N	N	N

Billboard	N	N	N	N	N	N	N	N	N	N	N

Directional	N	N	N	N	N	P	P	P	P	P	P

__________________________________________________________________________

A = Allowed without sign permit

P = Allowed only with sign permit

N = Not allowed

TABLE II. MAXIMUM TOTAL SIGN AREA PER LOT BY ZONING DISTRICT

____________________________________________________________________________

All	All	VC-1	VC-2	VC-3	BC-1	BC-2	BC-3	BH	BH-2	PCD

BR	VR

____________________________________________________________________________

The maximum total area of all signs on a lot except incidental, building marker, and identification signs, and flagsa shall not exceed the lesser of the following:

Maximum	6	6	8	32	32	100b	100b	1,000	100b	200c	800

Number of

Total Square

Feet

_____________________________________________________________________________

a. Flags of the United States, the state, the city, foreign nations having diplomatic relations with the United States, and any other flag adopted or sanctioned by an elected legislative body of competent jurisdiction, provided that such a flag shall not exceed sixty (60) square feet in area and shall not be flown from a pole, the top of which is more than forty (40) feet in height. These flags must be flown in accordance with protocol established by the Congress of the United States for the Stars and Stripes. The flag should never be used for advertising purposes in any manner whatsoever. Advertising signs should not be fastened to a staff or halyard from which the flag is flown. Any flag not meeting any one (1) or more of these conditions shall be considered a sign and shall be subject to regulation as such.

b. This figure applies to individual commercial uses occupying a site on which no other businesses are located. A multiple occupancy building or center shall be permitted one (1) freestanding sign to identify the building or center as well as one (1) wall sign for each business occupying the building or center. Such wall signs shall not exceed ten (10) percent of the facade of the portion of the buildings or center the business occupies. Signs shall be subject to the provisions of the master or common signage plan in section 20-457.

c.  see Amendment 00-11

TABLE III. NUMBER, DIMENSIONS AND LOCATION OF

INDIVIDUAL SIGNS BY ZONING DISTRICT

______________________________________________________________________________

All	All	VC-1	VC-2	VC-3	BC-1	BC-2	BC-3	BH	BH-2	PCD

BR	VR

______________________________________________________________________________

Individual signs shall not exceed the applicable maximum number dimensions or setbacks shown on this table and on Table IV.

Freestanding

Area (sq. ft.)	16b	16	8	32	64f	48	48	64	64	64	48a

Height (feet)c	5	5	8	14	14	14	14	20	20	20	14

Setback(feet)d	5	5	5	5	5	10	10	15	15	15	10

Number	1	1	1	1	2f	2 e	2e	1	2 e	2 e	1

permitted per

lot

Building

Area (max.	6	6	6	10	10	60	60	64	NA	150i	32

sq. ft.)

Wall area	NA	NA	NA	NA	NA	10	10	10	10j	NA	5

(percent)

______________________________________________________________________________

a.  This applies to primary access signs; secondary access signs may not exceed ten (10) square feet.

b.  Freestanding signs are permitted in residential districts at the entrance to subdivision or multi-family development.

c.  Maximum sign height is twenty (20) feet, and minimum setback is five (5) feet; however, in no case shall the actual sign height exceed the actual sign setback from any adjacent lot that is zoned and used for residential purpose.

d.  In addition to the setback requirements on this table, signs shall be located such that there is at every street intersection a clear view between heights of three (3) feet and ten (10) feet in a triangle formed by the corner and points on the curb thirty (30) feet from the intersection or entranceway. No freestanding sign may be closer than one hundred (100) feet from another freestanding sign.

e.  See section 20-455(i).

f. Two freestanding signs shall be allowed provided the signs are separated by a distance of 500 feet and the site is bounded by two public rights-of-way.  99-6

g. A wall sign may extend not more than two (2) feet above the roof line if it is attached to the building facia board and the facia board also extends above the roof line by a height not greater than two (2) feet. (00-14)

i. see amendment 00-11

j. see amendment 00-12

TABLE IV. NUMBER AND DIMENSIONS OF CERTAIN

INDIVIDUAL SIGNS BY SIGN TYPE

______________________________________________________________________________

Vertical Clearance

Number	Maximum Sign	From Sidewalk	From Public

Allowed	Area			or Private		Street

Drive or Parking

No sign shall exceed any applicable maximum numbers or dimensions, or encroach on any applicable minimum clearance shown on this table.

______________________________________________________________________________

Freestanding

Residential,		See Table III	See Table III		NA			NA

other and

incidental

Building

Banner			0a		0			NA			NA

Building marker	1 per bldg.	6 sq. ft.		NA			NA

Canopy		1 per bldg.	15% of vertical	9 ft.			12 ft.

surface of canopy

Incidental		Determined by	   3 sq. ft.		NA			NA

plan

Marquee		1 per bldg.	10% of façade		9 ft.			12 ft.

Projecting		1 per bldg.	8 sq. ft.		9 ft.			12 ft.

Real estate-Sale	1		6 sq. ft. residential

1		16 sq. ft. commercial

Real estate-Rental	1		6 sq. ft. residential/wall

Roof			0		0			NA			NA

Roof, integral		0		0			NA			NA

Suspended	1 per primary		NA			9 ft.			NA

entrance

Temporary	See § 20-465		See §§			NA			NA

20-455(f), 20-459

Wall		See Table III		See Table III		NA			NA

Window	NA			25% of window	NA			NA

area

TABLE IV. NUMBER AND DIMENSIONS OF CERTAIN

INDIVIDUAL SIGNS BY SIGN TYPE (continued)

______________________________________________________________________________

Vertical Clearance

Number	Maximum Sign	From Sidewalk	From Public

Allowed	Area			or Private		Street

Drive or Parking

No sign shall exceed any applicable maximum numbers or dimensions, or encroach on any applicable minimum clearance shown on this table.

______________________________________________________________________________

Miscellaneous

Banner		0a		NA			NA			NA

Flag			NA		60 sq. ft.		9 ft.			12 ft.

Portable		0		NA			NA			NA

Billboard		0		NA			NA			NA

Neon,			1		2 sq. ft.		NA			NA

open/closed

Neon, no		1		2 sq. ft.		NA			NA

vacancy

Neon, logo		1		3 sq. ft.		NA			NA

______________________________________________________________________________

Footnotes:

a. See amendment 94-2.

TABLE V. PERMITTED SIGN CHARACTERISTICS BY ZONING DISTRICT

______________________________________________________________________________

All	All	VC-1	VC-2	VC-3	BC-1	BC-2	BC-3	BH	BH-2	PCD

BR	VR

_____________________________________________________________________________

Animated	N	N	N	N	N	N	N	N	N	N	N

Changeable	N	N	N	N	N	P	P	P	P	P	P

copy

Illumination,	N	N	N	N	N	P	P	P	P	P	P

internal(freestanding) a

Illumination,	N	N	N	N	N	P	P	P	P	P	P

internal wall a

Illumination,	N	N	N	P	P	P	P	P	P	P	P

external

Illumination,	N	N	N	N	N	N	N	N	N	N	N

neon (external to building)

Illumination,	N	N	N	N	N	P	P	P	P	P	P

neon (2 sq. ft. open or closed or no vacancy, or 3 sq. ft. logo)b

______________________________________________________________________________

A = Allowed without sign permit

P = Allowed only with sign permit

N = Not allowed

a = Internal illumination may be permitted provided the sign surface has an opaque background and the light from illumination is only visible through letters, numbers or symbols on the sign surface.

b = These neon signs may not suggest movement, be animated in any way, or blink on and off.

### Sec. 20-456.	Computations.

The following principles shall control the computation of sign area and sign height:

(1) Computation of area of individual signs. The area of a sign (which is also the sign area of a wall sign or other sign with only one (1) face) shall be computed by means of the smallest square, circle, rectangle, triangle or combination thereof that will encompass the extreme limits of the writing, representation, emblem or other display, together with any material or color forming an integral part of the background or display or used to differentiate the sign from the backdrop or structure against which it is placed, but not including any supporting framework, bracing or decorative wall when such wall otherwise meets zoning ordinance regulations and is clearly incidental to the display itself.

(2) Computation of area of multifaced signs. The sign area for a sign with more than one (1) face shall be computed by adding together the area of all sign faces visible from any one (1) point. No sign shall have more than two (2) display surfaces. When two (2) identical sign faces are placed back-to-back so that both faces cannot be viewed from any point at the same time, and when such faces are part of the same sign structure and not more than twenty-four (24) inches apart, the sign area shall be computed by the measurement of one (1) of the faces.

(3) Computation of height. The height of a sign shall be computed as the distance from the base of the sign at normal grade to the top of the highest attached component of the sign. Normal grade shall be construed to be the lower of (1) existing grade prior to construction or (2) the newly established grade after construction, exclusive of any filling, berming, mounding or excavating solely for the purpose of locating the sign. The elevation at grade at the sign location shall be made part of the topographic data submitted for the site plan consideration.

(4) Computation of maximum total permitted sign area for a lot. The permitted sum of the area of all individual signs on a lot shall be computed by applying the formula contained in Table II, Maximum Total Sign Area, appropriate for the zoning district in which the lot is located.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-457.	Master or common signage plan.

(a) Required. No permit shall be issued for an individual sign requiring a permit unless and until a master signage plan or a common signage plan for the lot on which the sign will be erected has been submitted to the planner and approved by the planning board as conforming with this division.

(b) Master signage plan. For any lot on which the owner proposes to erect one (1) or more signs requiring a permit, unless such lot is included in a common signage plan the owner shall submit to the planner a master signage plan containing the following:

(1) An accurate plat of the lot, at such scale as the planner may reasonably require;

(2) Location of buildings, parking lots, driveways and landscaped areas on the lot;

(3) Computation of the maximum total sign area, the maximum area for individual signs, the height of signs and the number of freestanding signs allowed on the lot(s) included in the plan under this division;

(4) An accurate indication on the plat of the proposed location of each present and proposed sign of any type (except that incidental signs need not be shown); and

(5) Elevation of the ground at the location of any proposed freestanding sign;

(6) Consistency among all signs on the lot with regard to:

a. Color scheme;

b. Lettering or graphic style;

c. Lighting;

d. Location of each sign on the buildings;

e. Material; and

f. Area of windows which may be covered by window signs.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-458.	Common signage plan.

If the owners of two (2) or more contiguous (disregarding intervening streets and alleys) lots or the owner of a single lot with more than one (1) building (not including any accessory building) file with the planner for such lots a common signage plan conforming with the provisions of this section, a fifteen (15) percent increase in the maximum total sign area shall be allowed for each included lot. This bonus shall be allocated within each lot as the owner(s) elects.

(1) Provisions of common signage plan. The common signage plan shall contain all of the information required for a master signage plan.

(2) Other provisions of master or common signage plans. The master or common signage plan may contain such other restrictions as the owners of the lots may reasonably determine.

(3) Consent. The master or common signage plan shall be signed by all owners or their authorized agents in such form as the town shall require.

(4) Procedures. A master or common signage plan shall be included in any development plan, site plan, planned unit development plan, PCD plan or other official plan required by the town for the proposed development and shall be processed simultaneously with such other plan.

(5) Amendment. A master or common signage plan may be amended by filing a new master or common signage plan that conforms with all requirements of this division then in effect.

(6) Existing signs not conforming to common signage plan. If any new or amended common signage plan is filed for a property on which existing signs are located, it shall include a schedule for bringing into conformance, within three (3) years, all signs not conforming to the proposed amended plan or to the requirements of this division in effect on the date of submission. An additional fifteen (15) percent of display area shall be allowed to those business owners who voluntarily submit a master or common signage plan and bring all signs on their lot(s) into compliance with the regulations of this division prior to the three-year date from adoption of this division.

(7) Binding effect. After approval of a master or common signage plan, no sign shall be erected, placed, painted or maintained except in conformance with such plan, and such plan may be enforced in the same way as any provision of this division. In case of any conflict between the provisions of such a plan and any other provision of this division, this division shall control.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-459.	Signs in the public right-of-way or on public property.

No signs shall be allowed in the public right-of-way or on public property, except for the following:

(1) Permanent signs. Permanent signs, including:

a. Public signs erected by or on behalf of a governmental body to post legal notices, identify public property, convey public information and direct or regulate pedestrian or vehicular traffic;

b. Bus stop signs erected by a public transit company;

c. Informational signs of a public utility regarding its poles, lines, pipes or facilities; and

d. Directional signs whose size and location are approved by the town and the state department of transportation, do not exceed eight (8) square feet and are uniform in design with regard to size, color and shape. Directional signs shall be subject to the permit procedure set forth in section 20-462.

(2) Emergency signs. Emergency warning signs erected by a governmental agency, a public utility company or a contractor doing authorized or permitted work within the public right-of-way.

(3) Campaign signs. On election day, a political candidate standing for election may place a campaign sign at each poll location.

(4) Other signs may be confiscated. Any sign installed or placed on public property, except in conformance with the requirements of this division, may be subject to confiscation.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-460.	Signs exempt from regulation under this division.

The following signs shall be exempt from regulation under this division:

(1) Any public notice or warning required by a valid and applicable federal, state or local law, regulation or division;

(2) Any sign inside a building, not attached to a window or door;

(3) Holiday lights and decorations with no commercial message, which may be from mid-November until Mid-January, provided they meet the requirements of sec 2-414,(d) (13); and (02-29)

(4) Traffic control signs on private property, such as "Stop," "Yield" and similar signs, the faces of which meet department of transportation standards and which contain no commercial message of any sort.

(Ord. No. 91-14, 10-21-91; Ord. No. 95-16, 8-7-95)

### Sec. 20-461.	Signs prohibited under this division.

All signs not expressly permitted under this division or exempt from regulation hereunder in accordance with the previous section are prohibited in the town. Such signs include, but are not limited to:

(1) Animated, rotating or other moving or apparently moving signs;

(2) Beacons;

(3) Pennants; (94-2)

(4) Flashing lights (except as may be permitted to display time and temperature);

(5) Strings of lights;

(6) Portable signs except for vehicular signs used and displayed in accordance with the terms of the Town Code; (95-17)

(7) Inflatable signs and tethered balloons;

(8) Billboards; and

(9) Neon tube illumination exterior to a building and except as provided in Table V.

(10) Tourist-Oriented Directional Signs (TODS) (03-56)

(Ord. No. 91-14, 10-21-91; Ord. No. 95-17, 9-5-95)

### Sec. 20-462.	General permit procedures.

The following procedures shall govern the application for, and issuance of, all sign permits under this division, and the submission and review of common signage plans and master signage plans:

(1) Applications. All applications for sign permits of any kind and for approval of a master or common signage plan shall be submitted to the planner on an application form or in accordance with application specifications published by the town.

(2) Fees. Each application for a sign permit or for approval of a master or common signage plan shall be accompanied by the applicable fees, which shall be established by the governing body of the town from time to time by ordinance.

(3) Completeness. Within five (5) days of receiving an application for a sign permit or for a common or master signage plan, the planner shall review it for completeness. If the planner finds that it is complete, the application shall then be processed. If the planner finds that it is incomplete, the planner shall, within such five-day period, send to the applicant a notice of the specific ways in which the application is deficient, with appropriate references to the applicable sections of this division.

(4) Action. The planner shall issue a sign permit for each complete application submitted in accordance with and satisfying the requirements of this chapter. Planning board review and approval of the sign permit application is not required; however, the planner, in his discretion, may forward any application to the planning board for its review and approval. In such event the planner shall issue a sign permit for any application which the planning board approves as being in compliance with all applicable regulations of this chapter and the applicable master or common signage plan.

(5) Action on plan. On any application for approval of a master signage plan or common signage plan, the planning board shall take action on the submission if it is received in complete form at least twenty (20) days prior to a regularly scheduled planning board meeting.

(Ord. No. 91-14, 10-21-91; Ord. No. 94-2, 1-10-94)

### Sec. 20-463.	Permits to construct or modify signs.

Signs identified as "P" or "A" on Table I shall be erected, installed or created only in accordance with a duly issued and valid sign construction permit from the planner. Such permits shall be issued only in accordance with the following requirements and procedures:

(1) Permit for new sign or for sign modification. An application for construction, creation or installation of a new sign or for modification of an existing sign shall be accompanied by detailed drawings to show the dimensions, design, structure and location of each particular sign, to the extent that such details are not contained on a master signage plan or common signage plan then in effect for the lot. One (1) application and permit may include multiple signs on the same lot.

(2) Inspection. The codes enforcement officer shall cause an inspection of the lot for which each permit for a new sign or for modification of an existing sign is issued during the six (6) months after the issuance of such permit or at such earlier date as the owner may request. If the construction is not substantially complete at the time of inspection, the permit shall lapse and become void. If the construction is complete and in full compliance with this division and with the building and electrical codes, the codes enforcement officer shall affix to the premises a permanent symbol identifying the sign(s) and the applicable permit by number or other reference. If the construction is substantially complete but not in full compliance with this division and applicable codes, the codes enforcement officer shall give the owner or applicant notice of the deficiencies and shall allow an additional thirty (30) days from the date of inspection for the deficiencies to be corrected. If the deficiencies are not corrected by such date, the permit shall lapse. If the construction is then complete, the codes enforcement officer shall affix to the premises the permanent symbol described above.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-464.	Sign permits; continuing.

The owner of a lot containing signs requiring a permit under this division shall at all times maintain in force a sign permit for such property. Sign permits shall be issued for individual lots, notwithstanding the fact that a particular lot may be included with other lots in a common signage plan.

(1) Initial sign permit. An initial sign permit shall be automatically issued by the planner covering the period from the date of the inspection of the completed sign installation, construction or modification through the last day of that calendar year.

(2)Lapse of sign permit. A continuing permit shall lapse automatically if not renewed or if the business license for the premises lapses, is revoked or is not renewed. A sign permit shall also lapse if the business activity on the premises is discontinued for a period of one hundred eighty (180) days or more and is not renewed within thirty (30) days of a notice from the town to the last permittee, sent to the premises, that the sign permit will lapse if such activity is not renewed. (94-3)

(3) Assignment of sign permits. A current and valid sign permit shall be freely assignable to a successor as owner of the property or holder of a business license for the same premises, subject only to filing such application as the planner may require and paying any applicable fee. The assignment shall be accomplished by filing and shall not require approval.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-465.	Temporary sign permits (private property).

Temporary signs on private property shall be allowed upon written approval by the planner only upon the issuance of a temporary sign permit, which shall be subject to the following requirements:

(1) Term. A temporary sign permit shall allow the use of a temporary sign for a specified period of not more than thirty (30) days. Each temporary sign permit shall state thereon the commencement and termination dates of the sign term.

(2) Number. Only one (1) temporary sign permit shall be issued to the same business license holder on the same lot in any calendar year.

(3) Size. A temporary sign shall not exceed six (6) square feet (except as otherwise noted).

(4) Other conditions.

a. In addition to applicable fees otherwise payable, an applicant shall post a cash bond of fifty dollars ($50.00) for each sign authorized by the planner's stamp or temporary sign permit, which bond shall be held to ensure the removal of the sign and shall be refunded upon the surrender to the town of the actual sign for disposal or upon evidence submitted to the town of the sign's removal in accordance with the temporary sign permit and this chapter. The cash bond shall be forfeited upon the failure of the temporary sign permit holder to remove the sign on or before midnight of the last day of the temporary sign permit term or upon removal by the town of a sign displayed forty-five (45) days after the permit issuance date.

b. "Grand opening" banners and "end of season sale" banners are authorized as temporary signs in accordance with these regulations and the tables included in this chapter.

i. Businesses in a multi-business shopping center or commercial site may each have a "grand opening" banner for the payment of a fee of twenty-five dollars ($25.00) per banner.

ii. "End of season sale" banners may be used by each business in a multi-business commercial site as well as businesses on a single business commercial site. A fee of twenty-five dollars ($25.00) per business displaying an "end of season sale" banner shall be collected by the town before the banner is displayed. A banner may be displayed for a period not exceeding thirty (30) continuous days nor more than twice during each calendar year.

(5) Failure to remove. The failure to remove a temporary sign prior to or at the expiration date of the permit period shall be a violation of this division for which the town may collect a civil penalty as set forth in section 20-469 as an action in the nature of debt. This remedy shall be in addition to any other remedies available to the town.

(Ord. No. 91-14, 10-21-91; Ord. No. 94-2, 1-10-94)

### Sec. 20-466.	Time of compliance; nonconforming signs and signs without permits.

Except as otherwise provided herein, the owner of any lot or other premises on which exists a sign that does not conform with the requirements of this division or for which there is no current and valid sign permit shall be obligated to remove such sign or, in the case of a nonconforming sign, to bring it into conformity with the requirements of this division.

(1) Signs existing on effective date. For any sign existing in the town on October 21, 1991, an application for a sign permit must be submitted to the planner before April 18, 1992. Signs that are the subject of application received after the applicable date set forth in this section shall be subject to all of the terms and conditions of this division and shall not be entitled to the protection of section 20-466(3).

(2) Exemption from initial fees. Applicable permits for existing signs submitted before October 21, 1992, shall be exempt from the initial fees adopted under authority of this division, but not from renewal and subsequent fees.

(3) Nonconforming existing signs, permits and terms. A sign that would be permitted under this division only with a sign permit, but which was in existence on October 21, 1991, and which was constructed in accordance with the divisions and other applicable laws in effect on the date of its construction, but which by reason of its size, height, location, design or construction is not in conformance with the requirements of this division, shall be issued a nonconforming sign permit if an application in accordance with sections 20-457 or 20-458 of this division is timely filed.

Such permit shall allow the sign(s) subject to such permit, which were made nonconforming by the adoption of this division, to remain in place and be maintained for a period ending no later than January 1, 1994, provided that no action is taken which increases the degree or extent of the nonconformity. Such signs are also subject to the provisions of section 20-466(4). A change in the information on the face of an existing nonconforming sign is allowed. However, any nonconforming sign shall either be eliminated or made to conform with the requirements of this section when any proposed change, repair or maintenance would constitute an expense of more than twenty-five (25) percent of the lesser of the original value or replacement value of the sign.

(4) Lapse of nonconforming sign permit. A nonconforming sign permit shall lapse and become void under the same circumstance as those under which any other sign permit may lapse and become void.

(5) Sign removal required. A sign that was constructed, painted, installed or maintained in conformance with a permit under this division, but for which the permit has lapsed or not been renewed or for which the time allowed for the continuance of a nonconforming sign has expired shall be forthwith removed without notice or action from the town.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-467.	Violations.

(a) Any of the following shall be a violation of this division and shall be subject to the enforcement remedies and penalties provided by this division, by this chapter and by state law:

(1) To install, create, erect or maintain any sign in a way that is inconsistent with any plan or permit governing such sign or the lot on which the sign is located;

(2) To install, create, erect or maintain any sign requiring a permit without such a permit;

(3) To fail to remove any sign that is installed, created, erected or maintained in violation of this division or for which the sign permit has lapsed; or

(4) To continue any such violation. Each such day of a continued violation shall be considered a separate violation when applying the penalty portions of this division.

(b) Each sign installed, created, erected or maintained in violation of this division shall be considered a separate violation when applying the penalty portions of this division.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-468.	Enforcement and remedies.

(a) Any violation or attempted violation of this division or of any condition or requirement adopted pursuant hereto may be restrained, corrected or abated, as the case may be, by injunction or other appropriate proceedings pursuant to state law. A violation of this division shall be considered a violation of the zoning ordinance of the town. The remedies available to the town shall include the following:

(1) Issuing a stop-work order for any and all work on any signs on the same lot or lots;

(2) Seeking an injunction or other order of restraint or abatement that requires the removal of the sign(s) or the correction of the nonconformity;

(3) Imposing any penalties that can be imposed directly by the town under this chapter;

(4) Issuing a citation to cause the violation to be corrected and imposing a penalty for failure to do so;

(5) Seeking in court the imposition of any additional penalties that can be imposed by such court under this chapter; and

(6) In the case of a sign that poses an immediate danger to the public health or safety, taking such measures as are available to the town under the applicable provisions of this chapter and the building code for such circumstances

.

(b) The town shall have such other remedies as are and as may from time to time be provided for or allowed by state law for the violation of this chapter.

(Ord. No. 91-14, 10-21-91)

### Sec. 20-469.	Citation.

(a) If, through inspection, it is determined that a person has failed to comply with the provisions of this division, the codes enforcement officer or building inspections department shall issue a notice of violation by certified mail to the violator. Violations shall be corrected within ten (10) days of the receipt of such citation except that a violation for failing or refusing to remove a temporary sign after the permit has expired must be corrected within three (3) days of delivering a citation to the address of the applicant shown on the application. If the violation is not corrected within the specified time period, a citation subject to a twenty-five-dollar civil penalty shall be issued.

(b) If the offender does not correct the violation set forth in the citation within seventy-two (72) hours after being cited, a second citation subject to a twenty-five-dollar civil penalty for the same violation shall be issued. Failure to correct the violation set forth in the citation within seventy-two (72) hours shall subject the offender to a third citation of fifty dollars ($50.00) for the same violation. Subsequent citations subject to a fifty-dollar civil penalty for each day the penalty is not paid shall be issued.

(c) These civil penalties are in addition to any other penalties or actions imposed by a court for violation of the provisions of this division.

(d) All such remedies provided herein shall be cumulative. To the extent that state law may limit the availability of a particular remedy set forth herein for a certain violation or a part thereof, such remedy shall remain available for other violations or other parts of the same violation.

(e) For the purpose of calculating any time period imposed or required by this division, the first day of any activity or action required or authorized shall be excluded and the last day shall be included.

(Ord. No. 91-14, 10-21-91)

### Secs. 20-470--20-500.  Reserved.

## DIVISION 4.  MOBILE HOMES, MOBILE HOME PARKS, TRAILER   PARKS, ETC.*

### Sec. 20-501.	Generally.

(a) All mobile homes, trailers and campers shall hereafter be located for occupancy in a mobile home park or trailer park only as provided for in this chapter.

(b) No person may maintain, operate or occupy a mobile home or travel trailer park in the town unless such park has been located in accordance with this chapter.

(c) Except as otherwise provided, this division applies to mobile home and trailer parks.

(Ord. of 8-18-86, §§ 5.16, 5.16(A)(1))

### Sec. 20-502.	Nonconformities.

All mobile home parks existing August 18, 1986 and not meeting the minimum requirements established in this division for mobile home parks, shall be considered a nonconforming use. One (1) or two (2) mobile homes on a lot with a principal building or use, or on a separate lot, shall not be considered a nonconforming mobile home park; when any such mobile home is removed from the lot it may not be replaced with another mobile home.

(Ord. of 8-18-86, § 2.02)

### Sec. 20-503.	Application for park permits.

Application for a permit to develop, operate, alter or maintain a mobile home or trailer park shall be made to the planner upon forms supplied by that office. The permit fee shall be five dollars ($5.00) for each park plus the current minimum building permit fee established by the town will be required for each proposed space within the park. Any permits for buildings to be constructed on the site shall be accompanied by the normal fee. The application for a permit shall include the following:

(1) A plan for the general layout of the park containing the information required below:

a. The area to be used for the park showing property lines and adjacent zoning and land use.

b. Driveways, entrances, exits, roadways and walkways.

c. The location of mobile home or trailer spaces and buildings.

d. The location and quantity of proposed sanitary conveniences, including proposed toilets, washrooms, laundries, recreation and utility areas and utility rooms.

e. The method and plan of sewage disposal.

f. The location and quantity of refuse receptacles.

g. The plan for water supply.

h. The plan for electric lighting.

(2) Plans and specifications for any building to be constructed on the site.

(3) Such further information as may be required by the planner or county health department to enable them to determine if the proposed park will comply with the provisions of this chapter and other applicable laws.

(Ord. of 8-18-86, § 5.16(A)(2); Ord. No. 89-11, 4-17-89)

### Sec. 20-504.	Sanitary facilities, water supply, sewerage, refuse disposal and utilities.

In every park and related permanent building, all installations of plumbing and electrical wiring, and all gas and oil appliances shall comply with the provisions of the building, plumbing and electrical, heating and gas ordinances and codes and any other applicable regulations of the town. In addition, the following requirements must be met:

(1) Each mobile home space shall be provided with plumbing and electrical connections.

(2) A supply of pure running water for domestic purposes, from a source approved by the county health department, shall be provided for every park. The water supply for each mobile home shall be obtained only from approved connections located on each mobile home space or inside each mobile home.

(3) Every trailer park shall provide common sanitary and laundry facilities consisting of at least one (1) flush toilet, one (1) shower and one (1) lavatory for each sex for every ten (10) travel trailer spaces.

(4) Every shower and lavatory provided in a trailer park shall be equipped with hot and cold running water. The floors of every toilet room and lavatory room required herein shall be constructed of concrete or other nonabsorbent material, and a base of the same material shall extend upward from the floor at least six (6) inches on all walls. All such floors shall slope to a drain properly trapped. Every toilet room, shower room and lavatory room of every trailer park shall be kept clean, well-lighted, well-ventilated, screened with a wire mesh, adequately heated at all times, and shall be disinfected periodically, but at least once a day.

(5) Every park shall be provided with an approved community sewage disposal system constructed in compliance with the regulations of the county board of health. All sewage waters from each park including wastes from toilets, and toilet rooms, showers, lavatories and wash basins and wastes from refrigerator drains, sinks or faucets in mobile homes or nonmobile home spaces shall be piped into the park sewage disposal system. All sewage wastes from every trailer equipped with its own toilet facilities shall be piped into the park sewage disposal system.

(6) All garbage and refuse in every park shall be stored in suitable watertight and flytight receptacles in accordance with the ordinance requirements for businesses.

(Ord. of 8-18-86, § 5.16(A)(3))

### Sec. 20-505.	Registration.

It shall be the duty of the operator of the park to keep an accurate register containing a record of all mobile homes or trailers, owners, and occupants of the park. The register shall contain the following information:

(1) Name and address of owner and each occupant.

(2) License number and state of issue of each licensed vehicle.

(3) Space number in which the mobile home or trailer is parked.

(4) Date of entering park.

(5) Date of leaving park.

(Ord. of 8-18-86, § 5.16(A)(4))

### Sec. 20-506.	Specifications for mobile home parks.

The following provisions apply to mobile home parks:

(1) Every mobile home park shall consist of at least four (4) acres in area in single ownership or control. Individual spaces shall not be conveyed.

(2) The amount of land for each mobile home space shall be determined by the planner after an investigation of soil conditions, the proposed method of investigation of soil conditions, the proposed method of sewerage disposal, and proposed water system. In no case shall the size of a mobile home space be less than five thousand (5,000) square feet.

(3) Each mobile home space shall be at least forty (40) feet wide and clearly marked. There shall be at least twenty (20) feet of clearance between mobile homes including mobile homes parked end to end. No mobile home shall be located closer than fifteen (15) feet to any building within the park, within fifteen (15) feet of any exterior boundary line of the park and no closer than twenty (20) feet to the edge of any interior street.

(4) All mobile home spaces shall abut upon an interior drive with a right-of-way of not less than thirty (30) feet in width, which shall have unobstructed access to a public street or highway. It is the intent of this paragraph that individual mobile home spaces shall not have unobstructed access to public streets or highways except through the interior drive. All interior drives shall be graded and paved to provide an all-weather surface not less than eighteen (18) feet in width.

(5) Dead-end drives shall not exceed one thousand (1,000) feet in length. Any interior street designed to be permanently closed shall have a turnabout at the closed end with a minimum right-of-way diameter of eighty (80) feet. The entire right-of-way of such turnaround shall be graded and usable for the turning around of motor vehicles.

(6) Drives shall intersect as nearly as possible at right angles, and no drive shall intersect at less than seventy-five (75) degrees. Where a drive intersects a public street or highway, the design standards of the state department of transportation shall apply. Street jogs of less than one hundred twenty-five (125) feet shall not be allowed.

(7) Suitable vehicular access for firefighting equipment, delivery of fuel, removal of refuse, parking and removal of mobile homes and for other necessary services shall be provided.

(8) All mobile home parks shall have one (1) or more recreation areas which shall be easily accessible to all park residents. The size of the recreation areas shall be based upon a minimum of two hundred (200) square feet for each mobile home space within the park. No single outdoor recreation area shall contain less than two thousand five hundred (2,500) square feet. Recreation areas shall be so located as to be free of traffic hazards and should, where topography permits, be centrally located.

(9) Parking space sufficient to accommodate at least two (2) automobiles shall be located on each mobile home space.

(10) The mobile home park shall be located on ground that is not susceptible to flooding. The park shall be graded so as to prevent any water from ponding or accumulating on the premises. All ditch banks shall be sloped or seeded to prevent erosion.

(11) The mobile home park shall have a visual buffer such as shrubbery or fencing not less than six (6) feet in height between the park and any adjacent residential uses other than mobile homes.

(12) The area of the mobile home stand shall be improved to provide adequate foundation for the placement of the mobile home as required by the North Carolina State Building Code.

(13) Each mobile home shall be securely anchored to its foundation in accordance with the North Carolina State Building Code standards for hurricane areas.

(14) Each mobile home space shall be equipped with plumbing and electrical connections and shall be provided with electrical current in sufficient amount to safely meet the maximum anticipated requirements of a mobile home.

(15) Trailers shall not be permitted to be occupied in a mobile home park.

(16) Each mobile home space shall be provided with and shall be connected to sanitary sewerage and water supply systems as approved by the county health department. Electrical connections and wiring shall be in accordance with the North Carolina State Electrical Code.

(17) All structural additions to mobile homes other than those which are built into the unit and designed to fold out or extend from it shall be erected only after a building permit shall have been obtained, and such additions shall conform to the building code of the town, where applicable, or shall meet the standards of special regulations adopted with respect to such additions. The building permit shall specify whether such structural additions may remain permanently, must be removed when the mobile home is removed, or must be removed within a specified length of time after the mobile home is removed. Structural alterations existing on August 18, 1981 shall be removed within thirty (30) days after the mobile home which they serve is moved unless attached to another mobile home on the same site within that period.

(18) In each mobile home park the permittee or duly authorized attendant or caretaker shall be required at all times to keep the mobile home park, its facilities, and equipment in a clean, orderly, safe, and in a sanitary condition.

(19) Mobile home parks existing on August 18, 1981 which provide mobile home spaces having a width of area less than that herein described may continue to operate with spaces of existing width or area provided they meet the standards of the county health department. In no event shall any nonconforming park be allowed to expand unless such expansion shall meet the requirements of this chapter.

(Ord. of 8-18-86, § 5.16(B); Ord. No. 89-11, 4-17-89)

### Sec. 20-507.	Specifications for trailer parks.

The following provisions apply to trailer parks:

(1) Every trailer park shall contain a minimum of ten (10) spaces. Each travel trailer space shall be clearly marked and shall contain a minimum of two thousand (2,000) square feet.

(2) There shall be at least fifteen (15) feet of clearance between trailers when located on the trailer space. No trailer shall be located closer than fifteen (15) feet to any building within the park or within fifteen (15) feet of any exterior boundary line of the park or within ten (10) feet of any interior drive.

(3) Parking spaces sufficient to accommodate at least one (1) motor vehicle and trailer shall be constructed within each space. No more than one (1) trailer may be parked on any space.

(4) All spaces shall be graded to prevent any water from ponding or accumulating within the park. Each space shall be properly graded to obtain a reasonably flat site and to provide adequate drainage away from the space.

(5) All trailer spaces shall abut upon an interior drive of no less than twenty (20) feet in width which shall have unobstructed access to a public street or highway. It is the intent of this paragraph that individual trailer spaces shall not have unobstructed access to public streets or highways except through said interior drive. All interior drives shall be graded and paved to their full right-of-way width.

(6) The park shall be developed with proper drainage ditches. All banks shall be sloped and seeded to prevent erosion.

(7) Culs-de-sac or dead-end roads shall not exceed one thousand (1,000) feet in length measured from the entrance to the center of the turnaround. Any road designed to be permanently closed shall have a turnaround at the closed end with a minimum right-of-way diameter of eighty (80) feet.

(8) When the park has more than one (1) direct access to a public road, the access drives shall not be less than three hundred (300) feet apart nor closer than three hundred (300) feet to a public road intersection.

(9) Each park shall have a central structure or structures that will provide separate toilet facilities for each sex, in accordance with section 20-504. This structure may also contain coin-operated machines for the park residents' use only, provided there is no exterior advertising. Vending machines also may be permitted in a sheltered area.

(10) No swimming pool or bathing area shall be installed, altered, improved, or used without compliance with applicable regulations and the approval of the county health department. Each swimming pool shall be fenced to prevent unauthorized use.

(11) Signs for identification of parks shall comply with the provisions of this chapter.

(12) Each park shall provide recreation areas to serve the needs of the anticipated users. One-half acre of level, well-drained ground for every ten (10) spaces shall be utilized as a recreation area. The park owner is responsible for the development and maintenance of the recreation areas.

(13) It shall be unlawful to park or store a mobile home in a trailer park. However, one (1) mobile home may be allowed within a trailer park to be used as an office or residence of persons responsible for the operation and maintenance of the trailer park provided that it is located in accordance with the standards of section 20-506.

(14) Sewage dumping stations shall be approved by the county health department. Each trailer park shall provide at least one (1) sewage dumping station.

(Ord. of 8-18-86, § 5.16(C); Ord. No. 90-5, § 6(r), 4-23-90)

### Secs. 20-508, 20-509.  Reserved.

## DIVISION 5.  BUFFERS AND LANDSCAPING

### Sec. 20-510.	Buffers.

(a) The following definitions shall apply to this division:

Fence is as conventionally known except that the outside surface of the fence facing the adjacent property shall be the finished surface. The fence finish shall be coordinated with the finishes of other construction facilities on the site. For the purposes of this division, fence does not include chain link fences with opaque inserts.

Height is measured from the proposed average ground surface elevation immediately adjacent to the buffer.

Opaque excludes all visual contact screened by the buffer between the commercial site interior to the buffer and the abutting lands except that the lowest one (1) foot of height may allow visual contact. The opaque portion of a buffer shall be opaque year-around.

Redevelopment includes any change of use or site plan amendment requiring approval by the town.

Soil berm is constructed of clean, suitable native or borrow soil material. The finish slopes shall not exceed 1:2 (rise:run).

(b) Unless specified otherwise in the various district regulations, buffers within the commercial zone shall be required between all uses in commercial zones and abutting residential zones, except that buffers shall not be required between residential uses in a commercial zone and abutting residential uses or zones. Buffer shall not interfere with access and use of public utilities facilities.

(c) The buffers required by this section shall conform to Type A or Type B buffers except where Type C buffers are specifically allowed, and shall be located within, and along the outer perimeter of, the commercial site. Type C buffers are allowed along site perimeters abutting public or private rights-of-way, except rights-of-way along the rear lot line of the commercial site.

(d) Buffer details including vegetation type and size, and fence details shall be submitted for planning board review and for approval by the town council. Plants shall be selected for their hardiness, growth potential and suitability to the particular site. Plants should be indigenous or drought and salt tolerant.

(e) Above ground structures accessory to the principal use of the site, including but not limited to dumpsters, dumpster screens, sheds, parking and driveways, shall not be located in the buffer, except that access driveways may transverse the buffers where such driveways have been reviewed by the planning board and approved by the town council.

(f) Sites undergoing redevelopment shall be required to comply with this article.

(g) Buffer types:

(1) Type A buffer: An opaque vegetative buffer of a minimum width of ten (10) feet that will reach a height of six (6) feet in three (3) years. The buffer may include a stabilized soil berm as part or all of the required height.

(2) Type B buffer: An opaque buffer consisting of a fence screened on the outside by vegetation. The top of the fence shall have a height of not less than six (6) feet. The buffer minimum width shall be ten (10) feet. The buffer may include a stabilized soil berm as part or all of the required height. The screening vegetation shall be a minimum of fifty (50) percent opaque and shall reach a height of six (6) feet in three (3) years.

(3) Type C buffer: A fifty (50) percent opaque vegetative screening buffer of a minimum width of five (5) feet that will reach a height of three (3) feet in two (2) years. The buffer may include a stabilized soil berm as part or all of the required height. The buffer shall include local evergreen trees planted not more than twenty (20) feet on centers and which shall reach a height of six (6) feet in three (3) years. A list of acceptable plant species for each buffer type shall be established and maintained by the town planning department.

(h) Maintenance. Buffer shall be maintained to meet the criteria of this section. Maintenance of buffers is a continuing condition of site plan approval and a condition of compliance therewith and occupancy thereof, and failure to maintain the buffers shall constitute grounds for revocation of any occupancy permit. Buffer design and construction shall include the appropriate features necessary to maintain the buffer including, but not limited to, access and irrigation.

(i) Bonds. If the property owner, his agent, or a site plan applicant for the owner's property has commenced required land preparation activities, or has implemented an approved sedimentation and erosion control plan, or has planted ground cover or vegetation within a street right-of-way or within an area designated as a buffer or setback, open space, green area or common area on the site plan, map or project plan being reviewed by the town (hereinafter, all said activities being referred to collectively and singularly as land preparation activities) which have not been completed, or the planted vegetation established, or the adequacy of the land preparation activities ascertained by the town at the time the owner, agent, or site plan applicant requests final review of the material submitted for approval by the town, then the town may proceed with review and approval of the final plans, map, site plan, or permit provided the completion and sufficiency of the uncompleted land preparation activities has been secured as hereinafter provided.

The property owner, agent, or site plan applicant shall furnish the town with an original valid contract between the property owner, agent, or site plan applicant and a person or entity (hereinafter contractor) approved by the town for the cost of the work and material necessary to complete the land preparation activities including the reexecution of any work deemed unsatisfactory by the town and additional material. The price shall be a sum fixed for a term of one (1) year. The town shall be designated as a third party beneficiary of the contract with the right, but not the obligation, to enforce the contract should either the property owner, agent, or site plan applicant fail to complete the land preparation activities during the contract term or the town determines that the land preparation activities are inadequate to accomplish the purpose intended, then the town may require the property owner, agent, or site plan applicant to reexecute the plan or implement modifications to the plan which are reasonably necessary to complete the required land preparation activities.

In addition, the property owner, agent, or site plan applicant shall execute and deliver to the town a certified or cashier's check payable to the town (or other instrument or bond approved by the town and acceptable to the town), with the amount thereof equal to twice the amount of the contract furnished the town for the land preparation activities or such other sum as the property owner, agent, or site plan applicant and the town mutually agree is adequate for completion of the land preparation activity. The bond shall be secured by a deed of trust creating a first lien encumbering the property which is the subject of the town review. If the property owner, agent, or site plan applicant fails or refuses to complete the plan in accordance with the Town Code, the contract and the approval of the town, then the town shall notify the property owner, agent, or site plan applicant in writing of the deficiencies and specify the corrective measures necessary to complete the land preparation activities. The failure of the property owner, agent, or site plan applicant to implement and complete the corrective measures within thirty (30) days of notification by the town shall constitute a default under the deed of trust and the bond or other instrument secured by the deed of trust and the town may foreclose its lien as by law provided. After applying the proceeds of sale to the cost of sale, including trustee's fees and attorney's fees, the town may pay the contract amount (or such lesser sum necessary to correct the deficiencies if all the work and material of the contract are not required) to the contractor in order to complete the land preparation activity. After completion of the land preparation activity, by the property owner, agent, or site plan applicant or the town, the balance of the foreclosure sale proceeds remaining with the trustee shall be disbursed to the grantor of the deed of trust or such other person designated in writing by the grantor thereof.

The form and content of the bond or agreement evidencing the obligation to be performed, the deed of trust and the contract must be approved by the town, which approval may be denied for any reason in the town's sole discretion. The town may select and obtain from a licensed North Carolina attorney an opinion verifying the first lien status of the deed of trust. The attorney's fees and cost for the title examination and report shall be paid by the property owner, agent, or site plan applicant to the town prior to final plat approval.

In lieu of the bond or agreement evidencing the obligation to be performed and secured by the deed of trust as provided above, the owner may deposit cash in the amount equal to twice the contract amount with the town to be held in escrow for the purposes set forth above. If the contractor fails and refuses to perform the contract and the property owner, agent, or site plan applicant fails to enforce the contract, then the town may use any remedies, whether in law or equity, to enforce the contract. All costs, fees and expenses incurred by the town enforcing said contract shall be paid by the property owner, agent, or site plan applicant to the town and the payment thereof shall be secured by the deed of trust. Said cost, fees and expenses shall be paid to the town from the proceeds derived from the foreclosure sale of the lien of the deed of trust in addition to the sums paid by the town to complete the land preparation activities.

(Ord. No. 95-18, 9-5-95)

Appendix #1

COASTAL ECOSYSTEMS AND LANDSCAPING

COASTAL DEVELOPMENT WORKSHOP

DARE COUNTY – MARCH 3, 2000

I   HARDINESS ZONE

8A USDA Hardiness Zone map

II   EVVIRONMENTAL FACTORS

Sand – low nutrient content, low water retention

Heat

Salt Spray

Wind

Erosions – water and wind

High Water Table

III   PLANT FEATURES

Resilient Structure

Elliptical or Needle-like Leaves

Waxy, Leathery Leaves

IV PLANT COMMUNITIES

Sand Dune

American Beach Grass

Sea Oats

Seashore Elder

Panicum

Yucca

Southern Waxmyrtle

Northern Bayberry

Seaside Goldenrod

Bog, Freshwater, Brackish Water March

Black Needle Rush

Spartina

Muscadine Grape

Baccharis

Swamp Rose-mallow

Seashore Mallow

Inkberry

Lyonia

Maritime Forest

Live Oak

American Holly

Southern Redcedar

Yaupon Holly

Dwarf Palmetto

American Beautyberry

Redbay

Highbush Blueberry

Devilwood Osmanthus

Salt Marsh

Spartina

Black Needle Rush

Marshelder

Baccharis

Sea Ox-eye

V   EXOTIC PLANTS

Russian Olive (Autumn Olive)

Pittosporum

Crapemyrtle

Pampasgrass

Shore Juniper

Japanese Black Pine

Japanese Euonymous

Indian Hawthorn

Waxleaf Privet

Hollywood Juniper

Susan E. Ruis-Evans

Agricultural Extension Agent

Dare County Center of the

North Carolina Cooperative Extension

P.O. Box 968, Manteo, NC 27954

(252) 473-1101 ext. 241

email:  susan_ruiz-evans@ncsu.edu