NOTICE AND INSTRUCTIONS TO BIDDERS

Sealed bids will be received by the City of Monroe, North Carolina for the furnishing of a unitized asphalt machine capable of making permanent asphalt repairs under most weather conditions as specified until 2:00 pm on September 8, 2005, at the City Hall Library, 300 West Crowell Street, Monroe, NC  28110, at which time the bids received will be publicly opened and read aloud.

Any questions regarding this bid, contact Mike Keziah at 704-282-4603.

Bids must be submitted showing unit and prices fully extended and the proposal page completed in its' entirety, with the provided label attached to an envelope addressed to the Unitized Asphalt Machine Formal Bid Opening, City of Monroe, PO Box 69, 300 West Crowell Street, Monroe, North Carolina  28111-0069.  Bidder will be required to comply with all applicable statutes and regulations.

All bids shall be valid for a period of 60 days pending Council approval and awarding of contract(s).

Equipment must meet all minimum specifications and be the kind and type specified, or an approved equivalent.  Price quoted must be on the basis of delivery to Monroe, North Carolina.  Quoted price should not include any sales or usage taxes, but should only reflect the actual bid price of the equipment.

This is a formal bid.  No Bid bond will be required per NCGS 143-129.

The City of Monroe reserves the right to reject any and/or all bids received, or to select the bid which, in our opinion, is the best overall interest of the City.

All bids shall include delivery time.  Complete specifications shall be included on all equipment.  Bidder must meet all of the above requirements for bid to be considered.

PROPOSAL

TO CITY MANAGER AND CITY COUNCIL

CITY OF MONROE, NORTH CAROLINA 28110

The undersigned, as bidder, hereby declares that this proposal is made without connection with any other person, company, or parties making a similar bid or proposal, and that the proposal is in all respects fair and in good faith, without collusion or fraud.

The bidder has carefully examined the annexed form of specifications and instructions to bidders and hereby declares that he will furnish the equipment called for in the manner prescribed in the specifications and instructions to bidders for the following price :

| TYPE | DELIVERY TIME | UNIT PRICE |
| --- | --- | --- |

Warranty period: _______________

_____________________________		 	____________________________________

Name of Firm Submitting Proposal	  	         Signature of Firm Authorized Representative

____________________________________

PRINTED name of Firm Authorized Representative

____________________________________

Firm Address										Title

___________________________________		__________________/_________________

Representative Email Address				Phone 		/		Fax

CITY OF MONROE

MINIMUM SPECIFICATIONS

REQUIREMENT:	ASPHALT POTHOLE PATCH TRUCK

GENERAL:	THE UNITIZED ASPHALT MACHINE CAPABLE OF MAKING PERMANENT ASPHALT REPAIRS UNDER MOST WEATHER CONDITIONS WHILE MAINTAINING THE HIGHEST SAFETY STANDARDS POSSIBLE.  THE LATEST CURRENT MODEL WITH ALL EQUIPMENT NORMALLY 	FURNISHED BY THE MANUFACTURER AND TO INCLUDE THE FOLLOWING:

BIDDER MUST FILL IN BLANKS TO INDICATE COMPLIANCE, OR STATE EXCEPTION FOR OUR ACCEPTANCE.  FAILURE TO COMPLY WITH THIS REQUIREMENT MAY RENDER YOUR BID UNRESPONSIVE.

# TRUCK

# SPECIFICATION						COMPLIANCE OR EXCEPTION

- Engine:  300 H.P. diesel engine

- Automatic transmission- Allison 3500 RDS

transmission with PTO housing & transmission

cooling

- 80,000 PSI steel frame with “L” reinforcement

- 11,000 lb. front axle heavy duty with 12,000 lb.

auxiliary springs and stabilizing bar

- Single 23,000 lb. rear axle, equipped with auxiliary

spring and dual rear wheels

- Power steering

- ABS brakes- 15”x4” front cam air brakes,

16.5”x7” rear cam air brakes

- Vehicle GVWR min. 33,000 lb.

- Automatic moisture ejectors on the air tank

- Brake line air dryer with electrical heated dryer

- 15.9 CFM air compressor

- Air filter restriction indicator

- Front shock absorbers

- Spring type Maxi II parking brake

- Automatic Slack adjusters front and rear

- Air horn

- 138” minimum cab to center of axle

- 130 amp alternator

- Turn signals and 4-way heavy duty flashers

- 50-gallon dual fuel tanks (minimum)

- Integral air conditioning with heater

- AM/FM Stereo with clock

- Cab with driver’s and 2 man passenger’s

seat

- Tan vinyl interior

- Intermittent windshield wipers with washers

- Tinted glass all around

- Sun visors LH and RH

- Lap and shoulder seat belts

- 7”x16” stainless steel outside mirrors with spot

mirrors

- Engine alarm system- low oil pressure, high

water temperature

- Full instrumentation, including air pressure, oil

pressure, speedometer, tachometer, water in fuel,

low air buzzer, low coolant, transmission oil

temp., coolant temp., fuel, volts and hour meters

- Back-up alarm

- Two (2) 71-amp 535 CCA batteries with min.

of 1500 cold cranking amps, connected in series

- 1100 R22.5G (14 ply) tires with highway tread

- Front hub wheel bearings, oil lubricated

- Horizontal muffler

- Two (2) 1-½ qt. oil filters

- Coolant filter

- Fuel/Water separator

- Clutch fan

- Oil cooler

- Front bumper with two (2) tow hooks

- White paint

BODY DIMENSIONS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 112” long, 50” high, and 78” wide

- Diamond Tread safety step side fenders with

- Two (2) ladders and

- Body assist handles

- Extra large stop, turn, and emergency flashers

with dual element integral lighting

- Lockable tool storage compartment measuring

83” long, 22” deep, and 10” high

- Mud Flaps

BODY INSULATION

SPECIFICATION	COMPLIANCE OR EXCEPTION

- The asphalt aggregate hopper, road oil tank shall

be fully insulated with 2” industrial, high temp.

insulation encased in a double, steel jacket

CAPACITY

SPECIFICATION	COMPLIANCE OR EXCEPTION

- The asphalt aggregate hopper shall hold at least

4 cubic yards level full or at least 5 tons maximum

load

HEATING SYSTEMS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Dry radiant heat chamber heated by two (2) each

150,000 BTU propane burners with vertical

exhaust stacks.  The burners shall be thermostat

controlled and regulated at 10 PSI

- 12-volt DC ignition system with electronic ignitor

- Safety shut-off gas burner for loss of flame or

excessive temperature.  Automatic shut-off when

diesel pump is operating

- Two-stage burner that ignites on 10 oz. pressure,

operates at 10 PSI, and is regulated by the same

thermostat as the electric heater

ELECTRONIC CONTROLS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Solid state shock resistant

- Thermostat for tack oil tank shall be 0/250oF

- Thermostats shall have built-in high limit safety

shut-down switches

- Same thermostat shall control the 12-volt LP gas

fired system and the 220-volt electric heating system

LIQUID ASPHALT OIL TANK AND RE-EMULSIFYING SYSTEM

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 130-gallon tank capacity

- Capable of working emulsions or asphalt cutback

oils

- Heat transfer system that is capable of heating the

emulsion oil to approx. 180oF, using the truck

engine coolant system as the heat source

- Emulsion pump and all lines to be in an insulated

heated area

- 230-volt thermostat controlled electric heater

- Hot well for storing the spray wand in cold weather

- Built-in re-emulsifying system where water can be

added to thin oil.  No mixing paddles shall be used

in re-emulsifying system

ELECTRIC HEATING SYSTEM

SPECIFICATION	COMPLIANCE OR EXCEPTION

- The asphalt aggregate hopper and liquid asphalt

tank heated by a 220-volt, 62-kW immersion

heater, thermostat controlled with 25’ cord with

ground fault interrupt, capable of maintaining heated

heated material up to 300oF during non-working

hours

FRAME MOUNTED LP GAS BOTTLE

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 35-gallon LP gas bottle with quick fill connector,

10 psi regulator, and sight gauge

TEMPERATURE GAUGES

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Two (2) 3” dial type, stainless steel temperature

gauges for the road oil and asphalt hopper

HAND TORCH

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 200,000 BTU propane hand torch with shut-off

valve and 20’ hose mounted on a hose rack

LIQUID ASPHALT OIL PUMP

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Hydraulically drive, 1-½” self-priming, variable

speed, reversible, rotary gear pump with relief

valve.  Operates at 20 GPM @ 1200 RPM

LIQUID ASPHALT HAND SPRAYER

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 5’x3/8” wand, equipped with fan nozzle and

deadman control valve.  Sprayer hose to be

15’x5/8” with a reversible pump and diesel flush

system for cleaning

HYDRAULIC SYSTEM

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Pump- gear type, variable speed rated at 12 GPM

and operating pressure of 2000 PSI @ 900 RPM

- Oil Reservoir- 30-gallon tank capacity with sight

and temperature gauge, ventilated fill cap, 100

sump strainer, and a 5 micron return filter

- Oil Cooler- Hayden Temp-Troller Electra

Hydraulic Oil Cooler or equivalent.  Fan cooler

to operate on 12-volt DC and thermostat controlled

- Control Valve- Five (5) spool, two-way, variable

Flow with 2000-psi relief valve.  Valve to control

Hydraulic screw conveyor, hydraulic doors, asphalt

Agitator and liquid asphalt pump.  Valve to be

Operated from rear of machine by remote levers.

Valve to contain no electric solenoids

- Hydraulic hose and fittings- all hoses and crimped

Fittings to have a min. of 8000 psi burst pressure

SCREW CONVEYOR

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 10’ long by 6” dia. screw conveyor with 4” spaced

hard surfaced flights welded continuously on 2”

schedule 80 pipe and ball bearing mounted.

conveyor to be driven by a 9200 in..lb. of torque

variable speed hydraulic motor with forward and

reverse.  Shall have a delivery chute with 120

degrees of pivot

ASPHALT AGITATOR

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Built into premix hopper to break up bridged

asphalt.  Schedule 80 shaft with 12 spikes, ball

bearing mounted, and driven by a 9200 in..lb. of

torque variable speed hydraulic motor with

forward and reverse

HYDRO-THROTTLE CONTROL

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Muncie Hydro-Throttle control (or equivalent)

with manual rear override if engine is not full

electronic controlled

THROTTLE CONTROL

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Electronic engine throttle control remote rear

mounted

DIESEL FUEL / RELEASE AGENT  STORAGE TANK

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 18 gallon capacity tank, equipped with a 12-volt

DC pump and a hand sprayer with 15’ long hose for

cleaning tools and unit.  There shall be a connector

valve for flushing road oil pump and lines.  Reel to

be spring loaded auto-retractable

## OPTIONS TO BE INCLUDED IN BID:

LIGHTED TRAFFIC ADVISORY ARROWS

SPECIFICATION	COMPLIANCE OR EXCEPTION

Shall be Whelen model TA1652H or equivalent with the

following:

- 23-1/4” H x 5-1/4” W x 48” L minimum

- 16 Halogen lights

- Removable arrowheads with slide bolt mounting

studs

SPOILS BIN

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 36” wide with 1.16 cubic yard capacity, gravity

dump.  Allowance for discharge from both sides

of truck

VIBRATORY ROLLER

SPECIFICATION	COMPLIANCE OR EXCEPTION

- BOMAG MW55E or equivalent with 4 hp engine,

operating weight of 351 lb., 2250 lb. centrifugal

force, single drum vibratory roller 22” wide,

centrifugal clutch and water tank

HYDRAULIC REAR LIFT PLATFORM

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Hydraulically operated lift platform for loading or

unloading vibratory roller.  Platform to have a

safety latch in the up position, an in-cab “platform

down” indicator light and compactor hold down

brackets.  All controls for platform to be at rear

of unit

PAVEMENT BREAKER

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Stanley BR72/59 lb. hydraulic hammer with quick

couplers, asphalt cutter, tamper, and detachable

shank with lockable transporting bucket

HYDRAULIC OPERATED DOORS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Two (2) hydraulically operated bifold steel doors

with hex brake for rigidity.  Doors to be insulated

with 2” of high temperature insulation in a double

steel jacket.  Doors to have rain steel caps and flex

plates to keep asphalt out of hinges.  Each door to

have a 1-½” bore with a 13” stroke hydraulic

cylinder with bypass and flow control valves.

Controls for doors to be at the rear of the unit

WATER TANK

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Frame mounted eight (8) gallon stainless steel

tank with valve and hose

DRIP PAN

SPECIFICATION	COMPLIANCE OR EXCEPTION

- 10-gallon drip tank for waste oil combination

mounted on unit with necessary valve for draining

PTO

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Muncie electric over air shift or equal

HYDRAULIC TOOL LINE HOSE REEL

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Shall be 3000 PSI with dual 25’ hydraulic tool

lines.  Reel to be spring loaded auto-retractable

TOOL RINGS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Two (2) sets of spring loaded asphalt lute and

shovel holders on sides of unit

AIR LINE HOSE REEL

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Air line hose reel with 20’ of hose.  Reel to be

spring loaded auto-retractable

HYDRAULIC AIR COMPRESSOR

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Separate, self-contained 25 CFM @ 125 psi

hydraulically driven air compressor with cooling

fan hydraulic and electric controls to be located at

rear of unit.  Compressor to include quick couplers,

20-foot air Hose, 5-foot air blow gun with deadman

control valve, 5-gallon air tank with cable assisted

condensation drain and 150 psi relief valve

STROBE LIGHT

SPECIFICATION	COMPLIANCE OR EXCEPTION

- Pedestal mounted Meteorlight SY200, or equiv.

WORK DECK

SPECIFICATION	COMPLIANCE OR EXCEPTION

- A work deck shall be installed between the cab

and the hopper.  Deck shall be constructed of

¼” angle iron and 3/16” expanded metal with

assist hand rails.  Decks to provide easy and safe

access to emulsion oil tank and asphalt hopper.

FIRE EXTINGUISHERS

SPECIFICATION	COMPLIANCE OR EXCEPTION

- In-cab mounted 5 lb ABC fire extinguisher with

vehicle mounting bracket

- Outside mounted 10 lb ABC fire extinguisher with

vehicle mounting bracket

GENERAL STIPULATIONS FOR TRUCK, CHASSIS AND BODY SPECIFICATIONS

WARRANTY:  Full twelve (12) months against defects in material and workmanship of the body and on all parts and labor.

IGNITION KEYS AND DOOR KEYS: Two (2) sets

MANUALS AND TRAINING:  Two (2) operators, and two (2) parts service operators manuals and a training program for each machine and delivery.

1.	Emission control shall meet all Federal and State air quality standards for Union County, 	North Carolina.

2.	Truck and chassis furnished shall meet all Federal, State and Local Laws including 	compliance with NCDOT and OSHA regulations.

3.	Vendor shall furnish a manufacturer statement of compliance with these regulations at 	time of delivery of unit.

- Delivery date of unit must be included in vendors bid.

- Vendor must certify with delivery that cab and chassis meets all OSHA and NC Department of Labor Safety requirements in effect at date of bid.

Questions concerning any items in the above specifications may be directed to C.J. O’Neill, Civil Engineer II, City of Monroe Engineering Department, Monday through Friday 8:00 am to 5:00 p.m. at 704-282-4532.

Proposals to be submitted to:

Attn:  Mike Keziah, Adm. Services Mgr.

City of Monroe

Operations Center

2401 Walkup Avenue

Monroe, NC 28110